| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0559 |
| --- | --- |

| Title: | CE7-related: WD of QP dependent binarization in TS residual coding |  |
| --- | --- | --- |
| Status: | Input document to JVET |  |
| Purpose: | Proposal |  |
| Author(s) or Contact(s): | Marta Karczewicz, Yung-Hsuan Chao, Muhammed Coban (Qualcomm) | Tel: +1 858 658 3973 Email: martak@qti.qualcomm.com yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |

_____________________________

#### 7.3.7.11 Residual coding syntax

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| log2SbSize = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| numSbCoeff = 1 << ( log2SbSize << 1 ) |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − 2 * log2SbSize ) ) − 1 |  |
| inferSbCbf = 1 |  |
| MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight ) |  |
| for( i =0; i <= lastSubBlock; i++ ) { |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ][ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ][ i ][ 1 ] |  |
| if( ( i != lastSubBlock ¦ ¦ !inferSbCbf ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| MaxCcbs− − |  |
| } |  |
| if( coded_sub_block_flag[ xS ][ yS ] && i < lastSubBlock ) |  |
| inferSbCbf = 0 |  |
| /* First scan pass */ |  |
| inferSbSigCoeffFlag = 1 |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n != numSbCoeff − 1 ¦ ¦ !inferSbSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| MaxCcbs− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] { |  |
| coeff_sign_flag[ n ] | ae(v) |
| MaxCcbs− − |  |
| abs_level_gtx_flag[ n ][ 0 ] | ae(v) |
| MaxCcbs− − |  |
| } |  |
| AbsLevelPassX[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + abs_level_gtx_flag[ n ][ 0 ] |  |
| } |  |
| /* Greater than Tx scan passes (numGtTxFlags=3) */ |  |
| for( j = 0; j < 3; j++ ) { |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( ( j = = 0 && abs_level_gtx_flag[ n ][ 0 ] ) ¦ ¦ ( j > 0 && abs_level_gtTx_flag[ n ][ j – 1 ] ) ) { |  |
| abs_level_gtTx_flag[ n ][ j ] | ae(v) |
| MaxCcbs− − |  |
| } |  |
| if( j = = 0 ) |  |
| AbsLevelPassX[ xC ][ yC ] + = (T1 – 1) * abs_level_gtTx_flag[ n ][ j ] |  |
| else |  |
| AbsLevelPassX[ xC ][ yC ] + = (Tj+1 – Tj ) * abs_level_gtTx_flag[ n ][ j ] |  |
| } |  |
| } |  |
| /* remainder scan pass */ |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| abs_remainder[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * ( AbsLevelPassX[ xC ][ yC ] + abs_remainder[ n ] ) |  |
| } |  |
| } |  |
| } |  |

#### Residual coding semantics

The array AbsLevel[ xC ][ yC ] represents an array of absolute values of transform coefficient levels for the current transform block and the array AbsLevelPass1[ xC ][ yC ] represents an array of partially reconstructed absolute values of transform coefficient levels for the current transform block. The array indices xC and yC specify the transform coefficient location ( xC, yC ) within the current transform block. When the value of AbsLevel[ xC ][ yC ] is not specified in clause 7.3.7.11, it is inferred to be equal to 0. When the value of AbsLevelPass1[ xC ][ yC ] is not specified in clause 7.3.7.11, it is inferred to be equal to 0.

The variables CoeffMin and CoeffMax specifying the minimum and maximum transform coefficient values are derived as follows:

CoeffMin = −( 1  <<  15 )		(7-132)

CoeffMax = ( 1  <<  15 ) − 1		(7-133)

The array QStateTransTable[ ][ ] is specified as follows:

QStateTransTable[ ][ ] = { { 0, 2 }, { 2, 0 }, { 1, 3 }, { 3, 1 } }	(7-134)

last_sig_coeff_x_prefix specifies the prefix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_x_prefix shall be in the range of 0 to ( log2ZoTbWidth  <<  1 ) − 1, inclusive.

When last_sig_coeff_x_prefix is not present, it is inferred to be 0.

last_sig_coeff_y_prefix specifies the prefix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_y_prefix shall be in the range of 0 to ( log2ZoTbHeight  <<  1 ) − 1, inclusive.

When last_sig_coeff_y_prefix is not present, it is inferred to be 0.

last_sig_coeff_x_suffix specifies the suffix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_x_suffix shall be in the range of 0 to ( 1  <<  ( ( last_sig_coeff_x_prefix  >>  1 ) − 1 ) ) − 1, inclusive.

The column position of the last significant coefficient in scanning order within a transform block LastSignificantCoeffX is derived as follows:

- If last_sig_coeff_x_suffix is not present, the following applies:

LastSignificantCoeffX = last_sig_coeff_x_prefix		(7-135)

- Otherwise (last_sig_coeff_x_suffix is present), the following applies:

LastSignificantCoeffX = ( 1  <<  ( (last_sig_coeff_x_prefix  >>  1 ) − 1 ) ) *	(7-136)
			( 2 + (last_sig_coeff_x_prefix & 1 ) ) + last_sig_coeff_x_suffix

last_sig_coeff_y_suffix specifies the suffix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_y_suffix shall be in the range of 0 to ( 1  <<  ( ( last_sig_coeff_y_prefix  >>  1 ) − 1 ) ) − 1, inclusive.

The row position of the last significant coefficient in scanning order within a transform block LastSignificantCoeffY is derived as follows:

- If last_sig_coeff_y_suffix is not present, the following applies:

LastSignificantCoeffY = last_sig_coeff_y_prefix		(7-137)

- Otherwise (last_sig_coeff_y_suffix is present), the following applies:

LastSignificantCoeffY = ( 1  <<  ( ( last_sig_coeff_y_prefix  >>  1 ) − 1 ) ) *	(7-138)
			( 2 + ( last_sig_coeff_y_prefix & 1 ) ) + last_sig_coeff_y_suffix

coded_sub_block_flag[ xS ][ yS ] specifies the following for the subblock at location ( xS, yS ) within the current transform block, where a subblock is a (4x4) array of 16 transform coefficient levels:

- If coded_sub_block_flag[ xS ][ yS ] is equal to 0, the 16 transform coefficient levels of the subblock at location ( xS, yS ) are inferred to be equal to 0.

- Otherwise (coded_sub_block_flag[ xS ][ yS ] is equal to 1), the following applies:

- If ( xS, yS ) is equal to ( 0, 0 ) and ( LastSignificantCoeffX, LastSignificantCoeffY ) is not equal to ( 0, 0 ), at least one of the 16 sig_coeff_flag syntax elements is present for the subblock at location ( xS, yS ).

- Otherwise, at least one of the 16 transform coefficient levels of the subblock at location ( xS, yS ) has a non-zero value.

When coded_sub_block_flag[ xS ][ yS ] is not present, it is inferred to be equal to 1.

sig_coeff_flag[ xC ][ yC ] specifies for the transform coefficient location ( xC, yC ) within the current transform block whether the corresponding transform coefficient level at the location ( xC, yC ) is non-zero as follows:

- If sig_coeff_flag[ xC ][ yC ] is equal to 0, the transform coefficient level at the location ( xC, yC ) is set equal to 0.

- Otherwise (sig_coeff_flag[ xC ][ yC ] is equal to 1), the transform coefficient level at the location ( xC, yC ) has a non-zero value.

When sig_coeff_flag[ xC ][ yC ] is not present, it is inferred as follows:

- If ( xC, yC ) is the last significant location ( LastSignificantCoeffX, LastSignificantCoeffY ) in scan order or all of the following conditions are true, sig_coeff_flag[ xC ][ yC ] is inferred to be equal to 1:

- ( xC & ( (1 << log2SbW ) − 1 ), yC & ( (1 << log2SbH ) − 1 ) ) is equal to ( 0, 0 ).

- inferSbDcSigCoeffFlag is equal to 1.

- coded_sub_block_flag[ xS ][ yS ] is equal to 1.

- Otherwise, sig_coeff_flag[ xC ][ yC ] is inferred to be equal to 0.

abs_level_gtx_flag[ n ][ j ] specifies whether the absolute value of the transform coefficient level (at scanning position n) is greater than ( j << 1 ) + 1. When abs_level_gtx_flag[ n ][ j ] is not present, it is inferred to be equal to 0.

The variable Tx specifying the interval of binarization is derived as follows:

The list quantScale[ ] is specified as levelScale[ j ] = { 26214,23302,20560,18396,16384,14564 }

The variable qPts is derived as follows:

If ( intra_bdpcm_flag[ x0 ][ y0 ] = = 0)

qPts = Qp′Y  (clause 8.7.1)

else

qPts = Max ( Qp′Y – 3, 10 )

The variable iQBits is derived as 14 + ( qPts / 6 )

The variable maxTsLevel is derived as quantScale[ qPts % 6] *( (1 << BitDepthY) – 1 ) + ( 1 << ( iQBits – 1 ) ) ) >> iQBits );

If x = = 1, the variable Tx is derived as follows:

Tx = 1 + ( maxTsLevel >> 2 )

Otherwise, the variable Tx is derived as follows:

Tx = Tx-1 + ( maxTsLevel >> 2 )

abs_level_gtTx_flag[ n ][ j ] specifies whether the absolute value of the transform coefficient level (at scanning position n) is greater than Tx. When abs_level_gtx_flag[ n ][ j ] is not present, it is inferred to be equal to 0.

abs_remainder[ n ] is the remaining absolute value of a transform coefficient level that is coded with truncated unary code at the scanning position n. When abs_remainder[ n ] is not present, it is inferred to be equal to 0.

It is a requirement of bitstream conformance that the value of abs_remainder[ n ] shall be constrained such that the corresponding value of TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] is in the range of CoeffMin to CoeffMax, inclusive.

dec_abs_level[ n ] is an intermediate value that is coded with Golomb-Rice code at the scanning position n. Given ZeroPos[ n ] that is derived in clause 9.5.3.2 during the parsing of dec_abs_level[ n ], the absolute value of a transform coefficient level at location ( xC, yC ) AbsLevel[ xC ][ yC ] is derived using as follows:

- If dec_abs_level[ n ] is equal to ZeroPos[ n ], AbsLevel[ xC ][ yC ] is set equal to 0.

- Otherwise if dec_abs_level[ n ] is less than ZeroPos[ n ], AbsLevel[ xC ][ yC ] is set equal to dec_abs_level[ n ] + 1;

- Otherwise (dec_abs_level[ n ] is greater than ZeroPos[ n ]), AbsLevel[ xC ][ yC ] is set equal to dec_abs_level[ n ].

The value of AbsLevelPass1[ xC ][ yC ] is derived as follows:

AbsLevelPass1[ xC ][ yC ] = Min( 4 + ( AbsLevel[ xC ][ yC ] & 1 ), AbsLevel[ xC ][ yC ] ) 	(7-139)

It is a requirement of bitstream conformance that the value of dec_abs_level[ n ] shall be constrained such that the corresponding value of TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] is in the range of CoeffMin to CoeffMax, inclusive.

coeff_sign_flag[ n ] specifies the sign of a transform coefficient level for the scanning position n as follows:

- If coeff_sign_flag[ n ] is equal to 0, the corresponding transform coefficient level has a positive value.

- Otherwise (coeff_sign_flag[ n ] is equal to 1), the corresponding transform coefficient level has a negative value.

When coeff_sign_flag[ n ] is not present, it is inferred to be equal to 0.

# Reference

- B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, “Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip”, JVET document JVET-M0464, Marrackech, MA, Jan 2019.

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding”, JVET-N1027, 14th meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).