| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0559 |
| --- | --- |

| Title: | CE7-related: QP dependent binarization in TS residual coding |  |
| --- | --- | --- |
| Status: | Input document to JVET |  |
| Purpose: | Proposal |  |
| Author(s) or Contact(s): | Marta Karczewicz, Yung-Hsuan Chao, Muhammed Coban (Qualcomm) | Tel: +1 858 658 3973 Email: martak@qti.qualcomm.com yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |

_____________________________

# Abstract

In this proposal, a new binarization method for transform skip residual coding is proposed, where the binarization of absolute coefficient levels is adjusted based on the dynamic range of the quantized residuals. In addition, a QPoffset is introduced which modifies QP to be QP – QPoffset if the corresponding CU uses RDPCM mode. The proposed method is implemented on top of JVET-O0556 and CE7-3.10. Compared to CE7-3.10, the results show: Class F: -0.28% AI, -0.61% RA, -0.34% LB BD-Rate, and Class SCC: -1.22% AI, -0.72% RA, -0.29% LB BD-Rate versus JVET-O0556+CE7-3.10. Compared to VTM5.0 anchor, the results show Class F: -0.92% AI, -1.05% RA, -0.73% LB BD-Rate, and Class SCC: -2.35% AI, -1.60% RA, -0.91% LB BD-Rate.

# Introduction

In the transform skip residual coding scheme proposed in [1], absolute coefficient level absCoeffLevel are coded using sig_coeff_flag, abs_level_gtX_flag, where X = 1,2,..5,  par_level_flag, abs_remainder. On the decoder side, the reconstructed transform coefficient is derived by:

absCoeffLevel = 1 + abs_level_gt1_flag + par_level_flag + 2 * (abs_level_gt2_flag + abs_level_gt3_flag+…+abs_level_gt5_flag) + 2*abs_remainder

The syntax element abs_level_gt1_flags indicates if the absolute coefficient level is greater than 1, and the syntax abs_level_gtX_flag (X = 2,..5) indicates if the absolute level is greater than 2*(X-1). The binarization method, however, fails to reflect the dynamic range of the absolute coefficient levels for different quantization parameters (QP). In the proposal, a new binarization method is proposed for transform skip residual coding which adjust the binarization of absolute coefficient levels based on the corresponding QP value.

# Proposal

For an input quantization parameter QP, the corresponding dynamic range [0, maxTsLevel] of transform skip coefficient can be derived as follows in VTM5.0:

const int iQBits = QUANT_SHIFT + qpPer;

int  maxTsLevel =

(g_quantScales[0][qpRem] *((1 << channelBitDepth) - 1) + (1 << (iQBits - 1))) >> iQBits);

The integers qpPer and qpRem are derived as QP/6 and QP%6.

Once the dynamic range of coefficient is calculated. The range is divided into k intervals:

[2, T1], [T1+1, T2], …….. [Tk-2+1, Tk-1], [Tk-1+1, maxTsLevel]

On the encoder side, the syntax sig_coeff_flag, abs_level_gt1_flag, and coeff_sign_flag are signaled interleaved in the first pass, as in [1]. Starting from the second pass, syntax element abs_level_gtTx_flag is signaled indicating if the absolute coefficient level is greater than interval divider Tx. Similar to abs_level_gtX_flag in [1], the syntax elements abs_level_gtTx_flag are signaled bitplane by bitplane for different x, and coded with regular bins if the number of regular coded bin limit (currently set as 2*block width * block height in VTM5.0) is not reached. If the limit is reached, the rest of the syntax element are bypass coded.

After coding sig_coeff_flag, coeff_sign_flag , abs_level_gt1_flag and abs_level_gtTx_flag, syntax abs_remainder, which specifies the remainder within the interval where the absolute coefficient level falls into, would be coded in bypass mode (trancated unary coding is used in the test). For example, if the absolute coefficient level absCoeffLevel falls into the interval [tc-1+1, tc], the remainder absCoeffLevel – (tc-1+1) will be signaled in the last pass.

On the decoder side, the reconstructed transform coefficient is derived by:

absCoeffLevel =  1 + abs_level_gt1_flag + abs_level_gtT1_flag* (t1 – 1) + abs_level_gtT2_flag* (t2 – t1) + abs_level_gtT3_flag* (t3 – t2) …+ abs_level_gtk-1_flag* (tk-1 – tk-2) + abs_remainder)

For the experiment in this proposal, 4 intervals are used in the binarization and each interval contains maxTsLevel >> 2 levels. In addition, a QPoffset ( = 3 in this test) is introduced which modifies QP to be QP – QPoffset if the corrsponding CU uses RDPCM mode.

# Simulation results

The proposed method is implemented on top of JVET-O0556 (RDPCM replacement with TS level mapping) and CE7-3.10, and evaluated according to the CE7 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. IBC, HashME, and RDPCM are turned on only for class F and class TGM. Simulation results for standard QPs (22, 27, 32, 37) are shown in Table 1-3. Table 1 show the results of the proposed binarization and Qp offset on top of JVET-O0556 and CE7-3.10. Table 2 show additional results of the proposed binarization (without QP offset) on top of JVET-O0556 and CE7-3.10, Table 3 show additional results of QP offset (without new binarization) on top of JVET-O0556 and CE7-3.10.

Table 1: Results of proposed binarization and QP offset versus VTM5.0 and CE7-3.10+JVET-O0556

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.04% | 99% | 98% | 0.02% | 0.03% | 0.05% | 98% | 100% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 99% | 0.02% | 0.03% | 0.01% | 100% | 98% |
| Class B | -0.01% | -0.07% | -0.02% | 100% | 97% | 0.02% | -0.02% | -0.03% | 99% | 100% |
| Class C | 0.01% | -0.06% | 0.07% | 102% | 101% | 0.05% | 0.00% | 0.02% | 101% | 101% |
| Class E | -0.03% | -0.01% | 0.00% | 100% | 101% | 0.02% | 0.07% | 0.04% | 99% | 100% |
| Overall | 0.00% | -0.03% | 0.02% | 100% | 99% | 0.03% | 0.02% | 0.01% | 99% | 100% |
| Class D | -0.02% | 0.20% | -0.08% | 103% | 107% | 0.05% | 0.05% | -0.02% | 104% | 112% |
| Class F | -0.92% | -0.30% | -0.32% | 102% | 99% | -0.28% | 0.13% | 0.09% | 99% | 98% |
| TGM | -2.35% | -0.81% | -0.89% | 103% | 97% | -1.22% | 0.10% | 0.04% | 97% | 100% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | c | V | EncT | DecT |
| Class A1 | 0.01% | 0.14% | 0.05% | 100% | 100% | 0.01% | 0.15% | 0.02% | 100% | 99% |
| Class A2 | 0.03% | 0.01% | 0.02% | 99% | 98% | 0.00% | -0.11% | -0.08% | 102% | 101% |
| Class B | 0.00% | 0.04% | -0.13% | 100% | 101% | 0.01% | -0.02% | -0.12% | 100% | 101% |
| Class C | -0.01% | -0.14% | -0.24% | 103% | 110% | 0.05% | 0.10% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.01% | -0.09% | 101% | 102% | 0.02% | 0.03% | -0.06% | 101% | 100% |
| Class D | 0.04% | 0.08% | 0.10% | 105% | 109% | 0.07% | -0.10% | -0.18% | 103% | 106% |
| Class F | -1.05% | -0.56% | -0.27% | 101% | 100% | -0.61% | -0.16% | -0.06% | 100% | 100% |
| TGM | -1.60% | -0.63% | -0.79% | 102% | 101% | -0.72% | -0.08% | -0.13% | 99% | 101% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.06% | 0.05% | 0.12% | 103% | 110% | 0.06% | -0.04% | 0.40% | 102% | 110% |
| Class E | 0.09% | -0.11% | 0.60% | 101% | 108% | 0.09% | 0.04% | 0.44% | 100% | 110% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.07% | 0.26% | 1.42% | 107% | 121% | 0.07% | 0.31% | 0.81% | 108% | 122% |
| Class F | -0.73% | -0.18% | -0.64% | 100% | 103% | -0.34% | 0.44% | -1.11% | 99% | 100% |
| TGM | -0.91% | -0.90% | -0.80% | 100% | 99% | -0.29% | -0.13% | -0.05% | 97% | 92% |

Table 2: Results of proposed binarization versus VTM5.0 and CE7-3.10+JVET-O0556

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.04% | 100% | 99% | 0.02% | 0.03% | 0.05% | 99% | 100% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 99% | 0.02% | 0.03% | 0.01% | 99% | 98% |
| Class B | -0.01% | -0.07% | -0.02% | 100% | 100% | 0.02% | -0.02% | -0.03% | 99% | 102% |
| Class C | 0.01% | -0.06% | 0.07% | 100% | 97% | 0.05% | 0.00% | 0.02% | 99% | 97% |
| Class E | -0.03% | -0.01% | 0.00% | 100% | 101% | 0.02% | 0.07% | 0.04% | 99% | 100% |
| Overall | 0.00% | -0.03% | 0.02% | 100% | 99% | 0.03% | 0.02% | 0.01% | 99% | 100% |
| Class D | -0.02% | 0.20% | -0.08% | 105% | 117% | 0.05% | 0.05% | -0.02% | 106% | 122% |
| Class F | -0.79% | -0.66% | -0.61% | 102% | 101% | -0.14% | -0.24% | -0.20% | 99% | 100% |
| TGM | -1.34% | -1.30% | -1.32% | 105% | 99% | -0.20% | -0.40% | -0.40% | 99% | 101% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | c | V | EncT | DecT |
| Class A1 | 0.01% | 0.14% | 0.05% | 99% | 99% | 0.01% | 0.15% | 0.02% | 100% | 98% |
| Class A2 | 0.03% | 0.01% | 0.02% | 99% | 98% | 0.00% | -0.11% | -0.08% | 103% | 100% |
| Class B | 0.00% | 0.04% | -0.13% | 100% | 101% | 0.01% | -0.02% | -0.12% | 100% | 101% |
| Class C | -0.01% | -0.14% | -0.24% | 104% | 111% | 0.05% | 0.10% | -0.05% | 100% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.01% | -0.09% | 101% | 102% | 0.02% | 0.03% | -0.06% | 101% | 100% |
| Class D | 0.04% | 0.08% | 0.10% | 107% | 112% | 0.07% | -0.10% | -0.18% | 105% | 110% |
| Class F | -0.77% | -0.82% | -0.52% | 101% | 99% | -0.33% | -0.43% | -0.31% | 100% | 99% |
| TGM | -1.10% | -0.97% | -1.07% | 103% | 101% | -0.22% | -0.42% | -0.41% | 100% | 102% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.06% | 0.05% | 0.12% | 101% | 104% | 0.06% | -0.04% | 0.40% | 100% | 105% |
| Class E | 0.09% | -0.11% | 0.60% | 101% | 104% | 0.09% | 0.04% | 0.44% | 100% | 105% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.07% | 0.26% | 1.42% | 108% | 123% | 0.07% | 0.31% | 0.81% | 109% | 124% |
| Class F | -0.50% | -0.45% | -0.97% | 101% | 103% | -0.11% | 0.17% | -1.40% | 99% | 101% |
| TGM | -0.82% | -0.99% | -0.94% | 101% | 100% | -0.19% | -0.22% | -0.19% | 98% | 93% |

Table 3: Results of introduced QPoffset=3 versus VTM5.0 and CE7-3.10+JVET-O0556

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.01% | 101% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | -0.02% | -0.04% | 0.00% | 101% | 99% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | -0.03% | -0.05% | 0.01% | 101% | 101% | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class C | -0.03% | -0.06% | 0.04% | 101% | 97% | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class E | -0.06% | -0.08% | -0.04% | 101% | 102% | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall | -0.03% | -0.05% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.07% | 0.15% | -0.06% | 103% | 102% | 0.00% | 0.00% | 0.00% | 104% | 107% |
| Class F | -0.59% | 0.02% | -0.04% | 104% | 99% | 0.06% | 0.45% | 0.37% | 101% | 98% |
| TGM | -1.71% | -0.51% | -0.60% | 105% | 97% | -0.57% | 0.39% | 0.33% | 99% | 99% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | c | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.03% | 100% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.03% | 0.13% | 0.10% | 100% | 98% | 0.00% | 0.00% | 0.00% | 103% | 101% |
| Class B | -0.01% | 0.06% | -0.01% | 100% | 101% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | -0.06% | -0.24% | -0.20% | 103% | 109% | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.02% | -0.03% | 101% | 102% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.03% | 0.17% | 0.28% | 105% | 107% | 0.00% | 0.00% | 0.00% | 103% | 105% |
| Class F | -0.54% | -0.15% | 0.04% | 102% | 100% | -0.10% | 0.25% | 0.25% | 101% | 100% |
| TGM | -0.86% | 0.00% | -0.22% | 103% | 101% | 0.03% | 0.56% | 0.45% | 100% | 101% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.10% | -0.28% | 103% | 108% | 0.00% | 0.00% | 0.00% | 102% | 108% |
| Class E | -0.01% | -0.16% | 0.21% | 101% | 107% | 0.00% | 0.00% | 0.00% | 101% | 108% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.04% | 0.59% | 108% | 122% | 0.00% | 0.00% | 0.00% | 108% | 123% |
| Class F | -0.49% | -0.37% | -0.88% | 101% | 102% | -0.10% | 0.26% | -1.33% | 100% | 99% |
| TGM | -0.71% | -0.64% | -0.59% | 101% | 100% | -0.08% | 0.14% | 0.17% | 98% | 94% |

# Reference

- B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, “Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip”, JVET document JVET-M0464, Marrackech, MA, Jan 2019.

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding”, JVET-N1027, 14th meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).