| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0559 |
| --- | --- |

| Title: | CE7-related: QP dependent binarization in TS residual coding |  |
| --- | --- | --- |
| Status: | Input document to JVET |  |
| Purpose: | Proposal |  |
| Author(s) or Contact(s): | Marta Karczewicz, Yung-Hsuan Chao, Muhammed Coban (Qualcomm) | Tel: +1 858 658 3973 Email: martak@qti.qualcomm.com yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |

_____________________________

# Abstract

In this proposal, a new binarization method for transform skip residual coding is proposed, where the binarization of absolute coefficient levels is adjusted based on the dynamic range of the quantized residuals. In addition, a QPoffset is introduced which modifies QP to be QP – QPoffset if the corresponding CU uses RDPCM mode. The proposed method is implemented on top of JVET-O0556 and CE7-3.10. The results show: Class F: -0.28% AI, -0.61% RA, -0.34% LB BD-Rate, and Class SCC: -1.22% AI, -0.72% RA, -0.29% LB BD-Rate versus JVET-O0556+CE7-3.10. Compared to VTM5.0 anchor, the results show Class F: -0.92% AI, -1.05% RA, -0.73% LB BD-Rate, and Class SCC: -2.35% AI, -1.60% RA, -0.91% LB BD-Rate.

# Introduction

In the transform skip residual coding scheme proposed in [1], absolute coefficient level absCoeffLevel are coded using sig_coeff_flag, abs_level_gtX_flag, where X = 1,2,..5, par_level_flag, and abs_remainder.

The syntax element abs_level_gt1_flags indicates if the absolute coefficient level is greater than 1, and the syntax abs_level_gtX_flag (X = 2,..5) indicates if the absolute level is greater than 2*(X-1). In the proposal, a new binarization method is proposed for transform skip residual coding which adjust the binarization of absolute coefficient levels based on the corresponding QP value.

# Proposal

Let maxTsLevel be the maximum possible absolute TS coefficient (based on QP), the dynamic range [2, maxTsLevel] is divided into 4 intervals:

[ 2	,  1 + L ],  [ 2 + L ,  1 + 2*L],  [ 2 + 2*L,  1 + 3*L],  [ 2 + 3*L ,  maxTsLevel]

, where each interval contains L= maxTsLeve4 values.

On the encoder side, the syntax sig_coeff_flag, abs_level_gt1_flag, and coeff_sign_flag are signaled interleaved in the first pass, as in VTM5.0. Starting from the second pass, the encoder would signal which interval the absolute coefficient levels absCoeffLevel falls into and the remainder within the interval.

In addition to the binarization, a QPoffset ( = 3 in this test) is introduced which modifies QP to be QP – QPoffset if the corresponding CU uses RDPCM mode.

# Simulation results

The proposed method is implemented on top of JVET-O0556 and CE7-3.10 and evaluated according to the CE7 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. IBC, HashME, and RDPCM are turned on only for class F and class TGM. Simulation results for standard QPs (22, 27, 32, 37) are shown in Table 1.

Table 1: Results of proposed binarization and QP offset on top of CE7-3.10+JVET-O0556

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.04% | 99% | 98% | 0.02% | 0.03% | 0.05% | 98% | 100% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 99% | 0.02% | 0.03% | 0.01% | 100% | 98% |
| Class B | -0.01% | -0.07% | -0.02% | 100% | 97% | 0.02% | -0.02% | -0.03% | 99% | 100% |
| Class C | 0.01% | -0.06% | 0.07% | 102% | 101% | 0.05% | 0.00% | 0.02% | 101% | 101% |
| Class E | -0.03% | -0.01% | 0.00% | 100% | 101% | 0.02% | 0.07% | 0.04% | 99% | 100% |
| Overall | 0.00% | -0.03% | 0.02% | 100% | 99% | 0.03% | 0.02% | 0.01% | 99% | 100% |
| Class D | -0.02% | 0.20% | -0.08% | 103% | 107% | 0.05% | 0.05% | -0.02% | 104% | 112% |
| Class F | -0.92% | -0.30% | -0.32% | 102% | 99% | -0.28% | 0.13% | 0.09% | 99% | 98% |
| TGM | -2.35% | -0.81% | -0.89% | 103% | 97% | -1.22% | 0.10% | 0.04% | 97% | 100% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | c | V | EncT | DecT |
| Class A1 | 0.01% | 0.14% | 0.05% | 100% | 100% | 0.01% | 0.15% | 0.02% | 100% | 99% |
| Class A2 | 0.03% | 0.01% | 0.02% | 99% | 98% | 0.00% | -0.11% | -0.08% | 102% | 101% |
| Class B | 0.00% | 0.04% | -0.13% | 100% | 101% | 0.01% | -0.02% | -0.12% | 100% | 101% |
| Class C | -0.01% | -0.14% | -0.24% | 103% | 110% | 0.05% | 0.10% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.01% | -0.09% | 101% | 102% | 0.02% | 0.03% | -0.06% | 101% | 100% |
| Class D | 0.04% | 0.08% | 0.10% | 105% | 109% | 0.07% | -0.10% | -0.18% | 103% | 106% |
| Class F | -1.05% | -0.56% | -0.27% | 101% | 100% | -0.61% | -0.16% | -0.06% | 100% | 100% |
| TGM | -1.60% | -0.63% | -0.79% | 102% | 101% | -0.72% | -0.08% | -0.13% | 99% | 101% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.18% | 0.20% | 100% | 99% | 0.02% | 0.15% | 0.10% | 100% | 97% |
| Class C | 0.06% | 0.05% | 0.12% | 103% | 110% | 0.06% | -0.04% | 0.40% | 102% | 110% |
| Class E | 0.09% | -0.11% | 0.60% | 101% | 108% | 0.09% | 0.04% | 0.44% | 100% | 110% |
| Overall | 0.04% | 0.06% | 0.28% | 101% | 105% | 0.05% | 0.06% | 0.29% | 101% | 104% |
| Class D | 0.07% | 0.26% | 1.42% | 107% | 121% | 0.07% | 0.31% | 0.81% | 108% | 122% |
| Class F | -0.73% | -0.18% | -0.64% | 100% | 103% | -0.34% | 0.44% | -1.11% | 99% | 100% |
| TGM | -0.91% | -0.90% | -0.80% | 100% | 99% | -0.29% | -0.13% | -0.05% | 97% | 92% |

# Additional test results

In Table 2 and 3, additional results for binarization and QPoffset individually are provided. Table 2 shows the results of the proposed binarization (without QP offset) on top of JVET-O0556 and CE7-3.10 and Table 3 shows thel results of QP offset (without new binarization) on top of JVET-O0556 and CE7-3.10.

Table 2: Results of proposed binarization on top of CE7-3.10+JVET-O0556

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.04% | 100% | 99% | 0.02% | 0.03% | 0.05% | 99% | 100% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 99% | 0.02% | 0.03% | 0.01% | 99% | 98% |
| Class B | -0.01% | -0.07% | -0.02% | 100% | 100% | 0.02% | -0.02% | -0.03% | 99% | 102% |
| Class C | 0.01% | -0.06% | 0.07% | 100% | 97% | 0.05% | 0.00% | 0.02% | 99% | 97% |
| Class E | -0.03% | -0.01% | 0.00% | 100% | 101% | 0.02% | 0.07% | 0.04% | 99% | 100% |
| Overall | 0.00% | -0.03% | 0.02% | 100% | 99% | 0.03% | 0.02% | 0.01% | 99% | 100% |
| Class D | -0.02% | 0.20% | -0.08% | 105% | 117% | 0.05% | 0.05% | -0.02% | 106% | 122% |
| Class F | -0.79% | -0.66% | -0.61% | 102% | 101% | -0.14% | -0.24% | -0.20% | 99% | 100% |
| TGM | -1.34% | -1.30% | -1.32% | 105% | 99% | -0.20% | -0.40% | -0.40% | 99% | 101% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | c | V | EncT | DecT |
| Class A1 | 0.01% | 0.14% | 0.05% | 99% | 99% | 0.01% | 0.15% | 0.02% | 100% | 98% |
| Class A2 | 0.03% | 0.01% | 0.02% | 99% | 98% | 0.00% | -0.11% | -0.08% | 103% | 100% |
| Class B | 0.00% | 0.04% | -0.13% | 100% | 101% | 0.01% | -0.02% | -0.12% | 100% | 101% |
| Class C | -0.01% | -0.14% | -0.24% | 104% | 111% | 0.05% | 0.10% | -0.05% | 100% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.01% | -0.09% | 101% | 102% | 0.02% | 0.03% | -0.06% | 101% | 100% |
| Class D | 0.04% | 0.08% | 0.10% | 107% | 112% | 0.07% | -0.10% | -0.18% | 105% | 110% |
| Class F | -0.77% | -0.82% | -0.52% | 101% | 99% | -0.33% | -0.43% | -0.31% | 100% | 99% |
| TGM | -1.10% | -0.97% | -1.07% | 103% | 101% | -0.22% | -0.42% | -0.41% | 100% | 102% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.18% | 0.20% | 100% | 100% | 0.02% | 0.15% | 0.10% | 99% | 98% |
| Class C | 0.06% | 0.05% | 0.12% | 101% | 104% | 0.06% | -0.04% | 0.40% | 100% | 105% |
| Class E | 0.09% | -0.11% | 0.60% | 101% | 104% | 0.09% | 0.04% | 0.44% | 100% | 105% |
| Overall | 0.04% | 0.06% | 0.28% | 101% | 102% | 0.05% | 0.06% | 0.29% | 100% | 102% |
| Class D | 0.07% | 0.26% | 1.42% | 108% | 123% | 0.07% | 0.31% | 0.81% | 109% | 124% |
| Class F | -0.50% | -0.45% | -0.97% | 101% | 103% | -0.11% | 0.17% | -1.40% | 99% | 101% |
| TGM | -0.82% | -0.99% | -0.94% | 101% | 100% | -0.19% | -0.22% | -0.19% | 98% | 93% |

Table 3: Results of introduced QPoffset=3 on top of CE7-3.10+JVET-O0556

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.01% | 101% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | -0.02% | -0.04% | 0.00% | 101% | 99% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | -0.03% | -0.05% | 0.01% | 101% | 101% | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class C | -0.03% | -0.06% | 0.04% | 101% | 97% | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class E | -0.06% | -0.08% | -0.04% | 101% | 102% | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall | -0.03% | -0.05% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.07% | 0.15% | -0.06% | 103% | 102% | 0.00% | 0.00% | 0.00% | 104% | 107% |
| Class F | -0.59% | 0.02% | -0.04% | 104% | 99% | 0.06% | 0.45% | 0.37% | 101% | 98% |
| TGM | -1.71% | -0.51% | -0.60% | 105% | 97% | -0.57% | 0.39% | 0.33% | 99% | 99% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | c | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.03% | 100% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.03% | 0.13% | 0.10% | 100% | 98% | 0.00% | 0.00% | 0.00% | 103% | 101% |
| Class B | -0.01% | 0.06% | -0.01% | 100% | 101% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | -0.06% | -0.24% | -0.20% | 103% | 109% | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.02% | -0.02% | -0.03% | 101% | 102% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.03% | 0.17% | 0.28% | 105% | 107% | 0.00% | 0.00% | 0.00% | 103% | 105% |
| Class F | -0.54% | -0.15% | 0.04% | 102% | 100% | -0.10% | 0.25% | 0.25% | 101% | 100% |
| TGM | -0.86% | 0.00% | -0.22% | 103% | 101% | 0.03% | 0.56% | 0.45% | 100% | 101% |
|  | Low delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  | Over CE7-3.10 + JVET-O0556 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.03% | 0.03% | 0.10% | 101% | 100% | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class C | 0.00% | 0.10% | -0.28% | 103% | 108% | 0.00% | 0.00% | 0.00% | 102% | 108% |
| Class E | -0.01% | -0.16% | 0.21% | 101% | 107% | 0.00% | 0.00% | 0.00% | 101% | 108% |
| Overall | -0.01% | 0.01% | 0.00% | 101% | 104% | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class D | 0.00% | -0.04% | 0.59% | 108% | 122% | 0.00% | 0.00% | 0.00% | 108% | 123% |
| Class F | -0.49% | -0.37% | -0.88% | 101% | 102% | -0.10% | 0.26% | -1.33% | 100% | 99% |
| TGM | -0.71% | -0.64% | -0.59% | 101% | 100% | -0.08% | 0.14% | 0.17% | 98% | 94% |

# Reference

- B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, “Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip”, JVET document JVET-M0464, Marrackech, MA, Jan 2019.

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding”, JVET-N1027, 14th meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).