| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0562-v1 |
| --- | --- |

| Title: | AHG15: Flexible luma-to-chroma quantization parameter tables |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen Andrew Segall | Tel: Email: | +1 647 745 1691 fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada Ltd, Sharp Labs of America |  |  |

_____________________________

# Abstract

The current VVC draft defines a table that maps a luma quantization parameter value to a chroma quantization parameter value.  The table is substantially the same as what was used in previous standards, and it is the authors understanding that the table was originally developed to manage the bit-allocation between luma and chroma channels at lower rates.  However, compared to these previous standards, the VVC draft now contains additional tools that impact the balance of luma and chroma, such as dual tree support and cross-component linear model prediction (CCLM).  Furthermore, it is anticipated that the VVC standard will be used to compress a larger amount of content types – including both standard dynamic range and high dynamic range content – that contain different relationships between the luma and chroma channels.  As such, it is asserted that the current and fixed table may not be ideal, and it is proposed to have a programmable table to offer encoder flexibility.

# Proposal

A piece-wise linear function is encoded in the PPS to define the mapping table for luma to chroma QP values.

## Syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| qp_luma_to_chroma_map_present_flag | u(1) |
| if( qp_luma_to_chroma_map_present_flag ) { |  |
| qp_luma_to_chroma_joint_map_flag | u(1) |
| for( i = 0; i < ( qp_luma_to_chroma_joint_map_flag ? 1 : 3 ); i++ ) { |  |
| qp_map_num_points_minus1[ i ] | u(6) |
| for( j = 0; j <= qp_map_num_points_minus1[ i ]; j++ ) { |  |
| if( j == 0 ) |  |
| qp_map_pivot_delta_minus26[ i ] | se(v) |
| else |  |
| qp_map_pivot_delta_minus1[ i ][ j ] | ue(v) |
| qp_map_delta[ i ][ j ] | se(v) |
| } |  |
| } |  |
| if( qp_luma_to_chroma_joint_map_flag ) { |  |
| for( int i = 0; i < 3; i++ ) |  |
| qp_map_offset[ i ] | se(v) |
| } |  |
| } |  |
| … |  |

## Semantics

qp_luma_to_chroma_map_present_flag specifies whether a luma-to-chroma quantization parameter mapping table is present.

qp_luma_to_chroma_joint_map_flag specifies whether a common luma-to-chroma quantization parameter mapping table is used for Cb, Cr and CbCr.

qp_map_num_points_minus1[ i ], qp_map_pivot_delta_minus26[ i ], qp_map_pivot_delta_minus1[ i ][ j ], qp_map_delta[ i ][ j ] and qp_map_offset[ i ] define a piece-wise linear function for mapping a luma QP value to a chroma QP value. Values of a mapping table qp_map[ i ][ j ] are derived therefrom as follows:

for( i = 0; i < 3; i++) {

minQp = −24 // minimum for 12-bit content

maxQp = 69

k = qp_luma_to_chroma_joint_map_flag ? 0 : i

pivot = qp_map_pivot_delta_minus26[ k ] + 26

for( j = 0; j <= qp_map_num_points_minus1[ k ]; j++ ) {

if( j > 0 )

pivot = pivot + qp_map_pivot_delta_minus1[ k ][ j ] + 1

qp_map_pivot[ j ] = Clip3( minQp, maxQp, pivot )

}

qp_offset = qp_luma_to_chroma_joint_map_flag ? qp_map_offset[ i ] : 0

for( qp = minQp; qp < qp_map_pivot[ 0 ]; qp++ ) {

qp_map[ i ][ qp ] = qp + qp_map_delta[ k ][ 0 ] + qp_offset

}

for( j = 0; j < qp_map_num_points_minus1[ i ]; j++ ) {

slope_num = qp_map_delta[ k ][ j + 1] − qp_map_delta[ k ][ j ]

slope_den = qp_map_pivot[ j + 1 ] − qp_map_pivot[ j ];

for( qp = qp_map_pivot[ j ]; qp < qp_map_pivot[ j + 1 ]; qp++ ) {

offset = qp −  qp_map_pivot[ j ]

qp_map[ i ][ qp ] = qp + qp_map_delta[ k ][ j ] + Floor( slope_num * offset / slope_den + 0.5 ) + qp_offset

}

}

for( qp = qp_map_pivot[ qp_map_num_points_minus1[ k ] ]; qp <= maxQp; qp++ ) {

qp_map[ i ][ qp ] = qp + qp_map_delta[ k ][ qp_map_num_points_minus1[ k ] ] + qp_offset

}

}

When qp_luma_to_chroma_map_present_flag is false, and parameters for the piece-wise function are not present, qp_map[ i ][ qp ] = qp for i = 0..2 and qp = -24..69.

## Decoding process

Below is a modified version of section 8.7.1

When ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the following applies:

–	When treeType is equal to DUAL_TREE_CHROMA, the variable QpY is set equal to the luma quantization parameter QpY of the luma coding unit that covers the luma location ( xCb + cbWidth / 2, yCb + cbHeight / 2 ).

–	The variables qPCb, qPCr and qPCbCr are derived as follows:

qPiCb = Clip3( −24, 69, QpY + pps_cb_qp_offset + slice_cb_qp_offset )	(8-926)

qPiCr = Clip3( −24, 69, QpY + pps_cr_qp_offset + slice_cr_qp_offset )	(8-927)

qPiCbCr = Clip3( −24, 69, QpY + pps_joint_cbcr_qp_offset + slice_joint_cbcr_qp_offset )	(8-928)

qPCb = Clip3( −QpBdOffsetC, 63, qp_map[ 0 ][ qPiCb ] )	(8-xxx)

qPCr = Clip3( −QpBdOffsetC, 63, qp_map[ 1 ][ qPiCr ] )	(8-xxx)

qPCbCr = Clip3( −QpBdOffsetC, 63, qp_map[ 2 ][ qPiCbCr ] )	(8-xxx)

–	The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, and joint Cb-Cr coding Qp′CbCr are derived as follows:

Qp′Cb = qPCb + QpBdOffsetC		(8-929)

Qp′Cr = qPCr + QpBdOffsetC		(8-930)

Qp′CbCr = qPCbCr + QpBdOffsetC		(8-931)

# Results

## Encoder considerations

The piece-wise linear function may yield fractional chroma QP values, which are then rounded to the nearest integer to become the actual chroma QP values.

In an encoder, it is possible to consider the full-precision value of the piece-wise linear function to derive lambda parameters for chroma. Doing so yields smoother RD curves, as depicted in the graphs below, where the Cb PSNR is plotted against bitrate and luma QP. The Cactus sequence is used in this particular example.

## Coding efficiency

An experiment was run where a piece-wise linear function with 4 segments was used to approximate the existing mapping:

- at QPY=25, an offset of 0 is defined

- at QPY=33, an offset of -1 is defined

- at QPY=43, an offset of -6 is defined

The intent of this setting is to mimic the mapping table currently used in VTM5.

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.36% | -2.34% | -2.01% | -0.02% | 97% | 97% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.23% | -2.43% | -2.39% | -0.03% | 98% | 98% |
| Class C | 0.23% | -1.95% | -1.81% | -0.01% | 100% | 97% |
| Class E | 0.18% | -1.97% | -1.78% | -0.03% | 100% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.27% | -1.79% | -1.80% | 0.04% | 101% | 101% |
| Class F | 0.26% | -1.47% | -1.36% | -0.01% | 97% | 98% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | 59% | 63% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.10% | 0.07% | 0.25% | -0.07% | 97% | 97% |
| Class C | -0.13% | 0.59% | 0.43% | -0.06% | 99% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.07% | 0.57% | 0.49% | 0.01% | 101% | 101% |
| Class F | -0.16% | 0.84% | 0.73% | 0.00% | 100% | 100% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.26% | -3.87% | -3.85% | -0.13% | 102% | 102% |
| Class E | 0.19% | -4.01% | -2.53% | -0.15% | 95% | 96% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.18% | -4.33% | -3.99% | -0.18% | 101% | 97% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.19% | -4.22% | -3.99% | -0.21% | 99% | 95% |
| Class E | 0.19% | -3.95% | -3.10% | -0.16% | 99% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.20% | -4.44% | -4.41% | -0.18% | 96% | 96% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).