| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O566-v1 |
| --- | --- |

| Title: | Non-CE5: Consistent deblocking for chroma components |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Jiexi Wang, Li Zhang, Weijia Zhu, Yue Wang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the current VVC design, different on/off control and different deblocking filters can be applied to Cb and Cr components. Such an inconsistency makes parallel processing for Cb/Cr difficult when they are stored in an interleaved way. This document proposes to unify Cb/Cr deblocking filter. Luma BD-rate, encoding time and decoding time are summarized as follows:

CTC:	  AI: 0.00%, 100%, 99%; RA: 0.00%, 100%, 100%; LDB: -0.08%, 101%, 93%;

ALF off: AI: 0.00%, 100%, 100%; RA: 0.00%, 100%, 100%; LDB: 0.02%, 100%, 99%.

It is asserted that no obvious visual differences are found.

# Introduction

In HEVC, there is just one deblocking filter for chroma components, which influences one sample on P-side and the other one on Q-side. In the current VVC design, with the adoption of JVET-M0471 [1], long deblocking filters are introduced for both luma and chroma components. Thus, for a Cb or Cr edge, there exist two deblocking filters, i.e. long filter and normal filter, that can be applied. Also, in the current design, the deblocking decision for Cb/Cr is independent. It is also different from HEVC, where chroma deblocking decision follows the decision of luma component.

Considering that the granularity of chroma blocks and edges is small, in some implementation, Cb and Cr are stored in an interleaved way and a same processing is applied to the interleaved chroma samples parallelly. However, to make it feasible, the processing should be consistent for Cb and Cr components. It is noted that deblocking process takes a large portion of decoding time for a real time decoding. Thus, to make parallel deblocking for chroma components efficient, it is proposed in this document to make deblocking filters consistent for Cb and Cr components.

For example, since the edge size of chroma block can be two pixels, the edge filtering process for chroma can be performed for two successive samples in a single process with software 64-bit SIMD implementation. For general CPU architecture, the software SIMD implementation for two- sample chroma filtering can be described as:

1.	Load samples needed for calculation from memory into 64-bit vector registers, so that each register carries 8 elements for 8-bit case and 4 elements for higher bit-depth case.

2.	For 8-bit or 10-bit case, expand each element into 16-bit for further calculation, and then, each register carries 4 elements.

3.	Perform the filtering calculation with 64-bit SIMD instructions.

4.	For 8-bit or 10-bit case, clip the 16-bit filtering result into 8-bit or 10-bit filtered samples.

5.	Store the filtered samples into memory of DPB.

From the above description of SIMD implementation, 64-bit instructions can deal with 4 samples, while with the current design, they can only process 2 samples. If Cb and Cr components are consistent in processing, deblocking process can benefit from improvement of instruction-level parallelism.

# Proposed method

In the current VVC design, when one side of the chroma blocks are of intra, the boundary strength is set to 2. Otherwise, when one chroma block has non-zero transform coefficients, the boundary is set to 1. Thus, it is possible the bS=1 for Cb and bS=0 for Cr. We propose to simply set bS=1 for both Cb and Cr when one component has non-zero transform coefficients.

Then when bS is not equal to 0, a decision is made for each component to apply the long deblocking filter or the normal deblocking filter. We propose to simply apply normal deblocking filter for both Cb and Cr if normal deblocking filter is decided for one component.

# Experimental results

Experiments are conducted following the CE5 test conditions [2]. The results under CTC and with ALF off are summarized in table 1 and 2.

Table 1. Comparison between the proposed method with the VTM-5.0 anchor.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over vtm5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.03% | 99% | 100% |
| Class A2 | 0.00% | -0.14% | -0.04% | 100% | 99% |
| Class B | 0.00% | -0.07% | -0.13% | 99% | 97% |
| Class C | 0.00% | -0.02% | -0.03% | 100% | 101% |
| Class E | 0.00% | -0.05% | -0.01% | 101% | 98% |
| Overall | 0.00% | -0.05% | -0.06% | 100% | 99% |
| Class D | 0.00% | -0.01% | -0.05% | 102% | 99% |
| Class F | 0.00% | -0.01% | 0.00% | 102% | 100% |
|  | Random Access |  |  |  |  |
|  | Over vtm5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.30% | 0.02% | 99% | 100% |
| Class A2 | 0.00% | -0.05% | 0.05% | 100% | 101% |
| Class B | 0.01% | -0.05% | -0.23% | 100% | 100% |
| Class C | -0.02% | -0.15% | -0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.13% | -0.07% | 100% | 100% |
| Class D | -0.05% | 0.08% | 0.25% | 100% | 101% |
| Class F | 0.02% | 0.05% | 0.09% | 101% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over vtm5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.22% | -0.19% | -0.37% | 101% | 93% |
| Class C | 0.01% | -0.04% | -0.02% | 99% | 94% |
| Class E | 0.03% | -0.03% | -0.86% | 102% | 91% |
| Overall | -0.08% | -0.10% | -0.37% | 101% | 93% |
| Class D | 0.04% | -0.58% | 1.00% | 101% | 100% |
| Class F | 0.07% | 0.22% | 0.75% | 103% | 99% |

Table 2. Comparison between the proposed method with the VTM-5.0 anchor
(ALF off for both the anchor and the test).

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over vtm5.0 ALF0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.32% | 0.02% | 100% | 98% |
| Class A2 | 0.00% | -0.09% | -0.04% | 101% | 100% |
| Class B | 0.00% | -0.04% | -0.09% | 100% | 99% |
| Class C | 0.00% | -0.01% | -0.01% | 100% | 102% |
| Class E | 0.00% | -0.02% | 0.07% | 99% | 103% |
| Overall | 0.00% | 0.02% | -0.02% | 100% | 100% |
| Class D | 0.00% | 0.00% | -0.02% | 99% | 104% |
| Class F | 0.00% | 0.00% | 0.02% | 101% | 100% |
|  | Random Access |  |  |  |  |
|  | Over vtm5.0 ALF0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.12% | -0.13% | 100% | 100% |
| Class A2 | -0.01% | -0.06% | 0.06% | 101% | 100% |
| Class B | 0.01% | -0.05% | -0.17% | 101% | 100% |
| Class C | -0.01% | 0.10% | 0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | -0.06% | 100% | 100% |
| Class D | -0.04% | 0.25% | 0.02% | 100% | 104% |
| Class F | -0.02% | 0.04% | 0.03% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over vtm5.0 ALF0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.01% | -0.15% | 100% | 100% |
| Class C | 0.00% | -0.04% | 0.05% | 101% | 98% |
| Class E | 0.05% | -0.15% | -0.66% | 101% | 99% |
| Overall | 0.02% | -0.06% | -0.21% | 100% | 99% |
| Class D | 0.04% | 0.45% | 0.40% | 101% | 99% |
| Class F | -0.05% | 0.47% | 0.70% | 100% | 99% |

# Change to the working draft

Text changes, marked in yellow, based on JVET-N1001-v8 are shown below.

8.8.3.6 Edge filtering process for one direction

- Otherwise (cIdx is not equal to 0), the filtering process for edges in the chroma coding block of current coding unit specified by cIdx consists of the following ordered steps:

- The variable cQpPicOffset is derived as follows:

cQpPicOffset = cIdx = = 1  ?  pps_cb_qp_offset  :  pps_cr_qp_offset
(pps_cb_qp_offset + pps_cr_qp_offset + 1) >> 1	(8-1040)

- bS[ xDk ][ yDm ] for cIdx = 1 and 2 are modified to 1 if bS[ xDk ][ yDm ] for cIdx = 1 is equal to1 or bS[ xDk ][ yDm ] for cIdx = 2 is equal to 1.

- The decision process for chroma block edges as specified in clause 8.8.3.6.3 is invoked the chroma picture sample array recPicture, the location of the chroma coding block ( xCb, yCb ), the location of the chroma block ( xBl, yBl ) set equal to ( xDk, yDm ), the edge direction edgeType, the variable cQpPicOffset, the boundary filtering strength bS[ xDk ][ yDm ], and the variable maxFilterLengthCbCr set equal to maxFilterLengthPs[ xDk ][ yDm ] as inputs, the modified variable maxFilterLengthCbCr, and the variable tC as outputs.

- When maxFilterLengthCbCr for cIdx = 1 is equal to 1 or maxFilterLengthCbCr for cIdx = 2 is equal to 1, maxFilterLengthCbCr for cIdx = 1 and cIdx = 2 are modified to 1.

When maxFilterLengthCbCr is greater than 0, the filtering process for chroma block edges as specified in clause 8.8.3.6.4 is invoked with the chroma picture sample array recPicture, the location of the chroma coding block ( xCb, yCb ), the chroma location of the block ( xBl, yBl ) set equal to ( xDk, yDm ), the edge direction edgeType, the variable maxFilterLengthCbCr, and the variable tC as inputs, and the modified chroma picture sample array recPicture as output.

# Conclusions

Considering the benefit of the proposed solution and negligible impact on the coding performance, it is recommended adopting the proposed solution to VVC.

# References

[1]	M. Ikeda, T. Suzuki, D. Rusanovskyy, M. Karczewicz, W. Zhu, K. Misra, P. Cowan, A. Segall, K. Andersson, J. Enhorn, Z. Zhang, R. Sjöberg, “CE11.1.6, CE11.1.7 and CE11.1.8: Joint proposals for long deblocking from Sony, Qualcomm, Sharp, Ericsson,” JVET-M0471, Jan. 2019.

[2]	C.-Y. Chen, A. Norkin, “Description of Core Experiment 5 (CE5): Loop filtering,” JVET-N1025, Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).