| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0570 |
| --- | --- |

| Title: | Non-CE9: Unified gradient calculations in BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Li Zhang, Kai Zhang, Zhipin Deng, Na Zhang, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Proposal changes of specification on top of JVET-O0070

The working draft [1] should be changed as follows. The newly added parts are highlighted in yellow, and the deleted parts are highlighted in grey strikethrough.

#### 8.5.6.4	Bidirectional optical flow prediction process

…

- The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientHL0[ x ][ y ]  =  (predSamplesL0[ hx + 1 ][vy] − predSampleL0[ hx − 1 ][ vy ] ) >> shift1	(8-767)

gradientVL0[ x ][ y ]  =   (predSampleL0[ hx ][ vy + 1 ] − predSampleL0[ hx ][vy − 1 ] ) >> shift1	(8-768)

gradientHL1[ x ][ y ]  =  (predSamplesL1[ hx + 1 ][vy] − predSampleL1[ hx − 1 ][ vy ] ) >> shift1	(8-769)

gradientVL1[ x ][ y ]  =   (predSampleL1[ hx ][ vy + 1 ] − predSampleL1[ hx ][vy − 1 ] ) >> shift1	(8-770)

gradientHL0[x][y] = ( predSamplesL0[hx + 1][vy] >> shift1 ) − ( predSampleL0[hx − 1][vy] >> shift1 )	(8-769)

gradientVL0[x][y] = ( predSampleL0[hx][vy + 1] >> shift1 ) − ( predSampleL0[hx][vy − 1] >> shift1 )	(8-770)

gradientHL1[x][y] = ( predSamplesL1[hx + 1][vy] >> shift1 ) − ( predSampleL1[hx − 1][vy] >> shift1 )	(8-771)

gradientVL1[x][y] = ( predSampleL1[hx][vy + 1] >> shift1 ) − ( predSampleL1[hx][vy − 1] >> shift1 )	(8-772)

…

Changes on top of PROF [2] is as follow.

#### 8.5.6.7 Prediction refinement with optical flow process

Inputs to this process are:

- two variables sbWidth and sbHeight specifying the width and the height of the current subblock,

- one (sbWidth + borderExtension) x (sbHeight + borderExtension) prediction sample arrays, predSamples

- one (sbWidth x sbHeight) motion vector difference array, diffMv

Output of this process is the (sbWidth)x(sbHeight) array pbSamples of prediction sample values.

Variables bitDepth and shift1 are derived as follows:

- The variable shift1 is set equal to Max( 6, bitDepth − 6 ).

- The variable dILimit is set equal to 1 << (14 – 1).

For x =0.. sbWidth - 1, y =0.. sbHeight - 1, the following ordered steps apply:

  - The variables gradientH[ x ][ y ] and gradientV[ x ][ y ] are derived as follows:

gradientH[ x ][ y ]  =  ( predSamples[ x + 2 ][ y ] − predSamples[ x ][ y ] ) >> shift1

gradientV[ x ][ y ]  =  ( predSamples[ x ][ y + 2 ] − predSamples[ x ][ y ] ) >> shift1

gradientH[ x ][ y ]  =  ( predSamples[ x + 2 ][ y ]  >> shift1 ) − ( predSamples[ x ][ y ]  >> shift1 )

gradientV[ x ][ y ]  =  ( predSamples[ x ][ y + 2 ]  >> shift1 ) – ( predSamples[ x ][ y ] >> shift1 )

  - The variable dI is derived as follows:

dI = gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ] + gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ]

  - Prediction sample value at location ( x, y ) in the subblock is derived as follows:

pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + Clip3( -dILimit, dILimit - 1, ( dI + 1 ) >> 1 ) )

# Reference

[1]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 5), Joint Video Experts Team Document, JVET-N1001-v10, Geneva, June 2019.

[2]	J. Luo, Y. He, CE4-2.1: Prediction refinement with optical flow for affine mode, Joint Video Experts Team Document, JVET-O0070, Gothenburg, July 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).