| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0575-v1. |
| --- | --- |

| Title: | CE7-related: Reduction for Context Coded Bins in Transform Skip Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Li Zhang, Jizheng Xu, Kai Zhang, Hongbin Liu 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com lizhang.idm@bytedance.com xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In VTM-5.0, the bin2bit ratio for classF sequences are higher than other classes, which brings a throughput issue for screen content coding. It is proposed to limit the number of context coded bins for TS mode based on the block size, with less context coded bins limits for small blocks. Compared with VTM-5.0, the proposed method has {0.53%, 0.31%} and {0.25%, 0.18%} BD-rate changes on TGM sequences and class F sequences under the AI/RA configurations with common test conditions (CTC QPs). For low QPs, {0.92%, -2.67%} and {0.29%, 0.75%} BD-rate changes on TGM and class F sequences are reported. On TGM sequences, average transform coefficient bin to bit ratio under CTC shows the proposed method achieve -2.09%/-1.43% changes and -2.98%/-2.41% change under AI/RA configurations with CTC and low QPs respectively. On class F sequences, average transform coefficient bin to bit ratio under CTC shows the proposed method achieve -1.03%/-0.96% changes and -1.37%/-1.32% change under AI/RA configurations with CTC and low QPs respectively.

# Introduction

In the current VVC [1], the residual coding for TS/RDPCM is different from regular residual coding case in the following aspects:

- no signalling of the last x/y position;

- coded_sub_block_flag coded for every subblock except for the last subblock when all previous flags are equal to 0;

- sig_coeff_flag context modelling with reduced template;

- a single context model for abs_level_gtX_flag[0] and par_level_flag;

- context modeling for the sign flag, additional greater than 5, 7, 9 flags;

- modified Rice parameter derivation for the remainder binarization;

- 2 bins per sample is set as a limit for the number of context coded bins for one block.

To maintain a high throughput of CABAC, the maximum number context coded bins per block/per sample is set for TS blocks and a counter is used to record how many context coded bins within one block has been coded. In the current design, denote a luma block size as W*H, once the counter reaches K*W*H (K=2 in current VVC), the remaining bins are all bypass coded. However, even with such a constrains, it is reported that for classF, the bin2bit ratio is apparently higher than other classes. Not to mention that 4:2:0 TGM and 4:4:4 screen contents may even worsen the situation. It is desired to further reduce the number of context coded bins for TS blocks.

# The proposed method

We propose to further limit the number of context coded bins for TS blocks based on block size and coded mode. The number of context coded bins is limited to 1.5 bins per sample of IBC and BDPCM blocks with block size larger than 16. And the number of context coded bins is limited to 1.75 bins per sample for other blocks.

# Simulation Results

The proposed method is evaluated with VTM-5.0. The simulation follows the CE7 test condition [1]. The summarized results are given as follows. The anchor is VTM-5.0 and the test is the proposed scheme. Please note that the decoding time is not accurate because to collect the bits related information, the macro RExt__DECODER_DEBUG_BIT_STATISTICS is enabled.

Table 1. The performance of the proposed scheme versus VTM-5.0 under AI/RA using CTC QPs

| AI | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class TGM | 0.53% | 0.45% | 0.45% | 100% | 115% |
| Class F | 0.25% | 0.26% | 0.07% | 101% | 129% |

| RA | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class TGM | 0.31% | 0.34% | 0.23% | 101% | 121% |
| Class F | 0.18% | 0.14% | 0.25% | 101% | 140% |

Table 2. The performance of the proposed scheme versus VTM-5.0 under AI/RA using low QPs

| AI | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class TGM | 0.92% | 0.77% | 0.76% | 99% | 108% |
| Class F | 0.29% | 0.21% | 0.26% | 100% | 115% |

| RA | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class TGM | -2.67% | 0.54% | 0.58% | 100% | 118% |
| Class F | 0.75% | 0.22% | 0.15% | 99% | 124% |

Table 3. The bins to bits ration of the proposed scheme versus VTM-5.0 under AI/RA using CTC QPs

|  |  | Max – Bin2Bit Ratio | TRF of Max – Bin2Bit Ratio | Avg TRF – Bin2Bit Ratio | Avg – Bin2Bit Ratio |
| --- | --- | --- | --- | --- | --- |
| AI | TGM | -2.20% | -1.27% | -0.91% | -2.09% |
|  | Class F | -0.81% | -0.28% | -0.38% | -1.03% |
| RA | TGM | -2.40% | -0.45% | -0.57% | -1.43% |
|  | Class F | -1.11% | -0.88% | -0.52% | -0.96% |

Table 4. The bins to bits ration of the proposed scheme versus VTM-5.0 under AI/RA using low QPs

|  |  | Max – Bin2Bit Ratio | TRF of Max – Bin2Bit Ratio | Avg TRF – Bin2Bit Ratio | Avg – Bin2Bit Ratio |
| --- | --- | --- | --- | --- | --- |
| AI | TGM | -3.06% | -0.97% | -0.94% | -2.98% |
|  | Class F | -1.36% | -0.38% | -0.47% | -1.37% |
| RA | TGM | -2.96% | -0.75% | -0.64% | -2.41% |
|  | Class F | -1.98% | -0.32% | -0.38% | -1.32% |

# Conclusions

This document proposes to further limit number of context coded bins for TS blocks based on block size and coded mode to enhance the throughput. We recommend adopting this method into the next version of VVC draft.

# Text changes

The text changes, marked in grey, are on top of JVET-N1001-v8 [2].

7.3.7.11 Residual coding syntax

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| log2SbSize = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| numSbCoeff = 1 << ( log2SbSize << 1 ) |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − 2 * log2SbSize ) ) − 1 |  |
| inferSbCbf = 1 |  |
| blkSize = ( 1 << log2TbWidth ) * ( 1<< log2TbHeight ) |  |
| MaxCcbs = ((CuPredMode[ x0 ][ y0 ] == MODE_IBC ¦¦ intra_bdpcm_flag[ x0 ][ y0 ]) && blkSize > 16 ? 1.5 : 1.75) * blkSize |  |
| … |  |
| } |  |

# References

- H. Schwarzm, M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding”, JVET-N1027, Mar. 2019

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Mar. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).