| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0580 |
| --- | --- |

| Title: | CE4-related: Additional tests for prediction refinement with optical flow for affine mode (CE4-2.1) with 4x8 sub-block size |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuwen He, Wei Chen (InterDigital) Weimin Zeng, Eric Cai (Ubilinx) | Tel: Email: | Yuwen.He@InterDigital.com Weimin.Zeng@ublnx.com |
| Source: | InterDigital Communications, Inc. Ubilinx Technology, Inc. |  |  |

_____________________________

# Abstract

Based on the CE4 test 2.1, this contribution proposes to use 4x8 sub-block for affine bi-prediction for luma component. Two tests are included to test using 6-tap and 8-tap interpolation filter for bi-prediction, respectively. The simulation results reportedly show that comparing to VTM-5.0, average BD rate savings for RA and LD respectively are {-0.43%, -0.08%, -0.04%} and {-0.29%, -0.03%, -0.09%} for 6-tap filter; {-0.49%, -0.10%, -0.09%} and {-0.35%, -0.02%, -0.22%} for 8-tap filter.

# Introduction

Prediction refinement with optical flow (PROF) has been proposed in [1] to refine the sub-block based affine motion compensated prediction with optical flow in pixel level granularity without increasing the memory access bandwidth. In order to further reduce the memory bandwidth and complexity, this contribution proposes to use 4x8 subblock for affine bi-prediction.

# Complexity reduction with 4x8 affine sub-block

With the adoption of JVET-N0266 [2], JVET-N0068 [3] and JVET-N0196 [4] in Geneva meeting, the affine bi-prediction with 4x4 sub-block is no more the worst case in terms of memory access bandwidth. However, the computational complexity of 4x4 sub-block bi-prediction motion compensation is still high. In CE4-2.1 test b and c, 8x8 sub-block size for bi-prediction is tested for lower computational complexity. This contribution proposes to use 4x8 sub-block for affine bi-prediction. 4x8 sub-block has the same complexity as the 8x8 subblock, in terms of number of multiplications and additions per sample. But with a smaller sub-block size, it can achieve more accurate affine prediction.

# Simulation results

The proposal has been implemented on top of CE4-2.1 code. The test condition described in JVET-N1010 [5] used in the simulation. Two tests have been performed. In test 1, 6-tap interpolation filter is used for affine bi-prediction. In test 2, 8-tap interpolation filter is used for affine bi-prediction.

Table 1.  Test 1 CE-4.2.1 + 4x8 sub-block with 6-tap interpolation filter for affine bi-prediction

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.15% | 0.05% | 102% | 101% | - | - | - | - | - |
| Class A2 | -0.76% | 0.30% | 0.21% | 102% | 101% | - | - | - | - | - |
| Class B | -0.56% | -0.18% | -0.14% | 102% | 101% | -0.32% | -0.11% | -0.04% | 102% | 100% |
| Class C | -0.25% | -0.21% | -0.20% | 102% | 101% | -0.26% | -0.05% | -0.13% | 102% | 103% |
| Class E | - | - | - | - | - | -0.26% | 0.13% | -0.11% | 103% | 101% |
| Overall | -0.43% | -0.08% | -0.04% | 102% | 101% | -0.29% | -0.03% | -0.09% | 103% | 101% |
| Class D | -0.48% | 0.01% | 0.01% | 101% | 100% | -0.48% | -0.67% | 0.08% | 102% | 101% |
| Class F | -0.76% | -0.32% | -0.29% | 102% | 101% | -1.17% | -1.01% | -1.54% | 102% | 100% |

Table 2. Test 2 CE-4.2.1 + 4x8 sub-block with 8-tap interpolation filter for affine bi-prediction

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.22% | 0.07% | 101% | 101% | - | - | - | - | - |
| Class A2 | -0.81% | 0.30% | 0.19% | 101% | 101% | - | - | - | - | - |
| Class B | -0.62% | -0.22% | -0.23% | 101% | 101% | -0.38% | -0.07% | -0.10% | 102% | 100% |
| Class C | -0.35% | -0.16% | -0.26% | 101% | 102% | -0.37% | -0.20% | 0.06% | 102% | 102% |
| Class E | - | - | - | - | - | -0.27% | 0.32% | -0.78% | 102% | 101% |
| Overall | -0.49% | -0.10% | -0.09% | 101% | 101% | -0.35% | -0.02% | -0.22% | 102% | 101% |
| Class D | -0.63% | -0.20% | -0.04% | 101% | 101% | -0.87% | -0.64% | -0.25% | 101% | 100% |
| Class F | -0.78% | -0.32% | -0.39% | 101% | 101% | -1.22% | -1.45% | -1.43% | 101% | 101% |

The performance and computation complexity for different settings are listed in Table 3.

Table 3. Performance and bi-prediction MC complexity comparisons

| Tests | Settings for affine bi-prediction | BD rate relative to VTM-5.0 RA-Y; LB-Y | Multiplications per 4x4 block (uni/bi) | Additions per 4x4 block (uni/bi) |
| --- | --- | --- | --- | --- |
| VTM-5.0 | 4x4, 6 tap | 0%; 0% | 400/800 | 350/700 |
| CE4-2.1a | 4x4, 6 tap | -0.48%; -0.32% | 432/864 | 430/860 |
| CE4-2.1b | 8x8, 6 tap | -0.40%; -0.27% | 432/744 | 430/760 |
| CE4-2.1c | 8x8, 8 tap | -0.46%; -0.35% | 432/976 | 430/984 |
| Test 1 | 4x8, 6 tap | -0.43%; -0.29% | 432/744 | 430/760 |
| Test 2 | 4x8, 8 tap | -0.49%; -0.35% | 432/976 | 430/984 |

# Conclusions

Compared to VTM-5.0, Test 2 with 4x8 sub-block size and 6-tap interpolation filter saves BD rate by 0.43% and 0.29% for RA and LB, and reduces the number of multiplications.

# Reference

- J. Luo, Y. He, “CE2-related: Prediction refinement with optical flow for affine mode”, JVET-N0236, Geneva, CH, Mar. 2019.

- H. Liu, L. Zhang, K. Zhang, H. Liu, J. Xu, Y. Wang, “CE2-related: Disabling bi-prediction or inter prediction for small blocks”, JVET-N0226, Geneva, CH, Mar. 2019.

- M. Zhou, “CE2: On restriction of memory bandwidth consumption of affine mode (CE2-4.8)”, JVET-N0068, Geneva, CH, Mar. 2019.

- J. Li, C.-W. Kuo, C. S.Lim, “CE2: Using the shorter-tap filter for 4x4 sized partitions (Test 2.4.6)”, JVET-N0196, Geneva, CH, Mar. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ubilinx Technology, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).