| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0581 |
| --- | --- |

| Title: | Non-CE9: Block Boundary Prediction Refinement with Optical Flow for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Chen Yuwen He 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4855 Wei.Chen@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a method to reduce the PU boundary discontinuity with optical flow for DMVR. After the sub-block based motion compensation is performed, prediction sample at block boundary is refined by adding a difference derived by the optical flow equation, which is referred as block boundary prediction refinement with optical flow (BBPROF). The proposed BBPROF can achieve pixel level granularity without significantly increasing the complexity and keep the memory access bandwidth same as regular block level motion compensation. The simulation result reportedly shows on average 0.06% BD rate saving for RA, with 0.37% encoding time and 1.70% decoding time increase.

# Introduction

DMVR is used to increase the accuracy of the MVs of the bi-predictive merge mode. However, due to the high complexity and memory access bandwidth for generating pixel-based motion compensated prediction, the refined MV after DMVR is only used in block-based motion compensation method, where all samples within a DMVR sub-block share the same refined MV. It loses prediction accuracy at sample level.

# Proposed algorithm

To achieve a finer granularity of motion compensation and reduce the PU boundary discontinuity, this contribution proposes a new method named BBPROF to refine boundary samples with optical flow. The difference of MV offset between the current DMVR sub-block and four neighbouring DMVR sub-blocks are calculated and converted to a difference value derived by the optical flow equation, as shown in Figure 1. Luma prediction sample is then refined by the difference value.

Figure 1 Difference of MV offset between current DMVR sub-block and neighboring DMVR sub-blocks

The proposed BBPROF is described as following four steps.

Step 1) With refined MV after DMVR process, the sub-block-based motion compensation is performed to generate sample prediction Ii, j within each DMVR sub-block.

Step 2) MV offset difference MVdiff between current DMVR sub-block and neighbouring DMVR sub-blocks is calculated. It is then used to calculate sample level MV offset Δv(i,j).

Δvi,j=n=0Nwi, j,n* MVdiff(n)

where n is the index for a specific neighbouring sub-block, N is the total number of available neighbouring sub-blocks, and wi, j,n is the weighting factor when a neighbouring MVdiff(n) is applied for a specific pixel at location (i, j). For example, if left, above, right and bottom neighboring sub-block are available, N is equal to 4. The weighting factor is {1/4, 1/8, 1/16, 1/32}, which is applied to boundary samples from outside rows/columns to inside rows/columns respectively, if vertical or horizontal neighbouring sub-blocks are considered respectively. Note that DMVR sub-blocks are typically 16x16 blocks. And only 4 rows/columns of samples at each of the 4 boundaries are refined.

Step 3) The spatial gradients gx(i,j) and gy(i,j) of the sub-block prediction are calculated at each sample location using a 3-tap filter [-1, 0, 1], which is the same as in [1].

gxi, j=Ii+1, j-Ii-1, j

gyi, j=Ii, j+1-Ii, j-1

The sub-block prediction is extended by one pixel on each side for the gradient calculation. To reduce the memory bandwidth and complexity, the pixels on the extended borders are copied from the nearest integer pixel position in the reference picture. Therefore, additional interpolation for padding region is avoided.

Step 4) The luma prediction refinement is calculated by the optical flow equation.

ΔI(i,j)= gxi,j*Δvx(i,j)+gyi,j*Δvy(i,j)

Step 5) Finally, the luma prediction refinement is added to the sub-block prediction Ii, j.

I'(i,j)= Ii,j+ΔI(i,j)

# Simulation results

The contribution has been implemented on top of VTM-5.0. The test condition described in JVET-N1010 [2] used in the simulation. The result reported shows that the BD saving for random access is {-0.XX%, -0.XX%, -0.XX%}.

Table 1.  Applying BBPROF on CTC sequence

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.14% | -0.03% | 100% | 102% |
| Class A2 | -0.09% | 0.00% | -0.03% | 100% | 102% |
| Class B | -0.04% | -0.02% | -0.02% | 1.00 | 1.01 |
| Class C | -0.05% | -0.01% | -0.07% | 1.00 | 1.02 |
| Class E | - | - | - | - | - |
| Overall | -0.06% | -0.04% | -0.04% | 100% | 102% |
| Class D | 0.00% | -0.02% | -0.03% | 1.00 | 1.01 |
| Class F | -0.01% | 0.04% | 0.02% | 1.01 | 1.01 |

# Reference

- J. Luo, Y. He, “CE2-related: Prediction refinement with optical flow for affine mode”, JVET-N0236, Geneva, CH, Mar. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).