| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0583 |
| --- | --- |

| Title: | CE4-related: Prediction refinement with optical flow for chroma components for affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuwen He, Wei Chen 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4819 Yuwen.He@InterDigital.com Wei.Chen@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Based on the core experiment (CE) 4 test 2.1, this contribution proposes to extend the PROF to chroma components. The sub-block size for affine motion compensation of chroma component is set to 4x4, which is same as VTM-5.0. The simulation result reportedly shows that comparing to VTM-5.0, average BD rate for RA and LD respectively are {-0.53%, -0.56%, -0.69%} and {-0.33%, -0.30%, -0.75%}. Compared to luma PROF only test (CE4-2.1a), the additional BD rate for RA and LD respectively are {-0.04%, -0.46%, -0.62%} and {-0.01%, -0.26%, -0.39%}.

# Introduction

Prediction refinement with optical flow (PROF) has been proposed in [1] to refine the sub-block based affine motion compensated prediction with optical flow in pixel level granularity without increasing the memory access bandwidth. In the original proposal, PROF is only applied to luma component. This contribution proposes to apply PROF to both luma and chroma components.

# Proposed algorithm

The current VVC adopted a sub-block based affine motion compensation method, where a CU is divided into 4x4 sub-blocks, each of which is assigned with a MV derived from the affine CU’s control point MVs. Due to the sub-block nature of the affine mode, blocking artifacts can be observed on the edge of objects. PROF process refine the luma prediction in a finer granularity and reduce the blocking artifacts. However, PROF was not proposed to be applied chroma components. The luma edge and chroma edge may not be aligned if PROF is only applied to luma component, which causes boundaries artifacts. Moreover, a 4x4 sub-block in chroma components corresponding to an 8x8 sub-block in luma component. The larger sub-block area causes the blocking artifact in chroma more visible. As an example, blocking artifacts in chroma along the straight edge in the sequence “CatRobot” is shown in Figure 1. To reduce the artifacts and align the operation on both luma and chroma components, we propose to apply PROF on both luma and chroma components. In the proposed design the Δvx and Δvy calculated for the luma 4x4 sub-block can be reused for chroma 4x4 sub-block.

Figure 1. Applying PROF on luma component only. Chroma artifacts are shown on the straight edge due to the edge misalignment between luma and chroma components. (Frame 16 of sequence CatRobot encoded with RA configuration at QP 37)

# Simulation results

The proposal has been implemented on top of CE4-2.1 code. The test condition described in JVET-N1010 [2] used in the simulation. PROF is enabled for chroma component in CE4-2.1 test a, b and c, respectively. The result reported shows that the BD saving for RA and LD configurations are {-0.04%, -0.46%, -0.62%} and {-0.01%, -0.26%, -0.39%} for test a, {-0.05%, -0.41%, -0.51%} and {-0.01%, -0.35%, -0.32%} for test b, {-0.04%, -0.47%, -0.48%} and {-0.00%, -0.20%, -0.49%} for test c.

Table 1.  Applying PROF on both luma and chroma components with 4x4 sub-block size vs. VTM-5.0

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.19% | -0.19% | 101% | 101% | - | - | - | - | - |
| Class A2 | -1.00% | -0.79% | -0.99% | 102% | 101% | - | - | - | - | - |
| Class B | -0.62% | -0.72% | -0.87% | 101% | 102% | -0.41% | -0.20% | -0.39% | 101% | 100% |
| Class C | -0.32% | -0.44% | -0.60% | 101% | 102% | -0.28% | -0.52% | -0.65% | 102% | 103% |
| Class E | - | - | - | - | - | -0.25% | -0.18% | -1.49% | 103% | 102% |
| Overall | -0.53% | -0.56% | -0.69% | 101% | 102% | -0.33% | -0.30% | -0.75% | 102% | 101% |
| Class D | -0.56% | -0.52% | -0.40% | 101% | 102% | -0.44% | -0.82% | -0.27% | 101% | 103% |
| Class F | -0.92% | -1.08% | -1.11% | 101% | 102% | -1.38% | -1.48% | -1.81% | 102% | 99% |

Table 2.  Applying PROF on both luma and chroma components with 4x4 sub-block size vs. CE4-2.1a

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE4-2.1a |  |  |  |  | Over CE4-2.1a |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.03% | -0.30% | 100% | 101% | - | - | - | - | - |
| Class A2 | -0.15% | -1.06% | -1.15% | 100% | 100% | - | - | - | - | - |
| Class B | -0.01% | -0.44% | -0.68% | 100% | 102% | 0.00% | -0.15% | -0.11% | 100% | 99% |
| Class C | -0.01% | -0.37% | -0.39% | 100% | 102% | -0.06% | -0.57% | -0.49% | 100% | 103% |
| Class E | - | - | - | - | - | 0.05% | -0.04% | -0.70% | 100% | 103% |
| Overall | -0.04% | -0.46% | -0.62% | 100% | 101% | -0.01% | -0.26% | -0.39% | 100% | 101% |
| Class D | -0.02% | -0.39% | -0.44% | 100% | 102% | -0.07% | -0.47% | -1.38% | 100% | 104% |
| Class F | -0.07% | -0.64% | -0.64% | 100% | 102% | -0.05% | -0.15% | -0.89% | 100% | 100% |

Table 3.  Applying PROF on both luma and chroma components with 8x8 sub-block size and 6-tap interpolation filter for luma bi-prediction vs. CE4-2.1b

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE4-2.1b |  |  |  |  | Over CE4-2.1b |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.11% | -0.14% | 100% | 99% | - | - | - | - | - |
| Class A2 | -0.16% | -0.98% | -0.94% | 100% | 99% | - | - | - | - | - |
| Class B | -0.02% | -0.41% | -0.65% | 101% | 100% | 0.02% | -0.17% | -0.09% | 99% | 98% |
| Class C | -0.02% | -0.22% | -0.28% | 101% | 101% | -0.06% | -0.53% | -0.09% | 100% | 99% |
| Class E | - | - | - | - | - | 0.00% | -0.40% | -1.03% | 100% | 99% |
| Overall | -0.05% | -0.41% | -0.51% | 100% | 100% | -0.01% | -0.35% | -0.32% | 99% | 99% |
| Class D | -0.02% | -0.31% | -0.44% | 101% | 102% | -0.03% | -0.18% | -0.54% | 100% | 102% |
| Class F | -0.05% | -0.67% | -0.67% | 101% | 101% | -0.15% | -0.37% | -0.45% | 100% | 99% |

Table 4.  Applying PROF on both luma and chroma components with 8x8 sub-block size and 8-tap interpolation filter for luma bi-prediction vs. CE4-2.1c

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE4-2.1c |  |  |  |  | Over CE4-2.1c |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.20% | -0.25% | 99% | 100% | - | - | - | - | - |
| Class A2 | -0.16% | -1.05% | -0.94% | 99% | 100% | - | - | - | - | - |
| Class B | 0.00% | -0.59% | -0.56% | 100% | 100% | -0.02% | -0.33% | -0.51% | 99% | 98% |
| Class C | -0.03% | -0.08% | -0.22% | 100% | 102% | -0.02% | -0.17% | -0.68% | 99% | 99% |
| Class E | - | - | - | - | - | 0.06% | -0.05% | -0.20% | 99% | 99% |
| Overall | -0.04% | -0.47% | -0.48% | 100% | 101% | 0.00% | -0.20% | -0.49% | 99% | 99% |
| Class D | 0.00% | -0.22% | -0.32% | 100% | 103% | 0.02% | 0.49% | -0.53% | 100% | 100% |
| Class F | -0.03% | -0.63% | -0.68% | 100% | 101% | 0.00% | -0.66% | -1.34% | 99% | 99% |

Figure 2 shows the comparison between before and after applying PROF on chroma components. The figure shows that the chroma artifacts on the edge are reduced greatly, and luma and chroma edges are aligned.

Figure 2. Comparison between applying PROF to luma component only and to both all components. (Frame 16 of sequence CatRobot encoded with RA configuration at QP 37)

# Conclusions

PROF applied to both luma and chroma components can align the edge among different components for affine coded CUs. The subjective quality improvement is visible with PROF for all components compared to that without PROF (VTM-5.0) and that with PROF only applied to luma component (CE4-2.1). It is suggested that PROF for chroma components shall be considered in VVC.

# Reference

- J. Luo, Y. He, “CE2-related: Prediction refinement with optical flow for affine mode”, JVET-N0236, Geneva, CH, Mar. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).