| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0594 |
| --- | --- |

| Title: | CE4-related: Prediction sample padding unification for BDOF and PROF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Tel: Email: | 1-858-209-3851 {xiaoyuxiu, yiwenchen, tsung-chuanma, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, two methods are proposed to align the prediction sample padding schemes used for bidirectional optical flow (BDOF) and prediction refinement with optical flow (PROF). In the first method, the integer samples used to pad the prediction samples in the extended region of one BDOF/PROF block are obtained by flooring down the target fractional sample positions. In the second method, the integer samples that are closest to the target fractional samples are used for prediction sample padding. Simulation results reportedly show that the first method leads to {Y, U, V} BD-rate variations of {0.06%, 0.05%, 0.04%} and {0.06%, -0.01%, 0.23%} for RA and LD configurations, respectively. The second method results in {Y, U, V} BD-rate variations of {-0.05%, -0.07%, -0.04%} for RA configuration.

# Introduction

In the VVC working draft 5 [1], bidirectional optical flow (BDOF) is applied to enable the optical flow based motion refinement for bi-predicted coding units (CUs) that has both forward and backward predictions. In [2], prediction refinement with optical flow (PROF) is proposed to refine the subblock based affine motion compensation using the optical flow model.

For both the BDOF and the PROF, the gradients need to be calculated for each sample inside the current coding block, which requires generating one additional row/column of prediction samples on each side of the block. To avoid the additional computational complexity of sample interpolation, the prediction samples in the extended region around the block are copied from the reference samples at integer position (i.e., without interpolation). However, in the current BDOF and PROF designs, the reference samples at different integer positions are used to pad the prediction samples for the BDOF and the PROF. Specifically, the prediction samples in the extended region of the BDOF are always obtained by flooring down the target fractional sample positions while the integer reference samples that are closest to the target fractional samples are used for the PROF padding.

# Proposal

To achieve one more uniform design, two methods are proposed in the following to harmonize the prediction sample padding methods for the BDOF and the PROF.

Method One: The first method aligns the prediction sample padding method of the PROF to be the same as that of the BDOF. Specifically, by this method, the integer positions used to generate the extended prediction samples are obtained by flooring down the fractional sample positions, i.e., the selected integer samples are located left to the fractional samples (for horizontal gradients) and above the fractional sample position (for vertical gradients).

Method Two: The second method aligns the prediction sample padding method of the BDOF to be the same as that of the PROF. Specifically, when the second method is applied, the integer samples that are closest to the fractional samples are used to pad the prediction samples in the extended region.

Figure 1 Comparison of two prediction sample padding methods: (left) Method one; (right) Method two

Figure 1 shows examples to compare two prediction sample padding methods. In Figure 1, blank circles represent integer reference samples, triangles represent the fractional prediction samples and patterned circles represent the integer reference samples that used to fill the extended region of the current block.

# Simulation results

The proposed method was implemented based on the CE4-2.1a reference software [2] and tested following the common test conditions as specified in [3]. In the following tables, the anchors are the results of CE4-2.1 and the tests are the results of the anchors plus the proposed methods being applied.

Table 1 BD-rate performance of Method One compared to CE4-2.1a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-2.1a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.07% | 0.01% | 100% | 99% |
| Class A2 | 0.12% | 0.06% | -0.02% | 99% | 99% |
| Class B | 0.08% | 0.17% | 0.11% | 101% | 101% |
| Class C | 0.05% | -0.11% | 0.03% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.05% | 0.04% | 101% | 100% |
| Class D | 0.12% | -0.03% | -0.13% | 100% | 101% |
| Class F | 0.19% | 0.15% | 0.14% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-2.1a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.14% | 0.14% | 102% | 100% |
| Class C | 0.04% | -0.09% | 0.14% | 100% | 100% |
| Class E | 0.09% | -0.16% | 0.52% | 100% | 100% |
| Overall | 0.06% | -0.01% | 0.23% | 101% | 100% |
| Class D | 0.05% | -0.01% | -0.56% | 102% | 100% |
| Class F | 0.30% | 0.15% | -0.25% | 96% | 103% |

Table 2 BD-rate performance of Method Two compared to CE4-2.1a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-2.1a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.07% | 0.00% | 100% | 99% |
| Class A2 | -0.03% | -0.12% | 0.05% | 102% | 95% |
| Class B | -0.04% | -0.03% | -0.03% | 100% | 100% |
| Class C | -0.12% | -0.18% | -0.13% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.07% | -0.04% | 100% | 99% |
| Class D | -0.30% | -0.15% | -0.34% | 101% | 103% |
| Class F | -0.02% | -0.02% | -0.11% | 102% | 101% |

# Proposed text changes

The proposed text changes are made upon the specification text of CE4-2.1. All the changes beyond the VVC specification draft 5 are distinguished by using different colors, as specified as follows:

The text changes of CE4-2.1

The added texts and removed texts of the proposed methods are highlighted in yellow and strikethrough, respectively

## The proposed text changes of Method One

#### 8.5.6.3 Fractional sample interpolation process

##### 8.5.6.3.1 General

  - If bdofFlag is equal to TRUE or the inter_affine_flag[ xSb ][ ySb ] is equal to TRUE and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.6.3.3 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLX as inputs:

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + 1.

  - Otherwise, if inter_affine_flag[ xSb ][ ySb ] is equal to TRUE and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.7.3.3 with ( xIntRoundedL, yIntRoundedL ), ( xFracL, yFracL ) and refPicLX as inputs.

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + 1.

xIntRoundedL and yIntRoundedL are derived as following:

xIntRoundedL = xIntL + ( xFracL >> 3 )

yIntRoundedL = yIntL + ( yFracL >> 3 )

## The proposed text changes of Method Two

#### 8.5.6.3 Fractional sample interpolation process

##### 8.5.6.3.1 General

  - If bdofFlag is equal to TRUE and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.6.3.3 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLX as inputs:

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + 1.

  - Otherwise, if bdofFlag is equal to TRUE or inter_affine_flag[ xSb ][ ySb ] is equal to TRUE and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.7.3.3 with ( xIntRoundedL, yIntRoundedL ), ( xFracL, yFracL ) and refPicLX as inputs.

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + 1.

xIntRoundedL and yIntRoundedL are derived as following:

xIntRoundedL = xIntL + ( xFracL >> 3 )

yIntRoundedL = yIntL + ( yFracL >> 3 )

# References

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, March 2019, Geneva, Switzerland.

J. Luo, Y. He, “CE4: Prediction refinement with optical flow for affine mode (Test 2.1)”, JVET-O0070, July 2019, Gothenburg, Sweden.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).