| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0611 |
| --- | --- |

| Title: | CE2-3 related: On CCLM restriction for 32x32 luma latency |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Kyohei Unno Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 80 5066 9075 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

A decoding process of CCLM chroma block starts after the corresponding luma samples is reconstructed because the chroma block refers the reconstructed luma samples. This causes the latency issue of chrome decoding and the worst case of the latency and buffer size in the VTM 5.0 is 64x64 luma samples. This contribution proposes the CCLM restriction to reduce the chrome decoding latency from 64x64 luma samples to 32x32. For instance, CCLM enabled flag is introduced and set when the luma block size is equaled to 32x32 and the luma / chroma partitioning is the same. The flag is not changed at the remaining partitioning. This contribution also proposes an encoder optimization of chroma RDO to reduce coding loss. Y/U/V BD-rate performances are 0.32% / 2.81% / 2.94% and 0.20% / 2.74% / 2.88% for the all intra and the random access conditions, respectively.

Problem Statement

CCLM chroma intra prediction is one of the key coding tool to improve the coding performance. Chroma predicted samples are generated from corresponding reconstructed luma samples, which causes a potential latency issue when applying CCLM in large block or in separate luma/chroma tree. In the worst case, the chroma intra prediction cannot start until the whole 64x64 luma block is reconstructed. This issue was mentioned in AHG16 report JVET-M0016, and then two solutions were proposed in JVET-N0389 and JVET-N0390. Core experiments are established as CE2-3 in JVET-N1022.

A key idea of the above solutions was to restrict the block size and alignment on 32x32 luma grid and 16x16 chroma grid for chroma separate tree. A condition checks, however, were complicated from the hardware implementation viewpoint. Furthermore, the same latency issue existed for large block, i.e. 64x64 luma block, in single tree, which was not considered.

Proposed method

CCLM enabled flag

In this contribution, CCLM enabled flag is introduced into block level and determined at the specific CU depth.

Option 1: 32x32 luma sample block latency

CCLM enabled flag is set equal to TRUE when the following conditions are true.

- The luma block is partitioned into 32x32 sample blocks by a series of quad trees.

- The chroma block is partitioned into 16x16 sample blocks by a series of quad trees.

Further split is allowed but the flag is not changed. This condition check is applied to both single tree and chroma separate tree.

Option 2: two 32x32 luma sample block latency

When two 32x32 luma sample block latency is acceptable, coding loss caused by the partitioning restriction can be reduced. Condition check is, however, more complicated than the option 1. For example in single tree, 128x128 luma block is partitioned into four 64x64 by QT and then they are partitioned into two 32x64/64x32 by BT or four 32x32 by QT. In chroma separate tree, 128x128 luma block is partitioned into four 64x64 by QT and then they are partitioned into four 32x32 by QT while 64x64 chroma block is partitioned into four 32x32 by QT, which is always applied, and then they are partitioned into two 32x16 by BT or for 16x16 by QT. It is noted that 16x32 chroma block by BT is not allowed because luma and chroma block scan order is not aligned.

Encoder optimization

Chroma partition is checked by the RDO process with some early termination in the current VTM-5.0. The proposed restriction is not considered well in the RDO process. This causes the further degradation of coding performance.

To relax the above problem as the encoder optimization, two sequential QTs check of luma/chroma block and chroma block is always applied for single and separate tree case, respectively. It is noted that two sequential QTs check of luma block for separate tree case is not considered. The reason is that the luma and chroma partitioning is independently optimized and it is hard to consider a chroma cost for the luma partitioning in the current VTM implementation.

Specification of option 1

8.6.4 Derivation process for chroma intra prediction mode

Input to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current chroma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived.

A variable cclmEnabledFlag is derived as follows:

- If the current coding quadtree depth < 2, cclmEnabledFlag is set equal to FALSE. [Ed: quadtree depth is defined elsewhere.]

- Otherwise, if the current coding quadtree depth is eqal to 2, cclmEnabledFlag is derived as follows:

  - The value of split_qt_flag of the parent luma / chroma block is equal to TRUE and the value of split_qt_flag of the parent luma / chroma block of the parent luma / chroma block is equal to TRUE, cclmEnabledFlag is set equal to TRUE. [Ed: parent luma / chroma block is replaced by an appropriate representation.]

  - Otherwise, cclmEnabledFlag is set equal to FALSE.

- Otherwise, cclmEnabledFlag is inferred to be cclmEnableFlag in the parent block.

The corresponding luma intra prediction mode lumaIntraPredMode is derived as follows:

- If intra_mip_flag[ xCb ][ yCb ] is equal to 1, lumaIntraPredMode is derived using IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ] and sizeId set equal to MipSizeId[ xCb ][ yCb ] as specified in Table 8-4.

- Otherwise, lumaIntraPredMode is set equal to IntraPredModeY[ xCb + cbWidth / 2 ][ yCb + cbHeight / 2 ].

The chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode as specified in Table 8-5 and Table 8-6.

Table 8-5 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag cclmEnabledFlag is equal to 0

Table 8-6 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag cclmEnabledFlag is equal to 1

Simulation results

The proposed method is implemented on the VTM-5.0. A test condition follows the JVET common test condition [2]. The summary of BD-rate is shown in the following table. The first and the second test set are results of 32x32 luma block in Table 1 and two 32x32 luma block in Table 2, respectively. Even though the coding loss is larger than CE2.3 results, complexity of the CCLM enabled flag check is reduced.

Table 1 Simulation results of the all intra and random access by the proposed restriction 32x32.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.03% | 8.10% | 7.97% | 96% | 103% |
| Class A2 | 0.33% | 2.83% | 1.37% | 98% | 100% |
| Class B | 0.20% | 2.44% | 3.78% | 97% | 101% |
| Class C | 0.09% | 0.44% | 0.53% | 98% | 101% |
| Class E | 0.10% | 1.27% | 1.28% | 99% | 100% |
| Overall | 0.32% | 2.81% | 2.94% | 98% | 101% |
| Class D | 0.04% | 0.33% | 0.15% | 99% | 100% |
| Class F | 0.19% | 0.93% | 1.21% | 98% | 101% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.67% | 6.30% | 8.04% | 99% | 101% |
| Class A2 | 0.14% | 2.19% | 1.19% | 100% | 100% |
| Class B | 0.08% | 2.88% | 3.01% | 100% | 100% |
| Class C | 0.03% | 0.32% | 0.11% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.20% | 2.74% | 2.88% | 100% | 100% |
| Class D | 0.06% | 0.25% | 0.10% | 100% | 98% |
| Class F | 0.08% | 0.72% | 0.69% | 101% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 1.53% | 1.21% | 100% | 100% |
| Class C | 0.03% | 0.04% | 0.06% | 100% | 100% |
| Class E | 0.01% | 1.23% | 1.61% | 100% | 100% |
| Overall | 0.02% | 0.96% | 0.93% | 100% | 100% |
| Class D | 0.04% | -0.08% | 0.98% | 100% | 100% |
| Class F | 0.11% | 0.59% | 1.06% | 102% | 99% |

Table 2 Additional results of random access and Test 1 condition without the proposed method.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.76% | 5.12% | 5.95% | 99% | 102% |
| Class A2 | 0.25% | 1.97% | 1.08% | 99% | 100% |
| Class B | 0.11% | 1.42% | 2.00% | 99% | 101% |
| Class C | 0.05% | 0.23% | 0.25% | 99% | 100% |
| Class E | 0.04% | 0.67% | 0.65% | 99% | 100% |
| Overall | 0.22% | 1.74% | 1.89% | 99% | 101% |
| Class D | 0.01% | 0.13% | -0.03% | 99% | 101% |
| Class F | 0.08% | 0.35% | 0.51% | 99% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.29% | 2.40% | 2.89% | 101% | 100% |
| Class A2 | 0.08% | 1.28% | 0.70% | 100% | 100% |
| Class B | 0.03% | 1.45% | 1.38% | 100% | 100% |
| Class C | -0.06% | 0.11% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 1.25% | 1.18% | 100% | 100% |
| Class D | 0.02% | 0.03% | 0.14% | 100% | 99% |
| Class F | 0.03% | 0.29% | 0.41% | 101% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.75% | 0.35% | 100% | 99% |
| Class C | 0.05% | 0.00% | -0.19% | 100% | 101% |
| Class E | -0.07% | 0.45% | 0.60% | 100% | 100% |
| Overall | 0.01% | 0.43% | 0.23% | 100% | 100% |
| Class D | -0.03% | 0.63% | 0.47% | 100% | 101% |
| Class F | -0.04% | -0.16% | 0.01% | 103% | 100% |

Conclusion

In this contribution, we proposed CCLM enabled flag to reduce the worst case of the chrome decoding latency from 64x64 luma samples to 32x32. The enabled flag was set when the luma block size was equaled to 32x32 and the luma / chroma partitioning was the same. The encoder optimization of chroma RDO was also proposed to reduce coding loss. Y/U/V BD-rate performances are 0.32% / 2.81% / 2.94% and 0.20% / 2.74% / 2.88% for the all intra and the random access conditions, respectively.

References

- J. Chen, E. François, P. Yin, “Description of Core Experiment 2 (CE2): luma-chroma dependency reduction”, JVET-M1022, Geneva, CH, 19–27 Mar. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).