| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0614 |
| --- | --- |

| Title: | CE8-related: Combination of a simplified IBC mode and the palette mode in CE8-2.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Xuguang Zuo, Tsui-Shan Chang, Jian Lou | Tel: Email: | +1-858-349-2818 yuchen.s@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd |  |  |

_____________________________

- Abstract

This document proposed to simplify IBC mode. In VTM-5.0, residual decoding module is applied to refine IBC prediction. This proposal proposed to simplify IBC by forcing IBC CU to be transform skip mode. In other word, the transform module can be removed from IBC decoding path. An IBC CU is forced to be transform skip mode if the transform skip mode is supported for the size of the CU. For other CUs without transform skip mode support, IBC mode is disallowed. For 4:2:0 test sequences, the proposed simplification shows:

- 0.6%, 0.4%, and 0.3% BD-rate losses for luma in TGM sequences, with 96%, 97% and 98% in encoding time, and 102%, 101% and 105% in decoding time under AI, RA, and LD configurations, respectively.

- 1.3%, 1.0%, and 0.5% BD-rate losses for luma in class F sequences, with 97%, 98% and 98% for encoding time, and 97%, 101% and 104% in decoding time under AI, RA, and LD configurations, respectively.

The combination of the proposed simplified IBC mode and the palette mode in CE8-2.5 is also tested. For 4:2:0 test sequences, the combination shows:

- -4.7%, -1.6%, and -0.3% BD-rate gains for luma in TGM sequences, with 99%, 96% and 102% in encoding time, and 95%, 98% and 104% in decoding time under AI, RA, and LD configurations, respectively.

- 0.4%, 0.2%, and 0.1% BD-rate losses for luma in class F sequences, with 102%, 102% and 103% for encoding time, and 103%, 101% and 103% in decoding time under AI, RA, and LD configurations, respectively.

Proposal description

In VTM-5.0, if a CU is coded by IBC mode, residual decoding module is applied to refine IBC prediction. This proposal proposed to simplify IBC by forcing IBC CU to be transform skip mode. In other word, the transform module can be removed from IBC decoding path. An IBC CU is forced to be transform skip mode if the transform skip mode is supported for the size of the CU. For other CUs without transform skip mode support, IBC mode is disallowed.

The combination of the proposed simplified IBC mode and the palette mode in CE8-2.5 is also tested.

Simulation results

The proposed simplified IBC mode

The proposed method is evaluated according to the CE8 description [3] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The DualITree is turned on for all Classes. The IBC is turned on for all Classes, and HashME and RDPCM are turned on only for Class F and Class TGM. Table 1 shows the results compared with VTM5.0 on test sequences 4:2:0.

Table 1: Results of the proposed simplified IBC on test sequences 4:2:0

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.02% | -0.05% | 99% | 100% |
| Class A2 | 0.50% | 0.46% | 0.53% | 99% | 101% |
| Class B | 0.38% | 0.25% | 0.23% | 99% | 103% |
| Class C | 0.18% | 0.22% | 0.16% | 98% | 96% |
| Class E | 0.60% | 0.36% | 0.49% | 100% | 100% |
| Overall | 0.33% | 0.25% | 0.26% | 99% | 100% |
| Class D | 0.34% | 0.28% | 0.37% | 98% | 98% |
| Class F | 1.26% | 0.92% | 0.94% | 97% | 97% |
| TGM | 0.65% | 0.19% | 0.18% | 96% | 102% |
|  | Random Access |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.13% | 0.10% | 96% | 97% |
| Class A2 | 0.28% | 0.42% | 0.18% | 95% | 97% |
| Class B | 0.19% | 0.14% | 0.12% | 96% | 97% |
| Class C | 0.11% | -0.20% | -0.19% | 96% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.05% | 0.05% | 96% | 97% |
| Class D | 0.16% | -0.09% | -0.18% | 97% | 97% |
| Class F | 0.98% | 0.63% | 0.74% | 98% | 101% |
| TGM | 0.41% | 0.18% | 0.12% | 97% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.04% | 0.04% | 100% | 102% |
| Class C | 0.06% | 0.32% | 0.29% | 101% | 105% |
| Class E | 0.10% | 0.70% | 0.47% | 99% | 102% |
| Overall | 0.05% | 0.27% | 0.23% | 100% | 103% |
| Class D | 0.02% | -0.01% | -0.39% | 99% | 103% |
| Class F | 0.48% | 0.35% | 0.07% | 98% | 104% |
| TGM | 0.33% | 0.29% | 0.36% | 98% | 105% |

The combination of the proposed simplified IBC mode and the palette mode in CE8-2.5

The combination of the proposed simplified IBC mode and the palette mode in CE8-2.5 is evaluated according to the CE8 description [3] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The DualITree is turned on for all Classes. The IBC is turned on for all Classes, and HashME and RDPCM are turned on only for Class F and Class TGM. Table 2 shows the results of the combination compared with VTM5.0 on test sequences 4:2:0.

Table 2: Results of the combination on test sequences 4:2:0

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.03% | -0.06% | 104% | 100% |
| Class A2 | 0.55% | 0.48% | 0.60% | 104% | 105% |
| Class B | 0.43% | 0.27% | 0.27% | 104% | 107% |
| Class C | 0.22% | 0.29% | 0.17% | 102% | 106% |
| Class E | 0.66% | 0.48% | 0.58% | 104% | 105% |
| Overall | 0.38% | 0.30% | 0.30% | 104% | 105% |
| Class D | 0.38% | 0.39% | 0.31% | 101% | 103% |
| Class F | 0.38% | -0.02% | -0.17% | 102% | 103% |
| TGM | -4.71% | -4.89% | -4.27% | 99% | 95% |
|  | Random Access |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.22% | 0.13% | 99% | 96% |
| Class A2 | 0.30% | 0.29% | 0.15% | 102% | 100% |
| Class B | 0.21% | 0.12% | 0.20% | 102% | 101% |
| Class C | 0.19% | -0.07% | -0.20% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.19% | 0.12% | 0.07% | 101% | 99% |
| Class D | 0.17% | 0.09% | 0.01% | 103% | 102% |
| Class F | 0.21% | -0.50% | -0.42% | 102% | 101% |
| TGM | -1.62% | -2.62% | -2.21% | 96% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.03% | 0.64% | 105% | 101% |
| Class C | 0.01% | 0.42% | 0.56% | 105% | 104% |
| Class E | 0.10% | 0.79% | 0.68% | 104% | 102% |
| Overall | 0.04% | 0.33% | 0.62% | 105% | 102% |
| Class D | 0.08% | 0.00% | 0.00% | 105% | 105% |
| Class F | 0.12% | -0.62% | -0.60% | 103% | 103% |
| TGM | -0.34% | -0.60% | -0.35% | 102% | 104% |

- Reference

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-N1028, 14th meeting, Geneva, CH, March 2019.

- Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).