    - Transformation process for scaled transform coefficients

      - General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

When lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, numLfnstX, numLfnstY, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(8-968)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  36  :  16	(8-969)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(8-970)

nLfnstSize = 1  <<  log2LfnstSize		(8-971)

numLfnstX = ( nTbW > 8  &&  nTbH  = = 4 )  ?  2  :  1	(8-972)

numLfnstY = ( nTbW = = 4  &&  nTbH  > 8 )  ?  2  :  1	(8-973)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(8-974)

- The wide angle intra prediction mode mapping process as specified in clause 8.4.5.2.6 is invoked with predModeIntra, nTbW, nTbH and cIdx as inputs, and the modified predModeIntra as output.

- For xSbIdx = 0..numLfnstX − 1 and ySbIdx = 0..numLfnstY − 1, the following applies:

- The values of the list u[ x ] with x = 0..nonZeroSize − 1 are derived as follows:

xC = ( xSbIdx << log2LfnstSize ) + DiagScanOrder[ log2LfnstSize ][ log2LfnstSize ][ x ][ 0 ]	(8-975)

yC = ( ySbIdx << log2LfnstSize ) + DiagScanOrder[ log2LfnstSize ][ log2LfnstSize ][ x ][ 1 ]	(8-976)

u[ x ] = d[ xC ][ yC ] 			(8-977)

- The one-dimensional low frequency non-separable transformation process as specified in clause 8.7.4.2 is invoked with the input length of the scaled transform coefficients nonZeroSize, the transform output length nTrS set equal to nLfnstOutSize, the list of scaled non-zero transform coefficients u[ x ] with x = 0..nonZeroSize − 1, the intra prediction mode for LFNST set selection predModeIntra, and the LFNST index for transform selection in the selected LFNST set lfnst_idx[ xTbY ][ yTbY ] as inputs, and the list v[ x ] with x = 0..nLfnstOutSize − 1 as output.

- The array d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] with x = 0..nLfnstSize − 1, y = 0..nLfnstSize − 1 is derived as follows:

  - If  nLfnstSize is equal to 4, the following applies:

- If predModeIntra is less than or equal to 34, or equal to INTRA_LT_CCLM, INTRA_T_CCLM, or INTRA_L_CCLM, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = v[ x + ( y << log2LfnstSize ) ] 	(8-978)

- Otherwise, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = v[ y + ( x << log2LfnstSize ) ]  	(8-979)

- Otherwise (if nLfnstSize is equal to 8), the following applies:

- The valid area elementNum is defined with two set of table as

elementNum[2][8] = { {8, 6, 6, 6, 4, 4, 1, 1}, {6, 6, 6, 6, 4, 4, 4, 0} }

- The valiable cnt is set to 0.

- If predModeIntra is less than or equal to 34, or equal to INTRA_LT_CCLM, INTRA_T_CCLM, or INTRA_L_CCLM, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = 
	  ( x >= 4 && y >= 4 )  ? d[ ( xSbIdx << log2LfnstSize ) + x  ][ ( ySbIdx << log2LfnstSize ) + y  ]  : 
	( (x < elementNum[lfnst_idx[ xTbY ][ yTbY ] - 1][y] )  ?  v[ cnt++ ] : 0)

- Otherwise, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = 
	  ( x >= 4 && y >= 4 )  ? d[ ( xSbIdx << log2LfnstSize ) + x  ][ ( ySbIdx << log2LfnstSize ) + y  ]  : 	
	( (y < elementNum[lfnst_idx[ xTbY ][ yTbY ] - 1][x] )  ?  v[ cnt++ ] : 0)

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are both equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT or both sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA, trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? 1 : 0	(8-980)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? 1 : 0	(8-981)

- Otherwise, if cu_sbt_flag is equal to 1, trTypeHor and trTypeVer are specified in Table 8-18 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-17 depending on tu_mts_idx[ xTbY ][ yTbY ].

The variables nonZeroW and nonZeroH are derived as follows:

nonZeroW = Min( nTbW, ( trTypeHor > 0 )  ?  16  :  32 )	(8-982)

nonZeroH = Min( nTbH, ( trTypeVer > 0 )  ?  16  :  32 )	(8-983)

The (nTbW)x(nTbH) array r of residual samples is derived as follows:

- When nTbH is greater than 1, each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nonZeroH − 1 is transformed to e[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.7.4.4 for each column x = 0..nonZeroW − 1 with the height of the transform block nTbH, the non-zero height of the scaled transform coefficients nonZeroH, the list d[ x ][ y ] with y = 0..nonZeroH − 1 and the transform type variable trType set equal to trTypeVer as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbH − 1.

- When nTbH and nTbW are both greater than 1, the intermediate sample values g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 are derived as follows:

g[ x ][ y ] = Clip3( CoeffMin, CoeffMax, ( e[ x ][ y ] + 64 )  >>  7 )	(8-984)

- When nTbW is greater than 1, each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nonZeroW − 1, y = 0..nTbH − 1 is transformed to r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.7.4.4 for each row y = 0..nTbH − 1 with the width of the transform block nTbW, the non-zero width of the resulting array g[ x ][ y ] nonZeroW, the list g[ x ][ y ] with x = 0..nonZeroW − 1 and the transform type variable trType set equal to trTypeHor as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbW − 1.

Table 8-17 – Specification of trTypeHor and trTypeVer depending on tu_mts_idx[ x ][ y ]

| tu_mts_idx[ x0 ][ y0 ] | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| trTypeHor | 0 | 1 | 2 | 1 | 2 |
| trTypeVer | 0 | 1 | 1 | 2 | 2 |

Table 8-18 – Specification of trTypeHor and trTypeVer depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag

| cu_sbt_horizontal_flag | cu_sbt_pos_flag | trTypeHor | trTypeVer |
| --- | --- | --- | --- |
| 0 | 0 | 2 | 1 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 2 |
| 1 | 1 | 1 | 1 |

      - Low frequency non-separable transformation process

Inputs to this process are:

- a variable nonZeroSize specifying the transform input length,

- a variable nTrS specifying the transform output length,

- a list of of scaled non-zero transform coefficients x[ j ] with j = 0..nonZeroSize − 1,

- a variable predModeIntra specifying the intra prediction mode for LFNST set selection,

- a variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set.

Output of this process is the list of transformed samples y[ i ] with i = 0..nTrS − 1.

The transformation matrix derivation process as specified in clause 8.7.4.3 is invoked with the transform output length nTrS, the intra prediction mode for LFNST set selection  predModeIntra, and the LFNST index for transform selection in the selected LFNST set lfnstIdx as inputs, and the (nTrS)x(nonZeroSize) LFNST matrix lowFreqTransMatrix as output.

The list of transformed samples y[ i ] with i = 0..nTrS − 1 is derived as follows:

y[i] = Clip3( CoeffMin, CoeffMax, ( ( j=0nonZeroSize-1lowFreqTransMatrix[i][j]*x[j] ) + 64 ) >> 7 )	(8-985)

      - Low frequency non-separable transformation matrix derivation process

Inputs to this process are:

- a variable nTrS specifying the transform output length,

- a variable predModeIntra specifying the intra prediction mode for LFNST set selection,

- a variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set.

Output of this process is the transformation matrix lowFreqTransMatrix.

The variable lfnstTrSetIdx is specified in Table 8-19 – Specification of lfnstTrSetIdx depending on predModeIntra.

Table 8-19 – Specification of lfnstTrSetIdx

| predModeIntra | lfnstTrSetIdx |
| --- | --- |
| predModeIntra < 0 | 1 |
| 0 <= predModeIntra <= 1 | 0 |
| 2 <= predModeIntra <= 12 | 1 |
| 13 <= predModeIntra <= 23 | 2 |
| 24 <= predModeIntra <= 44 | 3 |
| 45 <= predModeIntra <= 55 | 2 |
| 56 <= predModeIntra <= 80 | 1 |
| 81 <= predModeIntra <= 83 | 0 |

The transformation matrix lowFreqTransMatrix is derived based on nTrS, lfnstTrSetIdx, and lfnstIdx as follows:

- If nTrS is equal to 16, lfnstTrSetIdx is equal to 0, and lfnstIdx is equal to 1, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{  108  -44  -15    1  -44   19    7   -1  -11    6    2   -1    0   -1   -1    0  }

{  -40  -97   56   12  -11   29  -12   -3   18   18  -15   -3   -1   -3    2    1  }

{   25  -31   -1    7  100  -16  -29    1  -54   21   14   -4   -7    2    4    0  }

{  -32  -39  -92   51   -6  -16   36   -8    3   22   18  -15    4    1   -5    2  }

{    8   -9   33   -8  -16 -102   36   23   -4   38  -27   -5    5   16   -8   -6  }

{  -25    5   16   -3  -38   14   11   -3  -97    7   26    1   55  -10  -19    3  }

{    8    9   16    1   37   36   94  -38   -7    3  -47   11   -6  -13  -17   10  }

{    2   34   -5    1   -7   24  -25   -3    8   99  -28  -29    6  -43   21   11  }

{  -16  -27  -39 -109    6   10   16   24    3   19   10   24   -4   -7   -2   -3  }

{   -9  -10  -34    4   -9   -5  -29    5  -33  -26  -96   33   14    4   39  -14  }

{  -13    1    4   -9  -30  -17   -3  -64  -35   11   17   19  -86    6   36   14  }

{    8   -7   -5  -15    7  -30  -28  -87   31    4    4   33   61   -5  -17   22  }

{   -2   13   -6   -4   -2   28  -13  -14   -3   37  -15   -3   -2  107  -36  -24  }

{    4    9   11   31    4    9   16   19   12   33   32   94   12    0   34  -45  }

{    2   -2    8  -16    8    5   28  -17    6   -7   18  -45   40   36   97   -8  }

{    0   -2    0  -10   -1   -7   -3  -35   -1   -7   -2  -32   -6  -33  -16 -112  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 0, and lfnstIdx is equal to 2, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{  119  -30  -22   -3  -23   -2    3    2  -16    3    6    0   -3    2    1    0  }

{  -27 -101   31   17  -47    2   22    3   19   30   -7   -9    5    3   -5   -1  }

{    0   58   22  -15 -102    2   38    2   10  -13   -5    4   14   -1   -9    0  }

{   23    4   66  -11   22   89   -2  -26   13   -8  -38   -1   -9  -20   -2    8  }

{  -19   -5  -89    2  -26   76  -11  -17   20   13   18   -4    1  -15    3    5  }

{  -10   -1   -1    6   23   25   87   -7  -74    4   39   -5    0   -1  -20   -1  }

{  -17  -28   12   -8  -32   14  -53   -6  -68  -67   17   29    2    6   25    4  }

{    1  -24  -23    1   17   -7   52    9   50  -92  -15   27  -15  -10   -6    3  }

{   -6  -17   -2 -111    7  -17    8  -42    9   18   16   25   -4    2   -1   11  }

{    9    5   35    0    6   21   -9   34   44   -3  102   11   -7   13   11  -20  }

{    4   -5   -5  -10   15   19   -2    6    6  -12  -13    6   95   69  -29  -24  }

{   -6   -4   -9  -39    1   22    0  102  -19   19  -32   30  -16  -14   -8  -23  }

{    4   -4    7    8    4  -13  -18    5    0    0   21   22   58  -88  -54   28  }

{   -4   -7    0  -24   -7    0  -25    3   -3  -30    8  -76  -34    4  -80  -26  }

{    0    6    0   30   -6    1  -13  -23    1   20   -2   80  -44   37  -68    1  }

{    0    0   -1    5   -1   -7    1  -34   -2    3   -6   19    5  -38   11 -115  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 1, and lfnstIdx is equal to 1, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{ -111   39    4    3   44   11  -12   -1    7  -16   -5    2    3   -1    4    2  }

{  -47  -27   15   -1  -92   43   20   -2   20   39  -16   -5   10   -5  -13    2  }

{  -35  -23    4    4  -17  -72   32    6  -59   18   50   -6    0   40    0  -13  }

{   13   93  -27   -4  -48   13  -34    4  -52   11    1   10    3   16   -3    1  }

{  -11  -27    1    2  -47   -4  -36   10   -2  -85   14   29  -20   -2   57    4  }

{    0  -35   32   -2   26   60   -3  -17  -82    1  -30    0  -37   21    3   12  }

{  -17  -46  -92   14    7  -10  -39   29  -17   27  -28   17    1  -15  -13   17  }

{    4  -10  -23    4   16   58  -17   26   30   21   67    2  -13   59   13  -40  }

{    5  -20   32   -5    8   -3  -46   -7   -4    2  -15   24  100   44    0    5  }

{   -4   -1   38  -18   -7  -42  -63   -6   33   34  -23   15  -65   33  -20    2  }

{   -2  -10   35  -19    5    8  -44   14  -25   25   58   17    7  -84  -16  -18  }

{    5   13   18   34   11   -4   18   18    5   58   -3   42   -2  -10   85   38  }

{   -5   -7  -34  -83    2   -1   -4  -73    4   20   15  -12    4   -3   44   12  }

{    0    4   -2  -60    5    9   42   34    5  -14    9   80   -5   13  -38   37  }

{   -1    2    7  -57    3   -7    9   68   -9    6  -49  -20    6   -4   36  -64  }

{   -1    0  -12   23    1   -4   17  -53   -3    4  -21   72   -4   -8   -3  -83  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 1, and lfnstIdx is equal to 2, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{   88  -55    6   -3  -66   27    9   -2   11   11  -13    1   -2   -7    1    2  }

{  -58  -20   27   -2  -27   75  -29    0   47  -42  -11   11   -9   -3   19   -4  }

{  -51   23  -22    5  -63    3   37   -5    1   64  -35   -4   29  -31  -11   13  }

{  -27  -76   49   -2   40   14    9  -17  -56   36  -25    6   14    3   -6    8  }

{   19   -4  -36   22   52    7   36  -23   28  -17  -64   15   -5  -44   48    9  }

{   29   50   13  -10    1   34  -59    1  -51    4  -16   30   52  -33   24   -5  }

{  -12  -21  -74   43  -13   39   18   -5  -58  -35   27   -5   19   26    6   -5  }

{   19   38  -10   -5   28   66    0   -5   -4   19  -30  -26  -40   28  -60   37  }

{   -6   27   18   -5  -37  -18   12  -25  -44  -10  -38   37  -66   45   40   -7  }

{  -13  -28  -45  -39    0   -5  -39   69  -23   16  -12  -18  -50  -31   24   13  }

{   -1    8   24  -51  -15   -9   44   10  -28  -70  -12  -39   24  -18   -4   51  }

{   -8  -22  -17   33  -18  -45  -57  -27    0  -31  -30   29   -2  -13  -53   49  }

{    1   12   32   51   -8    8   -2  -31  -22    4   46  -39  -49  -67   14   17  }

{    4    5   24   60   -5  -14  -23   38    9    8  -34  -59   24   47   42   28  }

{   -1   -5  -20  -34    4    4  -15  -46   18   31   42   10   10   27   49   78  }

{   -3   -7  -22  -34   -5  -11  -36  -69   -1   -3  -25  -73    5    4    4  -49  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 2, and lfnstIdx is equal to 1, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{ -112   47   -2    2  -34   13    2    0   15   -7    1    0    8   -3   -1    0  }

{   29   -7    1   -1 -108   40    2    0  -45   13    4   -1    8   -5    1    0  }

{  -36  -87   69  -10  -17  -33   26   -2    7   14  -11    2    6    8   -7    0  }

{   28   -5    2   -2  -29   13   -2    0  103  -36   -4    1   48  -16   -4    1  }

{  -12  -24   15   -3   26   80  -61    9   15   54  -36    2    0   -4    6   -2  }

{   18   53   69  -74   14   24   28  -30   -6   -7  -11   12   -5   -7   -6    8  }

{    5   -1    2    0  -26    6    0    1   45   -9   -1    0 -113   28    8   -1  }

{  -13  -32   18   -2   15   34  -27    7  -25  -80   47   -1  -16  -50   28    2  }

{   -4  -13  -10   19   18   46   60  -48   16   33   60  -48    1    0    5   -2  }

{   15   33   63   89    8   15   25   40   -4   -8  -15   -8   -2   -6   -9   -7  }

{   -8  -24  -27   15   12   41   26  -29  -17  -50  -39   27    0   35  -67   26  }

{   -2   -6  -24   13   -1   -8   37  -22    3   18  -51   22  -23  -95   17   17  }

{   -3   -7  -16  -21   10   24   46   75    8   20   38   72    1    2    1    7  }

{    2    6   10   -3   -5  -16  -31   12    7   24   41  -16  -16  -41  -89   49  }

{    4    8   21   40   -4  -11  -28  -57    5   14   31   70    7   18   32   52  }

{    0    1    4   11   -2   -4  -13  -34    3    7   20   47   -6  -19  -42 -101  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 2, and lfnstIdx is equal to 2, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{  -99   39   -1    2   65  -20   -5    0  -15   -2    5   -1    0    3   -1    0  }

{   58   42  -33    3   33  -63   23   -1  -55   32    3   -5   21   -2   -8    3  }

{  -15   71  -44    5  -58  -29   25    3   62   -7   -4   -4  -19    4    0    1  }

{   46    5    4   -6   71  -12  -15    5   52  -38   13   -2  -63   23    3   -3  }

{  -14  -54  -29   29   25   -9   61  -29   27   44  -48    5  -27  -21   12    7  }

{   -3    3   69  -42  -11  -50  -26   26   24   63  -19   -5  -18  -22   12    0  }

{   17   16   -2    1   38   18  -12    0   62    1  -14    5   89  -42    8   -2  }

{   15   54   -8    6    6   60  -26   -8  -30   17  -38   22  -43  -45   42   -7  }

{   -6  -17  -55  -28    9   30   -8   58    4   34   41  -52  -16  -36  -20   16  }

{   -2   -1   -9  -79    7   11   48   44  -13  -34  -55    6   12   23   20  -11  }

{    7   29   14   -6   12   53   10  -11   14   59  -15   -3    5   71  -54   13  }

{   -5  -24  -53   15   -3  -15  -61   26    6   30  -16   23   13   56   44  -35  }

{    4    8   21   52   -1   -1   -5   29   -7  -17  -44  -84    8   20   31   39  }

{   -2  -11  -25   -4   -4  -21  -53    2   -5  -26  -64   19   -8  -19  -73   39  }

{   -3   -5  -23  -57   -2   -4  -24  -75    1    3    9  -25    6   15   41   61  }

{    1    1    7   18    1    2   16   47    2    5   24   67    3    9   25   88  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 3, and lfnstIdx is equal to 1, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{ -114   37    3    2  -22  -23   14    0   21  -17   -5    2    5    2   -4   -1  }

{  -19  -41   19   -2   85  -60  -11    7   17   31  -34    2  -11   19    2   -8  }

{   36  -25   18   -2  -42  -53   35    5   46  -60  -25   19    8   21  -33   -1  }

{  -27  -80   44   -3  -58    1  -29   19  -41   18  -12   -7   12  -17    7   -6  }

{  -11  -21   37  -10   44   -4   47  -12  -37  -41   58   18   10  -46  -16   31  }

{   15   47   10   -6  -16  -44   42   10  -80   25  -40   21  -23   -2    3  -14  }

{   13   25   79  -39  -13   10   31   -4   49   45   12   -8    3   -1   43    7  }

{   16   11  -26   13  -13  -74  -20   -1    5   -6   29  -47   26  -49   54    2  }

{   -8  -34  -26    7  -26  -19   29  -37    1   22   46   -9  -81   37   14   20  }

{   -6  -30  -42  -12   -3    5   57  -52   -2   37  -12    6   74   10    6  -15  }

{    5    9   -6   42  -15  -18   -9   26   15   58   14   43   23  -10  -37   75  }

{   -5  -23  -23   36    3   22   36   40   27   -4  -16   56  -25  -46   56  -24  }

{    1    3   23   73    8    5   34   46  -12    2   35  -38   26   52    2  -31  }

{   -3   -2  -21  -52    1  -10  -17   44  -19  -20   30   45   27   61   49   21  }

{   -2   -7  -33  -56   -4   -6   21   63   15   31   32  -22  -10  -26  -52  -38  }

{   -5  -12  -18  -12    8   22   38   36   -5  -15  -51  -63   -5    0   15   73  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 3, and lfnstIdx is equal to 2, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{ -102   22    7    2   66  -25   -6   -1  -15   14    1   -1    2   -2    1    0  }

{   12   93  -27   -6  -27  -64   36    6   13    5  -23    0   -2    6    5   -3  }

{  -59  -24   17    1  -62   -2   -3    2   83  -12  -17   -2  -24   14    7   -2  }

{  -33   23  -36   11  -21   50   35  -16  -23  -78   16   19   22   15  -30   -5  }

{    0  -38  -81   30   27    5   51  -32   24   36  -16   12  -24   -8    9    1  }

{   28   38    8   -9   62   32  -13    2   51  -32   15    5  -66   28    0   -1  }

{   11  -35   21  -17   30  -18   31   18  -11  -36  -80   12   16   49   13  -32  }

{  -13   23   22  -36  -12   64   39   25  -19   23  -36    9  -30  -58   33   -7  }

{   -9  -20  -55  -83    3   -2    1   62    8    2   27  -28    7   15  -11    5  }

{   -6   24  -38   23   -8   40  -49    0   -7    9  -25  -44   23   39   70   -3  }

{   12   17   17    0   32   27   21    2   67   11   -6  -10   89  -22  -12   16  }

{    2   -9    8   45    7   -8   27   35   -9  -31  -17  -87  -23  -22  -19   44  }

{   -1   -9   28  -24   -1  -10   49  -30   -8   -7   40    1    4   33   65   67  }

{    5  -12  -24  -17   13  -34  -32  -16   14  -67   -7    9    7  -74   49    1  }

{    2   -6   11   45    3  -10   33   55    8   -5   59    4    7   -4   44  -66  }

{   -1    1  -14   36   -1    2  -20   69    0    0  -15   72    3    4    5   65  }

},

Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 0, and lfnstIdx is equal to 1 the following applies:

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15

lowFreqTransMatrixCol0to15 =

{

{ -114   34   15    2    3    1    2    1   37  -21   -2    0   -1    0   12   -1  }

{   35   86  -53    2  -10    1   -3    0   49  -29   10   -3    1   -1  -36   -4  }

{   11  -62    9   11    2    3    1    1   93   -2  -33    0   -4    0  -35   33  }

{  -14   20  -13   -1    1    0    1    0   12  112  -22  -18   -4   -4  -14  -27  }

{  -35  -43  -84   46   -4   10   -1    4  -32   -8   17  -13    6   -2  -32   13  }

{   -9    5   55  -21    4   -4    2   -2  -25   -6   15    5    0    1  -97   -7  }

{   -2   28    1   17   -5    2   -1    0  -39  -11 -100   29    5    4  -12   36  }

{    2  -20  -11    3    2    0    2    0   10  -23  -44   19   -1    4    1 -101  }

{   14   18   42  101  -29   11   -8    3    9   16   14   -1   11   -5   11   -7  }

{    6    3   -9  -34    9   -3    2    0   11   11  -17  -13   -2    1   42   28  }

{   12    2   28   13    9   -3    1   -1    2  -31   -2  -24    2    1   28   -5  }

{   -1    4   -5    1   22   -8    4   -1    8   21   30   89  -22    6    4   22  }

{   -3    8   12   -2   -3    1    1   -1   -9    5    0  -55    6   -1  -13   25  }

{    7   11   -2   34  109  -22   11   -4   -1   -2   -5  -20  -15   12   -8   -3  }

{   -5    1    3   12   17   -4    1   -1  -10   -2   -3   18   -7    2    0    0  }

{    0    7   -4    8   -8    5   -1    1   -5    3  -24   -6  -12    0   -1   28  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol16to31[ m - 16 ][ n ] with m = 16..31, n = 16..31

lowFreqTransMatrixCol16to31 =

{

{   -3    0   -1    0    2    0    0    0    0    0    3    0   -1    0    1    0  }

{   10    1    2    0    5   -3   -1    0    0    0   -8   -2    3    0    2   -1  }

{    5   -5    1   -1   -6   -3    6    0    0    0   -6    3    0    0   -2    0  }

{   32    0   -1    0   -4  -16    1    6    0    1    2   -5    0    0   -1   -3  }

{   22   -9    0   -1   27   -5   -6    3   -2    0   -2    5    3   -2    5   -1  }

{    9    7    5    2   44  -10  -12    1   -2    0   -6    4   -3    0    8   -1  }

{    8  -28    6   -4   -1   -4   23   -3   -2    0    5   -3    0    1    0    0  }

{    7    8    5    4   20   42  -17   -8   -1   -2   -4    6    9   -4    1    4  }

{  -25  -14    6    0   29    5  -11   -4   -3    1   -8    4    1   -6    3   -2  }

{   -8   -1   -8   -1   99    4  -14    2   -5   -1  -35   10   12   -1   11   -5  }

{   98  -18  -18    0    4  -27  -26   31    0    2   10    2  -18   -3   -3    1  }

{   28  -35    6   -2    5   36  -38  -16    6   -2   11  -12    2    3   -4    4  }

{   21  -15  -12   -1  -19  101   -2   -4   -1   -6  -17   -7   22    2    1   -2  }

{  -23    3  -28    4    5   -1   10   -4    5   -3   16   -2   -1   -1   -3    2  }

{    8   18   -4   -1  -39  -21  -20    4   13    1 -102    8   30  -14   25  -10  }

{   -5   77    3   -6   -6    9  -71   -5   31   -2   31   17   -9  -33  -10    1  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol32to35[ m - 32 ][ n ] with m = 32..35, n = 32..35

lowFreqTransMatrixCol32to35 =

{

{    0    0    1    1  }

{    0    0   -2    0  }

{    1    0   -2   -1  }

{    0    1    0   -1  }

{    0    0   -1    3  }

{   -2    0   -2    4  }

{    5   -1    1    0  }

{    1   -1   -1    0  }

{   -1    0   -3    1  }

{   -1    2   -8    2  }

{    0    2    2    0  }

{   -3   -4    2   -1  }

{   -1    6   -2   -1  }

{    1    1    2    0  }

{   -6    2  -10    4  }

{    8    1    3   -2  }

}

Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 0, and lfnstIdx is equal to 2 the following applies:

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15

lowFreqTransMatrixCol0to15 =

{

{  106  -49   -8   -1   -1   -1  -49    8   11    1    1    0   -8   10    1   -1  }

{   57   63  -44   -4   -6   -1   60  -38  -18    7   -1    0  -34  -17   19    4  }

{    3  -83   25   14    3    2   88   -2   -8   -6   -6   -1  -21    4    1   -4  }

{  -30  -30  -36   24    0    5  -25  -84   36   19    1    3  -24   30   34  -12  }

{   -2  -21  -80   25    4    4   18   13  -13   20   -2    0   84    5   -5   -5  }

{   25   13   62  -15   -5   -3   12  -73   12    2    1    2   73    3  -21   -2  }

{   12   29   -3   28  -12    3   27   26   77  -25  -13   -2   -3   49  -26  -40  }

{    2    5   14   67  -10    4  -10    3   44   12  -16    2   -3  -83    6   -6  }

{    7   11   15   85  -16    3  -13  -15  -64  -10  -10    4   -9   26  -37    1  }

{   13   19   34   26    8   -2   11   37   -4   52   -4   -2   22   31   64   -7  }

{   -1   -2  -10    1  -28    4   -7    1  -22  -81  -19   11   23   -7   40  -29  }

{    7   -7  -13   -4   -8    1   17    4   39  -32    4   -1   22  -21    0   44  }

{    3  -12   -4  -10    4    1    3   -3  -13   31  -11    2    3  -56  -17  -41  }

{    6    7    1   24  104  -18   -1   -7   -1  -40   41  -20    3   -9    5  -17  }

{   -2    4  -17   -5    8    0    3    6    6   25    9   -4   -6    9  -72    6  }

{    0    1    0  -25    6    1   -2   -1   -8   -1  -31    0   -3    5   -8  -77  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol16to31[ m - 16 ][ n ] with m = 16..31, n = 16..31

lowFreqTransMatrixCol16to31 =

{

{    0    0   -2    1   -2    0    0    0   -1    1    0    0   -1    0    0    0  }

{    1    1   -5    5    5   -3    0    0   -7   -1    1    0   -1    1    1    0  }

{    1    0  -10    6    2    0    2    0   -6    5   -1   -1   -2    1    0    0  }

{   -2   -2   15   17  -10   -8    1   -1    2    3   -2    0    3    2   -2   -1  }

{   -9   -1  -17  -17    5    1    1    1   -4    1    8   -1   -4   -1    1   -2  }

{   -1    0  -11    1   -4    0    2   -1   -5    4    0    1   -3    2   -1   -1  }

{    6   -2    3  -20  -32   13   11    0   -3   -5    3    7    1   -3   -3    1  }

{    0    5  -34    7   24   -7    2    0    2   15   -2  -10   -2    2   -2   -1  }

{   10    2   35   -8    8   11   -5   -1    1   -6    6   -7    3    0    3    0  }

{  -25   -3   22   37   -9  -37   -5   -1    4   -7  -17   -4    0   -1   -1   -2  }

{   25    5  -12   54  -14   -4   17  -10    3   -3  -21   13   -2    0    3    6  }

{    5   -2   90   12   25   -4  -27   -2    7    1   -7  -25   -1   -3   -3    3  }

{   -1    7   56   -2  -64    7   29    0   13  -18    1   33   -1    1    7   -2  }

{  -18   -3    2    6   -4   10   -6    2    3   -1   -2    5    1    0    0    2  }

{    6   -4  -12   93   -5   -6   -1   -4   12   24  -17   -5    0   -6   -4    5  }

{  -30   12   26    3   69   -7   10    7    9   33   26  -13    1    1  -11   -5  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol32to35[ m - 32 ][ n ] with m = 32..35, n = 32..35

lowFreqTransMatrixCol32to35 =

{

{    0    1    0    0  }

{   -2   -1    1    0  }

{   -1    1    0    0  }

{    0    1    0    0  }

{    0    1    2    0  }

{    0    0   -1    0  }

{   -1    0    1    0  }

{    2    2   -1   -2  }

{   -2    0    1   -2  }

{    1   -3   -2   -1  }

{    1   -2   -2    1  }

{   -3    1    0   -2  }

{   -2   -1    2    2  }

{    2    0    0    0  }

{    3   -2    1    1  }

{   -1    1   -1    2  }

}

Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 1, and lfnstIdx is equal to 1 the following applies:

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15

lowFreqTransMatrixCol0to15 =

{

{ -109   52    2    4    1    1    0    1   37    0  -11    0   -2    0   10  -13  }

{   41   14  -15    1   -3    0   -1    0   99  -47  -14    1   -2    0  -27  -27  }

{  -20   12   -5    3   -2    1   -1    0  -31  -56   31    3    4    0  -73   42  }

{   40  100  -43   -1   -7    0   -2    0  -34   10  -27    7   -1    1   -7   -6  }

{    8    2   -5   -1   -1    0   -1    0   44    1   19  -12    1   -3   18   68  }

{    3    4   31  -10    4   -2    1   -1   21   79  -24  -17   -1   -3  -76    6  }

{  -13  -13  -22   12   -1    3   -1    1   -8  -20   15   -6    2   -1  -51    4  }

{  -11  -43  -15    4    2    0    0    0   -4  -11  -53   18   -1    2  -16  -16  }

{  -17  -31  -98   23   -7    6   -3    2    9   37  -10   28   -5    3   -3   19  }

{    8    8  -15   15   -4    2   -2    1   22   25   38   -3   -5   -2   11   33  }

{    1   -6  -24   20   -4    6   -1    2   -2    7   56  -18   -7   -1  -12  -75  }

{   -4   -7  -20  -13    4   -2    0   -1   -9  -42  -39  -22    9   -3   18    8  }

{   -3   -1   -5   26   -3    3   -2    2   -8   -6   13  -33   -1   -2   -6   -5  }

{   -7  -11  -16  -65    6   -5    2   -2   -2    6  -18    2   10    0  -15  -15  }

{   -6    1  -26  -92   12  -11    4   -4    5    4   45    3   22   -3    4   -2  }

{   -4   -4  -23  -10  -14    1   -4    1   -5    8  -19  -86   -3    0   13   11  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol16to31[ m - 16 ][ n ] with m = 16..31, n = 16..31

lowFreqTransMatrixCol16to31 =

{

{   -1    2    0    0    5   -3    3    0    0    0    2   -3    1   -1    1   -1  }

{   17    3    2    1  -13   10   10   -3    0    0   -9    4    0   -3   -3    4  }

{   35  -10    3   -2   11   39  -18  -11    1   -2    4    1  -14    3    6    1  }

{   -4    8   -1    1   17  -12   -3    4   -3    1    3  -15    6    3    2   -4  }

{  -32  -20    1   -4   38  -24  -57   12    4    2   -2  -35    5   25    1   -7  }

{  -26    6    4    1   -8   26   -6    8    0    0    7   -3  -15    4    3   -6  }

{  -47   13    5    2  -10  -59   27   42   -4    6  -14   14   52  -10    2   12  }

{   10   36   -4    4   76    4   10    1  -16    1   15  -43    4   -4   -2   -5  }

{   16   -7   -6   -2  -38   10  -11   -8    3    0   -2    6   -8    9    2   -3  }

{   -2    8   -6   -1   53   11   46   -9  -21   -1   13   31  -11  -59    3  -10  }

{  -17  -41   13    0   31    8  -10   14   23   -1   37   -8  -11   15   -3  -18  }

{  -64   15   17    0   -7   34   -5   30    4   -1   24   31  -34    3    9  -24  }

{  -13   16   11    1   -6  -26   18  -28  -14   -4  -58  -31  -53  -15  -15   -3  }

{   15  -28   -3    2   40  -41  -13  -22   31    0  -34   38   -9    8    4    8  }

{   -8   28   -5   -2  -18   16   17    6  -18    3   10  -33   -1  -11  -17   -8  }

{   27  -16   43    3    9   30   16   15   12  -10  -21   -7   23   -4  -13   34  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol32to35[ m - 32 ][ n ] with m = 32..35, n = 32..35

lowFreqTransMatrixCol32to35 =

{

{    0    0    1    1  }

{   -1    0   -4   -2  }

{   -4    3    1    3  }

{    5    0   -1    1  }

{   14    4   -3    0  }

{   -2    6    2    1  }

{    2  -14   -2    2  }

{   24   -2   -2   -1  }

{    2    6    3    0  }

{  -41   -7   -2    1  }

{    8    8    1    0  }

{  -28   25    4    2  }

{   17   63   -1   -1  }

{  -41   21   -4    5  }

{   25    0    0   -4  }

{   10  -26   -5   -3  }

}

Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 1, and lfnstIdx is equal to 2 the following applies:

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15

lowFreqTransMatrixCol0to15 =

{

{   81  -50    5   -4    1   -1  -72   34    6   -1    2    0   22    5  -14    2  }

{   67    7  -17    0   -3    0   23  -62   21   -1    3   -1  -58   46    6   -8  }

{   50  -25   16   -4    1   -1   56    4  -29    2   -4    1   -7  -57   33    1  }

{  -39  -46   31   -7    5   -2  -32   21  -10    2    1    1  -53   24   20   -3  }

{   -9  -62   19    3    3    0   54   31    5  -17    2   -4  -37    8  -43   21  }

{  -23  -14  -13    8   -2    2  -41  -33   32  -11    5   -2  -38   -1    7    6  }

{   18   55    5    2   -2    1  -31   28  -42    3   -3    1  -49  -35   16   20  }

{   14   28  -24   19   -6    4   13   31   21  -23    3   -5   10   20  -31   15  }

{   -9  -33   21    2   -5    1   -7  -52  -20   -4   10   -1   30    0   27   23  }

{   -8  -13  -38   16   -3    3  -21  -26    7   -5   -2    0  -33  -60  -23   12  }

{    0  -11   16   -3    7   -2   -5  -47   -2    0   -7    1   39   -2    6   19  }

{   -9  -36  -68   16   -5    4    9   18   37   16   -9    4   16  -28   33  -36  }

{    5   13    5   17   -7    4    8   19   40  -17    7   -3   11   18   18   13  }

{   -3  -12  -25    0  -11    3    6    9  -15   31    4    4  -14    5   31   -6  }

{   -3   -8  -24   12  -12    4   -5  -13  -15  -30   15   -6   -2  -12  -29   13  }

{   -2    5   -6  -16    7   -4    3   12   -4   20  -10    4  -12   25   19  -23  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol16to31[ m - 16 ][ n ] with m = 16..31, n = 16..31

lowFreqTransMatrixCol16to31 =

{

{   -2    1   -4  -11    5    2   -1    0    2    5    1   -3   -2   -2   -2    1  }

{    3   -2   20   -1  -22    7   -2    1   -7   -8   11    1    3    4   -3   -3  }

{    2    0  -35   48    4  -15    4   -3   10   -5  -23   10   -5   -2   11    1  }

{    1   -1   18   43  -39   -1   -2    0   17  -40    1   17   -3   10   17  -11  }

{   -3    3   27  -30   37    3   -7    2  -17   16   -2  -20    2    0   -8   10  }

{    4    2  -31   35   38  -18    3   -3   -4   48  -50   -9   11  -21   -7   24  }

{   -4    5   55   -3   26  -19    0   -2  -28    2   -7   -8    6   12   -9    7  }

{   -5    2   24   13   -3  -19   -2   -2   16  -27  -49   29    8  -35   55    4  }

{  -16    5   25  -33    4  -41   16   -4  -42    9   -6   41   18    3   30  -18  }

{    6    2  -19  -21   24   26   -5    5   11  -30   37    4  -14   14   43  -21  }

{    7    2   53   24   32    2  -26    1   38  -36  -12  -49  -27    2   -4   22  }

{    9   -4   47   12  -15   -3    8   -5  -34   -5   -6   -3    4   13  -10   13  }

{   -5    0    3   32   30  -18   -3   -3    6   20   18  -13   -3   43   -8  -53  }

{  -19    4   25  -27   17   -9   22   -1   74   31    7    8  -12  -46   -7  -31  }

{   -7    3   -7   -9  -20  -22   12   -1    0  -30  -28    6  -34   -3  -32    4  }

{   -1    0  -24  -41    9  -33    8   -4   -5   -7  -24   -6  -68   38   13   36  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol32to35[ m - 32 ][ n ] with m = 32..35, n = 32..35

lowFreqTransMatrixCol32to35 =

{

{    1    1    0    0  }

{   -4   -1    2    1  }

{    1    1   -4   -2  }

{    3   -1  -11    0  }

{   -2    0    3   -3  }

{    2    6    8   -5  }

{    1   -8    1    2  }

{   -7   19  -10  -25  }

{  -11    7  -13   -7  }

{   -4   21  -17  -17  }

{    9    4    5   -8  }

{    1  -10    6   -1  }

{   -3   13  -66   16  }

{   -8   18    9   16  }

{   -3   -8  -17   86  }

{   16   33  -21  -21  }

}

Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 2, and lfnstIdx is equal to 1 the following applies:

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15

lowFreqTransMatrixCol0to15 =

{

{  119  -38   -2   -5   -1   -2    0   -1    6   -1   -1    0    0    0  -27    6  }

{   -3    4   -2    1   -1    0    0    0  119  -24   -8   -3   -2   -1   20   -1  }

{  -22   21   -6    3   -2    1    0    0   14    1   -2    0    0    0 -118   16  }

{   35   98  -36    9   -8    3   -3    1   14   22   -7    1   -1    0    3  -32  }

{   11   38  -19    4   -4    1   -1    1  -32   24   -7    2   -1    0   19   -7  }

{    6   17    0    0    0    0    0    0  -21 -102   33   -7    6   -1   -7  -53  }

{   -6    3   -2    0    0    0    0    0   -3   -8    6   -1    1    0  -23   17  }

{   11   36  -16    4   -3    1   -1    0  -10  -43    3   -2    2    0   18  100  }

{  -13  -40  -98   25  -13    6   -4    2   -6  -18  -49   10   -5    2    0   -5  }

{    4    4   11   -4    2   -1    1   -1    8   -2   11   -2    1   -1   -3   -2  }

{   -1   -6  -11    1   -1    0    0    0   -5  -30   22   -3    3   -1    3    0  }

{    5   14   47   -3    5   -1    2    0  -11  -31  -94   16   -9    4   -6  -18  }

{    2   -3   -9   -1    0    0    0    0    1   -1    3   -1    1    0    3   -5  }

{   -3  -11   -6    4   -2    1   -1    0    5   17   10   -3    1   -1   -6  -40  }

{   -3   -5  -29   -4   -3   -1   -1    0   -1   -2    4    3   -2    2   -7  -13  }

{    4   -4   30  102  -42   24   -9    7    0    1    5    0    3   -1   -1    1  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol16to31[ m - 16 ][ n ] with m = 16..31, n = 16..31

lowFreqTransMatrixCol16to31 =

{

{    1    1    0    0   -7    2    0    0    0    0   -2    1    0    0   -2    1  }

{   -2    0    0    0  -31    5    3    0    0    0  -13    2    1    0   -5    1  }

{    8    1    3    1  -25    0    4    0    1    0   17   -4   -1    0    6   -1  }

{    8   -2    1   -1   46  -14   -2   -1   -1    0    9    2    0    0   -7    0  }

{    5   -2    1   -1 -103    7   11    1    3    0  -40   -1    4    0    9   -2  }

{    9   -1    0    0  -10   27   -9    3   -1    0    2   18   -3    0    4    6  }

{   -2    1    0    0   34    7   -5    0   -1    0 -112    7    8    0  -40   -1  }

{  -18    3   -2    0    4   28    1    1    0    0   15  -33    5   -1    7   -5  }

{   19   -7    3   -1   11    6   16   -3    1   -1   -3    7    6   -1   17    2  }

{    2    1    0    0   20  -17   -1    1    0    0  -36  -17    2    0  110   -6  }

{   22   -3    3    0  -11  -98   16    0    1    0   -4  -61   -2    3  -21    4  }

{  -35    2   -3    0    4   -9   38   -6    4   -1   -5  -19    7   -1   -2   -2  }

{   -3    3   -1    0   -2    5   -5    0    0    0   10  -13    4   -2  -37   -8  }

{   -5    3   -1    1    5   57   -6    3   -1    0   -3  -95   16   -5   -5  -34  }

{  -95   18   -9    3  -11  -28  -62   -1   -3   -1    2    2   25   -7    5    7  }

{  -12  -42   17   -9   -2   -4  -15    6   -4    2   -1   -1   -4    4    1    4  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol32to35[ m - 32 ][ n ] with m = 32..35, n = 32..35

lowFreqTransMatrixCol32to35 =

{

{    0    0   -2   -1  }

{    0    0   -1   -2  }

{   -1    0    3    2  }

{    2    0   -4    0  }

{   -1    0    4    0  }

{   -1    0   -1    0  }

{    5    0   -5    4  }

{    0    0   -2   -2  }

{   -1    0    1   -2  }

{  -10    0   39    7  }

{   -5    0  -10   -2  }

{    2    0    3    2  }

{    4   -1   99   68  }

{   -4    0  -17   -9  }

{   19   -1   -4   -5  }

{    2   -1    2    3  }

}

Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 2, and lfnstIdx is equal to 2 the following applies:

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15

lowFreqTransMatrixCol0to15 =

{

{   92  -44    4   -4    1   -1  -68   25    3    1    1    1   24   -1   -7    1  }

{   70    3  -11    1   -3    0   49  -43    9   -3    1   -1  -71   29    4   -1  }

{   22  -48   25   -7    5   -2   75   28  -25    3   -5    0    5  -39   12    0  }

{   37   70  -32    7   -6    2    0  -48   15   -1    2    0   39  -18    7   -5  }

{   14  -13   -7    2    0    1   53  -13   15   -7    2   -1   71   21  -27    3  }

{   21   39   -9    2   -3    1   10   32  -26    8   -3    1   13  -67   32   -6  }

{    1   14    4    1    0    0  -18   10   -8   -1   -1    0  -59  -19   -5    4  }

{   -4  -11  -76   40  -13    8   13   50   54  -24    2   -3  -17  -25  -26    2  }

{  -21  -65  -20   10   -3    2  -12  -49   32   -9    4   -1    4  -14   19   -9  }

{   -5  -19  -47   -1  -10    3   -2    0  -17   30   -3    4    3    9   71  -34  }

{   -2  -10    6   -1    2    0   -3  -28    0   -3    0   -1   11  -18    0    5  }

{   -5  -16   12    3    1   -1  -12  -51    3  -10    2   -2   -5  -45    1    7  }

{    0    1  -18   10   -6    3    3   15  -16   -1    1   -1    7   44   19    8  }

{    4    3   33   78  -39   16   -3   -3  -33  -61   22   -7    5    9   24    7  }

{    1   -1  -19   -1    2   -1    1    3  -14    6   -3    1   -1   20   -1   10  }

{   -5  -20  -53    8   -9    1   -6  -23  -68    6   -5    2    2   -2    0   32  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol16to31[ m - 16 ][ n ] with m = 16..31, n = 16..31

lowFreqTransMatrixCol16to31 =

{

{   -1    0   -4   -5    3    0    0    0    1    2   -1    0   -1   -1    0    0  }

{    2    0   33   -2   -9    3   -1    0   -8   -6    4    1    1    3   -1    0  }

{    1    0  -47   26    5   -4    2   -1   30   -3   -9    4  -13   -1    6   -1  }

{    1   -2  -56   26   -2    1    1    0   25   -7   -4    1   -7   -1    2    0  }

{   -3    1    1  -48   15    3    0    0  -44   33    7   -6   26   -6  -12    3  }

{    2    0   51   23   -9   -7    2   -2  -54   -1   -1    6   25    2    1   -2  }

{   -1    1  -65  -18   26   -2    4    0  -28   52   -6   -2   48  -15  -11    4  }

{    6   -2    9    0   14    7   -6    2   17    5   -6   -8  -14   -5    2    5  }

{    6   -3  -17   51  -25    4   -2    0  -39  -18    5    5   36    7    0   -2  }

{    4   -5   -5  -41  -41    3    7   -1   19   44    9   11  -12   -8   -6   -6  }

{   -1    2   47    9   17   -8    2   -2   73   33  -30    6   59  -30    9    3  }

{    1    1   15   -6   33   -9    1   -2  -27   45  -28   -1  -70  -14   11    5  }

{   -2    2  -12    7    6  -25    5   -4  -28  -17  -53   23  -10  -43   52   -1  }

{   10   -6   -3   -8  -14   19  -15    5    5    4    4  -14    7    1    1    3  }

{   -3    2   -3   12   17  -14    3   -2   -9   -4  -22    8   43    1   12    3  }

{   -7    5    2  -11   50  -19    2   -1    6  -27    4  -12   -4   39  -17    5  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol32to35[ m - 32 ][ n ] with m = 32..35, n = 32..35

lowFreqTransMatrixCol32to35 =

{

{    1    1    0    0  }

{   -3    0    0    0  }

{    5    1   -3    0  }

{    2    1    0   -1  }

{   -9   -3    5    1  }

{   -8   -5    1    0  }

{  -21   -4    7    0  }

{    3    3    0   -4  }

{  -13   -3    1   -1  }

{   11   -6    4    3  }

{  -22    9    1   -5  }

{   28   10   -3   -3  }

{  -34   42  -11  -12  }

{   11   -3   -3    0  }

{  108   -8  -14   -3  }

{  -26  -18    4    3  }

}

Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 3, and lfnstIdx is equal to 1 the following applies:

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15

lowFreqTransMatrixCol0to15 =

{

{ -116   32   12    1    3    1    1    0   11  -31    8    0    1    0   22   -7  }

{   10   50  -16    0   -3    0   -1    0  -92    9   37   -5    5   -1   19  -52  }

{  -33   13  -12    6   -3    2   -1    1    2   77  -16  -16    0   -3  -40   -2  }

{  -16  -81   20    8    3    1    2    1    0    4    6  -11    1   -1    6  -49  }

{   25   63  -19    0   -3   -1   -2   -1   80   -1   33  -12   -1   -2  -11  -33  }

{   22    8   53  -14    2   -3    2   -2    5  -49   18   -9    2   -1   32    2  }

{   -9  -37   -1  -25    6   -4    2   -1   17    9   75   -4   -7   -2  -25  -31  }

{    0   16   42   -7    4   -2    2   -1  -19  -36   -1   27   -2    5  -96   -3  }

{  -10   -9   -4   -3    1   -3    1   -1  -21   -5   34  -25   -6   -1  -21   48  }

{   -8  -13  -49   -5    0   -1   -1    0    1   13   27   34   -6    3   14   21  }

{    4   17   84  -18    5   -3    2   -1    7   68   14   34   -9    4   32    9  }

{    1    2   -6  -23    4   -4    1   -2   -9   -3   32  -16  -15   -1   10   65  }

{    9   -2   -6   29   11    4    2    1    7  -15  -13  -13  -11   -2   28   -6  }

{   -2   10   14   29   17    5    2    2  -12    2   -8  -23  -19   -4   -9   19  }

{    4    7   17   56   -4    4    0    2    1    1  -19  -13    3    2   -2    4  }

{    2   -4   15   82  -12    8   -2    3    4   11   54   -5   45    1   -6   21  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol16to31[ m - 16 ][ n ] with m = 16..31, n = 16..31

lowFreqTransMatrixCol16to31 =

{

{   -9    2   -1    0    3    4   -2   -1    0    0    4    0    1   -1    1    1  }

{    5   10    0    2    9    3  -17    2    0    0    3   -2    1   -3    3    1  }

{   64  -10    1   -2   14  -44    7   20   -3    4   -1    0  -10    4    3   -6  }

{   17   28   -5    4   11    4  -62   11   11    0   -5   23   -1  -27    3   -1  }

{    8   30   -3    4   -4    7  -36    3   10    0   -8    9    5  -15    0   -1  }

{   50   -7  -14    1   -5  -40   12   51   -7    7    0    0  -40   13   -4    3  }

{   -8    5    4    4   15    5   40  -10  -23    0   -1  -26    4   53    0    3  }

{  -22    8   11    1   18  -29  -15  -12    7   -1    2    8  -10  -12    4   -3  }

{   21   48  -13   -2  -70   -1   -2    2   35   -3   11  -32   20  -19   -2   -7  }

{  -32   21   15   -2   -4  -37   -9   13   16    5    1    5  -69  -12   -6   20  }

{  -24   17   13   -1  -10    1   -4  -15   10    1    5   -5    4   -7   -1   -1  }

{   14   23   -5   -5   80    9   -6   -3    7    0   -1   42   19   -2   -3   -6  }

{    0   23   25   -1   12  -73   -9  -56   -1    5   49   -7   20   18   -4   -2  }

{    8   24   18    7  -32   20  -26  -14  -38    9  -33   46  -11   54  -22   -1  }

{   -6   37  -15   -6   50   27   12   18   37   -5   -9  -51  -17    9    9   12  }

{  -18  -37  -15   15   -3   -8   -3    4  -17   -8   16   17   -1  -11   -5    3  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol32to35[ m - 32 ][ n ] with m = 32..35, n = 32..35

lowFreqTransMatrixCol32to35 =

{

{    0    1    2    0  }

{   -2    1    1    1  }

{   -1    2    0    1  }

{    3    2   -2    1  }

{    1    2   -1    0  }

{   -5   -2    1   -1  }

{  -25    0   -1    1  }

{    4   -5    0    1  }

{   -6   16    2   -2  }

{    5  -47    4   -1  }

{    2    0    1    0  }

{   18   13   -2    0  }

{  -22   15    8   -2  }

{  -20  -40    3   -5  }

{  -42  -25   -2    4  }

{   18   10    2   -2  }

}

Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 3, and lfnstIdx is equal to 2 the following applies:

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..15

lowFreqTransMatrixCol0to15 =

{

{  115  -29   -7   -3   -2   -1  -33   27    0    1   -1    0  -23   -1    5    0  }

{   20  -28    7   -2    2   -1  101   -7  -19    0   -4    0  -29   52   -5   -2  }

{  -26   10    1    2   -1    0   -1   40   -7   -1   -2    0  -82   -4   36   -2  }

{   -9  -71   18   -2    4   -1   24  -11   -9    1   -1    1    6  -36    0    5  }

{   30   88  -22    3   -5    1   48    0   11   -5    2   -1  -13  -36    3    5  }

{  -28  -17   -8    4   -1    1   -5   73  -12   -3   -2   -1  -22    0    5    0  }

{    6  -19    3    2    1    1   36    0   41   -7    4   -2    2  -70   11   16  }

{    5    3   -6    3   -3    1   25   73  -15   -6   -1   -2   79   -1   40   -6  }

{   13   15   13   -3    2   -1   10  -10  -13    2   -2    1   25   21    5  -21  }

{    9    5   40   -4    4   -1   -7  -31   -4    1    2   -1   13    2   38    2  }

{    2   -4    5   -4    0   -1    8   18   19   -7    0   -1   10   -5  -14    7  }

{   -6  -29  -82   -2   -7    0    4  -25   53  -24    4   -5    5   15   39    2  }

{    3    2   68  -25    9   -6    7   16   56   10   -2    2   -2  -14  -10   -1  }

{   -1   11   27   -1    2   -2   -6   13   54  -13    2   -1    3   65   17   50  }

{    7    5   27   19   -1    3   -1  -23  -43  -31    5   -5   -2   -7   53   -8  }

{    6    2  -18    0    0    0    9   11   -8   27   -2    4   20   14  -51   16  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol16to31[ m - 16 ][ n ] with m = 16..31, n = 16..31

lowFreqTransMatrixCol16to31 =

{

{    1    0    2   -2   -1    0    0    0   -4    2    2   -1   -1    0    0    0  }

{   -1    0  -28    0   16   -2    2   -1   -2   -7    3    3   -4   -1    1    1  }

{    6   -1   28  -64    1   10    1    3   17    6  -26    1    2    6   -1   -6  }

{   -1    1   52    6  -46    3   -5    1  -15   55   -3  -17   -5   -1   20   -3  }

{    1    1   16   16  -29   -2   -1   -1  -17   29   15  -14   -2  -11   19    3  }

{    1    0    1   45    0  -15    1   -3  -29  -13   59   -2   15  -41   -5   23  }

{   -1    3   -3   -3   -4    4   -1    1    7  -50   -3   24   10    8  -58    7  }

{    1   -1  -14  -25    1   20   -1    4   -3    8  -24   -1   -7   10    7   -8  }

{    2   -4   92   -8   49   -5   -9    0   -4   -4   15   37   -8  -10  -21   20  }

{    0    0  -33  -45  -17   15    2    3    1   13   21  -11    5  -56    0   38  }

{   -3    2    1  -11   -3  -34    2   -7   94   12   43   -5  -17   23   18   39  }

{   -1    1   12  -11   16    9   -2    1   -6   -9    8   -3    9  -18   25    5  }

{    3    0    0   -4   34   -1   -5    1  -22  -15   -4    2   17  -11   41  -14  }

{   -9    6   26   18  -41   17   21    0   -5   -5   -2  -16  -19   11  -20    1  }

{  -12   -2   18   30   -6   18   12    1   20  -50   12  -34   33   18   16    2  }

{    8    1   29  -34  -26  -15   16   -1   11  -41  -12  -41   58  -19    4  -14  }

}

lowFreqTransMatrix[ m ][ n ] = lowFreqTransMatrixCol32to35[ m - 32 ][ n ] with m = 32..35, n = 32..35

lowFreqTransMatrixCol32to35 =

{

{   -3    0    0    0  }

{   -2    0    0    0  }

{    1    1    1    0  }

{   -2    2    3    4  }

{   -1   -4   -1    6  }

{   -2    4  -14   -1  }

{   -7   19   -7  -21  }

{   -1   -3    7    2  }

{   -4    6  -16   -6  }

{    7   13  -54   -6  }

{   -6  -14    1   12  }

{    7  -14   -7   31  }

{    8  -27    9   47  }

{   -6    7    8  -23  }

{   -6   -6   24   24  }

{   10   21  -20   11  }

}