| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0620 |
| --- | --- |

| Title: | Non-CE6: Simplification with new LFNST transform basis |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tianyang Zhou Tomonori Hashimoto Tomohiro Ikai | Tel: Email: | +81-50-5213-6338 zhou.tianyang@sharp.co.jp tomonori.hashimoto@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The Low-frequency Non-Separable Transform (LFNST) has been adopted in VVC-5. However, in the current design, LFNST requires 8KB table sizes for storing transform matrices. This contribution proposes the reduce LFNST matrices and LFNST computation by replacing 16x48 matrices with new 16x36 matrices. The scan order is switched by lfnstIdx to keep coding efficiency. It is reported that the proposed method achieves about 19 % table size reduction with 0.03 % loss in CTC IO condition.

Introduction

LFNST [1] have been adopted in VVC-5. In the current design, there are eight 16x16 matrices and eight 16x48 matrices. 16x16 and 16x48 matrices are employed in 4x4 blocks and over 4x4 blocks, respectively, and these matrices are selected based on intra prediction modes and signaled LFNST indices (lfnstIdx). As a result, the size of LFNST matrices is 8KByte (16x16x8 + 16x48x8 = 8192) in total.

This contribution proposes the table size reduction of LFNST and computation reduction.

| Size | # Transform basis | Sum |
| --- | --- | --- |
| 16x16 | 8 | 2048 |
| 16x48 | 8 | 6144 |
|  |  | 8192 |

Proposed methods

This contribution proposes to replace 16x48 matrices with smaller 16x36 matrices to achieve table size and computation reduction. The LFNST matrices has been retrained using CTC sequences. The table size of all LFNST matrices becomes 6656Byte (16x16x8 + 16x36x8 = 6656). In other words, the proposal saves 18.75% table size for LFNST matrices.

| Size | # Transform basis | Sum |
| --- | --- | --- |
| 16x16 | 8 | 2048 |
| 16x36 | 8 | 4608 |
|  |  | 6656 |

In VVC 5, LFNST derives 48 transform coefficients from 16 encoded coefficients using 16x48 matrices. Then the derived 48 coefficients is allocated in transform unit. The corresponding position is shown in Figure 1.

Figure 1: 48 coefficients pattern in VVC5

We propose to use 16x36 matrices. The corresponding position of 36 coefficients are decided depending on lfnstIdx. Specifically, if lfnstIdx is equal to 1, the corresponding 36 coefficients as shown in Figure 2 (Type 1) is used while if lfnstIdx is equal to 2, the corresponding 36 coefficients as shown in Figure 3 (Type 2) is used.

8 6 6 6 4 4 1 1 {8, 6, 6, 6, 4, 4, 1, 1} 8 6 6 6 4 4 1 1 {8, 6, 6, 6, 4, 4, 1, 1} Type 1

Figure 2: 36 coefficients pattern (proposal, lfnstIdx == 1)

6 6 6 6 4 4 4 0 {6, 6, 6, 6, 4, 4, 4, 0} 6 6 6 6 4 4 4 0 {6, 6, 6, 6, 4, 4, 4, 0} Type 2

Figure 3: 36 coefficients pattern (proposal, lfnstIdx == 2)

In this proposal, the scan order is the usual raster scan in nLfnstSize x nLfnstSize block but the valid area is defined with two set of table of elementNum[][], which defines which area is valid or not. Specifically it “x < elementNum[stIdx-1][y]”, it’s valid area thus v[cnt] is copied to the block, otherwise 0 is used. The pseudo code is shown below.

elementNum[2][8] = { {8, 6, 6, 6, 4, 4, 1, 1}, {6, 6, 6, 6, 4, 4, 4, 0} }
　if (PredModeIntra <= (34 or INTRA_LT_CCLM or INTRA_T_CCLM or INTRA_L_CCLM))
　{
　　cnt = 0
　　for (y = 0; y < nLfnstSize; y++)
　　　for (x = 0; x < nLfnstSize; x++)
　　　　if (x > = 4 && y > = 4) {
         d[(xSbIdx<<log2StSize)+x][(ySbIdx<<log2StSize)+y] = d[(xSbIdx<<log2StSize)+x][(ySbIdx<<log2StSize)+y]
　　　　} else {
　　　　　if (x < elementNum[lfnstIdx - 1][y]) 
　　　　　　d[(xSbIdx<<log2LfnstSize)+x][(ySbIdx<<log2LfnstSize)+y] = v[cnt++]
　　　　　else 
　　　　　　d[(xSbIdx<<log2LfnstSize)+x][(ySbIdx<<log2LfnstSize)+y] = 0
　}
  else
  {
　　cnt = 0
　　for (x = 0; x < nLfnstSize ; x++)
　　　for (y = 0; y < nLfnstSize ; y++)
　　　　if (x > = 4 && y > = 4) {
　　　　　d[(xSbIdx<<log2StSize)+x][(ySbIdx<<log2StSize)+y] = d[(xSbIdx<<log2StSize)+x][(ySbIdx<<log2StSize)+y]
　　　　} else {
　　　　　if (y < elementNum[lfnstIdx - 1][x]) 
　　　　　　d[(xSbIdx<<log2LfnstSize)+x][(ySbIdx<<log2LfnstSize)+y] = v[cnt++]
　　　　　else 
　　　　　　d[(xSbIdx<<log2LfnstSize)+x][(ySbIdx<<log2LfnstSize)+y] = 0
　}

Experimental Results

The simulations were conducted in the condition of ‘All Intra’, ‘Random Access’ defined in JVET-N1010 [2]. Table 1 shows the results (CTC). Table 2 shows Low QP (2, 7, 12, and 17) results

Table 1. Result of proposed LFNST (CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (Sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.06% | 0.08% | 101% | 100% |
| Class A2 | 0.04% | -0.04% | 0.00% | 100% | 100% |
| Class B | 0.04% | 0.00% | 0.08% | 101% | 100% |
| Class C | -0.01% | -0.08% | 0.13% | 101% | 100% |
| Class E | 0.01% | 0.14% | 0.09% | 100% | 100% |
| Overall | 0.03% | 0.01% | 0.08% | 101% | 100% |
| Class D | 0.01% | 0.25% | -0.06% | 101% | 100% |
| Class F | -0.02% | 0.00% | -0.06% | 100% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (Sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.09% | 0.11% | 100% | 100% |
| Class A2 | 0.04% | 0.06% | 0.06% | 100% | 100% |
| Class B | 0.03% | 0.06% | -0.06% | 100% | 100% |
| Class C | -0.03% | 0.10% | 0.04% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.04% | 0.02% | 100% | 100% |
| Class D | 0.04% | 0.07% | -0.12% | 100% | 101% |
| Class F | 0.03% | -0.06% | 0.19% | 100% | 101% |

Table 2. Result of proposed LFNST(Low QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (Sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.01% | -0.02% | -0.01% | 100% | 100% |
| Class C | 0.00% | 0.01% | 0.02% | 100% | 100% |
| Class E | 0.01% | 0.00% | 0.02% | 99% | 100% |
| Overall | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class D | -0.01% | -0.02% | -0.03% | 99% | 100% |
| Class F | 0.00% | -0.01% | 0.03% | 100% | 100% |

Conclusion

In this contribution, we propose a method of LFNST table size reduction with new 16x36 transform basis. The proposed method achieves about 19 % table size reductions with 0.03 % loss in CTC IO condition. It is recommended to adopt this method in the next VVC WD and VTM software.

Reference

[1]	M. Koo, M. Salehifar, J. Lim, S. Kim, “CE6: Reduced Secondary Transform (RST) (CE6-3.1),” JVET-N0193, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

[2]	F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).