| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0622-v2 |
| --- | --- |

| Title: | AHG2: Draft text to update BT.2100 references and ICTCP HLG coding equations |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Sean McCarthy, Fangjun Pu, Taoran Lu, Peng Yin, Walt Husak, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 940850 | Tel: Email: | {Sean.McCarthy,pyin}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the current VVC draft specification:

- replace all occurences of ITU-R BT.2100-1 with ITU-R BT.2100-2; and,

- add text and equations to support ICTCP HLG.

# Introduction

ITU-R BT.2100-2 (07/2018) [1] supersedes ITU-R BT.2100-1 (06/2017) [2].  In BT.2100-1, the ICTCP equations for derivation of colour difference signals for PQ and HLG are the same.  In BT-2100-2, the corresponding equations for ICTCP HLG are changed compared to BT.2100-1 but the ICTCP PQ equations are not changed.  Thus, the ICTCP equations for PQ and HLG are now different in BT.2100-2.

The current VVC draft specification [3] references BT.2100-1 rather than the currently in force BT.2100-2.  In addtion, VUI semantics in the currrent VVC draft specification provide incorrect equations for derivation of colour difference signals for ICTCP HLG.

# Proposal

It is proposed to replace ‘BT.2100-1’ with ‘BT.2100-2’ in the following:

- one place in Table E.2 – Colour primaries interpretation using the colour_primaries syntax element;

- two places in Table E.3 – Transfer characteristics interpretation using the transfer_characteristics syntax element; and,

- one place in the last sentence of chroma_sample_loc_type, chroma_sample_loc_type_top_field, and chroma_sample_loc_type_bottom_field before overscan_info_present_flag

It is additionally proposed to replace the following text and equations:

Otherwise (matrix_coeffs is equal to 14), the following equations apply:

E′Y = 0.5 * ( E′L + E′M )		(E-62)

E′PB = ( 6610 * E′L − 13613 * E′M + 7003 * E′S ) ÷ 4096	(E-63)

E′PR = ( 17933 * E′L − 17390 * E′M − 543 * E′S ) ÷ 4096	(E-64)

NOTE 11 – In this case, for purposes of the ICTCP nomenclature used in Table E.4, E′Y, E′PB, and E′PR of Equations E-62, E-63, and E-64 may be referred to as I, CT, and CP, respectively.

With the following:

Otherwise (matrix_coeffs is equal to 14), the following equations apply:

–	If transfer_characteristics is not equal to 18, the following equations apply:

E′Y = 0.5 * ( E′L + E′M )		(E-62)

E′PB = ( 6610 * E′L − 13613 * E′M + 7003 * E′S ) ÷ 4096	(E-63)

E′PR = ( 17933 * E′L − 17390 * E′M − 543 * E′S ) ÷ 4096	(E-64)

–	Otherwise, the following equations apply:

E′Y = 0.5 * ( E′L + E′M )		(E-65)

E′PB = ( 3625 * E′L − 7465 * E′M + 3840 * E′S ) ÷ 4096	(E-66)

E′PR = ( 9500 * E′L − 9212 * E′M − 288 * E′S ) ÷ 4096	(E-67)

NOTE 11 – In this case, for purposes of the ICTCP nomenclature used in Table E.4, E′Y, E′PB, and E′PR of Equations E-62, E-63, and E-64 or Equations E-65, E-66, and E-67 may be referred to as I, CT, and CP, respectively. Equations E-62 to E-64 were designed specifically for use with transfer_characteristics equal to 16 (PQ), and Equations E-65 to E-67 were designed specifically for use with transfer_characteristics equal to 18 (HLG).

Exisitng equations E-65 and above should be renumbered.

It is additionally proposed to update Table E.4 – Matrix coefficients interpretation using the matrix_coeffs syntax element as indicated by the yellow highlights below:

Table E.4 – Matrix coefficients interpretation using the matrix_coeffs syntax element

| Value | Matrix | Informative remark |
| --- | --- | --- |
| … | … | … |
| 9 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020-2 non-constant luminance system Rec. ITU-R BT.2100-2 Y′CbCr See Equations E-28 to E-30 |
| 10 | KR = 0.2627; KB = 0.0593 | Rec. ITU-R BT.2020-2 constant luminance system See Equations E-49 to E-58 |
| 11 | Y′D′ZD′X | SMPTE ST 2085 (2015) See Equations E-59 to E-61 |
| 12 | See Equations E-22 to E-27 | Chromaticity-derived non-constant luminance system See Equations E-28 to E-30 |
| 13 | See Equations E-22 to E-27 | Chromaticity-derived constant luminance system See Equations E-49 to E-58 |
| 14 | ICTCP | Rec. ITU-R BT.2100-2 ICTCP See Equations E-62 to E-64 for transfer_characteristics value 16 (PQ) See Equations E-65 to E-67 for transfer_characteristics value 18 (HLG) |
| 15..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

# References

- ITU-R BT.2100-2 (07/2018), “Image parameter values for high dynamic range television for use in production and international programme exchange”, July, 2018

- ITU-R BT.2100-1 (06/2017), “Image parameter values for high dynamic range television for use in production and international programme exchange”, June, 2017

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001-v9, Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).