| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0623-v1 |
| --- | --- |

| Title: | CE7-related: Interleaved coefficient coding for transform-skip mode |  |
| --- | --- | --- |
| Status: | Input document to JVET |  |
| Purpose: | Proposal |  |
| Author(s) or Contact(s): | Marta Karczewicz, Hongtao Wang, Muhammed Coban, Yung-Hsuan Chao (Qualcomm) | Tel: +1 858 658 5673 Email: martak@qti.qualcomm.com hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |

_____________________________

# Abstract

This contribution presents a modification to the coefficient coding of the transform-skip mode. Instead of encoding the abs_level_gtX_flags(X = 2,3,4,5) of a coding group (CG) as 4 separate bit-planes, the method described in this contribution encodes all coefficient components except abs_remainder in an interleaved way. Compared to VTM-5.0, the performance of the proposed method is summarized as follows:

- Standard QPs:

  - Overall all: TO BE UPDATED

  - Class F -0.02% (AI), -0.02% (RA) TO BE UPDATED (LB)

  - Class TGM 0.01%(AI), -0.04% (RA) TO BE UPDATED (LB)

- Low QPs:

  - Overall all: -0.01% (AI) TO BE UPDATED

  - Class F -0.15% (AI), TO BE UPDATED (RA) TO BE UPDATED (LB)

  - Class TGM -0.42%(AI), -1.13% (RA) TO BE UPDATED (LB)

# Introduction

## Transform-skip mode coefficient coding

In the current design of VVC (VTM-5.0), the method described in [1] was used to encode the quantized coefficients of transform-skip mode. According to the method, each coefficient may be separated into one or more components of different types, including sig_coeff_flag, coeff_sign_flag, abs_level_gt1_flag, par_level_flag, abs_level_gtX_flags(X = 2,3,4,5) and abs_remainder. For each coding group (CG), sig_coeff_flag, coeff_sign_flag, abs_level_gt1_flag and par_level_flag are coded coefficient by coefficient in an interleaved manner. And abs_level_gtX_flags of a CG are coded bit-plane by bit-plane. In the last pass, the abs_remainder part of each coefficient (if exists) is coded using Rice codes. Figure 1 shows how transform-skip coefficients are encoded by the current design of VVC (VTM-5.0).

Figure 1. TS Coefficient coding in VTM-5.0

# Technical description

In this contribution, instead of encoding abs_level_gtX_flags (X = 2,3,4,5) in a bit-plane by bit-plane manner, it is proposed to encode all the components except abs_remainder in an interleaved manner in the first pass. And the encoding of abs_remainder is unchanged. Figure 2 illustrates the proposed method of encoding transform-skip coefficients.

Figure 2. Proposed coefficient coding scheme

# Simulation results

## Standard QPs (22, 27, 32, 37)

Simulation results as shown in Table 1, the anchor is VTM-5.0 with the default setting, the tested method is the interleaved coding scheme proposed in this contribution. The configuration of screen content coding tools for class F in the common test condition was also applied to class TGM.

Table 1 Results of the proposed method vs. VTM-5.0, standard QPs

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | 0.01% | -0.01% | 100% | 102% |
| Class C | 0.01% | -0.04% | 0.04% | 100% | 100% |
| Class E | 0.01% | 0.06% | -0.04% | 101% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.15% | -0.05% | 99% | 100% |
| Class F | -0.02% | 0.06% | 0.01% | 101% | 99% |
| TGM | 0.01% | 0.13% | 0.10% | 100% | 101% |
|  | Random Access |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | 0.03% | 0.05% | 100% | 100% |
| Class C | -0.06% | -0.15% | 0.03% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | 0.19% | 0.03% | 101% | 100% |
| Class F | -0.02% | 0.09% | 0.05% | 101% | 101% |
| TGM | -0.04% | 0.06% | -0.02% | 99% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.12% | -0.31% | 98% | 99% |
| Class E | 0.03% | 0.35% | 0.23% | 99% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.05% | -0.34% | 0.27% | 100% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Low QPs (2, 7, 12, 17)

The low QP results of the proposed method is shown in Table 2 Results of the proposed method vs. VTM-5.0 Standard QPs. The low QP test condition described in [2] was used for the simulation. QP was set to 2, 7, 12, 17 and only the first 100 frames of each sequence were encoded.

Table 2 Results of the proposed method vs. VTM-5.0 low QPs

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class B | 0.01% | -0.01% | 0.00% | 102% | 100% |
| Class C | -0.01% | 0.02% | 0.03% | 102% | 98% |
| Class E | 0.00% | -0.01% | -0.01% | 102% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class D | -0.01% | 0.03% | 0.00% | 103% | 99% |
| Class F | -0.15% | -0.12% | -0.11% | 102% | 98% |
| TGM | -0.42% | -0.21% | -0.24% | 102% | 97% |
|  | Random Access |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | -0.03% | -0.01% | 105% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | 0.05% | -0.04% | 104% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | -1.13% | -0.44% | -0.43% | 102% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.02% | 0.00% | -0.02% | 104% | 99% |
| Class E | -0.02% | 0.02% | -0.05% | 104% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.05% | 0.07% | 103% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Reference

- B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, “Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip”, JVET document JVET-M0464, Marrackech, MA, Jan 2019.

- H. Schwarz, M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding”, JVET-N1027, 14th meeting, Geneva, CH, March 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).