| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0626-v2 |
| --- | --- |

| Title: | Non-CE8: Simplified IBC BV candidate list construction process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang, Kai Zhang, Hongbin Liu, Jizheng Xu, Zhipin Deng, Na Zhang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1(858)999-7093 lizhang.idm@bytedance.com zhangkai.video@bytedance.com liuhongbin.01@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, two methods are provided which are asserted to further simplify the IBC BV candidate list construction process. In the first method, the derivation of spatial merge candidates is skipped for coding units under a shared parent node. In the second method, the shared merge list is totally removed and the derivation of spatial merge candidates for 4×4 blocks is skipped. In both methods, the pruning operations among BV candidates and derivation of spatial neighboring blocks are totally removed. It is asserted that both proposed methods could be benefit for the MV throughput. Simulation results reportedly show that the first method has {0.04%, 0.06%, -0.11%} and {0.12%, 0.07%, 0.07%} BD-rate increase in {AI, RA, LDB} configurations for class F and class TGM, respectively; the second method has {0.04%, 0.04%, -0.14%} and {0.07%, -0.02%, 0.02%} BD-rate increase in {AI, RA, LDB} configurations for class F and class TGM, respectively.

# Introduction

The BV predictors for merge mode and AMVP mode in IBC will share a common BV candidate list, which may consist of the following elements:

- 2 spatial neighboring positions (A1, B1 as in Figure 1)

- 5 HMVP entries

- Zero vectors by default

Figure 1. Positions of spatial merge candidates.

Shared merge list was adopted in the 13rd JVET meeting [1] but has been removed from regular merge list in the last meeting. In the current design, shared merge candidates are still used for IBC BV candidate list. More specifically, when a shared ancestor node is met, a temporary HMVP table will be allocated and filled with all available HMVP candidates in the original HMVP table. In addition, all child nodes of the shared ancestor node use the temporary HMVP table to derive HMVP candidate and decoded motion information is used to update the original HMVP table. Therefore, two HMVP tables need to be maintained for coding small blocks. Such a design needs more complex logic to handle the HMVP tables.

# Proposed methods

Two methods are proposed to further simplify the IBC BV candidate list construction process.

- The derivation of spatial merge candidates is skipped for coding units under a shared parent node.

- The derivation of spatial merge candidates is skipped for 4×4 coding units and the shared merge list is removed.

In addition, the HMVP buffer is not updated for those small blocks, i.e., those belonging to a shared parent node or 4×4 block.

# Simulation results

The proposed methods have been implemented on top of VTM5.0 with the hash fix [2][3]. Simulations are performed following the common test conditions [4]. Two sets of simulations are tested, as follows:

- Test #1: Method #1.

- Test #2: Method #2.

The results of Test #1 and Test #2 are shown in Table 1 and Table 2, respectively.

Table 1: Coding performance of Test #1

|  |  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class F | 0.04% | 0.05% |  | 0.17% |  | 101% | 97% |
| Class TGM | 0.12% | 0.13% |  | 0.12% |  | 101% | 100% |
|  |  | Random access Main10 |  |  |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |  |
|  | Y | U |  |  | V | EncT | DecT |
| Class F | 0.06% | 0.02% |  |  | 0.08% | 100% | 99% |
| Class TGM | 0.07% | 0.13% |  |  | 0.12% | 100% | 100% |
|  |  | Low delay B Main10 |  |  |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |  |
|  | Y | U |  |  | V | EncT | DecT |
| Class F | -0.11% | -0.19% |  |  | -0.96% | 99% | 101% |
| Class TGM | 0.07% | 0.17% |  |  | 0.12% | 99% | 100% |

Table 2: Coding performance of Test #2

|  |  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class F | 0.04% | 0.02% |  | 0.02% |  | 100% | 100% |
| Class TGM | 0.07% | 0.09% |  | 0.09% |  | 101% | 100% |
|  |  | Random access Main10 |  |  |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |  |
|  | Y | U |  | V |  | EncT | DecT |
| Class F | 0.04% | 0.15% |  | 0.09% |  | 101% | 100% |
| Class TGM | -0.02% | 0.06% |  | -0.02% |  | 101% | 101% |
|  |  | Low delay B Main10 |  |  |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |  |
|  | Y | U |  |  | V | EncT | DecT |
| Class F | -0.14% | -0.14% |  |  | -0.50% | 99% | 102% |
| Class TGM | 0.02% | 0.07% |  |  | 0.09% | 100% | 99% |

# Conclusions

Two methods are provided to further simplify the IBC BV candidate list construction process. It is recommended adopting the second method to the next version of VVC.

# Proposed changes to working draft

The suggested specification changes on top of JVET-N1001_v7 is described as follows: deleted text is marked with strikethrough, and newly added text are highlighted in green and grey for the first aspect and second aspect, respectively.

## Method #1

8.6.2.2	Derivation process for IBC luma motion vector prediction

This process is only invoked when CuPredMode[ xCb ][ yCb ] is equal to MODE_IBC, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvL.

The variables xSmr, ySmr, smrWidth, smrHeight, and smrNumHmvpIbcCand are derived as follows:

xSmr = IsInSmr[ xCb ][ yCb ]  ?  SmrX[ xCb ][ yCb ]  :  xCb	(8-895)

ySmr = IsInSmr[ xCb ][ yCb ]  ?  SmrY[ xCb ][ yCb ]  :  yCb	(8-896)

smrWidth = IsInSmr[ xCb ][ yCb ]  ?  SmrW[ xCb ][ yCb ]  :  cbWidth	(8-897)

smrHeight = IsInSmr[ xCb ][ yCb ]  ?  SmrH[ xCb ][ yCb ]  :  cbHeight	(8-898)

smrNumHmvpIbcCand = IsInSmr[ xCb ][ yCb ]  ?  NumHmvpSmrIbcCand  :  NumHmvpIbcCand	(8-899)

The luma motion vector mvL is derived by the following ordered steps:

- When IsInSmr[ xCb ][ yCb ] is false, The derivation process for spatial motion vector candidates from neighboring coding units as specified in clause 8.6.2.3 is invoked with the luma coding block location ( xCb, yCb ) set equal to ( xSmr, ySmr ), the luma coding block width cbWidth, and the luma coding block height cbHeight set equal to smrWidth and smrHeight as inputs, and the outputs being the availability flags availableFlagA1, availableFlagB1 and the motion vectors mvA1 and mvB1.

- When IsInSmr[ xCb ][ yCb ] is false, The motion vector candidate list, mvCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mvCandList [ i++ ] = mvA1		(8-900)
if( availableFlagB1 )
	mvCandList [ i++ ] = mvB1

- When IsInSmr[ xCb ][ yCb ] is false, The variable numCurrCand is set equal to the number of merging candidates in the mvCandList.

- When numCurrCand is less than MaxNumMergeCand and smrNumHmvpIbcCand is greater than 0, the derivation process of IBC history-based motion vector candidates as specified in 8.6.2.4 is invoked with mvCandList, isInSmr set equal to IsInSmr[ xCb ][ yCb ], and numCurrCand as inputs, and modified mvCandList and numCurrCand as outputs.

- When numCurrCand is less than MaxNumMergeCand, the following applies until numCurrCand is equal to MaxNumMergeCand:

    - mvCandList[ numCurrCand ][ 0 ] is set equal to 0.

    - mvCandList[ numCurrCand ][ 1 ] is set equal to 0.

    - numCurrCand is increased by 1.

- The variable mvIdx is derived as follows:

mvIdx = general_merge_flag[ xCb ][ yCb ] ? merge_idx[ xCb ][ yCb ] : mvp_l0_flag[ xCb ][ yCb ]	(8-901)

- The following assignments are made:

mvL[ 0 ] = mergeCandList[ mvIdx ][ 0 ]		(8-902)

mvL[ 1 ] = mergeCandList[ mvIdx ][ 1 ]		(8-903)

8.6.2.4	Derivation process for IBC history-based motion vector candidates

Inputs to this process are:

- a motion vector candidate list mvCandList,

- a variable isInSmr specifying whether the current coding unit is inside a shared merging candidate region,

- the number of available motion vector candidates in the list numCurrCand.

Outputs to this process are:

- the modified motion vector candidate list mvCandList,

- the modified number of motion vector candidates in the list numCurrCand.

The variables isPrunedA1 and isPrunedB1 are set both equal to FALSE.

The array smrHmvpIbcCandList and the variable smrNumHmvpIbcCand are derived as follows:

smrHmvpIbcCandList = isInSmr  ?  HmvpSmrIbcCandList  :  HmvpIbcCandList	(8-906)

smrNumHmvpIbcCand = isInSmr  ?  NumHmvpSmrIbcCand  :  NumHmvpIbcCand	(8-907)

For each candidate in smrHmvpIbcCandList[ hMvpIdx ] with index hMvpIdx = 1.. smrNumHmvpIbcCand, the following ordered steps are repeated until numCurrCand is equal to MaxNumMergeCand:

- The variable sameMotion is derived as follows:

  - If isInSmr is false and all of the following conditions are true for any motion vector candidate N with N being A1 or B1, sameMotion and isPrunedN are both set equal to TRUE:

  - hMvpIdx is less than or equal to 1.

  - The candidate smrHmvpIbcCandList[smrNumHmvpIbcCand − hMvpIdx] is equal to the motion vector candidate N.

  - isPrunedN is equal to FALSE.

  - Otherwise, sameMotion is set equal to FALSE.

- When sameMotion is equal to FALSE, the candidate smrHmvpIbcCandList[smrNumHmvpIbcCand − hMvpIdx] is added to the motion vector candidate list as follows:

mvCandList[ numCurrCand++ ] = smrHmvpIbcCandList[ smrNumHmvpIbcCand − hMvpIdx ] (8-908)

8.6.2	Derivation process for motion vector components for IBC blocks

8.6.2.1	General

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vector in 1/16 fractional-sample accuracy mvL.

The luma motion vector mvL is derived as follows:

- The derivation process for IBC luma motion vector prediction as specified in clause 8.6.2.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight inputs, and the output being the luma motion vector mvL.

- When general_merge_flag[ xCb ][ yCb ] is equal to 0, the following applies:

- The variable mvd is derived as follows:

mvd[ 0 ] = MvdL0[ xCb ][ yCb ][ 0 ]		(8-883)

mvd[ 1 ] = MvdL0[ xCb ][ yCb ][ 1 ]		(8-884)

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to mvL, rightShift set equal to MvShift + 2, and leftShift set equal to MvShift + 2 as inputs and the rounded mvL as output.

- The luma motion vector mvL is modified as follows:

u[ 0 ] = ( mvL[ 0 ] + mvd[ 0 ] + 218 ) % 218	(8-885)

mvL[ 0 ] = ( u[ 0 ]  >=  217 ) ? ( u[ 0 ] − 218 ) : u[ 0 ]	(8-886)

u[ 1 ] = ( mvL[ 1 ] + mvd[ 1 ] + 218 ) % 218	(8-887)

mvL[ 1 ] = ( u[ 1 ]  >=  217 ) ? ( u[ 1 ] − 218 ) : u[ 1 ]	(8-888)

NOTE 1– The resulting values of mvL[ 0 ] and mvL[ 1 ] as specified above will always be in the range of −217 to 217 − 1, inclusive.

When IsInSmr[ xCb ][ yCb ] is false, The updating process for the history-based motion vector predictor list as specified in clause 8.6.2.6 is invoked with luma motion vector mvL.

The top-left location inside the reference block ( xRefTL, yRefTL ) and the bottom-right location inside the reference block ( xRefBR, yRefBR ) are derived as follows:

( xRefTL, yRefTL ) = ( xCb + ( mvL[ 0 ] >> 4 ), yCb + ( mvL[ 1 ] >> 4 ) ) 	(8-889)

( xRefBR, yRefBR ) = ( xRefTL + cbWidth − 1, yRefTL + cbHeight − 1 ) 	(8-890)

It is a requirement of bitstream conformance that the luma motion vector mvL shall obey the following constraints:

  - ...

## Method #2

7.4.8.2	Coding tree unit semantics

The CTU is the root node of the coding tree structure.

The array IsInSmr[ x ][ y ] specifying whether the sample at ( x, y ) is located inside a shared merging candidate list region, is initialized as follows for x = 0..CtbSizeY − 1 and y = 0..CtbSizeY − 1:

IsInSmr[ x ][ y ] = FALSE		(7-96)

7.4.8.4	Coding tree semantics

When all of the following conditions are true, IsInSmr[ x ][ y ] is set equal to TRUE for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

- IsInSmr[ x0 ][ y0 ] is equal to FALSE

- cbWidth * cbHeight / 4 is less than 32

- treeType is not equal to DUAL_TREE_CHROMA

When IsInSmr[ x0 ][ y0 ] is equal to TRUE. the arrays SmrX[ x ][ y ], SmrY[ x ][ y ], SmrW[ x ][ y ] and SmrH[ x ][ y ] are derived as follows for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

SmrX[ x ][ y ] = x0		(7-98)

SmrY[ x ][ y ] = y0		(7-99)

SmrW[ x ][ y ] = cbWidth		(7-100)

SmrH[ x ][ y ] = cbHeight		(7-101)

When all of the following conditions are true, IsInSmr[ x ][ y ] is set equal to TRUE for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

- IsInSmr[ x0 ][ y0 ] is equal to FALSE

- One of the following conditions is true:

- mtt_split_cu_binary_flag is equal to 1 and  cbWidth * cbHeight / 2 is less than 32

- mtt_split_cu_binary_flag is equal to 0 and  cbWidth * cbHeight / 4 is less than 32

- treeType is not equal to DUAL_TREE_CHROMA

When IsInSmr[ x0 ][ y0 ] is equal to TRUE. the arrays SmrX[ x ][ y ], SmrY[ x ][ y ], SmrW[ x ][ y ] and SmrH[ x ][ y ] are derived as follows for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

SmrX[ x ][ y ] = x0		(7-102)

SmrY[ x ][ y ] = y0		(7-103)

SmrW[ x ][ y ] = cbWidth		(7-104)

SmrH[ x ][ y ] = cbHeight		(7-105)

7.4.8.5	Coding unit semantics

When all the following conditions are true, the history-based motion vector predictor list for the shared merging candidate list region is updated by setting NumHmvpSmrIbcCand equal to NumHmvpIbcCand, and setting HmvpSmrIbcCandList[ i ] equal to HmvpIbcCandList[ i ] for i = 0..NumHmvpIbcCand − 1:

- IsInSmr[ x0 ][ y0 ] is equal to TRUE.

- SmrX[ x0 ][ y0 ]  is equal to x0.

- SmrY[ x0 ][ y0 ]  is equal to y0.

The following assignments are made for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1:

CbPosX[ x ][ y ] = x0		(7-106)

CbPosY[ x ][ y ] = y0		(7-107)

CbWidth[ x ][ y ] = cbWidth		(7-108)

CbHeight[ x ][ y ] = cbHeight		(7-109)

8.6.2	Derivation process for motion vector components for IBC blocks

8.6.2.1	General

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vector in 1/16 fractional-sample accuracy mvL.

The luma motion vector mvL is derived as follows:

- The derivation process for IBC luma motion vector prediction as specified in clause 8.6.2.2 is invoked with the luma location ( xCb, yCb ), the variables cbWidth and cbHeight inputs, and the output being the luma motion vector mvL.

- When general_merge_flag[ xCb ][ yCb ] is equal to 0, the following applies:

- The variable mvd is derived as follows:

mvd[ 0 ] = MvdL0[ xCb ][ yCb ][ 0 ]		(8-883)

mvd[ 1 ] = MvdL0[ xCb ][ yCb ][ 1 ]		(8-884)

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to mvL, rightShift set equal to MvShift + 2, and leftShift set equal to MvShift + 2 as inputs and the rounded mvL as output.

- The luma motion vector mvL is modified as follows:

u[ 0 ] = ( mvL[ 0 ] + mvd[ 0 ] + 218 ) % 218	(8-885)

mvL[ 0 ] = ( u[ 0 ]  >=  217 ) ? ( u[ 0 ] − 218 ) : u[ 0 ]	(8-886)

u[ 1 ] = ( mvL[ 1 ] + mvd[ 1 ] + 218 ) % 218	(8-887)

mvL[ 1 ] = ( u[ 1 ]  >=  217 ) ? ( u[ 1 ] − 218 ) : u[ 1 ]	(8-888)

NOTE 1– The resulting values of mvL[ 0 ] and mvL[ 1 ] as specified above will always be in the range of −217 to 217 − 1, inclusive.

When smrWidth  * smrHeight  is larger than K, The updating process for the history-based motion vector predictor list as specified in clause 8.6.2.6 is invoked with luma motion vector mvL.

The top-left location inside the reference block ( xRefTL, yRefTL ) and the bottom-right location inside the reference block ( xRefBR, yRefBR ) are derived as follows:

( xRefTL, yRefTL ) = ( xCb + ( mvL[ 0 ] >> 4 ), yCb + ( mvL[ 1 ] >> 4 ) ) 	(8-889)

( xRefBR, yRefBR ) = ( xRefTL + cbWidth − 1, yRefTL + cbHeight − 1 ) 	(8-890)

It is a requirement of bitstream conformance that the luma motion vector mvL shall obey the following constraints:

  - ...

8.6.2.2	Derivation process for IBC luma motion vector prediction

This process is only invoked when CuPredMode[ xCb ][ yCb ] is equal to MODE_IBC, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the luma motion vectors in 1/16 fractional-sample accuracy mvL.

The variables xSmr, ySmr, smrWidth, smrHeight, and smrNumHmvpIbcCand are derived as follows:

xSmr = IsInSmr[ xCb ][ yCb ]  ?  SmrX[ xCb ][ yCb ]  :  xCb	(8-895)

ySmr = IsInSmr[ xCb ][ yCb ]  ?  SmrY[ xCb ][ yCb ]  :  yCb	(8-896)

smrWidth = IsInSmr[ xCb ][ yCb ]  ?  SmrW[ xCb ][ yCb ]  :  cbWidth	(8-897)

smrHeight = IsInSmr[ xCb ][ yCb ]  ?  SmrH[ xCb ][ yCb ]  :  cbHeight	(8-898)

smrNumHmvpIbcCand = IsInSmr[ xCb ][ yCb ]  ?  NumHmvpSmrIbcCand  :  NumHmvpIbcCand	(8-899)

The varaible IslgrBlk is set to (cbWidth × cbHeight  is greater than 16 ? true: false).

The luma motion vector mvL is derived by the following ordered steps:

- When IslgrBlk is true, the The derivation process for spatial motion vector candidates from neighbouring coding units as specified in clause 8.6.2.3 is invoked with the luma coding block location ( xCb, yCb ) set equal to ( xSmr, ySmr ), the luma coding block width cbWidth, and the luma coding block height cbHeight set equal to smrWidth and smrHeight as inputs, and the outputs being the availability flags availableFlagA1, availableFlagB1 and the motion vectors mvA1 and mvB1.

- When IslgrBlk is true, the The motion vector candidate list, mvCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mvCandList [ i++ ] = mvA1		(8-900)
if( availableFlagB1 )
	mvCandList [ i++ ] = mvB1

- When IslgrBlk is true, the The variable numCurrCand is set equal to the number of merging candidates in the mvCandList.

- When numCurrCand is less than MaxNumMergeCand and smrNumHmvpIbcCand is greater than 0, the derivation process of IBC history-based motion vector candidates as specified in 8.6.2.4 is invoked with mvCandList, isInSmr set equal to IsInSmr[ xCb ][ yCb ], IslgrBlk and numCurrCand as inputs, and modified mvCandList and numCurrCand as outputs.

- When numCurrCand is less than MaxNumMergeCand, the following applies until numCurrCand is equal to MaxNumMergeCand:

    - mvCandList[ numCurrCand ][ 0 ] is set equal to 0.

    - mvCandList[ numCurrCand ][ 1 ] is set equal to 0.

    - numCurrCand is increased by 1.

- The variable mvIdx is derived as follows:

mvIdx = general_merge_flag[ xCb ][ yCb ] ? merge_idx[ xCb ][ yCb ] : mvp_l0_flag[ xCb ][ yCb ]	(8-901)

- The following assignments are made:

mvL[ 0 ] = mergeCandList[ mvIdx ][ 0 ]		(8-902)

mvL[ 1 ] = mergeCandList[ mvIdx ][ 1 ]		(8-903)

8.6.2.4	Derivation process for IBC history-based motion vector candidates

Inputs to this process are:

- a motion vector candidate list mvCandList,

- a variable isInSmr,

- a variable to indicate non-small block IslgrBlk

- the number of available motion vector candidates in the list numCurrCand.

Outputs to this process are:

- the modified motion vector candidate list mvCandList,

- a variable isInSmr specifying whether the current coding unit is inside a shared merging candidate region,

- the modified number of motion vector candidates in the list numCurrCand.

The variables isPrunedA1 and isPrunedB1 are set both equal to FALSE.

The array smrHmvpIbcCandList and the variable smrNumHmvpIbcCand are derived as follows:

smrHmvpIbcCandList = isInSmr  ?  HmvpSmrIbcCandList  :  HmvpIbcCandList	(8-906)

smrNumHmvpIbcCand = isInSmr  ?  NumHmvpSmrIbcCand  :  NumHmvpIbcCand	(8-907)

For each candidate in smrHmvpIbcCandList[ hMvpIdx ] with index hMvpIdx = 1.. smrNumHmvpIbcCand, the following ordered steps are repeated until numCurrCand is equal to MaxNumMergeCand:

- The variable sameMotion is derived as follows:

  - If IslgrBlk is true all of the following conditions are true for any motion vector candidate N with N being A1 or B1, sameMotion and isPrunedN are both set equal to TRUE:

  - hMvpIdx is less than or equal to 1.

  - The candidate smrHmvpIbcCandList[smrNumHmvpIbcCand − hMvpIdx] is equal to the motion vector candidate N.

  - isPrunedN is equal to FALSE.

  - Otherwise, sameMotion is set equal to FALSE.

- When sameMotion is equal to FALSE, the candidate smrHmvpIbcCandList[smrNumHmvpIbcCand − hMvpIdx] is added to the motion vector candidate list as follows:

mvCandList[ numCurrCand++ ] = smrHmvpIbcCandList[ smrNumHmvpIbcCand − hMvpIdx ] (8-908)

# Reference

- C.-C. Chen, Y.-C. Lin, M.-S. Chiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE4.3.1: Shared merging candidate list,” JVET-M0170, Marrakech, Morocco, Jan. 2019.

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/merge_requests/564

- JVET-N1010, “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).