| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0631-v1 |
| --- | --- |

| Title: | Non-CE4: DMVR on SMVD and AMVP mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Yan Ye, Jie Chen, Jiancong Luo Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | ruling.lrl@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to apply DMVR to SMVD mode and AMVP mode. A CU-level flag is signaled to indicate whether DMVR is applied. Besides, this flag of the neighboring block is inherited for a block predicted using regular merge mode or CIIP mode. It is reported that, under the CTC, the overall performance impact for {Y, U, V} is {0.12%, 0.10%, 0.12%} for RA with 112% encoding time and 98% decoding time.

Introduction

In VVC draft 5, DMVR is applied to a block predicted using regular merge mode or combined inter and intra prediction mode (CIIP). However, it does not apply to a block predicted using symmetric motion vector difference mode (SMVD) or advance motion vector prediction mode (AMVP). So the benefit of DMVR cannot be applied to these two modes.

In this contribution, it is proposed to apply DMVR to SMVD and AMVP modes. A CU-level flag is signaled to indicate whether DMVR is applied to a block predicted using SMVD or bi-prediction AMVP mode. In case of DMVR applied, the BCW index and two reference picture indices are no longer signaled. Instead, the BCW index is directly inferred to be equal weights. The two reference picture indices are derived at slice-level using similar process to that of SMVD. The reference picture derivation process is described as follows:

- The forward reference picture in reference picture list 0 which is nearest to current picture is searched.

- The backward reference picture in reference picture list 1 which is nearest to current picture is searched.

- If both forward and backward picture are found, following applies:

- The distances from both reference pictures to the current picture are the same, the DMVR on inter mode is marked as available. The reference picture of list 0 is set to the forward picture and the reference picture of list 1 is set to the backward picture.

- The distances from both reference pictures to the current picture are different, the DMVR on inter mode is marked as nonavailable, and the CU-level flag is not signaled.

- Otherwise, following applies:

- The backward reference picture picture in reference picture list 0 which is nearest to current picture is searched.

- The forward reference picture in reference picture list 1 which is nearest to current picture is searched.

- If both backward and forward picture are found, and the distances from both reference pictures to the current picture are the same, the DMVR on inter mode is marked as available. The reference picture of list 0 is set to the backward picture and the reference picture of list 1 is set to the forward picture.

- Otherwise, the DMVR on inter mode is marked as nonavailable, and the CU-level flag is not signaled.

In addition, the CU-level flag of neighboring block is inherited for a current block predicted using regular merge mode or CIIP mode. For example, DMVR is turned off for a neighboring block coded using SMVD. If the motion of a current block is inherited from the neighboring block, DMVR is not applied.

Simulation results

The proposed method is implemented on top of VTM-5.0. The results showed that there is 0.12% coding gain with 112% encoding time and 98% decoding time in random access configuration. It is worth noting that higher coding gain is obtained for larger resolution (e.g. 0.24% coding gain in class A2).

Table 1. Results of the proposed method

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.23% | -0.10% | 113% | 99% |
| Class A2 | -0.24% | -0.12% | -0.12% | 112% | 98% |
| Class B | -0.07% | -0.06% | -0.07% | 112% | 99% |
| Class C | -0.08% | -0.04% | -0.20% | 111% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.10% | -0.12% | 112% | 98% |
| Class D | -0.09% | -0.05% | -0.13% | 109% | 95% |
| Class F | -0.04% | 0.00% | -0.02% | 108% | 98% |

Conclusion

In this contribution, it is proposed to apply DMVR to SMVD and AMVP modes. It is reported that 0.12% coding gain is obtained under CTC condition. Considering this interesting results, we recommend to study this contribution in CE.

Proposed text

The proposed syntax change is highlighted in yellow.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_smvd_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && !inter_affine_flag[ x0 ][ y0 ] && RefIdxSymL0 > −1 && RefIdxSymL1 > −1 ) |  |
| sym_mvd_flag[ x0 ][ y0 ] | ae(v) |
| if( cbWidth >= 8 && cbHeight >= 8 && cbWidth * cbHeight >= 128 && RefIdxEqu inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && inter_affine_flag[ x0 ][ y0 ] = = 0 ) |  |
| dmvr_on_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( NumRefIdxActive[ 0 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] && !dmvr_on_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( NumRefIdxActive[ 1 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] && !dmvr_on_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( sps_bcw_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && luma_weight_l0_flag[ ref_idx_l0 [ x0 ][ y0 ] ] = = 0 && luma_weight_l1_flag[ ref_idx_l1 [ x0 ][ y0 ] ] = = 0 && chroma_weight_l0_flag[ ref_idx_l0 [ x0 ][ y0 ] ] = = 0 && chroma_weight_l1_flag[ ref_idx_l1 [ x0 ][ y0 ] ] = = 0 && cbWidth * cbHeight >= 256 && !dmvr_on_flag[ x0 ][ y0 ] ) |  |
| bcw_idx[ x0 ][ y0 ] | ae(v) |
| … |  |
| } |  |

- 7.4.8.5  Coding unit semantics

ref_idx_l0[ x0 ][ y0 ] specifies the list 0 reference picture index for the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When ref_idx_l0[ x0 ][ y0 ] is not present it is inferred as follows:

- If sym_mvd_flag[ x0 ][ y0 ] or dmvr_on_flag[ x0 ][ y0 ] is equal to 1, ref_idx_l0[ x0 ][ y0 ] is inferred to be equal to RefIdxSymL0.

- Otherwise (sym_mvd_flag[ x0 ][ y0 ] is equal to 0), ref_idx_l0[ x0 ][ y0 ] is inferred to be equal to 0.

- 8.3.5  Decoding process for symmetric motion vector difference reference indices

Output of this process are RefIdxEqu specifying the two distances from two reference pictures to the current picture are the same, and RefIdxSymL0 and RefIdxSymL1 specifying the list 0 and list 1 reference picture indices for symmetric motion vector differences and DMVR on inter mode, i.e., when sym_mvd_flag or dmvr_on_flag is equal to 1 for a coding unit.

The variable RefIdxSymLX with X being 0 and 1 is derived as follows:

- The variable currPic specifies the current picture.

- RefIdxSymL0 is set equal to −1.

- For each index i with i = 0..NumRefIdxActive[ 0 ] − 1, the following applies:

- When all of the following conditions are true, RefIdxSymL0 is set to i:

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > 0,

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1.

- RefIdxSymL1 is set equal to −1.

- For each index i with i = 0..NumRefIdxActive[ 1 ]  − 1, the following applies:

- When all of the following conditions are true, RefIdxSymL1 is set to i:

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < 0,

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1.

- When RefIdxSymL0 is equal to −1 or RefIdxSymL1 is equal to −1, the following applies:

- For each index i with i = 0..NumRefIdxActive[ 0 ]  − 1, the following applies:

- When all of the following conditions are true, RefIdxSymL0 is set to i:

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < 0,

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1.

- For each index i with i = 0..NumRefIdxActive[ 1 ]  − 1, the following applies:

- When all of the following conditions are true, RefIdxSymL1 is set to i:

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > 0,

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1.

The variable RefIdxEqu is derived as follows:

- When RefIdxSymL0 is equal to −1 or RefIdxSymL1 is equal to −1, RefIdxEqu is equal to 0.

- Otherwise, the following applies:

  - When Abs( DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) ) is equal to Abs( DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) ), RefIdxEqu is equal to 1.

  - Otherwise, RefIdxEqu is equal to 0.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).