| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0642 |
| --- | --- |

| Title: | Non-CE6: Reducing secondary transform kernel for specific residual block size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sandeep Shrestha Ankit Kumar Bumshik Lee Yonggil Lee Junsuk Park | Email: | sandeepshrestha@chosun.kr ankitku@chosun.kr bslee@chosun.ac.kr yglee@humaxdigital.com parkjs@humaxdigital.com |
| Source: | Chosun University and Humax |  |  |

_____________________________

- Abstract

In this contribution, reduced secondary transform for 32×4 and 4×32 TU blocks are proposed. Instead of using 16×16 secondary transform kernel, we apply the 8×16  secondary transform kernel, which is taken from 16×16 secondary transform kernel for left or upper parts of 32x4 and 4x32 TU blocks. With reduced kernel, 4×4 secondary transform block can produced 4×2 secondary transformed block.

Introduction

Based on the proposal that had been adopted [1], the secondary transform kernel used are  8×16 for 4×4 residual block size, 16×16 for N×4 or 4×N  residual block size and  16×48 for residual blocks with width≥8 and height≥8. This contribution proposes to use 8×16 secondary transform kernel instead of 16×16 secondary transform kernel for 4×32 and 32×4 TU blocks for reduction of the number of multiplication.

Proposed method

Figure 1  Proposed method for 32×4 and 4 ×32 residual block size

Figure 1 shows the proposed secondary transform for TU blocks of 32×4 and 4×32. The TU block size (32×4) is divided into two 4×4 block size and 24×4 block size. For first 4×4 (16×1 matrix in 1D) block is multiplied by the 8×16 secondary transform kernel and the resulting 8×1 secondary transformed coefficients can be obtained. The obtained coefficients are placed in the top part of the 4×4 matrix and remaining elements is taken as zeros. Similar process is done for the second first 4×4 residual block size. For the 4×32 TU residual block similar process is done for both first and second 4×4 residual block size.

Experimental Results

The proposed method has been implemented on VTM-5.0 reference software and experimentally evaluated for VTM test according to common test conditions for AI, RA and LDB configurations [2]

Table 1. Simulation results for the AI configuration over VTM-5.0. (MTS-On)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | 0.03% | 98% | 100% |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.02% | 0.07% | 99% | 100% |
| Class C | 0.02% | -0.05% | 0.03% | 100% | 100% |
| Class E | 0.04% | -0.02% | 0.08% | 100% | 100% |
| Overall |  |  |  |  |  |
| Class D | 0.01% | 0.14% | -0.04% | 98% | 101% |
| Class F | 0.01% | 0.05% | 0.02% | 99% | 99% |

Table 2. Simulation results for the RA configuration over VTM-5.0. (MTS-on))

|  | Random access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 3. Simulation results for the LDB configuration over VTM-5.0. (MTS-on))

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Reference

- M. Koo et al., “CE6: Reduced Secondary Transform (RST)(CE6-3.1)”, JVET-N0193, 14th meeting, Geneva, CH, 19-27 March 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET meeting, Geneva, 19–27 Mar. 2019.

Patent rights declaration(s)

Chosun University and Humax may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).