| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–17 July 2019 | Document: JVET-N0646-v3 |
| --- | --- |

| Title: | Non-CE4/8: Combination of Blending-off Switch (JVET-O0645) and Simplified Motion Storage (JVET-O0411) for TPM Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen1, Kevin Reuze, Yan Zhang, Wei-Jung Chien2, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +886-3-566-1980 1chunchic@qti.qualcomm.com, 2wchien@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to combine the two TPM-related methods as in JVET-O0645 (Non-CE4/8: Blending-off Switch for TPM Mode) and JVET-O0411 (Non-CE4: Simplification of bi-prediction MV generation for triangle partition mode storage). The former introduces a slice-level switch to disable the blending operation during the motion compensation stage of TPM mode, and the latter removes the derivation process of bi-prediction MV by taking only one MV from the two triangle partitions when both MVs of the selected TPM merge candidate point to the same reference picture list. Experiments were conducted on top of the VTM-5.0 with Random Access (RA) and Low Delay B (LDB) testing conditions for both CTC and CE8 testing sequences. Experimental results of the top performing one show that for CE8 testing sequences, the average BD-rate gain is 0.22% and 0.38% respectively for RA and LDB. In particular, it is reported that Class TGM1080p444 achieves the highest coding gain among all classes of sequences, with 0.62% and 1.05% BD-rate gain in RA and LDB, respectively. The average runtime changes observed from encoder and decoder, respectively, is around ±1%.

# Introduction

This contribution proposes to combine the simplified motion storage [1] and blending-off switch [2] for TPM mode. As illustrated in Fig. 1, there are three configurations as defined in [1] for simplified motion storage in the application of TPM mode.

- Test A replaces bi-prediction MV by using upper-left (if anti-diagonal split “/”) or upper-right if (diagonal split “\”) triangle partition’s uni-prediction MV.

- Test B replaces bi-prediction MV by using bottom-right (if anti-diagonal split “/”) or bottom-left if (diagonal split “\”) triangle partition’s uni-prediction MV.

- Test C replaces bi-prediction MV by using bottom-right (if anti-diagonal split “/”) or upper-right if (diagonal split “\”) triangle partition’s uni-prediction MV.

Each of these configurations will be tested together with the blending-off switch as specified in [2] to clip weighting values (denoted as w) to either 0/8 or 8/8 by using the following ordered steps.

- If w < 4/8, the value of w is clipped to 0/8;

- Otherwise, if w > 4/8, the value of w is clipped to 8/8;

- Otherwise, if w = 4/8, the value of w is clipped as follows:

- If the CU is split diagonally, w is clipped to 8/8;

- Otherwise, w is clipped to 0/8.

Figure 2 visualizes the patterns of weight values that are assigned to each sample on a coding unit.

| (a) VTM | (b) Test A |
| --- | --- |
| (c) Test B | (d) Test C |

Figure 1. (a) Storage of bi-prediction motion vectors as in VTM for TPM. In JVET-O0411, these bi-prediction motion vectors are all replaced by (b) MV1, (c) MV2 and (d) either MV1 or MV2 depending on the direction of diagonal splitting. [1]

| (a) VTM | (b) Proposed |
| --- | --- |

Figure 1. (a) Weighting map when blending is enabled; (b) weighting map when blending is disabled. [2]

# Experimental Results

The experiments were conducted on top of the VTM-5.0 software with the common test conditions [1] and CE8 testing conditions [2] to collect BD-rate numbers, encoding-time ratio and decoding-time ratio. The VTM anchor and testing results were generated using the VTM configurations with --PrintHexPSNR=1 and --SIMD=SSE42 for encoder and --SIMD=SSE42 for decoder. Macro settings as specified in the VTM-5.0 software are not changed. Parallel encoding and decoding were applied to Random Access case for both anchors and tests.

At the encoder, the introduced sps flag is always set enabled for each test. The introduced slice-level flag is configured by using the same hash-ratio based function as MMVD uses to swap between fractional and integer distance offset tables and the threshold value is set to be 40. Both flags are present in the bitstream among all the tests.

Four tests were conducted. The last one is the test for blending-off switch reference from JVET-O0645 [2]. The respective performance results are summarized in the table below.

(Test A) Simplified motion storage method A + Blending-off switch;

(Test B) Simplified motion storage method B + Blending-off switch;

(Test C) Simplified motion storage method C + Blending-off switch;

(Test D) Blending-off switch.

- Test A Performance results of CE8 sequences.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM1080 | -0.71% | -0.55% | -0.59% | 100% | 99% | -1.12% | -0.99% | -0.98% | 101% | 99% |
| TGM720 | -0.05% | -0.02% | -0.05% | 101% | 100% | -0.11% | -0.40% | -0.28% | 100% | 101% |
| Animation | 0.01% | 0.00% | 0.01% | 101% | 100% | 0.04% | -0.07% | 0.00% | 100% | 99% |
| Mixed | -0.13% | -0.10% | -0.16% | 98% | 98% | -0.18% | -0.03% | -0.04% | 102% | 102% |
| Camera | -0.01% | 0.09% | 0.06% | 102% | 102% | 0.09% | -0.01% | 0.08% | 100% | 101% |
| Overall | -0.23% | -0.16% | -0.19% | 100% | 99% | -0.35% | -0.39% | -0.33% | 101% | 100% |

- Test A Performance results of CTC sequences.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.15% | 0.03% | 99% | 99% | x | x | x | x | x |
| Class A2 | 0.02% | -0.01% | 0.06% | 102% | 102% | x | x | x | x | x |
| Class B | 0.02% | 0.07% | 0.00% | 100% | 99% | 0.09% | -0.02% | 0.20% | 101% | 101% |
| Class C | -0.02% | -0.04% | -0.19% | 100% | 101% | 0.10% | 0.15% | 0.16% | 102% | 101% |
| Class E | x | x | x | x | x | 0.07% | 0.40% | -0.49% | 102% | 102% |
| Overall | 0.01% | -0.02% | -0.03% | 100% | 100% | 0.09% | 0.14% | 0.01% | 101% | 101% |
| Class D | 0.04% | 0.15% | 0.29% | 100% | 103% | 0.13% | -0.17% | 0.35% | 101% | 101% |
| Class F | 0.00% | 0.11% | 0.09% | 101% | 101% | -0.05% | -0.15% | -0.48% | 102% | 98% |
| Class SCC | -0.60% | -0.40% | -0.37% | 100% | 100% | -0.75% | -0.63% | -0.57% | 99% | 99% |

- Test B Performance results of CE8 sequences.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM1080 | -0.53% | -0.48% | -0.50% | 100% | 98% | -1.01% | -0.92% | -0.89% | 101% | 100% |
| TGM720 | -0.06% | -0.03% | -0.01% | 101% | 99% | -0.04% | -0.29% | 0.01% | 101% | 100% |
| Animation | -0.05% | -0.02% | -0.03% | 101% | 101% | 0.02% | -0.08% | -0.07% | 100% | 100% |
| Mixed | 0.00% | 0.02% | -0.03% | 98% | 98% | -0.09% | -0.04% | 0.04% | 102% | 103% |
| Camera | -0.02% | -0.06% | 0.03% | 102% | 101% | 0.13% | 0.11% | 0.15% | 100% | 99% |
| Overall | -0.17% | -0.14% | -0.14% | 100% | 99% | -0.28% | -0.33% | -0.22% | 101% | 100% |

- Test B Performance results of CTC sequences.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.15% | 0.07% | 98% | 98% | x | x | x | x | x |
| Class A2 | 0.02% | 0.06% | 0.05% | 103% | 101% | x | x | x | x | x |
| Class B | 0.02% | -0.06% | 0.01% | 100% | 99% | 0.08% | 0.39% | 0.06% | 101% | 101% |
| Class C | -0.02% | -0.02% | -0.16% | 102% | 102% | 0.10% | 0.09% | 0.21% | 101% | 98% |
| Class E | x | x | x | x | x | 0.04% | 0.29% | -0.18% | 101% | 100% |
| Overall | 0.01% | -0.04% | -0.01% | 101% | 100% | 0.08% | 0.27% | 0.05% | 101% | 99% |
| Class D | 0.02% | 0.10% | 0.12% | 102% | 103% | 0.13% | -0.02% | 1.33% | 100% | 100% |
| Class F | 0.00% | 0.10% | 0.08% | 100% | 109% | -0.08% | -0.10% | 0.30% | 102% | 100% |
| Class SCC | -0.51% | -0.36% | -0.41% | 101% | 99% | -0.78% | -0.62% | -0.60% | 99% | 101% |

- Test C Performance results of CE8 sequences.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM1080 | -0.62% | -0.55% | -0.55% | 100% | 99% | -1.05% | -0.94% | -0.93% | 100% | 98% |
| TGM720 | -0.10% | -0.08% | -0.07% | 101% | 99% | -0.25% | -0.57% | -0.22% | 99% | 99% |
| Animation | -0.04% | -0.02% | 0.00% | 101% | 101% | 0.00% | 0.08% | 0.01% | 98% | 98% |
| Mixed | -0.08% | -0.04% | -0.08% | 98% | 98% | -0.22% | -0.12% | -0.26% | 102% | 102% |
| Camera | -0.01% | 0.03% | -0.06% | 102% | 101% | 0.10% | 0.08% | -0.02% | 100% | 101% |
| Overall | -0.22% | -0.17% | -0.19% | 100% | 99% | -0.38% | -0.40% | -0.36% | 100% | 100% |

- Test C Performance results of CTC sequences.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.08% | 99% | 98% | x | x | x | x | x |
| Class A2 | -0.02% | 0.04% | -0.01% | 101% | 100% | x | x | x | x | x |
| Class B | 0.01% | 0.13% | -0.09% | 101% | 99% | 0.07% | 0.21% | -0.03% | 101% | 100% |
| Class C | -0.01% | -0.13% | -0.09% | 100% | 100% | 0.07% | 0.10% | 0.08% | 101% | 99% |
| Class E | x | x | x | x | x | 0.04% | 0.16% | -0.41% | 102% | 100% |
| Overall | 0.00% | 0.02% | -0.04% | 100% | 100% | 0.06% | 0.16% | -0.09% | 101% | 100% |
| Class D | 0.04% | 0.21% | 0.18% | 100% | 102% | 0.01% | 0.01% | 0.43% | 100% | 101% |
| Class F | -0.01% | 0.13% | 0.07% | 101% | 101% | 0.02% | 0.48% | -0.26% | 103% | 99% |
| Class SCC | -0.66% | -0.48% | -0.49% | 100% | 100% | -0.84% | -0.65% | -0.63% | 99% | 100% |

- Test D Performance results of CE8 sequences.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM1080 | -0.60% | -0.49% | -0.50% | 101% | 100% | -0.91% | -0.77% | -0.81% | 101% | 99% |
| TGM720 | -0.07% | -0.01% | -0.02% | 101% | 100% | -0.15% | -0.13% | -0.31% | 99% | 97% |
| Animation | 0.01% | 0.01% | 0.01% | 101% | 101% | 0.00% | 0.00% | 0.00% | 98% | 94% |
| Mixed | -0.13% | -0.14% | -0.14% | 101% | 101% | -0.29% | -0.28% | -0.27% | 102% | 104% |
| Camera | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Overall | -0.20% | -0.16% | -0.16% | 101% | 100% | -0.34% | -0.29% | -0.35% | 100% | 99% |

- Test D Performance results of CTC sequences.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% | x | x | x | x | x |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% | x | x | x | x | x |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class C | 0.01% | 0.01% | 0.01% | 103% | 101% | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class E | x | x | x | x | x | 0.02% | 0.02% | 0.02% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.01% | 0.01% | 0.01% | 101% | 101% |
| Class D | 0.04% | 0.03% | 0.03% | 103% | 101% | 0.01% | 0.01% | 0.01% | 104% | 101% |
| Class F | -0.01% | 0.08% | 0.08% | 100% | 101% | 0.01% | -0.04% | -0.45% | 100% | 98% |
| Class SCC | -0.54% | -0.39% | -0.42% | 100% | 100% | -0.59% | -0.49% | -0.41% | 99% | 101% |

- Y BD-rate comparison among Test A, B, C and D.

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| YUV444 | Test A (Upper) | Test B (Lower) | Test C (Right) | Test D (n/a) |  | Test A (Upper) | Test B (Lower) | Test C (Right) | Test D (n/a) |
| TGM1080 | -0.71% | -0.53% | -0.62% | -0.60% |  | -1.12% | -1.01% | -1.05% | -0.91% |
| TGM720 | -0.05% | -0.06% | -0.10% | -0.07% |  | -0.11% | -0.04% | -0.25% | -0.15% |
| Animation | 0.01% | -0.05% | -0.04% | 0.01% |  | 0.04% | 0.02% | 0.00% | 0.00% |
| Mixed | -0.13% | 0.00% | -0.08% | -0.13% |  | -0.18% | -0.09% | -0.22% | -0.29% |
| Camera | -0.01% | -0.02% | -0.01% | 0.00% |  | 0.09% | 0.13% | 0.10% | 0.00% |
| Overall | -0.23% | -0.17% | -0.22% | -0.20% |  | -0.35% | -0.28% | -0.38% | -0.34% |
| EncT | 100% | 100% | 100% | 101% |  | 101% | 101% | 100% | 100% |
| Dec T | 99% | 99% | 99% | 100% |  | 100% | 100% | 100% | 99% |
|  | Random Access |  |  |  |  | Low Delay B |  |  |  |
| YUV420 | Test A (Upper) | Test B (Lower) | Test C (Right) | Test D (n/a) |  | Test A (Upper) | Test B (Lower) | Test C (Right) | Test D (n/a) |
| Class A1 | 0.01% | 0.01% | 0.01% | 0.00% |  | x | x | x | x |
| Class A2 | 0.02% | 0.02% | -0.02% | 0.00% |  | x | x | x | x |
| Class B | 0.02% | 0.02% | 0.01% | 0.00% |  | 0.09% | 0.08% | 0.07% | 0.00% |
| Class C | -0.02% | -0.02% | -0.01% | 0.01% |  | 0.10% | 0.10% | 0.07% | 0.00% |
| Class E | x | x | x | x |  | 0.07% | 0.04% | 0.04% | 0.02% |
| Overall | 0.01% | 0.01% | 0.00% | 0.00% |  | 0.09% | 0.08% | 0.06% | 0.01% |
| EncT | 100% | 101% | 100% | 101% |  | 101% | 101% | 101% | 101% |
| Dec T | 100% | 100% | 100% | 100% |  | 101% | 99% | 100% | 101% |
| Class D | 0.04% | 0.02% | 0.04% | 0.04% |  | 0.13% | 0.13% | 0.01% | 0.01% |
| Class F | 0.00% | 0.00% | -0.01% | -0.01% |  | -0.05% | -0.08% | 0.02% | 0.01% |
| TGM1080 | -0.60% | -0.51% | -0.66% | -0.54% |  | -0.75% | -0.78% | -0.84% | -0.59% |

# Proposed Draft Text

The proposed texts are drafted based on JVET-N1001 (v9) [5]. The following section highlights the change in VVC working draft of each respective test.

## Test A

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| if( sps_mmvd_enabled_flag ) |  |
| sps_fpel_mmvd_enabled_flag | u(1) |
| sps_triangle_enabled_flag | u(1) |
| if( sps_triangle_enabled_flag ) { |  |
| sps_tpm_blending_ctrl_flag | u(1) |
| } |  |
| sps_lmcs_enabled_flag | u(1) |
| … |  |

sps_tpm_blending_ctrl_flag equal to 1 specifies that triangle partition merge mode disables the blending operation applied to the boundary between two triangle partitions inside a coding unit. sps_tpm_blending_ctrl_flag equal to 0 specifies that the blending operation is always enabled. When not present, the value of sps_tpm_blending_ctrl_flag is inferred to be 0.

#### 7.3.5.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| … |  |
| if ( slice_type != I ) { |  |
| … |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| slice_fpel_mmvd_enabled_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 ) { |  |
| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| if( sps_tpm_blending_ctrl_flag ) |  |
| triangle_blending_off_flag | u(1) |
| } |  |
| } else if ( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| slice_qp_delta | se(v) |
| … |  |

triangle_blending_off_flag equal to 1 specifies that triangle partition merge mode disables the blending operation applied to the boundary between two triangle partitions inside each coding unit of the current slice. triangle_blending_off_flag equal to 0 specifies that the blending operation is always enabled in the current slice. When not present, triangle_blending_off_flag is inferred to be 0.

#### 8.5.7.2 Weighted sample prediction process for triangle merge mode

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW)x(nCbH) arrays predSamplesLA and predSamplesLB,

- a variable triangleDir specifying the partition direction,

- a variable cIdx specifying colour component index.

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

The variable nCbR is derived as follows:

nCbR = ( nCbW > nCbH )  ?  ( nCbW / nCbH )  :  ( nCbH / nCbW )	(8-821)

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

Variables shift1 and offset1 are derived as follows:

–	The variable shift1 is set equal to Max( 5, 17 − bitDepth).

–	The variable offset1 is set equal to 1  <<  ( shift1 − 1 ).

Depending on the values of triangleDir, wS and cIdx, the prediction samples pbSamples[ x ][ y ] with x = 0..nCbW − 1 and y = 0..nCbH − 1 are derived as follows:

–	The variable wIdx is derived as follows:

- If cIdx is equal to 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( x / nCbR − y ) + 4 ) )	(8-822)
	: ( Clip3( 0, 8, ( x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is equal to 0 and triangleDir is equal to 1, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( nCbH − 1 − x / nCbR − y ) + 4 ) ) 	(8-823)
		( Clip3( 0, 8, ( nCbW − 1 − x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is greater than 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( x / nCbR − y ) + 2 ) )	(8-824)
	: ( Clip3( 0, 4, ( x − y / nCbR ) + 2 ) )

- Otherwise (if cIdx is greater than 0 and triangleDir is equal to 1), the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( nCbH − 1 − x / nCbR − y ) + 2 ) ) 	(8-825)
		( Clip3( 0, 4, ( nCbW − 1 − x − y / nCbR ) + 2 ) )

–	The variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows:

wValue = ( cIdx = = 0 )  ?  Clip3( 0, 8, wIdx )  :  Clip3( 0, 8, wIdx * 2 )	(8-826)

–	If triangle_blending_off_flag is equal to 1, the following applies:

wValue = ( wValue != 4 ) ? ( wValue < 4 ? 0 : 8 ) : ( triangleDir == 1 ? 0 : 8 )	(8-827)

–	The prediction sample values are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, ( predSamplesLA[ x ][ y ] * wValue + 	(8-827)
		predSamplesLB[ x ][ y ] * ( 8 − wValue ) + offset1 ) >> shift1 )

#### 8.5.7.3 Motion vector storing process for triangle merge mode

This process is invoked when decoding a coding unit with MergeTriangleFlag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable triangleDir specifying the partition direction,

- the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB,

- the reference indices refIdxA and refIdxB,

- the prediction list flags predListFlagA and predListFlagB.

The variables numSbX and numSbY specifying the number of 4x4 blocks in the current coding block in horizontal and vertical direction are set equal to numSbX = cbWidth >> 2 and numSbY = cbHeight >> 2.

The variable minSb is set equal to min( numSbX, numSbY ).

The variable cbRatio is derived as follows:

cbRatio = ( cbWidth > cbHeight ) ? ( cbWidth / cbHeight ) : ( cbHeight / cbWidth )	(8-828)

The variable refIdxTempA is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with X set equal to predListFlagA and refIdxN set equal to refIdxA as inputs.

The variable refIdxTempB is derived by invoking the reference picture mapping process for triangle merge mode specified in clause 8.5.7.4 with the X set equal to predListFlagB and refIdxN set equal to refIdxB as inputs.

For each 4x4 subblock at subblock index ( xSbIdx, ySbIdx ) with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the following applies:

- The variables xIdx and yIdx are derived as follows:

xIdx = ( cbWidth > cbHeight )  ?  ( xSbIdx / cbRatio ) : xSbIdx	(8-829)

yIdx = ( cbWidth > cbHeight )  ?  ySbIdx  :  ( ySbIdx / cbRatio )	(8-830)

- The variable sType is derived as follows:

- If triangleDir is equal to 0, the following applies:

sType = ( xIdx = = yIdx )  ?  2  :  ( ( xIdx > yIdx )  ?  0  :  1 )	(8-831)

- Otherwise (triangleDir is equal to 1), the following applies:

sType = ( xIdx + yIdx = = minSb )  ?  2  :  ( ( xIdx + yIdx < minSb )  ?  0  :  1 )	(8-832)

- If sType is equal to 2 and predListFlagA + predListFlagB is not equal to 1, sType is set equal to 0.

- Depending on the value of sType, the following assignments are made:

- If sType is equal to 0, the following applies:

predFlagL0 = ( predListFlagA = = 0 )  ?  1  :  0		(8-833)

predFlagL1 = ( predListFlagA = = 0 )  ?  0  :  1		(8-834)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA : −1	(8-835)

refIdxL1 = ( predListFlagA = = 0 )  ?  −1  :  refIdxA	(8-836)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  0	(8-837)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  0	(8-838)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 0 ]	(8-839)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  0  :  mvA[ 1 ]	(8-840)

- Otherwise, if sType is equal to 1, the following applies:

predFlagL0 = ( predListFlagB = = 0 )  ?  1  :  0		(8-841)

predFlagL1 = ( predListFlagB = = 0 )  ?  0  :  1		(8-842)

refIdxL0 = ( predListFlagB = = 0 )  ?  refIdxB : −1	(8-843)

refIdxL1 = ( predListFlagB = = 0 )  ?  −1  :  refIdxB	(8-844)

mvL0[ 0 ] = ( predListFlagB = = 0 )  ?  mvB[ 0 ]  :  0	(8-845)

mvL0[ 1 ] = ( predListFlagB = = 0 )  ?  mvB[ 1 ]  :  0	(8-846)

mvL1[ 0 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 0 ]	(8-847)

mvL1[ 1 ] = ( predListFlagB = = 0 )  ?  0  :  mvB[ 1 ]	(8-848)

- Otherwise (sType is equal to 2), the following applies:

  - If predListFlagA + predListFlagB is equal to 1,

predFlagL0 = 1		(8-849)

predFlagL1 = 1		(8-850)

refIdxL0 = ( predListFlagA = = 0 )  ?  refIdxA  :  refIdxB	(8-851)

refIdxL1 = ( predListFlagA = = 0 )  ?  refIdxB  :  refIdxA	(8-852)

mvL0[ 0 ] = ( predListFlagA = = 0 )  ?  mvA[ 0 ]  :  mvB[ 0 ]	(8-853)

mvL0[ 1 ] = ( predListFlagA = = 0 )  ?  mvA[ 1 ]  :  mvB[ 1 ]	(8-854)

mvL1[ 0 ] = ( predListFlagA = = 0 )  ?  mvB[ 0 ]  :  mvA[ 0 ]	(8-855)

mvL1[ 1 ] = ( predListFlagA = = 0 )  ?  mvB[ 1 ]  :  mvA[ 1 ]	(8-856)

  - If predListFlagA + predListFlagB is equal to 0, the following applies:

predFlagL0 = 1		(8-857)

predFlagL1 = ( refIdxTempA = = −1  &&  refIdxTempB = = −1 )  ?  0  :  1	(8-858)

refIdxL0 = ( refIdxTempB != −1 )  ?  refIdxA  :  	(8-859)
	( ( refIdxTempA != −1 )  ?  refIdxB  :  refIdxA )

refIdxL1 = ( refIdxTempB != −1 )  ?  refIdxTempB  :  	(8-860)
	( ( refIdxTempA != −1 )  ?  refIdxTempA  :  −1 )

mvL0[ 0 ] = ( refIdxTempB != −1 )  ?  mvA[ 0 ]  :  	(8-861)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 0 ] : mvA[ 0 ] )

mvL0[ 1 ] = ( refIdxTempB != −1 )  ?  mvA[ 1 ]  :  	(8-862)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 1 ] : mvA[ 1 ] )

mvL1[ 0 ] = ( refIdxTempB != −1 )  ?  mvB[ 0 ]  :  	(8-863)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 0 ] : 0 )

mvL1[ 1 ] = ( refIdxTempB != −1 )  ?  mvB[ 1 ]  :  	(8-864)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 1 ] : 0 )

  - If predListFlagA + predListFlagB is equal to 2, the following applies:

predFlagL0 = ( refIdxTempA = = −1  &&  refIdxTempB = = −1 )  ?  0  :  1	(8-865)

predFlagL1 = 1		(8-866)

refIdxL0 = ( refIdxTempB != −1 )  ?  refIdxTempB  :  	(8-867)
	( ( refIdxTempA != −1 )  ?  refIdxTempA  :  −1 )

refIdxL1 = ( refIdxTempB != −1 )  ?  refIdxA  :  	(8-868)
	( ( refIdxTempA != −1 )  ?  refIdxB  :  refIdxA )

mvL0[ 0 ] = ( refIdxTempB != −1 )  ?  mvB[ 0 ]  :  	(8-869)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 0 ] : 0 )

mvL0[ 1 ] = ( refIdxTempB != −1 )  ?  mvB[ 1 ]  :  	(8-870)
	 ( ( refIdxTempA != −1 )  ?  mvA[ 1 ] : 0 )

mvL1[ 0 ] = ( refIdxTempB != −1 )  ?  mvA[ 0 ]  :  	(8-871)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 0 ] : mvA[ 0 ] )

mvL1[ 1 ] = ( refIdxTempB != −1 )  ?  mvA[ 1 ]  :  	(8-872)
	 ( ( refIdxTempA != −1 )  ?  mvB[ 1 ] : mvA[ 1 ] )

- The following assignments are made for x = 0..3 and y = 0..3:

MvL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL0	(8-873)

MvL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = mvL1	(8-874)

RefIdxL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL0	(8-875)

RedIdxL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = refIdxL1	(8-876)

PredFlagL0[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL0	(8-877)

PredFlagL1[ ( xSbIdx << 2 ) + x ][ ( ySbIdx << 2 ) + y] = predFlagL1	(8-878)

## Test B

Test B only changes one line of the Section 8.5.7.3 of Test A. The changed line is highlighted in the following.

#### 8.5.7.3 Motion vector storing process for triangle merge mode

This process is invoked when decoding a coding unit with MergeTriangleFlag[ xCb ][ yCb ] equal to 1.

…

- The variable sType is derived as follows:

- If triangleDir is equal to 0, the following applies:

sType = ( xIdx = = yIdx )  ?  2  :  ( ( xIdx > yIdx )  ?  0  :  1 )	(8-831)

- Otherwise (triangleDir is equal to 1), the following applies:

sType = ( xIdx + yIdx = = minSb )  ?  2  :  ( ( xIdx + yIdx < minSb )  ?  0  :  1 )	(8-832)

- If sType is equal to 2 and predListFlagA + predListFlagB is not equal to 1, sType is set equal to 1.

- Depending on the value of sType, the following assignments are made:

- If sType is equal to 0, the following applies:

…

## Test C

Test C only changes one line of the Section 8.5.7.3 of Test A. The changed line is highlighted in the following.

#### 8.5.7.3 Motion vector storing process for triangle merge mode

This process is invoked when decoding a coding unit with MergeTriangleFlag[ xCb ][ yCb ] equal to 1.

…

- The variable sType is derived as follows:

- If triangleDir is equal to 0, the following applies:

sType = ( xIdx = = yIdx )  ?  2  :  ( ( xIdx > yIdx )  ?  0  :  1 )	(8-831)

- Otherwise (triangleDir is equal to 1), the following applies:

sType = ( xIdx + yIdx = = minSb )  ?  2  :  ( ( xIdx + yIdx < minSb )  ?  0  :  1 )	(8-832)

- If sType is equal to 2 and predListFlagA + predListFlagB is not equal to 1, sType is set equal to ( triangleDir == 0 ? 0 : 1 ).

- Depending on the value of sType, the following assignments are made:

- If sType is equal to 0, the following applies:

…

# References

- K. Reuze, C.-C. Chen and et. al., “Non-CE4: Simplification of Bi-prediction MV Generation for Triangle Partition Mode Storage,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O0411, July 2019.

- C.-C. Chen, Y. Zhang and et. al., “Non-CE4/8: Blending-off Switch for TPM Mode,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O0645, July 2019.

- F. Bossen, J. Boyce and et. al., “JVET Common Test Conditions and Software Reference Configurations for SDR Video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1010, Mar. 2019.

- X. Xu, Y.-H. Chao and et. al., “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1028, Mar. 2019.

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1001, Mar. 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).