| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0647-v1 |
| --- | --- |

| Title: | Non-CE3: On signalling of intra coded blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van, Geert Van der Auwera, Adarsh K. Ramasubramonian, Vadim Seregin, Han Huang, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Email: | {lphamvan, geertv, aramasub, vseregin, hanhuang, martak}@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes several alternatives for signalling of the intra coding mode of luma blocks. In the first solution (Test 1), the MPM flag is signalled before MRL and ISP flags. In the second solution (Test 2), the MPM flag of VTM5.0 is redefined so that it indicates whether the block is a regular intra coded with a mode in MPM list. In this test, the modified MPM flag (reg_mpm_flag) is signalled before the MIP flag. In the third solution (Test 3), a regular flag is used to indicate whether the block is encoded using regular intra mode or not. The regular flag is context encoded with 4 contexts based on the width and height of the block before the MIP flag. In addition, in test 4 a context is added to encode the Planar flag while the first bin of the MPM index is encoded using 3 contexts. Under the AI and RA CTC configurations, the average (Y, U, V) BD rate performances of these above alternatives are reported as follows with negligible encoding and decoding time change:

Test 1: (0.01%, -0.04%, 0.00%) and (0.00%, -0.13%, -0.03%)

Test 2: (0.02%, -0.02%, -0.03%) and (0.02%, -0.01%, -0.02%)

Test 3: (0.00%, -0.03%, 0.00%) and (0.02%, -0.05%, -0.04%)

Test 4: (-0.09%, -0.14%, -0.08%) and (0.00%, -0.07%, -0.03%)

# Introduction

Current VTM5.0 supports several intra coding tools including MIP, MRL, ISP and regular intra coding. In addition, RDPCM is enabled for the screen content configuration. The intra prediction mode candidate is different for each intra mode coding. For MIP mode, the number of candidates is dependent on the block size index. Meanwhile, for the intra coded block with a non-zero MRL index, only non-Planar prediction modes in the MPM list are enabled. On the other hand, only prediction modes in MPM list are allowed for ISP mode that is being encoded with the zero MRL index.

Figure 1 illustrates the signalling syntax for intra mode information. It is obvious that the regular intra mode is implicitly signalled at the very end of the syntax. In the decoder, to detect whether the mode is regular, the MIP, MRL, ISP flags should be decoded. For decoding each flag, some conditional checks are tested to see if the mode is available for this block or not. Consequently, to detect a regular intra coded block, many conditional checks are needed. In our observation, the probability that a block is encoded using regular intra mode is highest (about 73%). Therefore, with the current design of signalling, there are many redundant checks. This proposal proposes several logical alternatives of signalling to solve this problem.

Figure 1. Illustration of signaling for intra modes

# Proposed Method

Three alternatives of intra signalling are proposed as follows:

Test 1: MPM flag is signalled before MRL.

In the current design, MRL index and ISP flag are still signalled for non-MPM mode, although they are not allowed. In this test, the MPM flag is signalled before MRL as depicted in Figure 2 (left). The rest of the syntax signalling is not changed. The signaling of MRL index and ISP flag is only performed when MPM flag is non-zero. Although this proposed signalling is shorter than the current VVC, the MPM flag is always signalled for MRL, ISP, and regular modes while it is not needed for MRL and ISP modes in the current design.

Test 2: Regular MPM flag is signalled before MIP flag.

The motivation of this proposed test is that the probability a block is being coded using regular mode with an MPM prediction mode is high. Meanwhile, MPM flag is implicitly signalled for MRL and ISP blocks. In this test, a regular MPM flag (reg_mpm_flag) is defined and the conventional MPM flag is removed from the syntax as depicted in Figure 2 (right). This flag is 1 when the block is encoded using the regular intra with an MPM mode. This flag is placed before the MIP flag of the syntax. When the flag is 0, signalling of MIP, MRL, ISP flags are kept unchanged. When MIP, MRL, ISP flag are zero, the block is implicitly signalled as a regular intra mode with a non-MPM prediction mode and mode index (remainder) is truncated binary decoded.

Since the reg_mpm_flag is placed at the beginning of the signalling syntax, the conditional checks for MIP, MRL, ISP flags are avoided for this kind of blocks. Without adding any flag, this solution provides a more logical and simpler design.

Figure 2. Illustration of signalling for Test 1 (left) and Test 2 (right)

Test 3: Regular intra flag is signalled before MRL.

Although signalling and conditional checks of MIP, MRL, and ISP flags may be avoided for the regular MPM modes in Test 2, they are still needed for the regular non-MPM mode. In this test, a regular intra mode flag (reg_intra_flag) is defined and the ISP flag is removed from the signalling syntax. The reg_intra_flag indicates whether the block is encoded using the regular intra mode or not. The proposed signalling syntax is illustrated in Figure 3. In this figure, the reg_intra_flag is signalled before the MIP flag. When this flag is 1, the signalling of the other flags including MIP, MRL, and ISP are terminated. Else, only MIP and MRL index is signalled. In this case, if MRL index is zero, the block is implicitly decided as ISP mode; hence, the ISP flag is removed from the syntax.

It can be seen from the proposed syntax, signalling and conditional checks of MIP, MRL, and ISP flags are avoided for all regular intra blocks.

Figure 3. Proposed signaling syntax with reg_intra_flag.

Improvement of encoding planar flag and the first bin of MPM index.

Currently, the planar flag is context encoded using two contexts (ISP or Regular) and the first bin of the MPM index is bypass encoded. In this improvement, one context is added for encoding of the planar flag and 3 contexts are used to encode the first bin of the MPM index. The context index of the first bin of MPM index is inherited for the context index of the Planar flag. If the current block is ISP, the context index of 0 is used. The context index for regular blocks is decided as follows:

ctxIdx = 1 + (left is regular && IPM of left is Planar || left is not regular) || (top is regular && IPM of top is Planar || top is not regular)

where left and top are two neighbours of the current block with the positions depicted as in Figure 4.

Figure 4. Top and left neighbours for the context index derivation

The conditional check whether they are regular or not utilizes the regular intra flags of their blocks, the context index derivation is simple. When top and the current block are not in the same CTU, the top block is not considered for obtaining the ctxIdx.

# Experimental Results

The proposed tests have been implemented on top of VTM5.0. The experimental results under the AI and RA CTC configurations are summarized in Table 2, Table 3, Table 4 and Table 5, respectively.

It can be seen from these tables that, the average (Y, U, V) BD rate performances of these above alternatives are reported as follows with negligible encoding and decoding time change:

Test 1: (0.01%, -0.04%, 0.00%) and (0.00%, -0.13%, -0.03%).

Test 2: (0.02%, -0.02%, -0.03%) and (0.02%, -0.01%, -0.02%).

Test 3: (0.00%, -0.03%, 0.00%) and (0.02%, -0.05%, -0.04%).

Test 4: (-0.09%, -0.14%, -0.08%) and (0.00%, -0.07%, -0.03%).

It should be noted that the target of this proposal is to make the signalling syntax more logical and simplify such that redundant conditional checks can be avoided. The conditional check for MIP, MRL, ISP, and MPM flags is elaborated in Table 1. In our proposed designs, these conditional checks are no longer needed for the regular intra mode that has highest probability (about 75%). Moreover, the conditional check is no longer need for ISP in Test 3 and Test 4 since the ISP flag is removed from the proposed syntax.

Table 1. Conditional checks for intra flags in VVC WD5

| Flag | Conditional check |
| --- | --- |
| MIP | if( sps_mip_enabled_flag && ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |
| MRL | sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) |
| ISP | if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |
| MPM | if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0) |

Table 2. Test 1: MPM flag before MRL

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.01% | 102% | 104% |
| Class A2 | 0.00% | -0.07% | -0.03% | 99% | 99% |
| Class B | 0.01% | -0.06% | 0.00% | 101% | 102% |
| Class C | 0.01% | -0.09% | 0.00% | 99% | 94% |
| Class E | 0.01% | 0.06% | 0.01% | 100% | 107% |
| Overall | 0.01% | -0.04% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.24% | 0.01% | 102% | 110% |
| Class F | 0.02% | 0.04% | 0.06% | 100% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.15% | 0.06% | 104% | 104% |
| Class A2 | 0.01% | 0.00% | 0.07% | 101% | 100% |
| Class B | 0.00% | -0.14% | -0.06% | 103% | 104% |
| Class C | 0.00% | -0.20% | -0.14% | 98% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.13% | -0.03% | 101% | 100% |
| Class D | 0.06% | -0.02% | 0.14% | 105% | 109% |
| Class F | 0.00% | 0.00% | 0.07% | 103% | 105% |

Table 3. Test 2: regular MPM flag before MIP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | -0.05% | 101% | 97% |
| Class A2 | 0.01% | -0.06% | -0.07% | 101% | 101% |
| Class B | 0.02% | -0.03% | 0.01% | 102% | 103% |
| Class C | 0.03% | 0.00% | 0.07% | 101% | 114% |
| Class E | 0.02% | -0.06% | -0.03% | 101% | 107% |
| Overall | 0.02% | -0.02% | -0.01% | 101% | 105% |
| Class D | 0.02% | 0.31% | 0.06% | 104% | 114% |
| Class F | 0.03% | 0.03% | 0.03% | 102% | 108% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.22% | 0.09% | 100% | 96% |
| Class A2 | 0.05% | -0.04% | -0.04% | 99% | 96% |
| Class B | 0.02% | 0.12% | -0.01% | 101% | 99% |
| Class C | 0.01% | 0.00% | -0.10% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.01% | -0.02% | 101% | 99% |
| Class D | 0.04% | -0.22% | -0.06% | 102% | 104% |
| Class F | -0.02% | -0.01% | 0.16% | 101% | 101% |

Table 4. Test 3: Regular intra flag before MIP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.04% | -0.06% | 99% | 97% |
| Class A2 | -0.01% | -0.08% | -0.09% | 98% | 100% |
| Class B | -0.01% | -0.03% | 0.04% | 103% | 107% |
| Class C | 0.02% | 0.01% | 0.04% | 100% | 115% |
| Class E | 0.01% | -0.03% | 0.02% | 99% | 111% |
| Overall | 0.00% | -0.03% | 0.00% | 100% | 107% |
| Class D | 0.02% | 0.28% | 0.02% | 96% | 97% |
| Class F | -0.02% | 0.07% | 0.06% | 105% | 109% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.20% | 98% | 95% |
| Class A2 | 0.03% | 0.05% | 0.07% | 102% | 99% |
| Class B | 0.03% | -0.01% | -0.18% | 100% | 98% |
| Class C | 0.02% | -0.21% | -0.10% | 102% | 111% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.05% | -0.04% | 101% | 101% |
| Class D | 0.06% | -0.09% | -0.02% | 96% | 94% |
| Class F | 0.05% | -0.08% | 0.31% | 100% | 99% |

Table 5. Regular intra flag before MIP and improvement of Planar and MPM index coding

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.16% | -0.07% | 100% | 97% |
| Class A2 | -0.08% | -0.11% | -0.12% | 100% | 102% |
| Class B | -0.11% | -0.12% | -0.10% | 100% | 101% |
| Class C | -0.04% | -0.03% | 0.01% | 99% | 107% |
| Class E | -0.09% | -0.31% | -0.16% | 98% | 100% |
| Overall | -0.09% | -0.14% | -0.08% | 100% | 101% |
| Class D | -0.03% | 0.09% | -0.16% | 99% | 103% |
| Class F | -0.05% | -0.02% | -0.01% | 100% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.14% | -0.07% | 101% | 99% |
| Class C | 0.01% | -0.01% | 0.14% | 103% | 111% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.04% | 0.24% | 0.15% | 100% | 101% |
| Class F | -0.03% | 0.05% | 0.05% | 102% | 102% |

# Conclusions

This proposal proposes several approaches to make the signalling syntax more logical and simpler such that redundant conditional check can be avoided. It is recommended to adopt signalling proposed Regular intra flag into the next version of VVC.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).