| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0654-v3 |
| --- | --- |

| Title: | AHG12/Non-CE5: Unification of boundary handling for adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated. |  |  |

_____________________________

# Abstract

In VTM-5.0, there are two ways to handle CTU boundaries for adaptive loop filter. Along picture and virtual boundaries, pixels are padded towards outside of the boundaries. On the other hand, in the line buffer reduction mechanism, re-scaled gradient and truncated symmetrical filtering are applied. It is desired to unify the CTU boundaries handling.

# Introduction

In VTM-5.0, there are two ways to handle unavailable pixels beyond boundaries for adaptive loop filter (ALF).

- Boundary handling 1 (padding): For picture/tile/slice and virtual boundaries, pixels are padded towards outside of the boundaries. Then after padding, Laplacian calculation and filtering process are performed without changes.

- Boundary handling 2 (line buffer): To reduce the line buffers caused by ALF between two coding tree units (CTUs), the samples located beyond the line buffer boundary are excluded in ALF process, where Laplacian is re-scaled and truncated symmetrical filtering is applied.

In the current VVC design, both boundary handlings need to be supported and handling for the same boundary is different depending on the type of the boundary. Therefore, it is desired to unify these two methods to handle boundaries for ALF.

# Solutions

There are two possible unifications by reusing existed methods and applying them for all types of boundaries:

- apply boundary handling 1 (padding)

- apply boundary handling 2 (line buffer reduction method)

# Experimental results

## Experimental results of method 1

## The experiments are performed in VTM-5.0 with common test conditions defined in JVET-N1010. SIMD for ALF is disabled in tests

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.09% | -0.07% | 101% | 151% |
| Class A2 | 0.03% | -0.02% | -0.02% | 101% | 142% |
| Class B | 0.02% | -0.03% | -0.03% | 100% | 137% |
| Class C | 0.00% | -0.03% | -0.03% | 99% | 127% |
| Class E | 0.05% | -0.01% | -0.02% | 100% | 146% |
| Overall | 0.03% | -0.03% | -0.03% | 100% | 139% |
| Class D | 0.01% | -0.02% | -0.01% | 100% | 125% |
| Class F | 0.00% | -0.03% | -0.03% | 101% | 136% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.13% | 0.10% | 100% | 157% |
| Class A2 | 0.02% | 0.10% | -0.02% | 100% | 153% |
| Class B | 0.04% | -0.07% | -0.11% | 100% | 147% |
| Class C | -0.03% | -0.11% | -0.23% | 100% | 142% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.06% | -0.08% | 100% | 149% |
| Class D | 0.00% | -0.01% | 0.23% | 99% | 138% |
| Class F | -0.02% | 0.03% | -0.01% | 99% | 132% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.03% | 0.04% | 100% | 149% |
| Class C | -0.06% | -0.09% | -0.21% | 100% | 145% |
| Class E | 0.14% | 0.00% | -0.61% | 100% | 136% |
| Overall | 0.03% | -0.02% | -0.21% | 100% | 144% |
| Class D | -0.03% | -0.34% | -0.03% | 100% | 145% |
| Class F | 0.07% | 0.09% | -0.02% | 99% | 140% |

## Experimental results of method 2

In this section, multiple-slice mode is tested based on VTM-5.0. In the tested multiple-slice mode, there are 2 slices in horizontal direction and 2 slices in vertical direction in one picture with uniform partition. In addition to common test condition for VTM-5.0, the following multiple-slice configuration is used:

SliceMode: 	3

SliceArgument:    1

NumTileColumnsMinus1: 1

NumTileRowsMinus1: 1

TileUniformSpacing: 1

Note that there are problems with VTM-5.0 with the above multiple-slice test condition for Class F. Therefore, class F results are not valid here.

### Performance of disabling loop filter crossing slice boundaries

At first, in the way described in JVET-N1001-v8, disabling ALF crossing slice boundaries is implemented in VTM-5.0,

Anchor 3.2.1:

VTM-5.0 + multiple-slice mode

Test 3.2.1:

VTM-5.0 + multiple-slice mode + disabling loop filter crossing slice boundaries (LFCrossSliceBoundaryFlag: 0)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor 3.2.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.20% | 0.21% | 99% | 136% |
| Class A2 | 0.03% | 0.12% | 0.09% | 99% | 130% |
| Class B | 0.07% | 0.20% | 0.28% | 99% | 126% |
| Class C | 0.10% | 0.30% | 0.42% | 100% | 123% |
| Class E | 0.20% | 0.60% | 0.67% | 99% | 133% |
| Overall | 0.09% | 0.27% | 0.33% | 99% | 129% |
| Class D | 0.12% | 0.55% | 0.65% | 100% | 120% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over Anchor 3.2.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.26% | 0.12% | 99% | 139% |
| Class A2 | 0.02% | 0.11% | 0.00% | 100% | 135% |
| Class B | 0.06% | 0.19% | 0.28% | 100% | 129% |
| Class C | 0.06% | 0.04% | 0.17% | 98% | 125% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.15% | 0.16% | 99% | 131% |
| Class D | 0.02% | -0.03% | 0.08% | 97% | 123% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over Anchor 3.2.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.10% | 0.32% | 99% | 136% |
| Class C | 0.06% | 0.10% | 0.00% | 100% | 135% |
| Class E | -0.05% | 0.15% | -0.03% | 100% | 127% |
| Overall | 0.03% | 0.03% | 0.13% | 99% | 133% |
| Class D | 0.09% | -0.14% | -0.20% | 99% | 130% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

### Performance of applying line buffer reduction method to all horizontal boundaries

Anchor 3.2.2: Test 3.2.1

Test      3.2.2: Test 3.2.1 + applying line buffer reduction method to all horizontal boundaries

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over Anchor 3.2.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.02% | 100% | 112% |
| Class A2 | 0.00% | 0.01% | 0.01% | 101% | 109% |
| Class B | 0.00% | 0.01% | 0.01% | 101% | 110% |
| Class C | 0.01% | 0.02% | 0.02% | 100% | 105% |
| Class E | -0.01% | 0.01% | 0.02% | 100% | 107% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 108% |
| Class D | 0.00% | 0.02% | 0.05% | 100% | 104% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random access Main10 |  |  |  |  |
|  | Over Anchor 3.2.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.02% | 100% | 111% |
| Class A2 | 0.01% | -0.07% | 0.04% | 100% | 110% |
| Class B | 0.00% | 0.07% | -0.04% | 100% | 110% |
| Class C | -0.01% | 0.06% | 0.02% | 101% | 110% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.00% | 100% | 110% |
| Class D | 0.03% | 0.14% | 0.23% | 102% | 110% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over Anchor 3.2.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.32% | 0.34% | 100% | 111% |
| Class C | -0.03% | 0.04% | 0.29% | 100% | 110% |
| Class E | 0.10% | 0.24% | 0.05% | 101% | 106% |
| Overall | 0.02% | 0.20% | 0.25% | 100% | 109% |
| Class D | 0.00% | -0.28% | -0.03% | 100% | 112% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

### Performance of applying line buffer reduction method to all horizontal and vertical boundaries

Anchor 3.2.3: Test 3.2.1

Test      3.2.3: Test 3.2.1 + applying line buffer reduction method to all horizontal and vertical boundaries

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Anchor 3.2.3 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.02% | 100% | 114% |
| Class A2 | 0.01% | 0.01% | 0.01% | 100% | 113% |
| Class B | 0.01% | 0.01% | 0.02% | 100% | 114% |
| Class C | 0.02% | 0.03% | 0.04% | 100% | 107% |
| Class E | 0.00% | 0.02% | 0.01% | 100% | 114% |
| Overall | 0.01% | 0.02% | 0.02% | 100% | 112% |
| Class D | 0.01% | 0.04% | 0.08% | 100% | 107% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over Anchor 3.2.3 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | 0.06% | 100% | 115% |
| Class A2 | 0.00% | -0.10% | 0.01% | 99% | 115% |
| Class B | 0.01% | 0.00% | 0.00% | 100% | 115% |
| Class C | 0.02% | 0.06% | 0.07% | 101% | 113% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.02% | 0.03% | 100% | 115% |
| Class D | 0.07% | 0.12% | 0.17% | 101% | 114% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over Anchor 3.2.3 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.29% | 0.27% | 101% | 116% |
| Class C | 0.03% | -0.13% | 0.08% | 100% | 114% |
| Class E | 0.16% | -0.18% | -0.22% | 100% | 110% |
| Overall | 0.06% | 0.03% | 0.08% | 100% | 114% |
| Class D | 0.05% | -0.43% | 0.21% | 100% | 111% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Modified working draft on top of JVET-N1001-v9

## Method 1

  - The reconstructed sample offsets r1, r2 and r3 are specified in Table 8-23 according to the horizontal luma sample position y and applyVirtualBoundary.

  - When applyVirtualBoundary is true, the follow applies

    - When y < CtbSizeY – 4, the follow applies

vy + j = y + j >= CtbSizeY – 4 ? yCtb + CtbSizeY - 5: vy + j

    - When y >= CtbSizeY – 4, the follow applies

vy + j = y + j <CtbSizeY – 4 ? yCtb + CtbSizeY – 4 :vy + j

  - The variable curr is derived as follows:

curr = recPictureL[ hx, vy ]		(8-1219)

  - The variable sum is derived as follows:

sum = f[ idx[ 0 ] ]   * (  Clip3( −c[ idx[ 0 ] ], c[ idx[ 0 ] ],     recPictureL[ hx, vy + r33 ] − curr ) + 
		Clip3( −c[ idx[ 0 ] ], c[ idx[ 0 ] ],     recPictureL[ hx, vy − r33 ] − curr ) ) + 
	f[ idx[ 1 ] ]   * (  Clip3( −c[ idx[ 1 ] ], c[ idx[ 1 ] ],     recPictureL[ hx + 1, vy + r22 ] − curr ) + 
		Clip3( −c[ idx[ 1 ] ], c[ idx[ 1 ] ],     recPictureL[ hx − 1, vy − r22 ] − curr ) ) + 
	f[ idx[ 2 ] ]   * (  Clip3( −c[ idx[ 2 ] ], c[ idx[ 2 ] ],     recPictureL[ hx, vy + r22 ] − curr ) + 
		Clip3( −c[ idx[ 2 ] ], c[ idx[ 2 ] ],     recPictureL[ hx, vy − r22 ] − curr ) ) + 
	f[ idx[ 3 ] ]   * (  Clip3( −c[ idx[ 3 ] ], c[ idx[ 3 ] ],     recPictureL[ hx − 1, vy + r22 ] − curr ) + 
		Clip3( −c[ idx[ 3 ] ], c[ idx[ 3 ] ],     recPictureL[ hx + 1, vy − r22 ] − curr ) ) + 
	f[ idx[ 4 ] ]   * (  Clip3( −c[ idx[ 4 ] ], c[ idx[ 4 ] ],     recPictureL[ hx + 2, vy + r11 ] − curr ) + 
		Clip3( −c[ idx[ 4 ] ], c[ idx[ 4 ] ],     recPictureL[ hx − 2, vy − r11 ] − curr ) ) + 
	f[ idx[ 5 ] ]   * (  Clip3( −c[ idx[ 5 ] ], c[ idx[ 5 ] ],     recPictureL[ hx + 1, vy + r11 ] − curr ) + 
		Clip3( −c[ idx[ 5 ] ], c[ idx[ 5 ] ],     recPictureL[ hx − 1, vy − r11 ] − curr ) ) + 
	f[ idx[ 6 ] ]   * (  Clip3( −c[ idx[ 6 ] ], c[ idx[ 6 ] ],     recPictureL[ hx, vy + r11 ] − curr ) + 
		Clip3( −c[ idx[ 6 ] ], c[ idx[ 6 ] ],     recPictureL[ hx, vy − r11 ] − curr ) ) + 	(8-1220)
	f[ idx[ 7 ] ]   * (  Clip3( −c[ idx[ 7 ] ], c[ idx[ 7 ] ],     recPictureL[ hx − 1, vy + r11 ] − curr ) + 
		Clip3( −c[ idx[ 7 ] ], c[ idx[ 7 ] ],     recPictureL[ hx + 1, vy − r11 ] − curr ) ) + 
	f[ idx[ 8 ] ]   * (  Clip3( −c[ idx[ 8 ] ], c[ idx[ 8 ] ],     recPictureL[ hx − 2, vy + r11 ] − curr ) + 
		Clip3( −c[ idx[ 8 ] ], c[ idx[ 8 ] ],     recPictureL[ hx + 2, vy − r11 ] − curr ) ) + 
	f[ idx[ 9 ] ]   * (  Clip3( −c[ idx[ 9 ] ], c[ idx[ 9 ] ],     recPictureL[ hx + 3, vy ] − curr ) + 
		Clip3( −c[ idx[ 9 ] ], c[ idx[ 9 ] ],     recPictureL[ hx − 3, vy ] − curr ) ) + 
	f[ idx[ 10 ] ] * (  Clip3( −c[ idx[ 10 ] ], c[ idx[ 10 ] ], recPictureL[ hx + 2, vy ] − curr ) + 
		Clip3( −c[ idx[ 10 ] ], c[ idx[ 10 ] ], recPictureL[ hx − 2, vy ] − curr ) ) + 
	f[ idx[ 11 ] ] * (  Clip3( −c[ idx[ 11 ] ], c[ idx[ 11 ] ], recPictureL[ hx + 1, vy ] − curr ) + 
		Clip3( −c[ idx[ 11 ] ], c[ idx[ 11 ] ], recPictureL[ hx − 1, vy ] − curr ) )

Table 8-23 – Specification of r1, r2, and r3 according to the horizontal luma sample position y and applyVirtualBoundary

| condition | r1 | r2 | r3 |
| --- | --- | --- | --- |
| ( y = = CtbSizeY − 5 ¦ ¦ y = = CtbSizeY − 4 ) && ( applyVirtualBoundary = = 1 ) | 0 | 0 | 0 |
| ( y = = CtbSizeY − 6 ¦ ¦ y = = CtbSizeY − 3 ) && ( applyVirtualBoundary = = 1 ) | 1 | 1 | 1 |
| ( y = = CtbSizeY − 7 ¦ ¦ y = = CtbSizeY − 2 ) && ( applyVirtualBoundary = = 1 ) | 1 | 2 | 2 |
| otherwise | 1 | 2 | 3 |

- The variables minY, maxY and ac are derived as follows:

- If ( y << 2 ) is equal to ( CtbSizeY − 8 ) and ( yCtb + CtbSizeY ) is less than pic_height_in_luma_samples − 1, minY is set equal to −2, maxY is set equal to 3 and ac is set equal to 96.

- Otherwise, if ( y << 2 ) is equal to ( CtbSizeY − 4 ) and ( yCtb + CtbSizeY ) is less than pic_height_in_luma_samples − 1, minY is set equal to 0, maxY is set equal to 5 and ac is set equal to 96.

- Otherwise, minY is set equal to −2 and maxY is set equal to 5 and ac is set equal to 64.

chroma filtering

  - The reconstructed sample offsets r1 and r2 are specified in Table 8-23 according to the horizontal luma sample position y and applyVirtualBoundary.

  - When applyVirtualBoundary is true, the follow applies

    - When y < ctbHeightC – 2, the follow applies

vy + j = y + j >= ctbHeightC – 2 ? yCtbC + ctbHeightC - 3: vy + j

    - When y >= ctbHeightC – 2, the follow applies

vy + j = y + j < ctbHeightC – 2 ? yCtbC + ctbHeightC – 2 :vy + j

  - The variable sum is derived as follows:

sum = f[ 0 ] * (  Clip3( −c[ 0 ], c[ 0 ], recPicture[ hx, vy + r22 ] − curr ) + 
		Clip3( −c[ 0 ], c[ 0 ], recPicture[ hx, vy − r22 ] − curr ) ) + 
	f[ 1 ] * ( Clip3( −c[ 1 ], c[ 1 ], recPicture[ hx + 1, vy + r11 ] − curr ) + 
		Clip3( −c[ 1 ], c[ 1 ], recPicture[ hx − 1, vy − r11 ] − curr ) ) + 
	f[ 2 ] * ( Clip3( −c[ 2 ], c[ 2 ], recPicture[ hx, vy + r11 ] − curr ) + 
		Clip3( −c[ 2 ], c[ 2 ], recPicture[ hx, vy − r11 ] − curr ) ) + 	(8-1272)
	f[ 3 ] * ( Clip3( −c[ 3 ], c[ 3 ], recPicture[ hx − 1, vy + r11 ] − curr ) + 
		Clip3( −c[ 3 ], c[ 3 ], recPicture[ hx + 1, vy − r11 ] − curr ) ) + 
	f[ 4 ] * ( Clip3( −c[ 4 ], c[ 4 ], recPicture[ hx + 2, vy ] − curr ) + 
		Clip3( −c[ 4 ], c[ 4 ], recPicture[ hx − 2, vy ] − curr ) ) + 
	f[ 5 ] * ( Clip3( −c[ 5 ], c[ 5 ], recPicture[ hx + 1, vy ] − curr ) + 
		Clip3( −c[ 5 ], c[ 5 ], recPicture[ hx − 1, vy ] − curr ) )

sum = curr + ( sum + 64 ) >> 7 )		(8-1273)

Table 8-24 – Specification of r1 and r2 according to the horizontal luma sample position y and applyVirtualBoundary

| condition | r1 | r2 |
| --- | --- | --- |
| ( y = = ctbHeightC − 2 ¦ ¦ y = = ctbHeightC − 3 ) && ( applyVirtualBoundary = = 1 ) | 0 | 0 |
| ( y = = ctbHeightC − 1 ¦ ¦ y = = ctbHeightC − 4 ) && ( applyVirtualBoundary = = 1 ) | 1 | 1 |
| otherwise | 1 | 2 |

## Method 2

Luma filtering

  - The variable picture_boundary_x is set as false.

  - The locations ( hx + i, vy + j ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples with i, j = −3..3 are derived as follows:

  - If xCtb + x is less than 3, picture_boundary_x is set as true and boundary_x_left is set as 0

  - If pic_width_in_luma_samples - xCtb - x is less than 4,  picture_boundary_x is set as true and boundary_x_right  is set as pic_width_in_luma_samples

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1 and xCtb + x − PpsVirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_ver_virtual_boundaries − 1, picture_boundary_x is set as true, boundary_x_left is set as PpsVirtualBoundariesPosX[ n ]   the following applies:

hx + i = Clip3( PpsVirtualBoundariesPosX[ n ], pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1213)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1 and PpsVirtualBoundariesPosX[ n ]− xCtb − x is greater than 0 and less than 4 for any n = 0..pps_num_ver_virtual_boundaries − 1, picture_boundary_x is set as true, boundary_x_right is set as PpsVirtualBoundariesPosX[ n ]   the following applies:

hx + i = Clip3( 0, PpsVirtualBoundariesPosX[ n ] − 1, xCtb + x + i )	(8-1214)

  - Otherwise, the following applies:

hx + i = Clip3( 0, pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1215)

  - The variable picture_boundary_y is set as false.

  - If yCtb + y is less than 3, picture_boundary_y is set as true and boundary_y_up is set as 0

  - If pic_height_in_luma_samples - yCtb - y is less than 4,  picture_boundary_y is set as true and boundary_y_down  is set as pic_height_in_luma_samples

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1 and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, picture_boundary_y is set as true and boundary_y_up is set as PpsVirtualBoundariesPosY[ n ] the following applies:

vy + j = Clip3( PpsVirtualBoundariesPosY[ n ], pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1216)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1 and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 4 for any n = 0..pps_num_hor_virtual_boundaries − 1, picture_boundary_y is set as true and boundary_y_down is set as PpsVirtualBoundariesPosY[ n ] the following applies:

vy + j = Clip3( 0, PpsVirtualBoundariesPosY[ n ] − 1, yCtb + y + j )	(8-1217)

  - Otherwise, the following applies:

vy + j = Clip3( 0, pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1218)

  - The variable applyVirtualBoundary is derived as follows:

  - If one or more of the following conditions are true, applyVirtualBoundary is set equal to 0:

  - The bottom boundary of the current coding tree block is the bottom boundary of the picture.

  - The bottom boundary of the current coding tree block is the bottom boundary of the brick and loop_filter_across_bricks_enabled_flag is equal to 0.

  - The bottom boundary of the current coding tree block is the bottom boundary of the slice and loop_filter_across_slices_enabled_flag is equal to 0.

  - The bottom boundary of the current coding tree block is one of the bottom virtual boundaries of the picture and pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1.

  - Otherwise, applyVirtualBoundary is set equal to 1.

  - The reconstructed sample offsets r1, r2 and r3 are specified in Table 8-23 according to the horizontal luma sample position y and applyVirtualBoundary.

  - The variable curr is derived as follows:

curr = recPictureL[ hx, vy ]		(8-1219)

  - The variable sum is derived as follows:

sum = f[ idx[ 0 ] ]   * (  Clip3( −c[ idx[ 0 ] ], c[ idx[ 0 ] ],     recPictureL[ hx, vy + r3 ] − curr ) + 
		Clip3( −c[ idx[ 0 ] ], c[ idx[ 0 ] ],     recPictureL[ hx, vy − r3 ] − curr ) ) + 
	f[ idx[ 1 ] ]   * (  Clip3( −c[ idx[ 1 ] ], c[ idx[ 1 ] ],     recPictureL[ hx + c1, vy + r2 ] − curr ) + 
		Clip3( −c[ idx[ 1 ] ], c[ idx[ 1 ] ],     recPictureL[ hx − c1, vy − r2 ] − curr ) ) + 
	f[ idx[ 2 ] ]   * (  Clip3( −c[ idx[ 2 ] ], c[ idx[ 2 ] ],     recPictureL[ hx, vy + r2 ] − curr ) + 
		Clip3( −c[ idx[ 2 ] ], c[ idx[ 2 ] ],     recPictureL[ hx, vy − r2 ] − curr ) ) + 
	f[ idx[ 3 ] ]   * (  Clip3( −c[ idx[ 3 ] ], c[ idx[ 3 ] ],     recPictureL[ hx − c1, vy + r2 ] − curr ) + 
		Clip3( −c[ idx[ 3 ] ], c[ idx[ 3 ] ],     recPictureL[ hx + c1, vy − r2 ] − curr ) ) + 
	f[ idx[ 4 ] ]   * (  Clip3( −c[ idx[ 4 ] ], c[ idx[ 4 ] ],     recPictureL[ hx + c2, vy + r1 ] − curr ) + 
		Clip3( −c[ idx[ 4 ] ], c[ idx[ 4 ] ],     recPictureL[ hx − c2, vy − r1 ] − curr ) ) + 
	f[ idx[ 5 ] ]   * (  Clip3( −c[ idx[ 5 ] ], c[ idx[ 5 ] ],     recPictureL[ hx + c1, vy + r1 ] − curr ) + 
		Clip3( −c[ idx[ 5 ] ], c[ idx[ 5 ] ],     recPictureL[ hx − c1, vy − r1 ] − curr ) ) + 
	f[ idx[ 6 ] ]   * (  Clip3( −c[ idx[ 6 ] ], c[ idx[ 6 ] ],     recPictureL[ hx, vy + r1 ] − curr ) + 
		Clip3( −c[ idx[ 6 ] ], c[ idx[ 6 ] ],     recPictureL[ hx, vy − r1 ] − curr ) ) + 	(8-1220)
	f[ idx[ 7 ] ]   * (  Clip3( −c[ idx[ 7 ] ], c[ idx[ 7 ] ],     recPictureL[ hx − c1, vy + r1 ] − curr ) + 
		Clip3( −c[ idx[ 7 ] ], c[ idx[ 7 ] ],     recPictureL[ hx + c1, vy − r1 ] − curr ) ) + 
	f[ idx[ 8 ] ]   * (  Clip3( −c[ idx[ 8 ] ], c[ idx[ 8 ] ],     recPictureL[ hx − c2, vy + r1 ] − curr ) + 
		Clip3( −c[ idx[ 8 ] ], c[ idx[ 8 ] ],     recPictureL[ hx + c2, vy − r1 ] − curr ) ) + 
	f[ idx[ 9 ] ]   * (  Clip3( −c[ idx[ 9 ] ], c[ idx[ 9 ] ],     recPictureL[ hx + c3, vy ] − curr ) + 
		Clip3( −c[ idx[ 9 ] ], c[ idx[ 9 ] ],     recPictureL[ hx − c3, vy ] − curr ) ) + 
	f[ idx[ 10 ] ] * (  Clip3( −c[ idx[ 10 ] ], c[ idx[ 10 ] ], recPictureL[ hx + c2, vy ] − curr ) + 
		Clip3( −c[ idx[ 10 ] ], c[ idx[ 10 ] ], recPictureL[ hx − c2, vy ] − curr ) ) + 
	f[ idx[ 11 ] ] * (  Clip3( −c[ idx[ 11 ] ], c[ idx[ 11 ] ], recPictureL[ hx + c1, vy ] − curr ) + 
		Clip3( −c[ idx[ 11 ] ], c[ idx[ 11 ] ], recPictureL[ hx − c1, vy ] − curr ) )

sum = curr + ( ( sum + 64 ) >> 7 )		(8-1221)

  - The modified filtered reconstructed luma picture sample alfPictureL[ xCtb + x ][ yCtb + y ] is derived as follows:

  - If pcm_loop_filter_disabled_flag and pcm_flag[ xCtb+ x ][ yCtb + y ] are both equal to 1, the following applies:

alfPictureL[ xCtb + x ][ yCtb + y ] = recPictureL[ hx, vy ]	(8-1222)

  - Otherwise (pcm_loop_filter_disabled_flag is equal to 0 or pcm_flag[ x ][ y ] is equal 0), the following applies:

alfPictureL[ xCtb + x ][ yCtb + y ] = Clip3( 0, ( 1 << BitDepthY ) − 1, sum )	(8-1223)

r1, r2 and r3 are set as follows

- if  applyVirtualBoundary is true, r1, r2 and r3 are set as

  - if (y = = CtbSizeY − 5  | |  y = = CtbSizeY – 4), r1 = r2 = r3 = 0

  - else if (y = = CtbSizeY − 6  | |  y = = CtbSizeY − 3 ), r1 = r2 = r3 = 1

  - else if (y = = CtbSizeY − 7  | |  y = = CtbSizeY − 2), r1 = 1, r2 = r3 = 2

Table 8-23 – Specification of r1, r2, and r3 according to the horizontal luma sample position y when applyVirtualBoundary is true

| condition | r1 | r2 | r3 |
| --- | --- | --- | --- |
| ( y = = CtbSizeY − 5 ¦ ¦ y = = CtbSizeY − 4 ) && ( applyVirtualBoundary = = 1 ) | 0 | 0 | 0 |
| ( y = = CtbSizeY − 6 ¦ ¦ y = = CtbSizeY − 3 ) && ( applyVirtualBoundary = = 1 ) | 1 | 1 | 1 |
| ( y = = CtbSizeY − 7 ¦ ¦ y = = CtbSizeY − 2 ) && ( applyVirtualBoundary = = 1 ) | 1 | 2 | 2 |
| otherwise | 1 | 2 | 3 |

- else if  picture_boundary_y is true, r1, r2 and r3 are set as

  - If yCtb + y = = boundary_y_up  | |  y + yCtb == boundary_y_down-1, r1 = r2 = r3 = 0

  - else if  yCtb + y = = boundary_y_up  + 1 || y + yCtb == boundary_y_down-2, r1 = r2 = r3 = 1

  - else if  yCtb + y = = boundary_y_up  + 2 || y + yCtb == boundary_y_down-3, r1 = 1, r2 = r3 = 2

- Otherwise, r1 = 1, r2 = 2, r3 = 3

c1, c2 and c3 are set as follows

- if  picture_boundary_x is true, c1, c2 and c3 are set as

  - If xCtb + x = = boundary_x_left  | |  x + xCtb == boundary_x_right-1, c1 = c2 = c3 = 0

  - else if  xCtb + x = = boundary_x_left  + 1 || x + xCtb == boundary_x_right-2, c1 = c2 = c3 = 1

  - else if  xCtb + x = = boundary_x_left  + 2 || x + xCtb == boundary_x_right-3, c1 = 1, c2 = c3 = 2

- Otherwise, c1 = 1, c2 = 2, c3 = 3

classification filter index and transpose index

The locations ( hx + i, vy + j ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples with i, j = −2..5 are derived as follows:

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1 and xCtb + x − PpsVirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 2 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( PpsVirtualBoundariesPosX[ n ], pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1224)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1 and PpsVirtualBoundariesPosX[ n ] − xCtb − x is greater than 0 and less than 6 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( 0, PpsVirtualBoundariesPosX[ n ] − 1, xCtb + x + i )	(8-1225)

  - Otherwise, the following applies:

hx + i = Clip3( 0, pic_width_in_luma_samples − 1, xCtb + x + i )	(8-1226)

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1 and yCtb + y − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 2 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( PpsVirtualBoundariesPosY[ n ], pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1227)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1 and PpsVirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 6 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( 0, PpsVirtualBoundariesPosY[ n ] − 1, yCtb + y + j ) 	(8-1228)

  - Otherwise, the following applies:

  - If yCtb + CtbSizeY is greater than or equal to pic_height_in_luma_samples, the following applies:

vy + j = Clip3( 0, pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1229)

  - Otherwise, if y is less than CtbSizeY − 4, the following applies:

vy + j = Clip3( 0, yCtb + CtbSizeY − 5, yCtb + y + j )	(8-1230)

  - Otherwise, the following applies:

vy + j = Clip3(  yCtb + CtbSizeY − 4, pic_height_in_luma_samples − 1, yCtb + y + j )	(8-1231)

The classification filter index array filtIdx and the transpose index array transposeIdx are derived by the following ordered steps:

- The variables filtH[ x ][ y ], filtV[ x ][ y ], filtD0[ x ][ y ] and filtD1[ x ][ y ] with x, y = − 2..CtbSizeY + 1 are derived as follows:

- If both x and y are even numbers or both x and y are uneven numbers, the following applies:

filtH[ x ][ y ] = Abs( ( recPicture[ hx, vy ]  <<  1 ) − recPicture[ hx − 1, vy ] − 	(8-1232)
	 recPicture[ hx + 1, vy ] )

filtV[ x ][ y ] = Abs( ( recPicture[ hx, vy ]  <<  1 ) − recPicture[ hx, vy − 1 ] − 	(8-1233)
	 recPicture[ hx, vy + 1 ] )

filtD0[ x ][ y ] = Abs( ( recPicture[ hx, vy ]  <<  1 ) − recPicture[ hx − 1, vy − 1 ] − 	(8-1234)
	recPicture[ hx + 1, vy + 1 ] )

filtD1[ x ][ y ] = Abs( ( recPicture[ hx, vy ]  <<  1 ) − recPicture[ hx + 1, vy − 1 ] − 	(8-1235)
	recPicture[ hx − 1, vy + 1 ] )

- Otherwise, filtH[ x ][ y ], filtV[ x ][ y ], filtD0[ x ][ y ] and filtD1[ x ][ y ] are set equal to 0.

- If ( y << 2 ) is equal to ( CtbSizeY − 8 ) and ( yCtb + CtbSizeY ) is less than pic_height_in_luma_samples − 1, minY is set equal to −2, maxY is set equal to 3 and ac is set equal to 96.

- Otherwise, if ( y << 2 ) is equal to ( CtbSizeY − 4 ) and ( yCtb + CtbSizeY ) is less than pic_height_in_luma_samples − 1, minY is set equal to 0, maxY is set equal to 5 and ac is set equal to 96.

- Otherwise, minY is set equal to −2 and maxY is set equal to 5 and ac is set equal to 64.

- The variables minY, maxY are derived as follows:

if  (y << 2) is equal to PpsVirtualBoundariesPosY[ n ] or (y << 2) is equal to 0 or (y << 2) is equal to CtbSizeY – 4, minY = 0;

otherwise minY = -2

if  (y << 2) is equal to PpsVirtualBoundariesPosY[ n ] - 4 or (y << 2) is equal to pic_height_in_luma_samples - 3 or (y << 2) is equal to CtbSizeY – 8, maxY = 3;

otherwise maxY = 5;

- The variables minX, maxX are derived as follows:

if  (x << 2) is equal to PpsVirtualBoundariesPosX[ n ] or (x << 2) is equal to 0, minX = 0;

otherwise minX = -2

if  (x << 2) is equal to PpsVirtualBoundariesPosX[ n ] - 4 or (x << 2) is equal to pic_width_in_luma_samples - 3, maxX = 3;

otherwise maxX = 5;

- The variable ac is derived as follows:

| (maxY – minY + 1) * (maxX – minX + 1) | 8*8 | 8*6 | 8*4 | 6*6 | 6*4 | 4*4 |
| --- | --- | --- | --- | --- | --- | --- |
| ac | 64 | 96 | 128 | 112 | 192 | 256 |

- The variables sumH[ x ][ y ], sumV[ x ][ y ], sumD0[ x ][ y ], sumD1[ x ][ y ] and sumOfHV[ x ][ y ] with x, y = 0..( CtbSizeY − 1 ) >> 2 are derived as follows:

sumH[ x ][ y ] = ij filtH[ h(x  <<  2 ) + i − xCtb ][ v(y  <<  2) + j − yCtb ] with i = −2..5 minX ..maxY, j = minY..maxY	(8-1236)

sumV[ x ][ y ] = ij filtV[ h(x  <<  2 ) + i − xCtb ][ v(y  <<  2) + j − yCtb ] with i = −2..5 minX ..maxY, j = minY..maxY 	(8-1237)

sumD0[ x ][ y ] = ij filtD0[ h(x  <<  2 ) + i − xCtb ][ v(y  <<  2) + j − yCtb ] with i = −2..5 minX ..maxY, j = minY..maxY 	(8-1238)

sumD1[ x ][ y ] = ij filtD1[ h(x  <<  2 ) + i − xCtb ][ v(y  <<  2) + j − yCtb ] with i = −2..5 minX ..maxY, j = minY..maxY	(8-1239)

sumOfHV[ x ][ y ] = sumH[ x ][ y ] + sumV[ x ][ y ]	(8-1240)

- The variables dir1[ x ][ y ], dir2[ x ][ y ] and dirS[ x ][ y ] with x, y = 0..CtbSizeY − 1 are derived as follows:

- The variables hv1, hv0 and dirHV are derived as follows:

- If sumV[ x >> 2 ][ y >> 2 ] is greater than sumH[ x >> 2 ][ y >> 2 ], the following applies:

hv1 = sumV[ x >> 2 ][ y >> 2 ]		(8-1241)

hv0 = sumH[ x >> 2 ][ y >> 2 ] 		8-1242)

dirHV = 1		(8-1243)

- Otherwise, the following applies:

hv1 = sumH[ x >> 2 ][ y >> 2 ]		(8-1244)

hv0 = sumV[ x >> 2 ][ y >> 2 ]		(8-1245)

dirHV = 3		(8-1246)

- The variables d1, d0 and dirD are derived as follows:

- If sumD0[ x >> 2 ][ y >> 2 ] is greater than sumD1[ x >> 2 ][ y >> 2 ], the following applies:

d1 = sumD0[ x >> 2 ][ y >> 2 ]		(8-1247)

d0 = sumD1[ x >> 2 ][ y >> 2 ]		(8-1248)

dirD = 0		(8-1249)

- Otherwise, the following applies:

d1 = sumD1[ x >> 2 ][ y >> 2 ]		(8-1250)

d0 = sumD0[ x >> 2 ][ y >> 2 ]		(8-1251)

dirD = 2		(8-1252)

- The variables hvd1, hvd0, are derived as follows:

hvd1 = ( d1 * hv0 > hv1 * d0 )  ?  d1  :  hv1	(8-1253)

hvd0 = ( d1 * hv0 > hv1 * d0 )  ?  d0  :  hv0	(8-1254)

- The variables dirS[ x ][ y ], dir1[ x ][ y ] and dir2[ x ][ y ] derived as follows:

dir1[ x ][ y ] = ( d1 * hv0 > hv1 * d0 )  ?  dirD  :  dirHV	(8-1255)

dir2[ x ][ y ] = ( d1 * hv0 > hv1 * d0 )  ?  dirHV  :  dirD	(8-1256)

dirS[ x ][ y ] = ( hvd1 > 2 * hvd0 )  ?  1  :  ( ( hvd1 * 2 > 9 * hvd0 )  ?  2  :  0 )	(8-1257)

- The variable avgVar[ x ][ y ] with x, y = 0..CtbSizeY − 1 is derived as follows:

varTab[ ] = { 0, 1, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 4 }	(8-1258)

avgVar[ x ][ y ] = varTab[ Clip3( 0, 15, ( sumOfHV[ x >> 2 ][ y >> 2 ] * ac ) >> ( 3 + BitDepthY ) ) ]	(8-1259)

- The classification filter index array filtIdx[ x ][ y ] and the transpose index array transposeIdx[ x ][ y ] with x = y = 0..CtbSizeY − 1 are derived as follows:

transposeTable[ ] = { 0, 1, 0, 2, 2, 3, 1, 3 }

transposeIdx[ x ][ y ] = transposeTable[ dir1[ x ][ y ] * 2 + ( dir2[ x ][ y ] >> 1 ) ]

filtIdx[ x ][ y ] = avgVar[ x ][ y ]

When dirS[ x ][ y ] is not equal 0, filtIdx[ x ][ y ] is modified as follows:

filtIdx[ x ][ y ] += ( ( ( dir1[ x ][ y ] & 0x1 ) << 1 ) + dirS[ x ][ y ] ) * 5	(8-1260)

Chroma filtering

For the derivation of the filtered reconstructed chroma samples alfPicture[ x ][ y ], each reconstructed chroma sample inside the current chroma coding tree block recPicture[ x ][ y ] is filtered as follows with x = 0..ctbWidthC − 1, y = 0..ctbHeightC − 1:

  - The variable picture_boundary_x is set as false.

  - The locations ( hx + i, vy + j ) for each of the corresponding chroma samples ( x, y ) inside the given array recPicture of chroma samples with i, j = −2..2 are derived as follows:

  - If xCtbC + x is less than 2, picture_boundary_x is set as true and boundary_x_left is set as 0

  - If pic_width_in_luma_samples / SubWidthC - xCtbC - x is less than 3,  picture_boundary_x is set as true and boundary_x_right  is set as pic_width_in_luma_samples / SubWidthC

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1 and xCtbC + x − PpsVirtualBoundariesPosX[ n ] / SubWidthC is greater than or equal to 0 and less than 2 for any n = 0..pps_num_ver_virtual_boundaries − 1, picture_boundary_x is set as true, boundary_x_left is set as PpsVirtualBoundariesPosX[ n ] / SubWidthC the following applies:

hx + i = Clip3( PpsVirtualBoundariesPosX[ n ] / SubWidthC,	(8-1263)
			pic_width_in_luma_samples / SubWidthC − 1, xCtbC + x + i )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1 and PpsVirtualBoundariesPosX[ n ] / SubWidthC − xCtbC − x is greater than 0 and less than 3 for any n = 0..pps_num_ver_virtual_boundaries − 1, picture_boundary_x is set as true, boundary_x_right is set as PpsVirtualBoundariesPosX[ n ] / SubWidthC the following applies:

hx + i = Clip3( 0, PpsVirtualBoundariesPosX[ n ] / SubWidthC − 1, xCtbC + x + i )	(8-1264)

  - Otherwise, the following applies:

hx + i = Clip3( 0, pic_width_in_luma_samples / SubWidthC − 1, xCtbC + x + i )	(8-1265)

  - The variable picture_boundary_y is set as false.

  - If yCtbC + y is less than 2, picture_boundary_y is set as true and boundary_y_up is set as 0

  - If pic_height_in_luma_samples - yCtbC - y is less than 3,  picture_boundary_y is set as true and boundary_y_down  is set as pic_height_in_luma_samples / SubHeightC

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1 and yCtbC + y − PpsVirtualBoundariesPosY[ n ] / SubHeightC is greater than or equal to 0 and less than 2 for any n = 0..pps_num_hor_virtual_boundaries − 1, picture_boundary_y is set as true and boundary_y_up is set as PpsVirtualBoundariesPosY[ n ] / SubHeightC the following applies:

vy + j = Clip3( PpsVirtualBoundariesPosY[ n ] / SubHeightC ,	(8-1266)
			pic_height_in_luma_samples / SubHeightC − 1, yCtbC + y + j )

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1 and PpsVirtualBoundariesPosY[ n ] / SubHeightC − yCtbC − y is greater than 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, picture_boundary_y is set as true and boundary_y_down is set as PpsVirtualBoundariesPosY[ n ] / SubHeightC the following applies:

vy + j = Clip3( 0, PpsVirtualBoundariesPosY[ n ] / SubHeightC − 1, yCtbC + y + j )	(8-1267)

  - Otherwise, the following applies:

vy + j = Clip3( 0, pic_height_in_luma_samples / SubHeightC − 1, yCtbC + y + j )	(8-1268)

  - The variable applyVirtualBoundary is derived as follows:

  - If one or more of the following conditions are true, applyVirtualBoundary is set equal to 0:

  - The bottom boundary of the current coding tree block is the bottom boundary of the picture.

  - The bottom boundary of the current coding tree block is the bottom boundary of the brick and loop_filter_across_bricks_enabled_flag is equal to 0.

  - The bottom boundary of the current coding tree block is the bottom boundary of the slice and loop_filter_across_slices_enabled_flag is equal to 0.

  - The bottom boundary of the current coding tree block is one of the bottom virtual boundaries of the picture and pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1.

  - Otherwise, applyVirtualBoundary is set equal to 1.

  - The reconstructed sample offsets r1 and r2 are specified in Table 8-23 according to the horizontal luma sample position y and applyVirtualBoundary.

  - The variable curr is derived as follows:

curr = recPicture[ hx, vy ]		(8-1269)

  - The array of chroma filter coefficients f[ j ] and the array of chroma clipping values c[ j ] is derived as follows with j = 0..5:

f[ j ] = AlfCoeffC[ slice_alf_aps_id_chroma ][ j ]		(8-1270)

c[ j ] = AlfClipC[ slice_alf_aps_id_chroma ][ j ]		(8-1271)

  - The variable sum is derived as follows:

sum = f[ 0 ] * (  Clip3( −c[ 0 ], c[ 0 ], recPicture[ hx, vy + r2 ] − curr ) + 
		Clip3( −c[ 0 ], c[ 0 ], recPicture[ hx, vy − r2 ] − curr ) ) + 
	f[ 1 ] * ( Clip3( −c[ 1 ], c[ 1 ], recPicture[ hx + c1, vy + r1 ] − curr ) + 
		Clip3( −c[ 1 ], c[ 1 ], recPicture[ hx − c1, vy − r1 ] − curr ) ) + 
	f[ 2 ] * ( Clip3( −c[ 2 ], c[ 2 ], recPicture[ hx, vy + r1 ] − curr ) + 
		Clip3( −c[ 2 ], c[ 2 ], recPicture[ hx, vy − r1 ] − curr ) ) + 	(8-1272)
	f[ 3 ] * ( Clip3( −c[ 3 ], c[ 3 ], recPicture[ hx − c1, vy + r1 ] − curr ) + 
		Clip3( −c[ 3 ], c[ 3 ], recPicture[ hx + c1, vy − r1 ] − curr ) ) + 
	f[ 4 ] * ( Clip3( −c[ 4 ], c[ 4 ], recPicture[ hx + c2, vy ] − curr ) + 
		Clip3( −c[ 4 ], c[ 4 ], recPicture[ hx −c2, vy ] − curr ) ) + 
	f[ 5 ] * ( Clip3( −c[ 5 ], c[ 5 ], recPicture[ hx + c1, vy ] − curr ) + 
		Clip3( −c[ 5 ], c[ 5 ], recPicture[ hx − c1, vy ] − curr ) )

sum = curr + ( sum + 64 ) >> 7 )		(8-1273)

  - The modified filtered reconstructed chroma picture sample alfPicture[ xCtbC + x ][ yCtbC + y ] is derived as follows:

  - If pcm_loop_filter_disabled_flag and pcm_flag[ ( xCtbC + x ) * SubWidthC ][ ( yCtbC + y ) * SubHeightC ] are both equal to 1, the following applies:

alfPicture[ xCtbC + x ][ yCtbC + y ] = recPictureL[ hx, vy ]	(8-1274)

  - Otherwise (pcm_loop_filter_disabled_flag is equal to 0 or pcm_flag[ x ][ y ] is equal 0), the following applies:

alfPicture[ xCtbC + x ][ yCtbC + y ] = Clip3( 0, ( 1 << BitDepthC ) − 1, sum )	(8-1275)

r1 and r2 are set as follows

- if  applyVirtualBoundary is true, r1 and r2 are set as

  - if (y = = CtbSizeY − 2  | |  y = = CtbSizeY – 3), r1 = r2 = 0

  - else if (y = = CtbSizeY − 1  | |  y = = CtbSizeY − 4 ), r1 = r2 = 1

- else if  picture_boundary_y is true, r1 and r2 are set as

  - If yCtb + y = = boundary_y_up  | |  y + yCtb == boundary_y_down-1, r1 = r2 = 0

  - else if  yCtb + y = = boundary_y_up  + 1 || y + yCtb == boundary_y_down-2, r1 = r2 = 1

- Otherwise, r1 = 1, r2 = 2

c1 and c2 are set as follows

- if  picture_boundary_x is true, c1 and c2 are set as

  - If xCtb + x = = boundary_x_left  | |  x + xCtb == boundary_x_right-1, c1 = c2 = 0

  - else if  xCtb + x = = boundary_x_left  + 1 || x + xCtb == boundary_x_right-2, c1 = c2 = 1

- Otherwise, c1 = 1, c2 = 2

Table 8-24 – Specification of r1 and r2 according to the horizontal luma sample position y and applyVirtualBoundary

| condition | r1 | r2 |
| --- | --- | --- |
| ( y = = ctbHeightC − 2 ¦ ¦ y = = ctbHeightC − 3 ) && ( applyVirtualBoundary = = 1 ) | 0 | 0 |
| ( y = = ctbHeightC − 1 ¦ ¦ y = = ctbHeightC − 4 ) && ( applyVirtualBoundary = = 1 ) | 1 | 1 |
| otherwise | 1 | 2 |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).