| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0669-v3 |
| --- | --- |

| Title: | Non-CE5: A Simplification of ALF Coefficient Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes a simplification by removing the coefficient predictions in ALF signaling. In VTM-5.0, ALF signaling may involve (i) recursive prediction of ALF coefficients and (ii) prediction using fixed filters. It is proposed to remove both prediction steps and relevant syntax elements. The experimental results show that the proposed changes lead to negligible coding losses over VTM-5.0 under CTC.

# Introduction

In VTM-5.0, the adaptive loop filter (ALF) coefficients can be either signaled directly, or coefficients are first predicted then the resulting differences (i.e., delta values) are signalled. Specifically, VTM-5.0 has two prediction methods for ALF coefficient signaling, where

- a recursive prediction is applied if a flag called alf_luma_coeff_delta_prediction_flag is enabled.

- fixed filters are used as predictors if alf_luma_use_fixed_filter_flag is enabled.

This contribution proposes a simplification by removing above prediction processes in VTM-5.0. On average, the proposed simplifications lead to 0.00% BD-rate loss in AI and RA configurations, and they provide 0.04% BD-rate gain in over VTM-5.0 under common test conditions (CTC).

# Description of method

The following specification changes on JVET-N1010-v8 are proposed to remove recursive prediction and fixed filter-based prediction from ALF coefficient signalling.

#### Adaptive loop filter data syntax

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| if( alf_luma_filter_signal_flag ) { |  |
| alf_luma_clip_flag | u(1) |
| alf_luma_num_filters_signalled_minus1 | tb(v) |
| if( alf_luma_num_filters_signalled_minus1 > 0 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | tb(v) |
| } |  |
| alf_luma_use_fixed_filter_flag | u(1) |
| if( alf_luma_use_fixed_filter_flag ) { |  |
| alf_luma_fixed_filter_set_idx | tb(v) |
| alf_luma_fixed_filter_pred_present_flag | u(1) |
| if( alf_luma_fixed_filter_pred_present_flag ) { |  |
| for( i = 0; i < NumAlfFilters; i++ ) |  |
| alf_luma_fixed_filter_pred_flag[ i ] | u(1) |
| } |  |
| } |  |
| alf_luma_coeff_signaldelta_flag | u(1) |
| if( !alf_luma_coeff_delta_flag && alf_luma_num_filters_signalled_minus1 > 0 ) |  |
| alf_luma_coeff_delta_prediction_flag | u(1) |
| alf_luma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_eg_order_increase_flag[ i ] | u(1) |

alf_luma_filter_signal_flag equal to 1 specifies that a luma filter set is signalled. alf_luma_filter_signal_flag equal to 0 specifies that a luma filter set is not signalled.

alf_chroma_filter_signal_flag equal to 1 specifies that a chroma filter is signalled. alf_chroma_filter_signal_flag equal to 0 specifies that a chroma filter is not signalled.

The variable NumAlfFilters specifying the number of different adaptive loop filters is set equal to 25.

alf_luma_clip_flag equal to 0 specifies that linear adaptive loop filtering is applied on luma component. alf_luma_clip_flag equal to 1 specifies that non-linear adaptive loop filtering may be applied on luma component.

alf_luma_num_filters_signalled_minus1 plus 1 specifies the number of adpative loop filter classes for which luma coefficients can be signalled. The value of alf_luma_num_filters_signalled_minus1 shall be in the range of 0 to NumAlfFilters − 1, inclusive.

The maximum value maxVal of the truncated binary binarization tb(v) is set equal to NumAlfFilters − 1.

alf_luma_coeff_delta_idx[ filtIdx ] specifies the indices of the signalled adaptive loop filter luma coefficient deltas for the filter class indicated by filtIdx ranging from 0 to NumAlfFilters − 1. When alf_luma_coeff_delta_idx[ filtIdx ] is not present, it is inferred to be equal to 0.

The maximum value maxVal of the truncated binary binarization tb(v) is set equal to alf_luma_num_filters_signalled_minus1.

alf_luma_use_fixed_filter_flag equal to 1 specifies that a fixed filter set is used to signal adaptive loop filter coefficients. alf_luma_use_fixed_filter_flag equal to 0 specifies that a fixed filter set is not used to signal adaptive loop filter coefficients.

alf_luma_fixed_filter_set_idx specifies a fixed filter set index. The value of alf_luma_fixed_filter_set_idx shall be in a range of 0 to 15, inclusive. The maximum value maxVal of the truncated binary binarization tb(v) is set equal to 15.

alf_luma_fixed_filter_pred_present_flag equal to 1 specifies that alf_luma_fixed_filter_pred_flag[ i ] is present. alf_luma_fixed_filter_pred_present_flag equal to 0 specifies that alf_luma_fixed_filter_pred_flag[ i ] is not present.

alf_luma_fixed_filter_pred_flag[ i ] equal to 1 specifies that a fixed filter is used to predict the i-th filter coefficients. alf_luma_fixed_filter_pred_flag[ i ] equal to 0 specifies that a fixed filter is not used to predict the i-th filter coefficients. When alf_luma_fixed_filter_pred_flag[ i ] is not present, it is inferred to be equal to 1.

alf_luma_coeff_signaldelta_flag equal to 1 indicates that alf_luma_coeff_flag[ sfIdx]    alf_luma_coeff_delta_prediction_flag is not is signalled. alf_luma_coeff_signaldelta_flag equal to 0 indicates that alf_luma_coeff_delta_prediction_flag may be is alf_luma_coeff_flag[ sfIdx] not signalled.

alf_luma_coeff_delta_prediction_flag equal to 1 specifies that the signalled luma filter coefficient deltas are predicted from the deltas of the previous luma coefficients. alf_luma_coeff_delta_prediction_flag equal to 0 specifies that the signalled luma filter coefficient deltas are not predicted from the deltas of the previous luma coefficients. When not present, alf_luma_coeff_delta_prediction_flag is inferred to be equal to 0.

alf_luma_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for luma filter coefficient signalling. The value of alf_luma_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_luma_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for luma filter coefficient signalling is incremented by 1. alf_luma_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for luma filter coefficient signalling is not incremented by 1.

The order expGoOrderY[ i ] of the exp-Golomb code used to decode the values of alf_luma_coeff_delta_abs[ sfIdx ][ j ] is derived as follows:

expGoOrderY[ i ] = ( i = = 0  ?  alf_luma_min_eg_order_minus1 + 1  :  expGoOrderY[ i − 1 ] ) +	(7-66)
	alf_luma_eg_order_increase_flag[ i ]

alf_luma_coeff_flag[ sfIdx ] equal 1 specifies that the coefficients of the luma filter indicated by sfIdx are signalled. alf_luma_coeff_flag[ sfIdx ] equal to 0 specifies that all filter coefficients of the luma filter indicated by sfIdx are set equal to 0. When not present, alf_luma_coeff_flag[ sfIdx ] is set equal to 1.

alf_luma_coeff_delta_abs[ sfIdx ][ j ] specifies the absolute value of the j-th coefficient delta of the signalled luma filter indicated by sfIdx. When alf_luma_coeff_delta_abs[ sfIdx ][ j ] is not present, it is inferred to be equal 0.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

golombOrderIdxY[ ] = { 0, 0, 1, 0, 0, 1, 2, 1, 0, 0, 1, 2 }		(7-67)

k = expGoOrderY[ golombOrderIdxY[ j ] ]		(7-68)

alf_luma_coeff_delta_sign[ sfIdx ][ j ] specifies the sign of the j-th luma coefficient of the filter indicated by sfIdx as follows:

- If alf_luma_coeff_delta_sign[ sfIdx ][ j ] is equal to 0, the corresponding luma filter coefficient has a positive value.

- Otherwise (alf_luma_coeff_delta_sign[ sfIdx ][ j ] is equal to 1), the corresponding luma filter coefficient has a negative value.

When alf_luma_coeff_delta_sign[ sfIdx ][ j ] is not present, it is inferred to be equal to 0.

The variable filtCoeff[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1, j = 0..11 is initialized as follows:

filtCoeff[ sfIdx ][ j ] = alf_luma_coeff_delta_abs[ sfIdx ][ j ] * 	(7-69)
	( 1 − 2 * alf_luma_coeff_delta_sign[ sfIdx ][ j ] )

When alf_luma_coeff_delta_prediction_flag is equal 1, filtCoeff[ sfIdx ][ j ] with sfIdx = 1..alf_luma_num_filters_signalled_minus1 and j = 0..11 are modified as follows:

filtCoeff[ sfIdx ][ j ] += filtCoeff[ sfIdx − 1 ][ j ]		(7-70)

The luma filter coefficients AlfCoeffL[ adaptation_parameter_set_id ] with elements AlfCoeffL[ adaptation_parameter_set_id ][ filtIdx ][ j ], with filtIdx = 0..NumAlfFilters − 1 and j = 0..11 are derived as follows:

AlfCoeffL[ adaptation_parameter_set_id ][ filtIdx ][ j ] = filtCoeff[ alf_luma_coeff_delta_idx[ filtIdx ] ][ j ]	(7-71)

When alf_luma_use_fixed_filter_flag is equal to 1 and alf_luma_fixed_filter_pred_flag[ filtIdx ] is equal to 1, the following applies for filtIdx = 0..NumAlfFilters − 1 and j = 0..11:

AlfCoeffL[ adaptation_parameter_set_id ][ filtIdx ][ j ] += 	(7-72)
	AlfFixFiltCoeff[ AlfClassToFiltMap[ alf_luma_fixed_filter_set_idx ][ filtIdx ] ][ j ]

The fixed filter coefficients AlfFixFiltCoeff[ i ][ j ] with i = 0..64, j = 0..11 and the class to filter mapping AlfClassToFiltMap[ m ][ n ] with m = 0..15 and n = 0..24 are derived as follows:

# Experimental results

The methods described in Section 2 are implemented in VTM-5.0, and the test results are presented in the following tables. Table 1 and 2 show separate results for removing recursive prediction of ALF coefficients and fixed filter-based prediction, respectively. The results of combining both simplifications are presented in Table 3.

Table 1: Results of removing recursive prediction in ALF coefficient signaling

Table 2: Results of fixed filter-based prediction in ALF coefficient signaling

Table 3: Results of combining both simplifications in ALF coefficient signaling

# Conclusions

This contribution proposes removal of the prediction steps of ALF coefficient in VTM-5.0, and the experimental results show that both recursive and fixed-filter based prediction do not have any benefit in terms of coding efficient. It is proposed to adopt both simplifications in VVC.

# Reference

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th meeting, Geneva, CH, 19-27 March 2019.

[2] N. Hu, V. Seregin, H. E. Egilmez, M. Karczewicz, “CE5: Coding tree block based adaptive loop filter (CE5-4)”, JVET-N0415, 14th meeting, Geneva, CH, 19-27 March 2019.

[3] J. Taquet, P. Onno, C. Gisquet, G. Laroche, “CE5: Results of tests CE5-3.1, CE5-3.2, CE5-3.3 and CE5-3.4 on Non-Linear Adaptive Loop Filter”, JVET-N0242, 14th meeting, Geneva, CH, 19-27 March 2019.

[4] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, 14th meeting, Geneva, CH, 19-27 March 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).