| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0673-v4 |
| --- | --- |

| Title: | Harmonized MIP/CCLM boundary downsampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thibaud Biatek, Geert Van Der Auwera, Marta Karczewicz | Email: | tbiatek@qti.qualcomm.com geertv@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes harmonization of boundary downsampling used in both MIP and CCLM. Results show limited losses over anchor with 0.04%(Y)/0.34%(U)/0.36%(V) BD-Rate in all-intra and 0.05%(Y)/0.47%(U)/0.43%(V) BD-Rate in random-access.

# Introduction

In the current VVC specification the prediction of a given block using MIP follows three steps:

- Reduced block boundary is computed via averaging the samples. The output vector is used as input for the prediction.

- Matrix-vector multiplication with addition of an offset is achieved with reduced-boundary reference vector as input. The output is a low-resolution prediction.

- Linear interpolation is achieved on both row and column if needed to generate full-resolution prediction signal followed by a clipping operation.

For CCLM, the chroma samples are predicted based on the reconstructed luma samples of the same CU by using a linear model as follows:

predCi, j=α·recL'i, j+ βpredCi,j=α·recL'i,j+β

The CCLM parameters (αα and ββ) are derived with at most four neighbouring chroma samples and their corresponding downsampled luma samples. Two types of downsampling filters are applied to luma samples to achieve 2 to 1 downsampling ratio in both horizontal and vertical directions.

It is noticed that both CCLM and MIP downsample their neighbouring luma samples during prediction. However, MIP uses simple averaging while CCLM uses up to 6-tap filters. In this contribution, it is proposed to harmonize boundary downsampling of both MIP and CCLM. Benefits are reduced complexity in (αα and ββ) parameters derivation and harmonization of MIP/CCLM boundary down-sampling functions.

# Proposed Method

This proposal simplifies the down-sampling process of neighbouring luma samples required to derive (αα and ββ) parameters in CCLM.

Luma/chroma sample alignment related to the sps_cclm_colocated_chroma_flag is not considered during the luma boundary downsampling process. Top row and left columns samples are used as input to the MIP downsampler. Illustration of luma positions after downsampling are provided in Figure 1.

Figure 1: Luma location after downsampling (red)

The rest of the CCLM process remains unchanged. Suppose the current chroma block dimensions are W×H, then W’ and H’ are set as

- W’=W, H’=H when LM mode is applied;

- W’=W+H when LM-A mode is applied;

- H’=H+W when LM-L mode is applied;

For the above neighbouring positions denoted as S[0, -1]…S[W’-1, -1] and the left neighbouring positions denoted as S[-1, 0]…S[-1, H’-1], four neighbouring samples are selected:

- S[W’/4, -1], S[3W’/4, -1], S[-1, H’/4], S[-1, 3H’/4] when LM mode is applied and both above and left neighbouring samples are available;

- S[W’/8, -1], S[3W’/8, -1], S[5W’/8, -1], S[7W’/8, -1] when LM-A mode is applied or only the above neighbouring samples are available;

- S[-1, H’/8], S[-1, 3H’/8], S[-1, 5H’/8], S[-1, 7H’/8] when LM-L mode is applied or only the left neighbouring samples are available;

The four selected neighbouring values are compared four times to find two smaller values: x0A and x1A, and two larger values: x0B and x1B. Their corresponding chroma sample values are denoted as y0A, y1A, y0B and y1B. Then xA, xB, yA and yB are derived as:

Xa=(x0A + x1A +1)>>1; Xb=(x0B + x1B +1)>>1; Ya=(y0A + y1A +1)>>1; Yb=(y0B + y1B +1)>>1

Finally, the linear model parameters α and β are obtained according to the following equations.

α=Ya -YbXa-Xb

β=Yb-α·Xb

# Specification changes

“

  - When numSampL is greater than 0, the downsampling function defined in clause 8.4.5.2.3 is invoked as follows:

    - Input:

      - nTbs = SubHeightC * numSampL

      - refS = pY[-2][y], with y=0...nTbs-1

      - boundarySize = numSampL

      - needUpsBdry = 0

    - Output :

      - pDsYL[y], x = 0.. numSampL-1

  - When numSampT is greater than 0, the downsampling function defined in clause 8.4.5.2.3 is invoked as follows:

    - Input:

      - nTbs = SubWidthC * numSampT

      - refS = pY[x][-1], with x=0...nTbs-1

      - boundarySize = numSampT

      - needUpsBdry = 0

    - Output :

      - pDsYT[x], x = 0.. numSampT-1

  - When numSampL is greater than 0, the selected neighbouring left chroma samples pSelC[ idx ] are set equal to p[ −1 ][ pickPosL[ idx ] ] with idx = 0..cntL − 1, and the selected down-sampled neighbouring left luma samples pSelDsY[ idx ] with idx = 0..cntL − 1 are derived as follows:

    - The variable y is set equal to pickPosL[ idx ].

    - If both SubWidthC and SubHeightC are equal to 1, the following applies:

pSelDsY[ idx ] = pY[ − 1][ y ]			(8-177)

    - Otherwise the following applies:

pSelDsY[idx] = pDsYL[y]

      - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

        - If y is greater than 0 or availTL is equal to TRUE, pSelDsY[ idx ]  is derived as follows:

pSelDsY[ idx ] = ( F3[ 1 ][ 0 ] * pY[ − SubWidthC ][ SubHeightC * y − 1 ] +
		F3[ 0 ][ 1 ] * pY[ −1 − SubWidthC ][ SubHeightC * y ] +
		F3[ 1 ][ 1 ] * pY[ −SubWidthC ][ SubHeightC * y ]  + 	(8-178)
		F3[ 2 ][ 1 ] * pY[ 1 − SubWidthC ][ SubHeightC * y ] +
		F3[ 1 ][ 2 ] * pY[ −SubWidthC ][ SubHeightC * y + 1 ] + 4 ) >> 3

        - Otherwise (y is equal to 0), pSelDsY[ idx ]  is derived as follows:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ −1 − SubWidthC ][ 0 ] +
		F2[ 1 ] * pY[ −SubWidthC ][ 0 ] +	(8-179)
		F2[ 2 ] * pY[ 1 − SubWidthC ][ 0] + 2 ) >> 2

      - Otherwise (sps_cclm_colocated_chroma_flag is equal to 0), the following applies:

pSelDsY[ idx ] = ( F4[ 0 ][ 1 ] * pY[ −1 − SubWidthC ][ SubHeightC * y ] +
		F4[ 0 ][ 2 ] * pY[ −1 − SubWidthC ][ SubHeightC * y + 1 ] +
		F4[ 1 ][ 1 ] * pY[ −SubWidthC ][ SubHeightC * y ] +	(8-180)
		F4[ 1 ][ 2 ] * pY[ −SubWidthC ][ SubHeightC * y + 1] +
		F4[ 2 ][ 1 ] * pY[ 1 − SubWidthC ][ SubHeightC * y ] +
		F4[ 2 ][ 2 ] * pY[ 1 − SubWidthC][ SubHeightC * y + 1 ] + 4 ) >> 3

  - When numSampT is greater than 0, the selected neighbouring top chroma samples pSelC[ idx ] are set equal to p[ pickPosT[ idx − cntL ] ][ -1 ] with  idx = cntL..cntL + cntT − 1, and the down-sampled neighbouring top luma samples pSelDsY[ idx ] with idx = cntL..cntL + cntT – 1 are specified as follows:

    - The variable x is set equal to pickPosT[ idx − cntL ].

    - If both SubWidthC and SubHeightC are equal to 1, the following applies:

pSelDsY[ idx ] = pY[ x ][ − 1]			(8-181)

    - Otherwise, the following applies:

pSelDsY[idx] = pDsYT[y]

      - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

        - If x is greater than 0, the following applies:

          - If bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F3[ 1 ][ 0 ] * pY[ SubWidthC * x ][ − 1 − SubHeightC ] +
		F3[ 0 ][ 1 ] * pY[ SubWidthC * x − 1 ][ −SubHeightC ] +
		F3[ 1 ][ 1 ] * pY[ SubWidthC * x ][ −SubHeightC] +	(8-182)
		F3[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ −SubHeightC] +
		F3[ 1 ][ 2 ] * pY[ SubWidthC * x ][ 1 − SubHeightC ] + 4 ) >> 3

          - Otherwise (bCTUboundary is equal to TRUE), the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ SubWidthC * x − 1 ][ −1 ] +
		F2[ 1 ] * pY[ SubWidthC * x ][ −1 ]  +	(8-183)
		F2[ 2 ] * pY[ SubWidthC * x + 1 ][ −1 ] + 2 ) >> 2

        - Otherwise (x is equal to 0), the following applies:

          - If availTL is equal to TRUE and bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F3[ 1 ][ 0 ] * pY[ −1 ][ − 1 − SubHeightC ] +
		F3[ 0 ][ 1 ] * pY[ −1 ][ −SubHeightC ] +
		F3[ 1 ][ 1 ] * pY[ 0 ][ −SubHeightC ] +	(8-184)
		F3[ 2 ][ 1 ] * pY[ 1 ][ −SubHeightC ] +
		F3[ 1 ][ 2 ] * pY[ −1 ][ 1 − SubHeightC ] + 4 ) >> 3

          - Otherwise if availTL is equal to TRUE and bCTUboundary is equal to TRUE, the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ −1 ][ −1 ] + F2[ 1 ] * pY[ 0 ][ −1 ]  +
		F2[ 2 ] * pY[ 1 ][ −1 ] + 2 ) >> 2	(8-185)

          - Otherwise if availTL is equal to FALSE and bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ 0 ][ −1 ] + F2[ 1 ] * pY[ 0 ][ −2 ] +
		F2[ 2 ] * pY[ 0 ][ −1 ] + 2 ) >> 2	(8-186)

          - Otherwise (availTL is equal to FALSE and bCTUboundary is equal to TRUE), the following applies:

pSelDsY[ idx ] = pY[ 0 ][ −1 ]		(8-187)

      - Otherwise (sps_cclm_colocated_chroma_flag is equal to 0), the following applies:

        - If x is greater than 0, the following applies:

          - If bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F4[ 0 ][ 1 ] * pY[ SubWidthC x − 1 ][ −2 ] +
		F4[ 0 ][ 2 ] * pY[ SubWidthC * x − 1 ][ −1 ] +
		F4[ 1 ][ 1 ] * pY[ SubWidthC * x ][ −2 ] +	(8-188)
		F4[ 1 ][ 2 ] * pY[ SubWidthC * x ][ −1] +
		F4[ 2 ][ 1 ] * pY[ SubWidthC * x + 1 ][ −2 ] +
		F4[ 2 ][ 2 ] * pY[ SubWidthC * x + 1 ][ −1 ] + 4 ) >> 3

          - Otherwise (bCTUboundary is equal to TRUE), the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ SubWidthC * x − 1 ][ −1 ] +
		F2[ 1 ] * pY[ SubWidthC * x ][ −1 ] +	(8-189)
		F2[ 2 ] * pY[ SubWidthC * x + 1][ −1 ] + 2 ) >> 2

        - Otherwise (x is equal to 0), the following applies:

          - If availTL is equal to TRUE and bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F4[ 0 ][ 1 ] * pY[ −1 ][ −2 ] + F4[ 0 ][ 2 ] * pY[ −1 ][ −1 ] +
		F4[ 1 ][ 1 ] * pY[ 0 ][ −2 ] + F4[ 1 ][ 2 ] * pY[ 0 ][ −1 ] +	(8-190)		F4[ 2 ][ 1 ] * pY[ 1 ][ −2 ] + F4[ 2 ][ 2 ] * pY[ 1 ][ −1 ] + 4 ) >> 3

          - Otherwise if availTL is equal to TRUE and bCTUboundary is equal to TRUE, the following applies:

pSelDsY[ idx ] = ( F2[ 0 ] * pY[ −1 ][ −1 ] + F2[ 1 ] * pY[ 0 ][ −1 ] +
		F2[ 2 ] * pY[ 1 ][ − 1 ] + 2 ) >> 2	(8-191)

          - Otherwise if availTL is equal to FALSE and bCTUboundary is equal to FALSE, the following applies:

pSelDsY[ idx ] = ( F1[ 1 ] * pY[ 0 ][ −2 ] + F1[ 0 ] * pY[ 0 ][ −1 ] + 1 ) >> 1	(8-192)

          - Otherwise (availTL is equal to FALSE and bCTUboundary is equal to TRUE), the following applies:

pSelDsY[ idx ] = pY[ 0 ][ −1 ]		(8-193)

“

# Experimental Results

The proposed method was integrated into VTM5.0 and tested under the CTC [1]. Results are reported below.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.85% | 0.94% | 101% | 98% |
| Class A2 | 0.04% | 0.21% | 0.04% | 100% | 101% |
| Class B | 0.02% | 0.21% | 0.33% | 100% | 102% |
| Class C | 0.06% | 0.48% | 0.49% | 100% | 101% |
| Class E | 0.00% | 0.01% | 0.00% | 100% | 102% |
| Overall | 0.04% | 0.34% | 0.36% | 100% | 101% |
| Class D | 0.03% | 0.23% | 0.15% | 99% | 101% |
| Class F (optional) | 0.13% | 0.73% | 0.83% | 102% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.43% | 0.89% | 102% | 99% |
| Class A2 | 0.04% | 0.40% | 0.15% | 101% | 101% |
| Class B | 0.02% | 0.54% | 0.43% | 100% | 98% |
| Class C | 0.01% | 0.47% | 0.30% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.47% | 0.43% | 101% | 100% |
| Class D | 0.05% | 0.56% | 0.31% | 99% | 99% |
| Class F (optional) | 0.09% | 0.67% | 0.70% | 103% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.19% | 0.51% | 101% | 95% |
| Class C | 0.02% | 0.31% | -0.02% | 103% | 104% |
| Class E | -0.01% | -0.28% | 0.77% | 102% | 102% |
| Overall | 0.01% | 0.11% | 0.40% | 102% | 100% |
| Class D | 0.03% | -0.07% | 0.94% | 101% | 105% |
| Class F (optional) | 0.00% | 0.14% | 0.10% | 103% | 96% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.01% | 0.62% | 101% | 102% |
| Class C | 0.00% | 0.04% | 0.28% | 103% | 105% |
| Class E | -0.02% | 0.24% | 0.11% | 102% | 103% |
| Overall | 0.00% | 0.07% | 0.38% | 102% | 103% |
| Class D | 0.07% | 0.09% | 0.09% | 100% | 102% |
| Class F (optional) | 0.09% | 0.53% | 0.76% | 104% | 96% |

# Additional results: sps_cclm_colocated_chroma_flag = 1

The proposed method has been tested on HDR test material following HDR/WCG CTC [2]. For type-2 sequences the sps_cclm_collocated_chroma_flag is set to 1 as recommended in [3]. Results are reported below.

|  | All Intra |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y |  | U | V | EncT | DecT |
| Class H1 | #VALUE! | #VALUE! | 0.12% | 1.60% | 3.40% | 0.12% |  | 1.43% | 2.96% | 100% | 101% |
| Class H2 |  |  |  |  |  | #VALUE! |  | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | 0.12% | 1.60% | 3.40% | #VALUE! |  | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y |  | U | V | EncT | DecT |
| Class H1 | #VALUE! | #VALUE! | 0.07% | 1.99% | 2.75% | 0.08% |  | 1.93% | 2.37% | 99% | 103% |
| Class H2 |  |  |  |  |  | #VALUE! |  | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | 0.07% | 1.99% | 2.75% | #VALUE! |  | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusion

This contribution proposes harmonization of boundary downsampling used in both MIP and CCLM. Results show limited losses over anchor with 0.04%(Y)/0.34%(U)/0.36%(V) BD-Rate in all-intra and 0.05%(Y)/0.47%(U)/0.43%(V) BD-Rate in random-access.

# Acknowledgement

We would like to thank Interdigital for crosschecking our contribution.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sürhing, “JVET Common Test Condition and Software Reference Configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

- A. Segall, E. Francois, W. Husak, S. Iwamura, D. Rusanovskky, “JVET common test conditions and evaluation procedures for HDR/WCG video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0101, Geneva, CH, 19–27 Mar. 2019.

- P. Hanhart and Y. Ye, “CE3: Modified CCLM downsampling filter for “type-2” content (Test 2.4)", Joint Video Exploration Team (JVET) of  ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M0142, Marrakech, MA, 9–18 Jan. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).