| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0674-v2 |
| --- | --- |

| Title: | Non-CE3: Multiple reference set for CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Po-Han Lin Jhih-You Deng | Email: | PoHanLin@fginnov.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

This contribution proposes a multiple reference set method for CCLM. The proposed method increases the performance of the chroma components. The simulation result shows -1.03/-1.06 gains on the U/V components with negligible encoding/decoding time change compared with VTM5.0.

Introduction

Cross-component linear model (CCLM) predicts a chroma block based on the reconstructed luma samples and the neighboring chroma samples. The chroma predictor is generated by applying a linear model to the luma samples described as follows:

predCi,j=α·recL'i,j+β

In VTM5.0, three LM modes are available and each of mode uses different position of at most 4 samples to derive the parameter (αα and ββ) of the linear model. The 3 modes are LM, LM_A and LM_L. Denote the reference samples of the chroma block in the above line and left line as R[-1, 0]…R[-1, W+H] and R[0, -1]…R[W+H, -1] and W’=H’=W+H, the position of the selected reference samples are described as follows.

LM:	R[W/4, -1], R[3W/4, -1], R[-1, H/4] and R[-1, 3H/4]

LM_A:	R[W’/8, -1], R[3W’/8, -1] , R[5W’/8, -1] and R[7W’/8, -1]

LM_L: R[-1, H’/8], R[-1, 3H’/8], R[-1, 5H’/8] and R[-1, 7H’/8]

After the chroma samples at these positions and the corresponding down-sampled luma samples are selected. These samples are divided into 2 groups by the 2 smaller and 2 larger luma values. Then, the parameter αα and ββ are calculated by the averaged luma and chroma values of each group.

Proposed method

The proposed method introduces 4 new LM modes, which are denoted as LM_LA1, LM_LA2, LM_LA3 and LM_LA4. Each of the 4 modes select 4 reference samples from both the above and the left side of the current chroma block, which is similar to the original LM mode. The selected samples of the 4 modes are described in the following.

LM_LA1: R[W’/8, -1], R[7W’/8, -1], R[-1, H’/8] and R[-1, 7H’/8]

LM_LA2: R[3W’/8, -1], R[5W’/8, -1], R[-1, 3H’/8] and R[-1, 5H’/8]

LM_LA3: R[W’/8, -1], R[5W’/8, -1], R[-1, H’/8] and R[-1, 5H’/8]

LM_LA4: R[3W’/8, -1], R[7W’/8, -1], R[-1, 3H’/8] and R[-1, 7H’/8]

All the modes reuse some of the reference samples from LM_A and LM_L modes. Therefore, no additional neighboring region and reference sample are needed. After the chroma and down-sampled luma samples are selected. The derivation method of the linear model parameters is the same as the existed LM modes.

Simulation results

The following experiments were conducted using the VTM-5.0 software with the VTM configuration and the JVET common test conditions [1] to collect the BD-rate, encoding time and decoding time of the proposed techniques in the RA and LB configurations. The proposed method has a BD-rate performance of -0.01%/-1.03%/-1.06% for Y/Cb/Cr without encoding/decoding time increasing for the AI configuration, and BD-rate performance of -0.01%/-1.00%/-0.99% for Y/Cb/Cr without encoding/decoding time increasing for the RA configuration.

Table 1: Simulation results of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -1.97% | -1.64% | 99% | 99% |
| Class A2 | -0.06% | -1.12% | -0.63% | 99% | 99% |
| Class B | 0.01% | -0.99% | -1.39% | 100% | 99% |
| Class C | -0.01% | -0.99% | -1.04% | 99% | 100% |
| Class E | 0.00% | -0.13% | -0.41% | 99% | 100% |
| Overall | -0.01% | -1.03% | -1.06% | 99% | 100% |
| Class D | 0.00% | -0.68% | -0.85% | 100% | 100% |
| Class F | -0.03% | -0.97% | -1.17% | 98% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -1.42% |  | -1.15% |  | 100% | 100% |  |
| Class A2 | -0.01% | -0.89% |  | -0.37% |  | 100% | 100% |  |
| Class B | 0.02% | -0.90% |  | -1.48% |  | 100% | 100% |  |
| Class C | -0.04% | -0.90% |  | -0.73% |  | 100% | 100% |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.01% | -1.00% |  | -0.99% |  | 100% | 100% |  |
| Class D | 0.01% | -0.50% |  | -0.18% |  | 100% | 100% |  |
| Class F | 0.03% | -0.65% |  | -0.57% |  | 100% | 100% |  |

Conclusions

This contribution proposes 4 new LM modes to improve the performance of chroma components without increasing the hardware complexity. The simulation results show -0.01%/-1.03%/-1.06% BD-rate improvement in Y/U/V component and negligible encoding/decoding time. It is suggested to adopt the proposed modification as part of VVC.

Reference

- F. Bossen, J. Boyce, X. Li ,V. Seregin and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, Apr. 2019, Geneve, CH.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).