| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0680-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0546 (Non-CE3/6: Transform selection for MRL) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jason Jung Dongcheol Kim Geonjung Ko Ju-hyung (John) Son Jin Sam Kwak | Tel: Email: | +82-31-712-0523 jason.jung@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-O0546 where MTS (Multiple Transform Selection) is not allowed for intra-coded blocks with non-zero reference line index in order to reduce encoder complexity. It is confirmed that the software implementation is aligned with the proposal description and the test results provided by the proponent of JVET-O0546 are matched with the crosscheck results.

Crosscheck results

In JVET-O0546 [1], it is proposed to use only DCT-II or transform skip mode for intra-coded blocks with non-zero reference line index.

The SW provided by the proponent of JVET-O0546 was studied and tested under the common test condition (CTC) [2]. Table 1 shows summarized experimental results.

Table 1. Coding performance of JVET-O0546 under the CTC

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.01% | 100% | 99% |
| Class A2 | 0.01% | -0.04% | -0.04% | 99% | 100% |
| Class B | 0.02% | -0.03% | -0.02% | 99% | 100% |
| Class C | -0.01% | -0.07% | -0.04% | 99% | 100% |
| Class E | 0.00% | -0.01% | 0.07% | 99% | 100% |
| Overall | 0.00% | -0.03% | -0.01% | 99% | 100% |
| Class D | 0.02% | 0.19% | -0.04% | 99% | 100% |
| Class F | 0.03% | 0.04% | -0.13% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.11% | 0.04% | 100% | 100% |
| Class A2 | 0.01% | 0.09% | 0.09% | 100% | 99% |
| Class B | 0.02% | 0.03% | -0.01% | 100% | 100% |
| Class C | -0.03% | -0.12% | -0.04% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.10% | -0.30% | 99% | 100% |
| Class F | -0.03% | -0.10% | -0.11% | 100% | 100% |

Conclusion

In this contribution, performance of JVET-O0546 was verified. It is reported that the SW implementation is aligned with the proposal description and test results provided by the proponent of JVET-O0546 are matched with the crosscheck results.

References

- L. Zhao, X. Zhao, X. Li and S. Liu, “Non-CE3/CE6: Transform selection for MRL,” JVET-O0546, 15th meeting, Gothenburg, SE, 3-12 July 2019

- B. Bross, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-N1010, 14th meeting, Geneva, CH, 19-27 March 2019