| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 Jul. 2019 | Document: JVET-O0692-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0122: CE7-3.9 + parity plane change |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tung Nguyen Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-230 tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the crosscheck results for the input document JVET-O0122. The crosschecker confirms the experimental results provided by the proponent. The crosschecker inspected the provided source code and confirms that the description and the implementation seem to be correct.

# Experimental results

The presented experimental results are for the CTC QP configuration, i.e., the QP range is from 22 to 37, inclusively, with a step size equal to five.

Note that the encoding and decoding times are not reliable. Due to issues with the cluster, the results are partially only.

Table I: Results for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | -0.01% | 100% | 108% |
| Class A2 | 0.00% | 0.02% | -0.01% | 99% | 103% |
| Class B | 0.00% | 0.01% | -0.01% | 100% | 111% |
| Class C | 0.01% | -0.01% | -0.03% | 99% | 118% |
| Class E | 0.00% | 0.03% | 0.04% | 99% | 109% |
| Overall | 0.00% | 0.01% | -0.01% | 100% | 110% |
| Class D | -0.01% | 0.12% | 0.09% | 100% | 115% |
| Class F | -0.04% | -0.05% | -0.06% | 101% | 104% |
| Class TGM | -0.04% | -0.03% | -0.03% | 100% | 117% |

Table II: Results for the RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.00% | 99% | 102% |
| Class A2 | 0.01% | -0.07% | -0.03% | 104% | 100% |
| Class B | 0.00% | -0.06% | 0.11% | 98% | 100% |
| Class C | -0.03% | -0.16% | 0.06% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.08% | 0.05% | 99% | 100% |
| Class D | 0.04% | -0.03% | 0.11% | 99% | 107% |
| Class F | 0.02% | 0.12% | 0.02% | 99% | 102% |
| Class TGM | 0.06% | 0.05% | 0.01% | 98% | 102% |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).