| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0704-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0389: On Supporting 64x64 Chroma Transform Unit |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xin Zhao | Tel: Email: | xinzzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-O0389, which proposes to support the maximum transform unit size of chroma components in 4:2:0 format up to 64x64 for inter CU to make the maximum chroma TU size for 4:4:4, 4:2:2 and 4:2:0 format identical. When inter CU size in luma sample unit is larger or equal to 128x128, the size of transform unit of chroma components is 64x64. Otherwise, the maximum transform unit size of chroma is 32x32. The software implementation as provided by the proponent of JVET-O0389 has been investigated and confirmed to be aligned with the proposal description. Simulations results are reportedly matching with the results provided by proponent of JVET-O0389.

# Crosscheck report

In VTM-5.0, the 64x64 transforms for luma and chroma component in 4:4:4 and 4:2:2 color format are supported. For pictures in 4:2:0 format, the maximum supported luma transform size is 64×64 and the maximum supported chroma transform size is 32×32.

In JVET-O0389, it is proposed to use maximum transform TB size 64x64 for both luma and chroma components for inter CU in P and B slices in which CTU is partitioned by using a single tree. When inter CU size in luma samples is larger or equal to 128x128, the size of transform unit of chroma components is 64x64. Otherwise, the maximum transform unit size of chroma is 32x32.

Simulated are performed using common test condition (CTC) [1], the anchor is VTM-5.0. Below table summarizes the results. The run-time is not accurate due to the variation of the work load on the cluster.

Table 1: Coding performance of JVET-O0389.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.51% | -0.35% | 100% | 100% |
| Class A2 | 0.02% | -0.14% | -0.02% | 100% | 100% |
| Class B | 0.01% | -0.14% | -0.22% | 100% | 100% |
| Class C | -0.01% | -0.04% | -0.01% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.19% | -0.15% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Class F | 0.01% | -0.01% | -0.06% | 101% | 104% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.