| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0705-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0205: CE8-related: Improvements on BDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xin Zhao | Tel: Email: | xinzzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-O0205 method 1, which proposes to set the intra prediction mode for the BDPCM by the one used for the actual prediction, either horizontal or vertical modes. The software implementation as provided by the proponent of JVET-O0205 has been investigated and confirmed to be aligned with the proposal description. Simulations results are reportedly matching with the results provided by proponent of JVET-O0205.

# Crosscheck report

In JVET-O0205, the intra prediction mode for the BDPCM is set to the one used for the actual prediction, either horizontal or vertical modes. If the BDPCM mode is set to 1, the syntax elements for the intra prediction modes remain not parsed and inferred one of those two modes.

Simulated are performed using common test condition (CTC) [1] for Class F and TGM sequences, since the proposed method only has an impact when BDPCM is enabled, the anchor is VTM-5.0. Below table summarizes the results. The run-time is not accurate due to the variation of the work load on the cluster.

Table 1: Coding performance of JVET-O0205.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -0.12% | 0.04% | -0.08% | 101% | 99% | -0.09% | 0.08% | 0.10% | 101% | 101% | 0.02% | -0.21% | -0.44% | 100% | 99% |
| TGM | -0.18% | -0.06% | -0.07% | 102% | 100% | -0.18% | -0.02% | -0.13% | 101% | 99% | -0.04% | 0.01% | 0.04% | 101% | 100% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.