| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0706-v2 |
| --- | --- |

| Title: | CE8-related: 4x4 sub-block based dual-tree chroma IBC block-vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hua Yang 4 Research Way 3rd Floor, Princeton, NJ 08540 USA Yuwen He 9276 Scranton Road, Suite 300 San Diego, CA 92121 USA | Tel: Email: | +1-609-608-0479 Hua.Yang@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Block-vector (BV) derivation in the dual-tree chroma Intra-Block-Copy (IBC) mode is studied in CE8-1.5. CE8-1.5.a uses CU center BV as the chroma CU BV, while CE8-1.5.b bears the same 2x2 chroma sub-block based BV derivation approach as in VTM-5.0 and uses CU center BV as the default BV in case a collocated 4x4 luma sub-block does not have a valid BV. However, the 2x2 chroma sub-block based dual-tree chroma IBC mode is not efficient in terms of hardware (HW) throughput and memory access, where a minimum 16 samples per cycle is generally preferred. In this proposal, to enhance HW throughput, we change the chroma sub-block size from 2x2 to 4x4 in dual-tree chroma IBC BV derivation. Simulation results show that: for screen content coding sequences (i.e. Class TGM), this change alone yields -0.07%/-0.04%/0.02% Y-BD-rate change for AI/RA/LDB compared to VTM-5.0. If combined with the extra change of using CU center BV as luma sub-block default BV (i.e. as in CE8-1.5.b), it yields -0.30%/-0.12%/-0.05% Y-BD-rate change for AI/RA/LDB.

# Introduction

The IBC mode is very efficient for screen content coding (SCC) in VTM-5.0. In the current design, for a dual-tree chroma CU, IBC mode is allowed only when all the collocated luma 4x4 sub-blocks are coded in IBC mode and all the related block vectors (BVs) are valid with respect to the corresponding 2x2 chroma sub-blocks, individually.

(Note that in the current VTM-5.0, encoder checking only makes sure the reference chroma 2x2 sub-block from a collocated luma derived chroma BV is available (i.e. in the decoded area of the current picture). However, the availability of reference is not equivalent to the validity of the chroma BV, which requires not only the availability of reference, but also that the BV be within the valid range as defined by the IBC reference memory restrictions in the VVC specification. For testing/comparison convenience, we only identify this software bug herein, but not fixed it in our tests.)

# Technical description

One problem with the current design is that: for the decoding of a dual-tree IBC coded chroma CU, IBC compensation would be conducted for each 2x2 chroma sub-blocks, which does not meet with the preferred minimum 16 samples per cycle requirement from the HW efficiency point of view. Therefore, in this proposal, we change the chroma sub-block size from 2x2 to 4x4 to enhance throughput.

With this change, the collocated luma sub-block size would be increased from 4x4 to 8x8. Since the luma IBC coding information is still stored in 4x4 blocks (due to the minimum luma CU size being 4x4), we need to consolidate the BVs from the 4 involved luma 4x4 sub-blocks in a collocated luma 8x8 sub-block. For this problem, we use the same center position BV approach as in CE8-1.5 [1], where in our case, the center position BV of the 8x8 luma sub-block is used as the consolidated BV of that collocated 8x8 luma sub-block. If the center position is not IBC coded, the invalid zero BV will be assumed.

Note that currently only 2x2, 2x4, 4x2 chroma CU sizes are removed for dual-tree coding [2], and thus, there still exists 2xN, Nx2 (N≥8) chroma CUs for dual-tree coding. In these cases, we still use the chroma 2x2 sub-block size for chroma BV derivation.

Dual-tree chroma BV derivation is studied in CE8-1.5 [3]. CE8-1.5.a uses the CU center BV as the chroma CU BV, while CE8-1.5.b bears the same 2x2 chroma sub-block based BV derivation approach as in VTM-5.0 and uses CU center BV as the default BV in case a collocated 4x4 luma sub-block does not have a valid BV. In both CE8-1.5 tests, the encoder only checks IBC mode for a dual-tree chroma CU if the center BV is valid for the chroma CU. Also, in VTM-5.0, the encoder only checks IBC mode for a dual-tree chroma CU if all the chroma 2x2 sub-blocks have valid BVs derived from the collocated luma 4x4 sub-block BVs.

Therefore, for fair comparison, we also conduct and enforce the similar encoder conformance check in our tests. Additionally, we also tested our methods without encoder conformance check along with the default IBC compensation mode as in CE8-1.4, where better coding efficiency can be achieved.

# Simulation results

CTC tests are performed based on VTM-5.0 as the anchor [4]. For the proposed testing methods, IBC is turned on for all the classes as prescribed in CE8 [3].

## Test 1: Chroma 4x4 change on VTM-5.0

Firstly, from the original dual-tree chroma IBC BV derivation approach in VTM-5.0, we only change the involved 2x2 chroma sub-block size to 4x4, and use center BV as the BV of a collocated luma 8x8 sub-block.

Table 1. Results of chroma 4x4 change only

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.05% | 0.02% | 142% | 100% |
| Class A2 | -0.78% | -1.08% | -1.00% | 147% | 102% |
| Class B | -0.80% | -0.69% | -0.86% | 157% | 100% |
| Class C | -0.35% | -0.38% | -0.34% | 157% | 99% |
| Class E | -1.32% | -1.27% | -1.37% | 161% | 99% |
| Overall | -0.66% | -0.68% | -0.71% | 153% | 100% |
| Class D | -0.66% | -0.51% | -0.69% | 149% | 97% |
| Class F | -0.02% | 0.00% | 0.04% | 102% | 100% |
| Class TGM | -0.07% | -0.08% | -0.12% | 101% | 100% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | -0.11% | 0.24% | 105% | 102% |
| Class A2 | -0.06% | -0.32% | -0.21% | 106% | 103% |
| Class B | -0.13% | -0.35% | -0.72% | 108% | 99% |
| Class C | 0.15% | -0.27% | -0.18% | 113% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.27% | -0.28% | 108% | 100% |
| Class D | 0.10% | -0.31% | 0.23% | 112% | 97% |
| Class F | -0.01% | 0.08% | -0.04% | 101% | 100% |
| Class TGM | -0.04% | -0.02% | -0.10% | 100% | 99% |

|  | Low delay B Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.10% | 0.06% | 118% | 101% |
| Class C | 0.11% | 0.17% | 0.03% | 124% | 101% |
| Class E | 0.08% | -0.84% | 0.33% | 112% | 102% |
| Overall | 0.08% | -0.11% | 0.12% | 118% | 101% |
| Class D | 0.06% | -0.04% | 0.05% | 125% | 102% |
| Class F | -0.03% | -0.07% | 0.22% | 100% | 101% |
| Class TGM | 0.02% | -0.12% | 0.03% | 100% | 100% |

## Test 2: Chroma 4x4 change on CE8-1.5.b (i.e. CU center BV as default BV)

Secondly, we test further include the change of using center BV as default BV from CE8-1.5.b in addition to chroma 4x4 change. In this case, if a collocated luma 8x8 sub-block BV is either not available or valid, the CU center BV will be further used as the default BV.

Table 2. Results of chroma 4x4 change + CU center BV as default BV

|  | All Intra Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.05% | 0.04% | 142% | 101% |
| Class A2 | -0.82% | -1.08% | -0.98% | 148% | 101% |
| Class B | -0.82% | -0.78% | -1.00% | 157% | 101% |
| Class C | -0.37% | -0.44% | -0.34% | 157% | 99% |
| Class E | -1.32% | -1.34% | -1.41% | 161% | 99% |
| Overall | -0.67% | -0.73% | -0.75% | 154% | 100% |
| Class D | -0.67% | -0.54% | -0.71% | 149% | 97% |
| Class F | -0.05% | -0.10% | -0.11% | 102% | 100% |
| Class TGM | -0.30% | -0.37% | -0.45% | 101% | 100% |

|  | Random Access Main 10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | -0.15% | 0.29% | 106% | 101% |
| Class A2 | -0.08% | -0.32% | -0.07% | 106% | 102% |
| Class B | -0.16% | -0.50% | -0.81% | 108% | 100% |
| Class C | 0.15% | -0.34% | -0.16% | 112% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.35% | -0.27% | 108% | 100% |
| Class D | 0.08% | -0.18% | 0.06% | 111% | 97% |
| Class F | -0.03% | -0.14% | -0.08% | 101% | 101% |
| Class TGM | -0.12% | -0.12% | -0.23% | 101% | 100% |

|  | Low delay B Main10 - VTM configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.02% | -0.25% | 118% | 102% |
| Class C | 0.11% | 0.04% | -0.25% | 124% | 101% |
| Class E | 0.06% | -0.65% | 0.24% | 113% | 102% |
| Overall | 0.06% | -0.16% | -0.13% | 119% | 102% |
| Class D | 0.05% | -0.29% | 0.22% | 125% | 102% |
| Class F | 0.02% | 0.13% | -0.05% | 101% | 102% |
| Class TGM | -0.05% | -0.17% | -0.07% | 101% | 101% |

## Test 3: No encoder conformance check + CE8-1.4 default IBC compensation mode

Additionally, we also tested our methods without encoder conformance check along with the default IBC compensation mode as in CE8-1.4 for better coding efficiency. For Class TGM, “Test 3.a chroma 4x4 change on VTM-5.0” yields -0.48%/-0.17%/0.03% Y-BD-rate change for AI/RA/LDB. “Test 3.b chroma 4x4 change on CE8-1.5.b” yields -0.61%/-0.25%/-0.07% gain for AI/RA/LDB.

# Conclusion

To enhance throughput, we change the chroma sub-block size for dual-tree IBC BV derivation from 2x2 to 4x4. On VTM-5.0, for Class TGM, this change only yields -0.07%/-0.04%/0.02% Y-BD-rate change for AI/RA/LDB, while if combined with using CU center BV as default BV (i.e. CE8-1.5.b), -0.30%/-0.12%/-0.05% Y-BD-rate change can be achieved for AI/RA/LDB.

# References

- Y. Han, W.-J. Chien, M. Karczewicz, J. Li, C. Lim, H. Jang, J. Nam, J. Lim, "CE8-1.4 and CE8-1.5: Default Processing for IBC Mode", JVET-O0117, 15th meeting, July 2019, Gothenburg, SE.

- T. Zhou, T. Ikai, “CE3-2.1: Intra chroma partitioning and prediction restriction”, JVET-N0137, 14th meeting, Mar 2019, Geneva, CH.

- X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu, "CE8: Summary Report on Screen Content Coding Tools", JVET-O0028, 15th meeting, July 2019, Gothenburg, SE.

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th meeting, Mar. 2019, Geneva, CH.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).