| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0737 |
| --- | --- |

| Title: | Chroma Delta QP for Separate Tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu Han, Geert Van Der Auwera, Muhammed Coban, Wei-Jung, Chien, Yung-Hsuan Chao, Alican Nalci, Marta Karczewicz | Tel: Email: | +1-858-845-1793 yuhan@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes a flexible signaling method for chroma delta QP when dual tree is enabled. The proposed method is the same as delta QP signaling for luma in VTM-5.0. Moreover, signaling of the CU level delta QP for chroma is also proposed allowing for more flexibility when deriving chroma QP.

# Introduction

In VTM-5.0, chroma QP is derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 69, QpY + pps_cb_qp_offset + slice_cb_qp_offset ),

qPiCr = Clip3( −QpBdOffsetC, 69, QpY + pps_cr_qp_offset + slice_cr_qp_offset ),

where QpY is the luma QP, QpBdOffsetC is the chroma QP range offset, and pps_cb_qp_offset and pps_cr_qp_offset are PPS and slice level QP offsets.

In case of dual tree, QpY is equal to the luma QP of the coding unit (CU) that covers the luma position (xCb + cbWidth / 2, yCb + cbHeight / 2 ). Thus, as shown in Fig. 1 one chroma CU may correspond to more than one luma CUs (e.g. 4 luma CUs in Figure 1). Moreover, these luma CUs can belong to different quantization groups (QGs) and may involve very different QP values.

As a result, a luma QP (QpY) derived using individual luma CUs may not be a good prediction for the chroma QP. Moreover, even though PPS and slice level QP offset adjustment for chroma may be sufficient for single tree, these offsets can be limited in dual tree case for chroma. Therefore, a flexible method for chroma QP adjustment is of interest.

Qp = 27

Qp = 27

Qp = 37

Qp = 27

Figure 1: An example of a single chroma CU that spans multiple luma CUs when dual tree is enabled

# Technical description

In order to provide a flexible chroma QP adjustment and to overcome the issues detailed above, following syntax change is proposed on top of VTM-5.0, where new syntax is highlighted with yellow and removed syntax is crossed out with red. The proposed change is the same as delta QP signaling for luma.

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| … |  |
| If(treeType != DUAL_TREE_CHROMA) { |  |
| if( ( tu_cbf_luma[ x0 ][ y0 ] ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) && treeType != DUAL_TREE_CHROMA ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| } |  |
| else if( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) { |  |
| if( chroma_cu_qp_delta_enabled_flag && !IsChromaCuQpDeltaCoded ) { |  |
| chroma_cu_qp_delta_abs | ae(v) |
| if( chroma_cu_qp_delta_abs ) |  |
| chroma_cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

In the proposed method, a chroma delta QP value is signalled using both absolute value for chroma delta QP and sign values for chroma delta QP for both Cb and Cr components. Consequently, new variables qPiCb and qPiCr can be derived as follows (Proposal #1):

qPiCb = Clip3( −QpBdOffsetC, 69, QpY + pps_cb_qp_offset + slice_cb_qp_offset + …

ChromaCuQpDeltaVal)

qPiCr = Clip3( −QpBdOffsetC, 69, QpY + pps_cr_qp_offset + slice_cr_qp_offset + …

ChromaCuQpDeltaVal)

where ChromaCuQpDeltaVal = chroma_cu_qp_delta_abs * ( 1 − 2 * chroma_cu_qp_delta_sign_flag ).

In addition to Proposal #1, CU level QP offset variables can also be incorporated such that qPiCb and qPiCr are derived as follows (Proposal #2):

ChromaCuQpDeltaVal = chroma_cu_qp_delta_abs * ( 1 − 2 * chroma_cu_qp_delta_sign_flag ),

qPiCb = Clip3( −QpBdOffsetC, 69, QpY + pps_cb_qp_offset + slice_cb_qp_offset + …

ChromaCuQpDeltaVal + CuQpOffsetCb)

qPiCr = Clip3( −QpBdOffsetC, 69, QpY + pps_cr_qp_offset + slice_cr_qp_offset + …

ChromaCuQpDeltaVal + CuQpOffsetCr)

where CuQpOffsetCb = cb_qp_offset_list[cu_chroma_qp_offset_idx] is the CU level QP offset for Cb and CuQpOffsetCr = cr_qp_offset_list[cu_chroma_qp_offset_idx] is the CU level QP offset for Cr. Proposal #2 uses both PPS, slice and CU level QP offsets for chroma when dual tree is enabled.

# Conclusions

This contribution proposes a flexible signaling method for chroma delta QP when dual tree is enabled. The proposed approach is the same as delta QP signaling for luma in VTM-5.0 and hence unifies the luma and chroma delta QP scheme when dual tree is enabled. This approach also has the potential to provide better QP predictions for chroma, especially when one chroma CU corresponds to multiple luma CUs with different QP values.

# References

- B. Bross, J. Chen, S. Liu, JVET-N1001, “Versatile Video Coding (Draft 5)”, Geneva, March 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# The syntax modification on VVC draft is as below:

Based on JVET-N1001 version 8, the newly added parts are highlighted in yellow and the text to be removed is marked as red with strikethrough.

7.3.2.4 Picture parameter set RBSP syntax

| cu_qp_delta_enabled_flag | u(1) |
| --- | --- |
| if( cu_qp_delta_enabled_flag ) |  |
| cu_qp_delta_subdiv | ue(v) |
| chroma_cu_qp_delta_enabled_flag | u(1) |
| if( chroma_cu_qp_delta_enabled_flag ) |  |
| chroma_cu_qp_delta_subdiv | ue(v) |

7.3.7.2 Coding tree unit sytax

| dual_tree_implicit_qt_split( x0, y0, cbSize, cqtDepth ) { | Descriptor |
| --- | --- |
| cbSubdiv = 2 * cqtDepth |  |
| if( cbSize > 64 ) { |  |
| if( cu_qp_delta_enabled_flag && cbSubdiv <= cu_qp_delta_subdiv ) { |  |
| if(( treeType = = DUAL_TREE_LUMA && cu_qp_delta_enabled_flag && cbSubdiv <= cu_qp_delta_subdiv) ¦¦ ( treeType = = DUAL_TREE_CHROMA && chroma_cu_qp_delta_enabled_flag && chromaCbSubdiv <= chroma_cu_qp_delta_subdiv)) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| x1 = x0 + ( cbSize / 2 ) |  |
| y1 = y0 + ( cbSize / 2 ) |  |
| dual_tree_implicit_qt_split( x0, y0, cbSize / 2, cqtDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| dual_tree_implicit_qt_split( x1, y0, cbSize / 2, cqtDepth + 1 ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| dual_tree_implicit_qt_split( x0, y1, cbSize / 2, cqtDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| dual_tree_implicit_qt_split( x1, y1, cbSize / 2, cqtDepth + 1 ) |  |
| } else { |  |
| coding_tree( x0, y0, cbSize, cbSize, 1, cbSubdiv, cqtDepth, 0, 0, 0, DUAL_TREE_LUMA ) |  |
| coding_tree( x0, y0, cbSize, cbSize, 0, cbSubdivchromaCbSubdiv, cqtDepth, 0, 0, 0, DUAL_TREE_CHROMA ) |  |
| } |  |
| } |  |

7.3.7.4 Coding tree syntax

| coding_tree( x0, y0, cbWidth, cbHeight, qgOn, cbSubdiv, cqtDepth, mttDepth, depthOffset, partIdx, treeType ) { | Descriptor |
| --- | --- |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ¦ ¦ allowSplitQT ) &&( x0 + cbWidth <= pic_width_in_luma_samples ) && (y0 + cbHeight <= pic_height_in_luma_samples ) ) |  |
| split_cu_flag | ae(v) |
| if( cu_qp_delta_enabled_flag && qgOn && cbSubdiv <= cu_qp_delta_subdiv ) { |  |
| if((treeType != DUAL_TREE_CHROMA && cu_qp_delta_enabled_flag && qgOn && cbSubdiv <= cu_qp_delta_subdiv) ¦¦ (treeType == DUAL_TREE_CHROMA && chorma_cu_qp_delta_enabled_flag && qgOn && cbSubdiv <= chroma_cu_qp_delta_subdiv) ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| } |  |
| if( split_cu_flag ) { |  |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ) && allowSplitQT ) |  |
| split_qt_flag | ae(v) |
| if( !split_qt_flag ) { |  |
| if( ( allowSplitBtHor ¦ ¦ allowSplitTtHor ) && ( allowSplitBtVer ¦ ¦ allowSplitTtVer ) ) |  |
| mtt_split_cu_vertical_flag | ae(v) |
| if( ( allowSplitBtVer && allowSplitTtVer && mtt_split_cu_vertical_flag ) ¦ ¦ ( allowSplitBtHor && allowSplitTtHor && !mtt_split_cu_vertical_flag ) ) |  |
| mtt_split_cu_binary_flag | ae(v) |
| if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_BT_VER ) { |  |
| depthOffset += ( x0 + cbWidth > pic_width_in_luma_samples ) ? 1 : 0 |  |
| x1 = x0 + ( cbWidth / 2 ) |  |
| coding_tree( x0, y0, cbWidth / 2, cbHeight, qgOn, cbSubdiv + 1, cqtDepth, mttDepth + 1, depthOffset, 0, treeType ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_tree( x1, y0, cbWidth / 2, cbHeightY, qgOn, cbSubdiv + 1, cqtDepth, mttDepth + 1, depthOffset, 1, treeType ) |  |
| } else if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_BT_HOR ) { |  |
| depthOffset += ( y0 + cbHeight > pic_height_in_luma_samples ) ? 1 : 0 |  |
| y1 = y0 + ( cbHeight / 2 ) |  |
| coding_tree( x0, y0, cbWidth, cbHeight / 2, qgOn, cbSubdiv + 1, cqtDepth, mttDepth + 1, depthOffset, 0, treeType ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| coding_tree( x0, y1, cbWidth, cbHeight / 2, qgOn, cbSubdiv + 1, cqtDepth, mttDepth + 1, depthOffset, 1, treeType ) |  |
| } else if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_TT_VER ) { |  |
| x1 = x0 + ( cbWidth / 4 ) |  |
| x2 = x0 + ( 3 * cbWidth / 4 ) |  |
| qgOn = qgOn && ( cbSubdiv + 2 <= cu_qp_delta_subdiv ) |  |
| if(treeType != DUAL_TREE_CHROMA) |  |
| qgOn = qgOn && ( cbSubdiv + 2 <= cu_qp_delta_subdiv ) |  |
| else |  |
| qgOn = qgOn && ( cbSubdiv + 2 <= chroma_cu_qp_delta_subdiv ) |  |
| coding_tree( x0, y0, cbWidth / 4, cbHeight, qgOn, cbSubdiv + 2, cqtDepth, mttDepth + 1, depthOffset, 0, treeType ) |  |
| coding_tree( x1, y0, cbWidth / 2, cbHeight, qgOn, cbSubdiv + 1, cqtDepth, mttDepth + 1, depthOffset, 1, treeType ) |  |
| coding_tree( x2, y0, cbWidth / 4, cbHeight, qgOn, cbSubdiv + 2, cqtDepth, mttDepth + 1, depthOffset, 2, treeType ) |  |
| } else { /* SPLIT_TT_HOR */ |  |
| y1 = y0 + ( cbHeight / 4 ) |  |
| y2 = y0 + ( 3 * cbHeight / 4 ) |  |
| qgOn = qgOn && ( cbSubdiv + 2 <= cu_qp_delta_subdiv ) |  |
| If(treeType != DUAL_TREE_CHROMA) |  |
| qgOn = qgOn && ( cbSubdiv + 2 <= cu_qp_delta_subdiv ) |  |
| else |  |
| qgOn = qgOn && ( cbSubdiv + 2 <= chroma_cu_qp_delta_subdiv ) |  |
| coding_tree( x0, y0, cbWidth, cbHeight / 4, qgOn, cbSubdiv + 2, cqtDepth, mttDepth + 1, depthOffset, 0, treeType ) |  |
| coding_tree( x0, y1, cbWidth, cbHeight / 2, qgOn, cbSubdiv + 1, cqtDepth, mttDepth + 1, depthOffset, 1, treeType ) |  |
| coding_tree( x0, y2, cbWidth, cbHeight / 4, qgOn, cbSubdiv + 2, cqtDepth, mttDepth + 1, depthOffset, 2 , treeType) |  |
| } |  |
| } else { |  |
| x1 = x0 + ( cbWidth / 2 ) |  |
| y1 = y0 + ( cbHeight / 2 ) |  |
| coding_tree( x0, y0, cbWidth / 2, cbHeight / 2, qgOn, cbSubdiv + 2, cqtDepth + 1, 0, 0, 0, treeType ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_tree( x1, y0, cbWidth / 2, cbHeight / 2, qgOn, cbSubdiv + 2, cqtDepth + 1, 0, 0, 1, treeType ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| coding_tree( x0, y1, cbWidth / 2, cbHeight / 2, qgOn, cbSubdiv + 2, cqtDepth + 1, 0, 0, 2, treeType ) |  |
| if( y1 < pic_height_in_luma_samples && x1 < pic_width_in_luma_samples ) |  |
| coding_tree( x1, y1, cbWidth / 2, cbHeight / 2, qgOn, cbSubdiv + 2, cqtDepth + 1, 0, 0, 3, treeType ) |  |
| } |  |
| } else |  |
| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |

7.3.7.10 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| … |  |
| If(treeType != DUAL_TREE_CHROMA) { |  |
| if( ( tu_cbf_luma[ x0 ][ y0 ] ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) && treeType != DUAL_TREE_CHROMA ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| } |  |
| else if( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) { |  |
| if( chroma_cu_qp_delta_enabled_flag && !IsChromaCuQpDeltaCoded ) { |  |
| chroma_cu_qp_delta_abs | ae(v) |
| if( chroma_cu_qp_delta_abs ) |  |
| chroma_cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

7.4.3.4 Picture parameter set RBSP semantics

chroma_cu_qp_delta_enabled_flag equal to 1 specifies that the chroma_cu_qp_delta_subdiv syntax element is present in the PPS and that chroma_cu_qp_delta_abs may be present in the transform unit syntax. chroma_cu_qp_delta_enabled_flag equal to 0 specifies that the chroma_cu_qp_delta_subdiv syntax element is not present in the PPS and that chroma_cu_qp_delta_abs is not present in the transform unit syntax.

chroma_cu_qp_delta_subdiv specifies the maximum cbSubdiv value of coding units that convey chroma_cu_qp_delta_abs and chroma_cu_qp_delta_sign_flag. The value range of chroma_cu_qp_delta_subdiv is specified as follows:

- If slice_type is equal to I, the value of chroma_cu_qp_delta_subdiv shall be in the range of 0 to 2 * ( log2_ctu_size_minus2 − log2_min_qt_size_intra_slice_minus2 + MaxMttDepthY ), inclusive.

When not present, the value of chroma_cu_qp_delta_subdiv is inferred to be equal to 0.

#### 7.4.8.10 Transform unit semantics

chroma_cu_qp_delta_abs specifies the absolute value of the difference ChromaCuQpDeltaVal between the quantization parameter of the current coding unit and its prediction.

chroma_cu_qp_delta_sign_flag specifies the sign of ChromaCuQpDeltaVal as follows:

- If chroma_cu_qp_delta_sign_flag is equal to 0, the corresponding ChromaCuQpDeltaVal has a positive value.

- Otherwise (chroma_cu_qp_delta_sign_flag is equal to 1), the corresponding ChromaCuQpDeltaVal has a negative value.

When chroma_cu_qp_delta_sign_flag is not present, it is inferred to be equal to 0.

When chroma_cu_qp_delta_abs is present, the variables IsCuQpDeltaCoded and CuQpDeltaVal are derived as follows:

IsChromaCuQpDeltaCoded = 1

ChromaCuQpDeltaVal = chroma_cu_qp_delta_abs * ( 1 − 2 * chroma_cu_qp_delta_sign_flag )

8.7 Scaling, transformation and array construction process

8.7.1 Derivation process for quantization parameters

When ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the following applies:

–	When treeType is equal to DUAL_TREE_CHROMA, the variable QpY is set equal to the luma quantization parameter QpY of the luma coding unit that covers the luma location ( xCb + cbWidth / 2, yCb + cbHeight / 2 ).

–	The variables qPCb, qPCr and qPCbCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 69, QpY + pps_cb_qp_offset + slice_cb_qp_offset + ChromaCuQpDeltaVal)	(8-928)

qPiCr = Clip3( −QpBdOffsetC, 69, QpY + pps_cr_qp_offset + slice_cr_qp_offset + ChromaCuQpDeltaVal)	(8-929)