| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0740-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0706 (CE8-related: 4x4 sub-block based dual-tree chroma IBC block-vector derivation) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jizheng Xu 8910 University Center Ln San Diego, CA 92122, USA | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document is a crosscheck report of JVET-O0706. For the test points tested, results match with those in the proposal.

# Introduction

JVET-O0706 proposes to change the subblock size of chroma IBC from 2x2 to 4x4. There are four tests:

Test 1: Chroma IBC subblock to be 4x4 and use the center BV.

Test 2: Further using center BV as default BV from CE8-1.5.b.

Test 3a: Test 1 and skip encoder conformance check and add a default IBC compensation mode when a BV is invalid.

Test 3b:  Test 2 and skip encoder conformance check and add a default IBC compensation mode when a BV is invalid.

# Results

ClassF/4:2:0TGM are used in the tests. Results match with those presented in the proposal.

Test 1, test 2, test 3a and test3b results are summarized in table 1-4 respectively.

Table 1. Results of test1.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.02% | 0.00% | 0.04% | 101% | 102% |
| 4:2:0 TGM | -0.07% | -0.08% | -0.12% | 102% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.01% | 0.08% | -0.04% | 101% | 103% |
| 4:2:0 TGM | -0.04% | -0.02% | -0.10% | 101% | 102% |

Table 2. Results of test 2.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 IBC=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.05% | -0.10% | -0.11% | 102% | 102% |
| 4:2:0 TGM | -0.30% | -0.37% | -0.45% | 102% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM 5.0 IBC=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.03% | -0.14% | -0.08% | 101% | 102% |
| 4:2:0 TGM | -0.12% | -0.12% | -0.23% | 101% | 102% |

Table 3. Results of test3a.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 IBC=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.04% | -0.47% | -0.59% | 101% | 99% |
| 4:2:0 TGM | -0.48% | -1.14% | -1.20% | 103% | 98% |
|  | Random Access |  |  |  |  |
|  | Over VTM 5.0 IBC=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.06% | -0.42% | -0.39% | 100% | 103% |
| 4:2:0 TGM | -0.17% | -0.61% | -0.61% | 101% | 100% |

Table 4. Results of test3b.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 IBC=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.08% | -0.49% | -0.59% | 102% | 98% |
| 4:2:0 TGM | -0.61% | -1.26% | -1.37% | 103% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM 5.0 IBC=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.08% | -0.44% | -0.40% | 101% | 102% |
| 4:2:0 TGM | -0.25% | -0.64% | -0.68% | 100% | 101% |