| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0756v3 |
| --- | --- |

| Title: | HDRTools: VTM software integration |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information / Proposal |  |  |
| Author(s) or Contact(s): | Dominik Mehlem Alexis Michael Tourapis David Singer One Apple Park Way Cupertino, California, 95014 United States | Tel: Email: | +1-408-228-7983 dmehlem@apple.com atourapis@apple.com singer@apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

This contribution presents our recent improvements to the HDRTools software package and in particular its integration with the VTM reference software. Such integration permits the direct computation of metrics such as Delta-E and PSNR-L from within the VTM encoder. This could potentially be helpful for HDR related experiments within the context of the VVC activity since a user can now generate such information without having to generate and process the reconstructed EXR output files, thus reducing both storage and processing requirements. Other improvements of the HDRTools package are also briefly discussed.

# Introduction

The current HDR content coding evaluation process in the context of the VVC standardization activity requires the computation of several additional distortion metrics beyond the computation of PSNR compared to the original source. These metrics include the Delta-E and PSNR-L metrics, which require the use of the HDRTools package [1] for their computation. In particular, the process involves first encoding and decoding the non-constant luminance PQ 4:2:0 YCbCr video data using the VTM reference software, followed by the use of the HDRConvert tool for converting such data to a linear light RGB representation stored as a series of half float EXR files. These files are then used for the computation of these two metrics using the HDRMetrics tool (Figure 1-1).

Figure 1-1. Anchor process for Delta-E and PSNR-L distortion computation

Unfortunately this process can considerably burden these simulations since it adds additional, rather considerable costs in file storage, I/O, and processing of the EXR files. It was therefore requested if it was possible that the entire process for computing the Delta-E and PSNR-L metrics was integrated within the VTM, thus avoiding the generation of the EXR files and all other steps that may be associated with this process.

# HDRTools integration in the VTM

To achieve the desire functionality we tried to modify the HDRTools package so as to allow more easily to import the resulting HDRLib library in other software. This was done by fixing namespace support in the software, resolving issues with variable naming, adding cmake support, and fixing several other issues, such as warnings related to the use of strncpy and strncat, that may impact software compilation. The HDRLib library was then combined with the VTM encoder by introducing new code prior to the distortion computation process that first converts the data from a non-constant luminance PQ 4:2:0 YCbCr 10 bit representation to a linear, floating point RGB representation using the appropriate HDRTools structures. This information is then directly used to compute the Delta-E and PSNR-L metrics, which are then immediately reported after the encoding process of each frame. This avoids the generation of the intermediate EXR files and saves both storage and processing time.

Our integration was performed on version 5.0 of the VTM reference software. Enabling the desired functionality currently requires running the cmake process for the modified VTM software using the option “-DEXTENSION_HDRTOOLS=1”, while also setting “CALCULATE_HDRMETRICS” to 1 in TypeDef.h prior to compilation.

The newest version of the HDRLib library can be found in branch “0.19-dev” of the HDRTools package. We strongly encourage the use of this version, as older version are not compatible with the implementation.

The process currently only works for PQ YCbCr material and should support both the 420 and 444 chroma formats. It is currently not supported for HLG content or for ICtCp representations. It also supports cropping, which can be specified during the encoding process, while the Delta-E and PSNR-L measurements are computed for only a single whitepoint. This whitepoint can be specified also in the encoder configuration file. In particular, the following parameters were introduced into the VTM software to support this functionality:

| Parameter | Description |
| --- | --- |
| CalculateHdrMetrics | Enables/disables HDR related metric computation |
| MaxSampleValue | Sample range of the linear representation (in cd/m2) |
| WhitePointDeltaE1 | White point value used for Delta E measurements |
| EnableTFunctionLUT | Expected transfer characteristics of the content |
| InputColorPrimaries | Expected color primaries of the content |
| InputSampleRange | Expected sample range |
| ChromaLocation | Chroma sampling location |
| ChromaUpsampleFilter | Chroma upsampling filter |
| CropOffsetLeft | Cropping window left offset |
| CropOffsetTop | Cropping window top offset |
| CropOffsetRight | Cropping window right offset |
| CropOffsetBottom | Cropping window bottom offset |

The following exemplary encoder call shows how these parameters could be used for the showgirl sequence:

./bin/EncoderApp -c cfg/encoder_randomaccess_vtm.cfg -c cfg/per-class/classH1.cfg -c cfg/per-sequence-HDR/H1_ShowGirl.cfg  --InputFile=orig/ShowGirl2.yuv --CalculateHdrMetrics=1 --MaxSampleValue=10000.0 --WhitePointDeltaE1=100.0 --EnableTFunctionLUT=1 --InputColorPrimaries=1 --InputSampleRange=0 --ChromaLocation=2 --ChromaUpsampleFilter=1 --CropOffsetLeft=10 --CropOffsetTop=10 --CropOffsetRight=-10 --CropOffsetBottom=-10

# Test results

Simulations were performed using version 5.0 of the VTM reference software. Five frames from four sequences, Hurdles, Market3, ShowGirl2, and Starting were tested using the new implementation versus the anchor method. The results are shown in Table 2-1.

Minor differences in performance can be observed between the Anchor measurements and the new VTM integrated measurements. These relate to the distortion introduced by the conversion to the half float EXR representation that is utilized by the Anchor data, which is avoided by the new method. In a sense, the new method is also more accurate.

Table 2-1. Performance comparison of the anchor method versus 
the combined VTM+HDRTools implementation

| Sequences | QP | Anchor |  | Test results |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Delta-E100[dB] | PSNR-L100[dB] | Delta-E100[dB] | PSNR-L100[dB] |
| Hurdles | 22 | 35.294 | 35.518 | 35.2945 | 35.5173 |
|  | 27 | 34.800 | 34.150 | 34.8001 | 34.1497 |
|  | 32 | 34.251 | 32.906 | 34.2510 | 32.9052 |
|  | 37 | 33.768 | 32.062 | 33.7676 | 32.0614 |
| Market3 | 22 | 34.663 | 35.970 | 34.6635 | 35.9699 |
|  | 27 | 34.165 | 34.681 | 34.1655 | 34.6799 |
|  | 32 | 33.509 | 33.280 | 33.5089 | 33.2794 |
|  | 37 | 32.783 | 32.008 | 32.7826 | 32.0077 |
| ShowGirl2 | 22 | 36.561 | 39.751 | 36.5591 | 39.7502 |
|  | 27 | 36.188 | 38.859 | 36.1868 | 38.8579 |
|  | 32 | 35.785 | 37.820 | 35.7837 | 37.8192 |
|  | 37 | 35.186 | 36.635 | 35.1845 | 36.6349 |
| Starting | 22 | 34.920 | 38.041 | 34.9235 | 38.0464 |
|  | 27 | 34.527 | 36.903 | 34.5297 | 36.9060 |
|  | 32 | 34.033 | 35.584 | 34.0344 | 35.5861 |
|  | 37 | 33.423 | 34.139 | 33.4241 | 34.1390 |

It should be noted that because of our modifications it might be easier to also integrate other processing steps of HDRTools, e.g. rescaling or format conversion, in the VTM or other tools.

Although we can provide our modifications as a separate software that integrates HDRTools and the VTM outside of the official reference software, we recommend adopting these modifications in the official VTM software so as to ease maintainance of the software and allow such support to persist across future versions of the VTM.

# References

- HDRTools Master branch: https://gitlab.com/standards/HDRTools/

- HDRTools 0.19-dev branch: https://gitlab.com/standards/HDRTools/tree/0.19-dev

- VVC VTM Reference software: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

Commit from 6/25 - 33e05832df8feddc95cb858547a9fcf9b5a8bd8c