| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0765 |
| --- | --- |

| Title: | Cross-check of JVET-O0341 Non-CE3: Unification of intra interpolation filter selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Fabien Racapé, Gagan Rath | Tel: Email: | (+1) 510-359-7732 fabien.racape@interdigital.com Gagan.rath@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This document is a cross-check report of the contribution JVET-O0341 on the unification of intra interpolation filter selection. It is reported that the average BD-rates match those provided by the proponents. No discrepancies have been found between the provided software and the description of the tests. The execution times are within the measurement error range.

# Description

The contribution JVET-O034 proposes to unify the selection of interpolation filters, which depends on the intra mode: MRL, ISP, or regular in VTM-5.0. The selection does not depend on ISP anymore, and the selection process with regular(method1) or MRL(method2).

Interpolation filter selection in the method1

| Intra prediction mode | MRL | Regular/ISP |
| --- | --- | --- |
| Integer sample copy modes | Cubic (fractional sample position 0) |  |
| Other angular modes | Cubic | Cubic or Gaussian (MDIS condition) |

Interpolation filter selection in the method2

| Intra prediction mode | MRL/ISP | Regular |
| --- | --- | --- |
| Integer sample copy modes | Cubic (fractional sample position 0) |  |
| Other angular modes | Cubic | Cubic or Gaussian (MDIS condition) |

# Experimental Results

Experiments were performed based on VTM-5.0, in All Intra configuration and Random access.

## Test 1

|  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.04% | 100% | 99% |
| Class A2 | 0.00% | -0.07% | -0.03% | 101% | 101% |
| Class B | 0.00% | -0.06% | 0.02% | 102% | 100% |
| Class C | 0.01% | -0.09% | 0.11% | 100% | 97% |
| Class E | 0.00% | -0.06% | -0.09% | 101% | 101% |
| Overall | 0.00% | -0.06% | 0.00% | 101% | 100% |
| Class D | 0.01% | 0.06% | -0.03% | 101% | 100% |
| Class F | 0.01% | 0.07% | 0.09% | 103% | 106% |

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

## Test 2

|  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | 0.04% | 100% | 99% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 99% |
| Class B | 0.00% | -0.06% | 0.01% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.03% | 99% | 97% |
| Class E | -0.01% | 0.00% | -0.06% | 101% | 101% |
| Overall | 0.00% | -0.01% | 0.01% | 100% | 99% |
| Class D | 0.01% | 0.08% | 0.01% | 101% | 100% |
| Class F | -0.01% | -0.02% | -0.03% | 104% | 106% |

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Patent rights declaration

Interdigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).