| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0793-v3 |
| --- | --- |

| Title: | CE3-related: MIP with simplified mode index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this proposal, three methods are proposed to simplify the VTM5.0 matrix-based intra prediction (MIP). Method 1 removes mapping between non-MIP and MIP modes and uses power-of-2 modes in each MIP group. To remove the non-MIP-to-MIP mapping table, the MPMs in MIP mode are removed, and MIP mode indices are coded using fixed length binarization without using any context variables. To remove MIP-to-non-MIP mapping table, during the non-MIP MPMs derivation and the direct mode (DM) derivation of a chroma intra block, MIP mode index is mapped to planar mode index. To efficiently utilize the fixed length binarization codewords, only power-of-2 modes are used in each MIP group. The number of MIP modes in the three MIP groups are reduced from (35, 19, 11) to (32, 16, 8). Based on method 1, method 2 further simplifies the matrix index derivation by changing the composition of the MIP mode index from “matrixIndex + (numberOfMode/2)*isTranspose” to “2*matrixIndex + isTranspose.” Therefore, the matrix index can be easily derived by right shifting the MIP mode index by one bit, and whether the MIP mode is transposed mode is derived by checking the parity bit of the MIP mode index. Method 3 further simplifies method 2 by reducing the numbers of MIP modes in the three MIP mode groups from (32, 16, 8) to (8, 8, 8).

Test results of method 2 are reported as follows.
VTM5.0-AI: {BD-rates = 0.03% (Y), -0.03% (Cb), 0.00% (Cr); EncT = 98%; DecT = 100%}
VTM5.0-RA: {BD-rates = 0.02% (Y), -0.08% (Cb), 0.09% (Cr); EncT = 99%; DecT = 99%}
VTM5.0-LB: {BD-rates = -0.01% (Y), 0.32% (Cb), 0.15% (Cr); EncT = 99%; DecT = 99%}

Test results of method 3 are reported as follows.
VTM5.0-AI: {BD-rates = 0.05% (Y), -0.01% (Cb), 0.02% (Cr); EncT = 97%; DecT = 100%}
VTM5.0-RA: {BD-rates = 0.04% (Y), 0.02% (Cb), 0.03% (Cr); EncT = 99%; DecT = 97%}
VTM5.0-LB: {BD-rates = 0.06% (Y), 0.28% (Cb), 0.10% (Cr); EncT = 99%; DecT = 99%}

# Introduction

In VTM5.0, all MIP blocks are categorized into three MIP mode groups according to the width and height of the coding block (CB). Group 0 contains 4x4 CB, group 1 contains 4x8, 8x4, and 8x8 CBs, and group 2 contains all other sizes. In group 0, group 1, and group 2, 35, 19, and 11 MIP modes are supported. To support 35, 19, and 11 modes, 18, 10, and 6 matrices are used in group 0, 1, and 2, respectively.

Before matrix vector multiplication, a matrix index is derived as follows. First, whether the MIP mode index (MipModeIdx) represents a transposed mode is checked. If the MipModeIdx is larger than half of the total number of modes in that group (numberOfMode), then it is a transposed mode, and then the matrix index is derived by MipModeIdx - numberOfMode/2. Otherwise, the matrix index is equal to MipModeIdx. For example, in gourp 0, if the MipModeIdx is larger than 17, then it is a transposed mode, and the matrix index is derived by MipModeIdx – 17.

In the MIP mode index coding, since three MPMs are used in MIP mode, the mode coding of the MIP mode index contains two parts. First, a syntax element intra_mip_mpm_flag is coded to indicate whether the current MIP mode is a MPM mode. If intra_mip_mpm_flag is true, a MPM index is coded using truncated unary binarization, and intra_mip_mpm_flag is coded using one context variable. If intra_mip_mpm_flag is false, the remaining mode index is coded using fixed length binarization.

Moreover, two mapping tables (MIP-to-non-MIP and non-MIP-to-MIP) are required to map between MIP mode index and non-MIP mode index. In MIP MPM derivation, if a neighbouring block is coded in non-MIP intra mode, the non-MIP mode index is mapped to one MIP mode index using the non-MIP-to-MIP mapping table. In non-MIP MPM derivation, if a neighbouring block is coded in MIP mode, the MIP mode index is mapped to non-MIP mode index using the MIP-to-non-MIP mapping table. Similarly, when one chroma intra block is coded in DM, and if the collocated luma block is MIP mode, the MIP mode index is mapped to a non-MIP mode index using the MIP-to-non-MIP mapping.

In this contribution, three methods are proposed to simplify the above processes in MIP.

# Proposed methods

## Method 1 – Removing mapping tables between MIP mode index and non-MIP mode index and using power-of-two modes in each MIP mode group

To remove the non-MIP-to-MIP mapping table, the MPMs are removed in MIP. The context variable used to code the flag that specifies whether the MIP mode index is a MPM is also removed. The MIP mode is coded using fixed length Binarization with bypass coding. To remove the MIP-to-non-MIP mapping table, in non-MIP MPM derivation and chroma intra block’s DM derivation, MIP mode index is mapped to planar mode index (INTRA_PLANAR). To efficiently use the fixed length binarization codewords, only power-of-2 MIP modes are used in each MIP group. The numbers of MIP modes in group 0, group 1, and group 2 are reduced from (35, 19, 11) to (32, 16, 8). In group 0, matrix #16 and matrix #17 are removed. In group 1, matrix #8 and matrix #9 are removed. In group 2, matrix #4 and matrix #5 are removed.

## Method 2 Simplified matrix index derivation

Based on method 1, the matrix index derivation is further simplified. The matrix index is set equal to MipModeIdx/2 and whether the current MIP mode is a transposed mode is specified by the parity bit of MipModeIdx.

## Method 3 – Method 2 with 8 modes available in all MIP mode groups

Method 3 further simplifies method 2 by reducing the number of MIP modes in group 0, group 1 and group 2 from (32, 16, 8), to (8, 8, 8). In group 0, matrix #5, matrix #9, matrix #15, and matrix #17 are used. In group 1, matrix #0, matrix #1, matrix #3, and matrix #8 are used. In group 2, matrix #0, matrix #1, matrix #2, and matrix #3 are used.

# Experiment results

The proposed methods have been implemented on VTM5.0 [1]. The common test conditions (CTC) [2] are used for evaluation. The results of method 2 and method 3 are summarized in Table 1 and Table 2, respectively.

Table 1.  Results of method 2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.06% | 0.08% | 98% | 99% |
| Class A2 | 0.05% | -0.04% | -0.02% | 97% | 100% |
| Class B | 0.02% | -0.07% | 0.01% | 97% | 99% |
| Class C | 0.02% | -0.03% | 0.00% | 98% | 100% |
| Class E | 0.03% | -0.02% | -0.08% | 97% | 106% |
| Overall | 0.03% | -0.03% | 0.00% | 98% | 100% |
| Class D | 0.05% | 0.12% | 0.03% | 98% | 99% |
| Class F | 0.00% | 0.02% | -0.02% | 99% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.09% | 0.26% | 99% | 99% |
| Class A2 | 0.05% | 0.08% | 0.11% | 99% | 99% |
| Class B | 0.03% | 0.01% | 0.16% | 99% | 100% |
| Class C | -0.04% | -0.29% | -0.15% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.08% | 0.09% | 99% | 99% |
| Class D | 0.05% | 0.16% | -0.03% | 99% | 99% |
| Class F | 0.04% | -0.04% | 0.10% | 99% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.41% | -0.29% | 99% | 98% |
| Class C | 0.01% | 0.22% | -0.02% | 99% | 100% |
| Class E | -0.04% | 0.31% | 1.11% | 100% | 99% |
| Overall | -0.01% | 0.32% | 0.15% | 99% | 99% |
| Class D | 0.04% | 0.28% | 1.20% | 97% | 101% |
| Class F | -0.01% | -0.11% | 0.41% | 98% | 98% |

Table 2.  Results of method 3

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.06% | 0.09% | 96% | 100% |
| Class A2 | 0.06% | 0.04% | 0.00% | 96% | 100% |
| Class B | 0.03% | -0.05% | 0.04% | 97% | 100% |
| Class C | 0.06% | -0.01% | 0.04% | 97% | 99% |
| Class E | 0.07% | -0.06% | -0.06% | 97% | 100% |
| Overall | 0.05% | -0.01% | 0.02% | 97% | 100% |
| Class D | 0.12% | 0.23% | 0.05% | 99% | 101% |
| Class F | 0.06% | 0.00% | -0.03% | 98% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | 0.30% | 99% | 96% |
| Class A2 | 0.04% | 0.14% | 0.06% | 99% | 99% |
| Class B | 0.05% | 0.12% | 0.05% | 99% | 96% |
| Class C | 0.03% | -0.20% | -0.14% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.02% | 0.05% | 99% | 97% |
| Class D | 0.08% | 0.00% | 0.21% | 99% | 100% |
| Class F | 0.05% | 0.00% | 0.04% | 100% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.65% | 0.35% | 99% | 99% |
| Class C | 0.08% | 0.26% | -0.20% | 99% | 99% |
| Class E | 0.08% | -0.30% | 0.10% | 100% | 97% |
| Overall | 0.06% | 0.28% | 0.10% | 99% | 99% |
| Class D | 0.01% | 0.31% | 0.49% | 98% | 99% |
| Class F | 0.04% | 0.29% | 0.22% | 97% | 98% |

# Conclusions

In this contribution, three methods are proposed to simplify the MIP effectively. The coding efficiency impact is reported to be very minor.

# References

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).