| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019; Document: JVET-O0066-v1. | Document: JVET-N0825-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0277 (CE3-related: Simplification and unification for intra reference sample filtering) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jay N Shingala Ashwin Natesan | Tel: Email: | +91 8066601000 jay.shingala@ittiam.com ashwin.natesan@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd |  |  |

_____________________________

# Abstract

This document reports crosscheck results of JVET-O0277 (CE3-related: Simplification and unification for intra reference sample filtering). The crosscheck results were verified and the implementation in the reference software was also inspected.

# Introduction

In JVET-N0277, the size-dependent threshold in interpolation filter selection is modified to simplify and unify the intra reference sample filter selections. There are two tests related to this proposal both of which were verified and tested. It is noted that the software implementation is in line with the idea submitted in the proposal. It is also noted that the value of intraHorVerDistThres for nTbs=2 is indicated as 16 in the proposal (as in the existing draft specification JVET-N1001-v10) while it is actually 20 in the anchor VTM-5.0 software. This confusion arises from the difference in the value specified in the specification and VTM5.0 reference software and does not directly affect the proposal and its results.

# Simulation Results

Simulation results under AI and RA configurations over VTM-5.0 anchor are shown below.

Table 1. Results of test 1 of JVET-O0277 over VTM-5.0 anchor

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.01% | 97% | 94% |
| Class A2 | 0.01% | -0.01% | -0.03% | 91% | 95% |
| Class B | 0.01% | -0.02% | -0.02% | 100% | 99% |
| Class C | -0.01% | -0.06% | -0.03% | 103% | 96% |
| Class E | 0.00% | -0.01% | -0.08% | 107% | 100% |
| Overall | 0.00% | -0.02% | -0.03% | 100% | 97% |
| Class D | -0.05% | 0.17% | -0.09% | 104% | 96% |
| Class F | -0.03% | 0.02% | -0.04% | 104% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.25% | 0.17% | 94% | 99% |
| Class A2 | 0.02% | -0.05% | 0.09% | 95% | 91% |
| Class B | 0.02% | 0.09% | -0.02% | 98% | 97% |
| Class C | -0.04% | -0.17% | -0.08% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.08% | 0.02% | 97% | 97% |
| Class D | 0.01% | 0.03% | 0.27% | 102% | 93% |
| Class F | -0.01% | 0.02% | -0.04% | 101% | 96% |

Table 2. Results of Test 2 of JVET-O0227 over VTM-5.0 anchor

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | -0.01% | 93% | 98% |
| Class A2 | 0.01% | -0.05% | -0.03% | 95% | 102% |
| Class B | 0.01% | -0.07% | -0.02% | 100% | 100% |
| Class C | -0.02% | -0.12% | 0.05% | 106% | 97% |
| Class E | 0.00% | 0.00% | -0.08% | 101% | 100% |
| Overall | 0.00% | -0.06% | -0.01% | 100% | 100% |
| Class D | -0.05% | 0.13% | -0.05% | 101% | 95% |
| Class F | -0.03% | 0.03% | -0.11% | 103% | 93% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | 0.16% | 96% | 104% |
| Class A2 | 0.03% | 0.09% | 0.15% | 94% | 99% |
| Class B | 0.00% | -0.04% | -0.13% | 99% | 95% |
| Class C | -0.05% | -0.15% | -0.26% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.05% | -0.05% | 98% | 99% |
| Class D | -0.03% | -0.09% | -0.04% | 102% | 93% |
| Class F | 0.00% | 0.04% | 0.19% | 100% | 99% |

# Conclusions

The BDRate data in the tables above exactly match the data in the original contribution with minor variations in the encoding and decoding times.

# References

- Chia-Ming Tsai, Chih-Wei Hsu, Yu-Wen Huang and Shaw-Min Lei, “CE3-related: Simplification and unification for intra reference sample filtering”, JVET-O0277, 15th Meeting: Gothenburg, SE, 3–12 July 2019

- Benjamin Bross, Jianle Chen, Shan Liu, “Verstaile Video Coding (Draft 5)”, JVET-N1001, 14th Meeting: Geneva, SE, 19–27 Mar. 2019