| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0845-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0343 (CE3-related: Simplification on CCLM process) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2782 masaru.ikeda@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution reports cross-check results of JVET-O0343 which is CE3-related: Simplification on CCLM process [1]. The simulation was conducted with VTM-5.0 under Common Test Conditions [2]. It was confirmed that the test results were matched with those proponents provided.

Results

The simulation was conducted with VTM-5.0 for AI and RA [3] under CTC. The decoding time was locally measured on the same machines. The simulation for the method-2 is still running. It was confirmed that results for the method-1 were matched with those proponents provided.

Method 1

Table 1: BD-Rate performance for AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | 0.01% | 101% | 101% |
| Class A2 | 0.00% | 0.01% | -0.01% | 98% | 101% |
| Class B | 0.00% | 0.00% | 0.01% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.05% | 100% | 102% |
| Class E | 0.00% | -0.03% | -0.04% | 98% | 101% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 101% |
| Class D | -0.01% | 0.10% | 0.06% | 97% | 100% |
| Class F (optional) | 0.00% | -0.01% | -0.02% | 100% | 101% |

Table 2: BD-Rate performance for RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.27% | 0.06% | 103% | 100% |
| Class A2 | 0.01% | -0.01% | 0.03% | 102% | 100% |
| Class B | 0.01% | -0.13% | 0.02% | 100% | 100% |
| Class C | -0.03% | -0.09% | -0.13% | 103% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.12% | -0.01% | 102% | 100% |
| Class D | 0.04% | -0.03% | 0.41% | 103% | 100% |
| Class F (optional) | -0.01% | -0.01% | -0.02% | 98% | 100% |

Method 2

Table 3: BD-Rate performance for AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.16% | 0.20% | 100% | 99% |
| Class A2 | 0.01% | 0.00% | 0.01% | 98% | 100% |
| Class B | 0.00% | -0.04% | 0.04% | 101% | 99% |
| Class C | 0.01% | 0.03% | 0.12% | 99% | 101% |
| Class E | -0.01% | -0.03% | 0.00% | 96% | 100% |
| Overall | 0.00% | 0.02% | 0.08% | 99% | 100% |
| Class D | 0.00% | 0.07% | -0.13% | 99% | 100% |
| Class F (optional) | 0.01% | 0.19% | 0.27% | 100% | 100% |

Table 4: BD-Rate performance for RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.03% | 0.27% | 102% | 100% |
| Class A2 | 0.05% | 0.04% | -0.02% | 99% | 100% |
| Class B | 0.02% | 0.15% | 0.14% | 100% | 100% |
| Class C | -0.03% | -0.11% | 0.05% | 105% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.02% | 0.11% | 102% | 100% |
| Class D | 0.03% | 0.19% | 0.16% | 102% | 100% |
| Class F (optional) | 0.01% | 0.21% | 0.21% | 100% | 99% |

Reference

[1] D.-Y. Kim, J.-O. Lee (Chips&Media), S.-C. Lim, J. Lee, J. Kang (ETRI), “CE3-related: Simplification on CCLM process”, JVET-O0343, July 2019

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019.

[3] G. Van der Auwera, L. Li, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode  Coding”, JVET-N01023, March 2019.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).