| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0853 |
| --- | --- |

| Title: | Crosscheck of JVET-O0465 (Non-CE6 : Simplified SBT mode coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Zhipin Deng Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-O0465, where a simplified sub-block transform mode coding is proposed. The simulation results reportedly match those provided by the proponents.

# Verification

In the JVET-O0465 proposal [1], the cu_sbt_quad_flag, cu_sbt_horizontal_flag and cu_sbt_pos_flag syntax elements are coded in bypass mode. We verified that

- The source code implements match with what was described in the proposal.

- The standard specification changes match with what was proposed in the proposal.

- Simulation results match with those provided in the proposal.

CTC testing [2] is performed with the provided source code based on VTM-5.0 [3]. The coding results of the proposed method compared to VTM-5.0 anchor are summarized in Table 1.

The BD rate differences match with the results from the proposal [1]. The run times are basically in accordance with that from the proposal. Note that timing results are not well comparable due to differences in the simulation environment.

Table 1. Results of the proposed method compared to VTM-5.0 anchor

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.17% | 0.09% | 101% | 101% |
| Class A2 | 0.01% | -0.02% | 0.07% | 101% | 101% |
| Class B | 0.02% | 0.01% | -0.01% | 101% | 102% |
| Class C | 0.01% | -0.17% | -0.13% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.08% | -0.01% | 101% | 102% |
| Class D | 0.02% | -0.12% | 0.24% | 101% | 102% |
| Class F | 0.03% | 0.02% | 0.01% | 101% | 102% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.23% | 0.24% | 101% | 104% |
| Class C | 0.05% | -0.25% | -0.21% | 101% | 106% |
| Class E | -0.02% | 0.10% | -0.14% | 101% | 104% |
| Overall | 0.02% | 0.04% | 0.00% | 101% | 105% |
| Class D | -0.06% | -0.69% | 0.35% | 101% | 103% |
| Class F | 0.04% | -0.04% | 0.06% | 103% | 102% |

# References

F. Le Léannec, F. Urban, K. Naser, T. Poirier, “Non-CE6 : interaction between MTS and LFNST” , JVET-O0465, July 2019, Gothenburg, SE.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, CH.

VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0.