| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0878-v1 |
| --- | --- |

| Title: | Cross-check of JVET-O0619 (Non-CE7: Simplification of transform skip residual coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck of a Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution reports the cross-check results of the JVET-O0619 (Non-CE7: Simplification of transform skip residual coding). The source codes provided by the proponent were carefully analyzed and found to be consistent with the description in JVET-O0619. The cross-checker’s results matched with the results reported in JVET-O0619. The measured encoder and decoder runtime are similar to those in JVET-O0619.

Note: Some results are not finished and will be updated in a revision of the document.

Source code Analysis

JVET-O0619 proposed mainly three modifications to TS residual coding as below:

Modification #1: Reverse scanning order

Modification #2: Single pass coding of abs_level_gtx_flag[x], x=1 to 4, where two options are available as blow:

Method1:

Method2:

Modification #3: Unified by-pass coding, where the by-pass coding process of transform and TS residual coding are unified. In addition, the rice parameter is derived based on locSumAbs and baseLevel.

Source codes were carefully analyzed, and it is confirmed that implementations are consistent with the description in JVET-O0619.

Simulation Results

Following simulations were carried out under the common test condition with TGM sequences:

- Test1:	VTM-5.0 vs Modification#1 (reverse scanning order)

- Test2: VTM-5.0 vs Method 1 in Modification#2 (forward scanning + single pass coding of abs_level_gtx_flag)

- Test3: VTM-5.0 vs Method 2 in Modification#2 (forward scanning + single pass coding of abs_level_gtx_flag)

- Test4: VTM-5.0 vs Test2 + Modification#3 (unified by-pass coding)

Table 2-1 to 2-4 summarizes results of each Test1, Test2, Test3 and Test4, respectively.

Table 2-1: Results of Test1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.08% | 0.03% | 0.11% | 100% | 100% |
| Class E | 0.06% | 0.07% | 0.03% | 100% | 100% |
| Overall |  |  |  |  |  |
| Class D | 0.09% | 0.34% | 0.11% | 100% | 100% |
| Class F | 0.25% | 0.16% | 0.16% | 100% | 100% |
| Class SCC | 0.31% | 0.27% | 0.24% | 100% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.00% | -0.29% | -0.38% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.04% | 0.14% | 0.02% | 100% | 99% |
| Class F | 0.17% | 0.14% | 0.23% | 99% | 100% |
| Class SCC | 0.13% | 0.13% | 0.04% | 99% | 99% |

Table 2-2: Results of Test2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.02% | 0.00% | 102% | 102% |
| Class C | 0.00% | -0.01% | 0.01% | 101% | 100% |
| Class E | 0.00% | 0.01% | -0.04% | 102% | 102% |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.01% | -0.08% | 102% | 100% |
| Class F | 0.00% | -0.03% | 0.02% | 101% | 101% |
| Class SCC | 0.01% | 0.01% | 0.01% | 101% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.05% | -0.06% | 101% | 101% |
| Class C | -0.03% | -0.22% | -0.09% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.05% | -0.02% | 0.18% | 101% | 100% |
| Class F | -0.01% | -0.17% | 0.08% | 101% | 102% |
| Class SCC | -0.09% | -0.02% | -0.05% | 100% | 100% |

Table 2-3: Results of Test3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.00% | 0.02% | -0.01% | 100% | 100% |
| Class E | 0.00% | 0.04% | 0.04% | 100% | 100% |
| Overall |  |  |  |  |  |
| Class D | -0.01% | 0.07% | -0.13% | 101% | 100% |
| Class F | -0.04% | 0.09% | 0.13% | 101% | 101% |
| Class SCC | -0.04% | -0.01% | 0.00% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.04% | -0.21% | -0.22% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.01% | -0.05% | -0.13% | 100% | 100% |
| Class F | -0.01% | -0.04% | 0.19% | 101% | 101% |
| Class SCC | 0.00% | 0.04% | -0.09% | 100% | 99% |

Table 2-4: Results of Test4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.02% | 0.00% | 0.03% | 100% | 100% |
| Class E | 0.00% | 0.06% | 0.03% | 100% | 100% |
| Overall |  |  |  |  |  |
| Class D | 0.02% | 0.12% | -0.01% | 100% | 101% |
| Class F | 0.14% | 0.12% | 0.20% | 100% | 100% |
| Class SCC | 0.34% | 0.30% | 0.30% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.03% | -0.13% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.17% | 0.20% | 100% | 100% |
| Class F | 0.11% | 0.09% | 0.14% | 101% | 101% |
| Class SCC | 0.15% | 0.23% | 0.12% | 99% | 100% |

Reference

[1]  M. G. Sarwer, Y. Ye, “Non-CE7: Simplification of transform skip residual coding, ” JVET-O0619

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).