| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0886-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0486 (Non-CE3: Harmonization of 8-Bit MIP with Unified-MPM and LFNST using a single type of MIP prediction modes) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Z. Zhang | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document reports the results of crosschecking JVET-O0486 proposed by HHI. JVET-O0486 proposed to combine several changes which were proposed in other documents in Gothenburg 15th meeting.

JVET-O0486 proposed to use JVET-O0485 as starting point. The proposed change in JVET-O0485 were as follows:

- JVET-O0485 proposed to use 8-bit MIP implementation which was proposed JVET-O0481 as starting point.

- JVET-O0485 proposed to include the MIP simplifications which were proposed in JVET-O0084.

- JVET-O0485 proposed to enable LFNST for a MIP coded block if both width and height are equal or larger than 16. The MIP mode mapped to a conversional intra prediction mode by a LUT to do LFNST transform set selection.

- JVET-O0485 proposed to modify the conversional MPM list to exclude intra DC prediction mode when one of the two neighbouring blocks has an angular intra prediction mode. This is one aspect in JVET-O0484.

JVET-O0486 proposed to disable MIP on 4 x 4 block.

JVET-O0486 proposed to use 64 x 7 matrix for 4 x 8, 8 x 4 and 8 x 8 blocks. The memory storage for MIP matrix reduced to 2.3625 KB.

The update of the code had been reviewed and confirmed consistent with the proposal description.

The verification simulation results on top of VTM5.0 matched exactly the simulation results provided in JVET-O0486.

The encoding time for AI was 93% compared to VTM5.0. The reason was the full-RD checks were reduced for encoding an intra block.

# Verification

HHI had provided the results of their proposed methods in JVET-O0486.

The simulation was conducted under the common test conditions (CTC). The verification simulation results matched the results in JVET-O0486. It noticed that the encoding time for AI reduced to 93% compared to VTM5.0.

Verification results compared to VTM5.0:

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,14% | -0,23% | -0,23% | 91% | 97% |
| Class A2 | -0,15% | -0,18% | -0,14% | 93% | 98% |
| Class B | -0,16% | -0,20% | -0,11% | 90% | 98% |
| Class C | -0,07% | -0,08% | -0,08% | 99% | 104% |
| Class E | -0,20% | -0,17% | -0,22% | 91% | 98% |
| Overall | -0,14% | -0,17% | -0,15% | 93% | 99% |
| Class D | 0,05% | 0,18% | -0,07% | 92% | 95% |
| Class F | -0,06% | 0,04% | -0,06% | 95% | 98% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,07% | -0,50% | -0,32% | 94% | 94% |
| Class A2 | -0,01% | -0,17% | 0,03% | 95% | 94% |
| Class B | -0,01% | 0,09% | -0,13% | 96% | 99% |
| Class C | 0,00% | -0,25% | -0,06% | 95% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0,02% | -0,17% | -0,12% | 95% | 96% |
| Class D | 0,10% | -0,02% | 0,12% | 95% | 98% |
| Class F | 0,00% | -0,07% | 0,03% | 102% | 96% |

Tool off test results:

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-O0486 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,69% | 0,52% | 0,41% | 94% | 101% |
| Class A2 | 0,44% | 0,21% | 0,14% | 91% | 97% |
| Class B | 0,42% | 0,15% | 0,16% | 93% | 98% |
| Class C | 0,32% | 0,17% | 0,23% | 88% | 95% |
| Class E | 0,43% | 0,16% | 0,22% | 93% | 98% |
| Overall | 0,45% | 0,23% | 0,22% | 92% | 98% |
| Class D | 0,31% | 0,26% | 0,08% | 92% | 99% |
| Class F | 0,16% | 0,21% | 0,14% | 96% | 98% |

# References

[1]. Non-CE3: Harmonization of 8-Bit MIP with Unified-MPM and LFNST using a single type of MIP prediction modes

[2]. JVET-O0485: Non-CE3: Harmonization of 8-Bit MIP with Unified-MPM and LFNST

[3]. JVET-O0481: Variations of the 8-bit implementation of MIP

[4]. JVET-O0084: 8-bit implementation and simplification of MIP

[5]. JVET-O0484: CE3-related: Improved intra mode coding based on unified MPM list generation

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).