| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0888 |
| --- | --- |

| Title: | AHG10: Adaptive Coding Sub-set for encoder optimization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Bhat, David Gommelet, Jean-Marc Thiesse, Didier Nicholson VITEC 99 rue Pierre Semard 92320 Chatillon, France | Tel: Email: | +33 1 46 73 06 06 madhukar.bat@vitec.com david.gommelet@vitec.com jean-marc.thiesse@vitec.com didier.nicholson@vitec.com |
| Source: | VITEC |  |  |

_____________________________

# Abstract

This contribution introduces an additional flexibility capacity at the encoder level. This proposal tries to optimize the coding cost of specific parameters in case of underuse of the full standard capacity. Two scenarios are considered: underuse due to specific content at a given encoding point and desired underuse of modes due to a specific encoder implementation. While several mechanisms are already available to handle these underuses in previous standards and in current VVC draft, including high level flags and CABAC probability update, it is asserted that higher flexibility might bring a better trade-off for encoder design. The contribution proposes a subset signalization of specific coding modes at PPS or Slice level. This is presented in this contribution for two coding elements: the SAO offsets and the intra coding modes. Three way of sub-set signaling are evaluated in the contribution and it is asserted that early results show a potential to be further investigated.

# Introduction

This contribution tries to bring an additional flexibility to the encoder side for handling underuse of the full standard capacity. First, underuse of specific tools is common in the implementation of encoders, both in hardware and software, in order to meet a given speed/consumption target. While it is possible to fully switch off some tool when the complexity/gain trade-off is obvious [1], there are tools that are rather mandatory but they come with a significant bits overhead when not fully exploited. As an illustration, one can refer to the definition of x265 presets list [2] for an HEVC implementation. CTU sizes are limited to 32x32 while AMP and SAO for example are switched off in earlier presets: those choices are fully managed by the signalization. In the same time, Merge mode candidates are limited to 2 until the preset number 6 (slow mode) but this lead most of the time to the inability to reach the temporal candidate. One classical HEVC implementation choice is also to limit the number of intra prediction mode available, in particular for challenging small block sizes (4x4) or in the higher ones (64x64) due to the limited compression gain. A subset with only 11 intra modes {DC, Planar, 2, 6, 10, 14, 18, 22, 26, 30, 34} is for example commonly used. In that case, MPM and CABAC processing allow to limit the impact of modes underuse but there is still a remaining overhead. Smallest sets of intra modes were also considered [3] in the firsts JCT-VC meetings when designing the HEVC standard but keeping a fixed number of modes. This proposal introduces the signalization of subset of coding modes or parameters to match the implementation choice. Secondly, this can also help to leverage other coding options underuse which are content dependent. One encoder can for example run an analysis of the most relevant mode subset in a first pass encoding and then use this optimal subset for the second pass encoding. The motivation behind this is also to limit a potential sub-optimal choice made when designing the standard. While a plurality of content, encoding configuration and QP are considered by the group, it is assumed by authors that default over-configuration of coding choice can be critical in some applications or bitrates.

Thus, by providing an additional signalization flexibility, this contribution tries to address both:

- the early/low-complexity/real-time encoder implementation of the standard which can save several signalizations overhead in case of underuse of a specific tool with large number of coding choice and associated signalization.

- the former/enhanced implementation of the standard which can leverage this additional flexibility to increase the performance of their encoder by adapting the subset for a specific content and encoding point.

While the issue considered in this study was already present in former standard, it is asserted that the increasing coding modes and parameters choice in competition within VVC make such an approach even more relevant to help the VVC adoption thanks to efficient encoder implementation.

# Proposed method

## Overview

In this contribution, a method called Adaptive Coding Sub-set (ACS) is proposed to provide the ability to the encoder to define a subset of coding options for specific tool. This is enabled at PPS level according to pps_adaptive_coding_subset_enabled_flag flag and then the ACS mode used for each coding element handled by the method is specified at slice level in a syntax function: adaptive_coding_subset_data(). The intend by having it at slice level is to potentially change the data depending on the content and picture type. It is also possible to move these data at PPS level to limit the overhead in case of subset definition fixed for a whole encoding.

Several subset definition modes PARAM_acs_mode have been defined inspired by classical fast encoder implementation in order to limit the signalization burden. PARAM stands for the coding parameter considered for a given subset, for example sao or intra. Two modes are considered in this contribution:

- Decimation mode: in this mode, only one syntax element is transmitted, the decimation factor PARAM_acs_decimation_factor_minus1 which is in the range of 0 to log2 of the number of elements in the considered PARAM reference list. Thanks to this factor, the subset is built of 1 of decimation factor elements.

- Explicit mode: in this mode, two syntax elements are transmitted, the size of the subset PARAM_acs_explicit_num which provides the number of elements in the subset and which is in the range of 0 to the number of elements in the considered PARAM reference list; and the subset itself PARAM_acs_explicit_subset.

The proposed approach had been applied to SAO offsets and intra angular modes as detailed below. Other potential coding elements include Merge candidate list, SAO edge offset class and MMVD distance index for example.

## Application to SAO offsets

There are 8 and 32 SAO offset value in current version of VVC draft 5, respectively for 8 and 10-bit video signal. When studying the usage of each offsets, especially in 10-bit, it appears that the full range of the 32 offsets are not used. Considering also that covering the full range of offsets has a non-negligible cost, the proposed approach has been applied to these SAO offsets.

The proposed additional signalization for SAO is:

- at Slice level, within adaptive_coding_subset_data():

  - sao_acs_mode  equals to 0 to 2 specifies the adaptive coding subset mode used for SAO offsets subset generation.  sao_acs_mode equals 1 specifies fixed decimation of SAO offsets by an integer factor.  sao_acs_mode equals 2 specifies that SAO offsets are explicitly defined from a table.

  - sao_luma_acs_decimation_factor_minus1  specifies the decimation factor for the offsets subset generation in luma minus 1 when  sao_acs_mode  =1

  - sao_chroma_acs_decimation_factor_minus1  specifies the decimation factor for the offsets subset generation in chroma minus 1 when sao_acs_mode  =1

  - sao_acs_explicit_num  specifies number offsets when sao_acs_mode =2

  - sao_acs_explicit_subset   specifies selection of a specific pre-defined table offsets when  sao_acs_mode =2

- Changes in CABAC:

  - unary_max_eqprob(absOffset, maxOffsetQVal ) has been changed to take into account the new value of maxOffsetQVal depending on the number of offset defined in the subset.

## Application to intra mode candidates

The proposed approach is applied to intra mode for reducing the number of angular modes as represented in figure 1.

Figure 1. Decimation factor applied to intra angular modes (factors = 0, 1, 2, 3).

To be more detailled.

# Results

## SAO

The proposed approach applied to SAO have been implemented on top of VTM 5.0. Common test conditions are used for evaluation.  Two modes have been tested:

- Explicit mode where the selected offsets are {0,1,4,8,16,31}. There are signalized at PPS level as described in section 2.

- Decimation mode with the following settings:

Table 1: Decimation factor selection based on QP, picture type and temporal layer

| Decimation factor | getTLayer()=0 |  | getTLayer()=1 | getTLayer()=2 | getTLayer()>2 |
| --- | --- | --- | --- | --- | --- |
|  | I_SLICE | B_SLICE |  |  |  |
| 0 | QP <= 24 |  |  |  |  |
| 2 for chroma |  |  | QP < 35 | QP < 38 |  |
| 2 for luma and chroma | QP > 24 | All QP | QP >= 35 | QP >= 38 | All QP |

### Explicit mode

Table 2: Results of explicit table selection for All Intra

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EncoderApp_explicit_offsets_optimized.exe |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | -0,04% | -0,09% | 100% | #DIV/0! |
| Class A2 | 0,00% | -0,05% | -0,04% | 100% | #DIV/0! |
| Class B | 0,00% | -0,03% | -0,04% | 100% | #DIV/0! |
| Class C | 0,01% | -0,09% | -0,11% | 94% | #DIV/0! |
| Class E | 0,02% | -0,03% | -0,02% | 101% | #DIV/0! |
| Overall | 0,01% | -0,05% | -0,06% | 99% | #DIV/0! |
| Class D | 0,02% | -0,03% | -0,02% | 111% | #DIV/0! |
| Class F (optional) | -0,03% | -0,22% | -0,24% | 102% | #DIV/0! |

Table 3: Results of explicit table selection Low delay

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EncoderApp_explicit_offsets_optimized.exe |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.07% | -0.10% | -0.44% | 101% |  |
| Class E | 0.35% | 0.37% | -0.71% | 101% |  |
| Overall |  |  |  |  |  |
| Class D | 0.19% | -0.17% | 0.47% | 100% |  |
| Class F (optional) |  |  |  |  |  |

Table 4: Results of explicit table selection for Random Access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EncoderApp_explicit_offsets_optimized.exe |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.05% | -0.02% | 0.07% | 104% |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.19% | 0.14% | 0.27% | 99% |  |
| Class F (optional) |  |  |  |  |  |

### Decimation mode

Table 5: Results of optimized decimation factors for All Intra

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EncoderApp_SAO_decimation.exe |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.04% | -0.14% | 103% |  |
| Class A2 | 0.01% | -0.12% | -0.11% | 100% |  |
| Class B | 0.00% | -0.06% | -0.08% | 99% |  |
| Class C | 0.03% | -0.19% | -0.24% | 95% |  |
| Class E | 0.01% | -0.08% | -0.09% | 101% |  |
| Overall | 0.01% | -0.10% | -0.13% | 99% |  |
| Class D | 0.02% | -0.09% | -0.09% | 103% |  |
| Class F (optional) |  |  |  |  |  |

Table 6: Results of optimized decimation factors for Low delay

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EncoderApp_SAO_decimation.exe |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.06% | -0.47% | -0.81% | 102% |  |
| Class E | 0.23% | 0.13% | -0.59% | 102% |  |
| Overall |  |  |  |  |  |
| Class D | 0.15% | -0.34% | 0.56% | 99% |  |
| Class F (optional) |  |  |  |  |  |

Table 7: Results of optimized decimation factors for Random Access

|  | Over EncoderApp_SAO_decimation.exe |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.02% | -0.09% | -0.05% | 106% |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.16% | 0.19% | 0.37% | 100% |  |
| Class F (optional) |  |  |  |  |  |

## Intra modes in RA-10 (VTM 3.0)

This test has been run in VTM 3.0 and an update of the results in VTM 5.0 is running.

The proposed scheme is applied on all pictures with the decimation mode resulting to 19 intra modes and keeping the same number of intra RD checks. In table 8, the performances are compared against the reference with 67 modes while table 9 reports the results when compared to a reference with the same amount of intra modes (19) but using the full syntax.

It is reported that two sequences perform better than the reference with all the intra candidate (SlideEditing and SlideShow with -0.83% and -0.32% in luma). The overall bitrate savings when compared to the reference without ACS signalization is -0.41%, -0.39% and -0.60% in Y, U and V component.

Table 8: Results of intra ACS of 19 modes in Random Access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3_intra67mpm6.exe |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.16% | 0.71% | 1.84% |  |  |
| Class A2 | 1.35% | 1.75% | 2.01% |  |  |
| Class B | 1.69% | 1.50% | 1.20% |  |  |
| Class C | 1.71% | 1.82% | 1.55% |  |  |
| Class E |  |  |  |  |  |
| Overall | 1.52% | 1.48% | 1.58% |  |  |
| Class D | 0.84% | 2.35% | 0.52% |  |  |
| Class F (optional) | 0.66% | 1.21% | 0.25% |  |  |

Table 9: Results of intra ACS of 19 modes in Random Access against a reference with the same 19 intra modes.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM_3.0_Intra67_19_all.exe |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | 0.09% | -0.49% |  |  |
| Class A2 | -0.27% | -0.43% | -0.22% |  |  |
| Class B | -0.48% | -0.63% | -1.04% |  |  |
| Class C | -0.44% | -0.41% | -0.44% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.41% | -0.39% | -0.60% |  |  |
| Class D | -0.44% | -0.79% | -0.93% |  |  |
| Class F (optional) | -0.57% | -0.89% | -1.78% |  |  |

# Conclusion

In this contribution, a signalization dedicated to coding subset definition has been proposed. The intend was to provide an additional flexibility for the encoder implementers to exploit the expected coding tools powerfulness of VVC. Thanks to the proposed ACS signalization it is expected to limit the signalization overhead in case of underuse of the standard both for complexity or content adaptation reason. Early adaptation to two coding tools (SAO offsets and intra modes) was reported with results which are considered promising by the authors. Above all, the proposed approach is intended to be more straightforward to manage the significantly increasing number of coding elements in competition than current approach which is mostly build on a brute force optimization of the full set of coding elements. Further study of the approach is requested within the AHG 10 activity.

# References

[1] W.-J. Chien, J. Boyce, Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, S. Liu, D. Luo, “JVET AHG report: Tool reporting procedure (AHG13)”, JVET-N0013, March 2019.

[2] https://x265.readthedocs.io/en/default/presets.html

[3] M. Budagavi, “Angular intra prediction and ADI simplification”, JCTVC-B118, March 2012.

# Patent rights declaration(s)

VITEC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).