| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0891 |
| --- | --- |

| Title: | AHG6/AHG17: Cubemap-based projection syntax for 360-degree videos |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ya-Hsuan Lee, Jian-Liang Lin, Ying-Jui Chen, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 { louise.lee, jl.lin, yr.chen, cc.ju}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes the syntax for signalling cubemap-based projections which is flexible to adjust the sampling distribution by setting different coefficients of the mapping function. The proposed syntax can represent the same mapping as CMP, HEC, and an approximation of EAC.

Introduction

In the JVET 360 category [1], there are many efficient projections proposed. Many of them are based on the cubemap projection such as EAC [2] and HEC [3]. They adjust the sampling distribution of the conventional CMP to achieve better compression efficiency. However, instead of defining many types of projection formats, the proposed syntax design has the ability to cover many kinds of cubemap projections.

Syntax and semantics

The cubemap-based projection has six square faces which are defined as Front, Back, Right, Left, Top and Bottom. Each face is defined by a two-dimensional (u, v) plane as illustrated in Figure 1 with u and v in the range of [-1, 1].

Figure 1. Coordinates definition for a cubemap projection

To compress 360 videos, the video content on the sphere is projected to the cubemap faces. Adjustment functions, such as EAC and HEC, can be applied on the cubemap faces to achieve more uniform sampling distribution. The cube are then unfolded, and its faces, with or without rotation, are arranged by some pre-defined packing type to form a 2D picture for coding. The proposed syntax such as face index and rotation is aligned with 360Lib [1].

However, there may have discontinuous edges inside the cubemap picture which will create seam artifacts when reconstructing 360 videos. To deal with this issue, padding can be applied on the discontinuous edges and/or frame boundaries to protect the content.

Syntax

| vr_content( ) { | Descriptor |
| --- | --- |
| 360_video_projection_type | u(4) |
| if( 360_video_geometry_type == FORMAT_360_CMP ) { |  |
| cubemap_packing_type | u(2) |
| for( i = 0; i < num_cube_face_equal_to_6; i++ ) { |  |
| face_index[ i ] | u(3) |
| face_rotation[ i ] | u(2) |
| cmp_x_axis_adjustment_function_enabled_flag[ i ] | u(1) |
| if( cmp_x_axis_adjustment_function_enabled_flag[ i ] ) { |  |
| cmp_x_axis_coeff[ i ] | u(7) |
| cmp_x_axis_y_position_related_flag[ i ] | u(1) |
| } |  |
| cmp_y_axis_adjustment_function_enabled_flag[ i ] | u(1) |
| if( cmp_y_axis_adjustment_function_enabled_flag[ i ] ) { |  |
| cmp_y_axis_coeff[ i ] | u(7) |
| cmp_y_axis_x_position_related_flag[ i ] | u(1) |
| } |  |
| cmp_padding_enabled_flag | u(1) |
| if( cmp_padding_enabled_flag ) { |  |
| cmp_padding_type | u(1) |
| cmp_padding_width_minus1 | u(4) |
| } |  |
| } |  |
| } |  |

Semantics

360_video_projection_type specifies the projection format used in the frame.

cubemap_packing_type specifies the packing arrangement of the six faces of the cubemap projection as specified in Table 1. The number in the sqaure indicates the face index defined in face_index[ i ] and Table 2. By identifying cubemap_packing_type, the width and height of a cube face in luma samples can be determined by pic_width_in_luma_samples and pic_height_in_luma_samples, respectively. The value of cubemap_packing_type shall be in the range of 0 to 3, inclusive.

Table 1 – Specification of face arrangement, face width and height in luma samples based on cubemap_packing_type

| cubemap_packing_type | Face arragement | Face width in luma samples | Face height in luma samples |
| --- | --- | --- | --- |
| 0 |  | pic_width_in_luma_samples | pic_height_in_luma_samples / 6 |
| 1 |  | pic_width_in_luma_samples / 2 | pic_height_in_luma_samples / 3 |
| 2 |  | pic_width_in_luma_samples / 3 | pic_height_in_luma_samples / 2 |
| 3 |  | pic_width_in_luma_samples / 6 | pic_height_in_luma_samples |

face_index[ i ] identifies the index of each face in the cubemap projection and the corresponding coordintes of 3D (X, Y, Z) and 2D (u, v) as specified in Table 2. The value of face_index[ i ] shall be in the range of 0 to 5, inclusive.

Table 2 – Specification of cube face based on face_index[ i ]

| face_index[ i ] | Corresponding cube face | X | Y | Z |
| --- | --- | --- | --- | --- |
| 0 | Front face with positive X axis value | 1.0 | −v | −u |
| 1 | Back face with negative X axis value | −1.0 | −v | u |
| 2 | Top face with positive Y axis value | u | 1.0 | v |
| 3 | Bottom face with negative Y axis value | u | −1.0 | −v |
| 4 | Right face with positive Z axis value | u | −v | 1.0 |
| 5 | Left face with negative Z axis value | −u | −v | −1.0 |

face_rotation[ i ] specifies the rotation to be applied on each face as specified in Table 3. The value of face_rotation[ i ] shall be in the range of 0 to 3, inclusive.

Table 3 – Specification of counterclockwise rotation angle based on face_rotation[ i ]

| face_rotation[ i ] | Rotation angle (counterclockwise) |
| --- | --- |
| 0 | 0 |
| 1 | 90 |
| 2 | 180 |
| 3 | 270 |

cmp_x_axis_adjustment_function_enabled_flag[ i ] equal to 0 specifies that the sampling is not changed in x-axis. cmp_x_axis_adjustment_function_enabled_flag[ i ] equal to 1 specifies that the cubemap adjustment function is used to adjust the sampling with a coefficient in x-axis.

cmp_x_axis_coeff[ i ] specifies the coefficient used in the cubemap adjustment function in x-axis. The value of cmp_x_axis_coeff[ i ] shall be in the range of 0 to 27 – 1, inclusive.

When cmp_x_axis_coeff[ i ] is not present, it is inferred to be equal to 0.

cmp_x_axis_y_position_related_flag[ i ] equal to 1 specifies that the sampling in x-axis referes to the value of y position. When cmp_x_axis_y_position_related_flag[ i ] is equal to 0, the value of y position is not used.

When cmp_x_axis_y_position_related_flag[ i ] is not present, it is inferred to be equal to 0.

cmp_y_axis_adjustment_function_enabled_flag[ i ] equal to 0 specifies that the sampling is not changed in y-axis. cmp_y_axis_adjustment_function_enabled_flag[ i ] equal to 1 specifies that the cubemap adjustment function is used to adjust the sampling with a coefficient in y-axis.

cmp_y_axis_coeff[ i ] specifies the coefficient used in the cubemap adjustment function in y-axis. The value of cmp_y_axis_coeff[ i ] shall be in the range of 0 to 27 – 1, inclusive.

When cmp_y_axis_coeff[ i ] is not present, it is inferred to be equal to 0.

cmp_y_axis_x_position_related_flag[ i ] equal to 1 specifies that the sampling in y-axis referes to the value of x position. When cmp_y_axis_x_position_related_flag[ i ] is equal to 0, the value of x position is not used.

When cmp_y_axis_x_position_related_flag[ i ] is not present, it is inferred to be equal to 0.

Given a 3D position (x, y, 1) on a cube face, the adjusted sampling location (u, v) on the cubemap is derived as follows:

if( cmp_x_adjustment_function_enabled_flag[ i ] ) {

cmpCoeffX[ i ] = cmp_x_axis_coeff[ i ] / 27

t = cmpCoeffX[ i ] * x * ( cmp_x_axis_y_position_related_flag[ i ] * v2 – 1 )

if( t == 0 )

u = x

else		(1)

u = ( 1 – Sqrt( 1 – 4 * t ( x – t ) ) ) / ( 2 * t )

} else {

u = x

}

if( cmp_y_adjustment_function_enabled_flag[ i ] ) {

cmpCoeffY[ i ] = cmp_y_axis_coeff[ i ] / 27

t = cmpCoeffY[ i ] * y * ( cmp_y_axis_x_position_related_flag[ i ] * u2 – 1 )

if( t == 0 )

v = y

else		(2)

v = ( 1 – Sqrt( 1 – 4 * t ( y – t ) ) ) / ( 2 * t )

} else {

v = y

}

cmp_padding_enabled_flag equal to 0 specifies that no padding is applied on the cubemap projection. When cmp_padding_enabled_flag is equal to 1, the cubemap projection includes padding areas.

cmp_padding_type specifies the padding type of the cubemap projection. When cmp_padding_type is equal to 0, padding is applied on the discontinuous edges. When cmp_padding_type is equal to 1, padding is applied both on the discontinuous edges and frame boundaries. The location of the padding shall refer to cubemap_packing_type as specified in Table 4.

Table 4 – Specification of padding location based on cubemap_packing_type and cmp_padding_type

| cubemap_packing_type | cmp_padding_type | Location of padding |
| --- | --- | --- |
| 0 | 0 |  |
| 0 | 1 |  |
| 1 | 0 |  |
| 1 | 1 |  |
| 2 | 0 |  |
| 2 | 1 |  |
| 3 | 0 |  |
| 3 | 1 |  |

cmp_padding_width_minus1 plus 1 specifies the padding width in luma samples used in the cubemap projection. The value of cmp_padding_width shall be in the range of 0 to 15, inclusive.

Cubemap-based Projection

The forward adjustment function of the proposed cubemap-based projection is as follows:

|  | fxu, v=u1+ax(1-bxv2)(1-u2)fyu, v=v1+ay(1-byu2)(1-v2) | (3) |
| --- | --- | --- |

There are two coefficients in each axis that can make the sampling distribution have different behavior. ax and ay shall be in the range of [0, 1], and bx and by are binary numbers.

CMP

CMP can be exactly performed by setting a = 0 for both x- and y-axis. The forward and backward functions are described in (4) and (5), respectively.

|  | fxu=ufyv=v | (4) |
| --- | --- | --- |
|  | gxu=ugyv=v | (5) |

Approximated EAC

By setting a = 0.28 and b = 0 for both x- and y-axis, EAC can be simulated by using (6) and (7) for the forward and backward adjustment functions, respectively. With the proposed functions, EAC no longer needs the implementation of tangent and arctangent. Figure 2 shows the curve of the forward and backward adjustment functions comparing between EAC and approximated EAC using the above coefficients.

|  | fxu=u1+a(1-u2)fyv=v1+a(1-v2) | (6) |
| --- | --- | --- |
|  | gxu=u , if t=01-1-4t(u-t)2t, otherwise , t=-augyv=v , if t=01-1-4t(v-t)2t, otherwise , t=-av | (7) |

Figure 2. Forward and backward adjustment functions of EAC and approximated EAC

HEC

For the Top and Bottom faces of HEC, the same coefficients of the approximated EAC are used in both x- and y-axis. For the Left, Front, Right, Back faces, ax = 0.28 and bx = 0 are set to approximate EAC in x-axis while in y-axis, ay = 0.4 and by = 1 are set to perform the same mapping function of HEC. The forward and backward adjustment functions are as follows:

|  | fxu=u1+ax(1-u2)fyv=v1+ay(1-u2)(1-v2) | (8) |
| --- | --- | --- |
|  | gxu=u , if t=01-1-4t(u-t)2t, otherwise , t=-axugyv=v , if t=01-1-4t(v-t)2t, otherwise , t=-ayv(gx(u)2-1) | (9) |

Conclusion

In this contribution, the syntax of signaling a cubemap-based projection is proposed. With this format, many types of cubemap projections can be supported. Padding of the cubemap projection is also designed in this syntax.

References

- Y. Ye, and J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 9)”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1004, Jan. 2019, Marrakech, MA.

- M. Zhou, “AHG8: A study on Equi-Angular Cubemap projection (EAC),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G0056, Jul. 2017, Torino, IT.

- Y.-H. Lee, J.-L. Lin, S.-K. Chang, C.-C. Ju, “CE13: Modified Cubemap Projection in JVET-J0019 (Test 5),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0131, Jul. 2018, Ljubljana, SI.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).