| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0892 |
| --- | --- |

| Title: | Crosscheck of JVET-O0067 (Non-CE5: Modified coding method for ALF clipping parameter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Nan Hu 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | nanh@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution is a cross-check report of JVET-O0067. It is confirmed that the cross-checked results are identical with the proponent’s results except the simulation time is unreliable.

# Introduction

Brief technical description of JVET-O0067.

In the current VVC design, clipping parameters used in the non-linear adaptive loop filter process are coded in a similar way as filter coefficients. However, there are only four choices allowed for a clipping parameter. Using the Exponential-Golomb code with an adaptive order is not necessary. In addition, whether to signal a clipping parameter is dependent on whether the corresponding filter coefficient is unequal to zero. Such a dependency may impact the throughput. In this contribution, two aspects are proposed: 1) using fixed-length coding for ALF clipping parameters; 2) removing the dependency on values of ALF coefficients. Simulation results reportedly show negligible impact on compression performance.

# Simulation results

The simulation is conducted using VTM-5.0 with the test condition described in JVET-N1010.

## Test 1

In this test: exponential-Golomb binarization method is replaced by a fixed-length (2 bits) binarization method.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.01% | 101% | 101% |
| Class A2 | 0.00% | -0.01% | -0.01% | 101% | 101% |
| Class B | -0.01% | -0.01% | -0.01% | 100% | 99% |
| Class C | -0.02% | -0.02% | -0.02% | 99% | 98% |
| Class E | -0.02% | -0.02% | -0.02% | 100% | 102% |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.02% | -0.02% | -0.02% | 100% | 95% |
| Class F | -0.02% | -0.02% | -0.02% | 100% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class B | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class C | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.02% | -0.02% | -0.02% | 99% | 99% |
| Class F | -0.02% | -0.02% | -0.02% | 99% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.01% | 100% | 96% |
| Class C | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class E | -0.02% | -0.02% | -0.02% | 100% | 100% |
| Overall | -0.02% | -0.02% | -0.02% | 100% | 98% |
| Class D | -0.01% | -0.01% | -0.01% | 99% | 99% |
| Class F | -0.04% | -0.04% | -0.04% | 100% | 100% |

## Test 2

In this test, in addition to the method in test 1, the restriction that clipping parameter is only signaled for a non-zero filter coefficient is removed. With the proposed method, the dependency between parsing clipping parameters and the reconstruction of filter coefficients is removed.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | -0.01% | -0.01% | -0.01% | 100% | 99% |
| Class C | -0.02% | -0.02% | -0.02% | 99% | 102% |
| Class E | -0.02% | -0.02% | -0.02% | 100% | 103% |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 101% |
| Class D | -0.02% | -0.01% | -0.01% | 99% | 98% |
| Class F | -0.02% | -0.02% | -0.02% | 100% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | -0.01% | -0.01% | -0.01% | 100% | 101% |
| Class B | -0.01% | -0.01% | -0.01% | 100% | 99% |
| Class C | -0.01% | -0.01% | -0.01% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.01% | -0.01% | -0.01% | 99% | 100% |
| Class F | -0.02% | -0.02% | -0.02% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.01% | 100% | 97% |
| Class C | -0.01% | -0.01% | -0.01% | 101% | 101% |
| Class E | -0.01% | -0.01% | -0.01% | 99% | 100% |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 99% |
| Class D | -0.01% | -0.01% | -0.01% | 99% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

It is confirmed that the cross-checked results are identical with the proponent’s results except the running time is not reliable in the cross-checking.