| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1013 |
| --- | --- |

| Title: | Crosscheck of JVET-O0273: CE2-related: Luma-chroma latency reduction for chroma separate tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Tangi Poirier | Tel: Email: | +33.2.56.85.62.76 tangi.poirier@InterDigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results for JVET-O0273 [1], where split constraints are proposed for chroma tree and ISP is disabled for 64x64 CU, in case of chroma separated tree (CST), to reduce the luma-chroma latency to 32x32 chroma samples. Three methods are proposed. Results reportedly shows 0.18% / 2.35 % / 2.30 % BD rate loss in AI for method 1, 0.04% / 1.19% / 1.17% BD rate loss in AI for method 2, 0.10% / 1.53% / 1.49% BD rate loss in AI for method 3.

.

# Verification

We verified that

- The source code implements what was described in the proposal.

- Simulation results match with those provided in the proposal.

Only All Intra configuration is tested with the provided source code based on VTM-5.0. The code is compiled with GCC 6.3.1 compiler. The tests are conducted on a cluster with CentOS Linux 7 (Core) OS with hyperthreading off and turbo boost on. The coding results compared to VTM-5.0 anchors are shown in Table 1, Table 2 and Table 3. Please note that the encoding and decoding times are not reliable.

# Results

## Test 1

Table 1: results of method 1 in All Intra configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.30% | 3.87% | 3.17% | 118% | 111% |
| Class A2 | 0.19% | 2.43% | 1.41% | 125% | 112% |
| Class B | 0.17% | 2.59% | 3.23% | 129% | 115% |
| Class C | 0.08% | 0.47% | 0.60% | 129% | 114% |
| Class E | 0.17% | 2.84% | 3.01% | 126% | 111% |
| Overall | 0.18% | 2.35% | 2.30% | 126% | 113% |
| Class D | 0.04% | 0.51% | 0.26% | 131% | 114% |
| Class F (optional) | 0.17% | 1.16% | 1.45% | 124% | 112% |

## Test 2

Table 2: results of method 2 in All Intra configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 1.67% | 1.43% | 132% | 111% |
| Class A2 | 0.07% | 1.38% | 0.85% | 136% | 113% |
| Class B | 0.05% | 1.32% | 1.59% | 138% | 115% |
| Class C | 0.02% | 0.26% | 0.35% | 136% | 114% |
| Class E | 0.02% | 1.55% | 1.59% | 134% | 111% |
| Overall | 0.04% | 1.19% | 1.17% | 136% | 113% |
| Class D | 0.01% | 0.37% | 0.06% | 138% | 115% |
| Class F (optional) | 0.04% | 0.70% | 0.68% | 129% | 112% |

## Test 3

Table 3: results of method 3 in All Intra configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | 2.12% | 1.88% | 124% | 111% |
| Class A2 | 0.14% | 1.59% | 1.00% | 128% | 112% |
| Class B | 0.08% | 1.62% | 1.92% | 130% | 114% |
| Class C | 0.05% | 0.37% | 0.46% | 131% | 113% |
| Class E | 0.10% | 2.29% | 2.24% | 129% | 109% |
| Overall | 0.10% | 1.53% | 1.49% | 129% | 112% |
| Class D | 0.02% | 0.36% | 0.18% | 132% | 113% |
| Class F (optional) | 0.10% | 0.62% | 0.64% | 125% | 111% |

# Reference

[1] “CE2-related: Luma-chroma latency reduction for chroma separate tree “, JVET-O0273, 15th Meeting: Gothenburg, SE, 3–12 July 2019