| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1026-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0608 (CE2-related: On latency reduction for chroma residual scaling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 zhi-yi.lin@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document reports the results of crosschecking JVET-O0608 proposed by Kwai. JVET-O0608 proposes two methods to reduce the latency for the chroma residual scaling. In the first method, the luma prediction samples obtained prior to decoder motion vector refinement (DMVR), bidirectional optical flow (BDOF), and combined inter and intra prediction (CIIP) are used to calculate the chroma residual scaling factor. In the second method, luma uni-prediction samples are used to derive the scaling factor for chroma residuals. JVET-O0608 provides two additional results that combines their two proposed methods with CE2-1.1. The verification task was done, and the BD-rate results of all of the tests match exactly with those in JVET-O0608.

# Verification

Kwai provided the source code of their proposed method described in JVET-O0608 [1]. Four verification tests were carried out. Verification 1 and verification 2 are the two proposed methods in JVET-O0608. In the first method, the luma prediction samples obtained prior to DMVR, BDOF, and CIIP are used to calculate the chroma residual scaling factor. In the second method, luma uni-prediction samples are used to derive the scaling factor for chroma residuals. Verification 3 and verification 4 are the tests that combines the above two proposed methods with the subblock based chroma residual scaling method in CE2-1.1 [2].

The test is conducted under the common test conditions (CTC) defined in [3] using the VTM5.0 software [4]. Only the random access (RA) condition is tested. Table 1, Table 2, Table 3, and Table 4 show the results of using VTM5.0 as the anchor and enabling the proposed method 1, method 2, method 1 with CE2-1.1, and method 2 with CE2-1.2, respectively. The BD-rates from our experimental results match exactly those in JVET-O0608.

Table 1. The results of test 1 using VTM5.0 as anchor

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.10% | 0.06% | 101% | 100% |
| Class A2 | -0.01% | 0.03% | -0.05% | 101% | 101% |
| Class B | 0.00% | -0.03% | -0.14% | 102% | 96% |
| Class C | 0.00% | -0.04% | -0.08% | 102% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.06% | 101% | 98% |
| Class D | 0.02% | 0.11% | -0.07% | 101% | 99% |
| Class F | 0.00% | 0.04% | -0.03% | 102% | 97% |

Table 2. The results of test 2 using VTM5.0 as anchor

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.11% | 0.10% | 101% | 101% |
| Class A2 | -0.01% | 0.03% | -0.07% | 101% | 99% |
| Class B | 0.00% | -0.02% | -0.12% | 101% | 97% |
| Class C | 0.01% | -0.06% | -0.08% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.05% | 101% | 99% |
| Class D | 0.02% | 0.09% | -0.05% | 101% | 102% |
| Class F | -0.01% | 0.05% | -0.03% | 101% | 100% |

Table 3. The results of test 1 combined with CE2-1.1 using VTM5.0 as anchor

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.16% | 0.02% | 101% | 98% |
| Class A2 | 0.01% | 0.07% | -0.01% | 101% | 101% |
| Class B | 0.00% | -0.02% | -0.05% | 101% | 96% |
| Class C | 0.00% | -0.03% | -0.17% | 102% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.06% | 101% | 97% |
| Class D | 0.00% | -0.01% | 0.16% | 101% | 98% |
| Class F | 0.01% | 0.03% | 0.01% | 101% | 96% |

Table 4. The results of test 2 combined with CE2-1.1 using VTM5.0 as anchor

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.13% | 0.01% | 101% | 98% |
| Class A2 | 0.00% | 0.08% | -0.01% | 101% | 101% |
| Class B | 0.00% | -0.06% | -0.04% | 101% | 98% |
| Class C | 0.00% | -0.03% | -0.17% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.06% | 101% | 99% |
| Class D | 0.00% | -0.03% | 0.20% | 101% | 106% |
| Class F | 0.01% | 0.03% | 0.01% | 101% | 98% |

# References

[1] X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, S. Ye, X. Wang, “CE2-related: On latency reduction for chroma residual scaling,” Document of Joint Video Experts Team, JVET-O0608.

[2] Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE2-1.1: Simplified LMCS”, July 2019, Gothenburg, Sweden

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[4] VTM5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).