| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET- O1027-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-O0188 (Non-CE5: Non-linear ALF simplifications) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ivan Zupancic Fraunhofer HHI Einsteinufer 37, 10587 Berlin, Germany | Tel: Email: | +49 30 31002-181 ivan.zupancic@ hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results for JVET-O0188 (Non-CE5: Non-linear ALF simplifications), Test 3. The source code provided by the proponent was examined, and it has been confirmed that it matches the description from the input document JVET-O0188. Moreover, the specification text is aligned with the description in JVET-O0188, apart from a typo which is present in both JVET-O0188 and the specification text document. The cross checker could exactly reproduce the BD-rate results reported in JVET-O0188 (version 1) for the common test conditions, as described in JVET-N1010. Encoder and decoder run times are also very similar to the ones presented in the proposal JVET-O0188.

# Specification text

There is a typo in Eq. (7-87) in the specification text and in the corresponding part of JVET-O0188:

AlfClipC[ adaptation_parameter_set_id ][ j ] = Round( 2(BitDepthC * ( 4 − alf_chroma_clip_idx ][ j ] ) / 4 ) - 1)	(7-87)

There is an extra closing square bracket after alf_chroma_clip_idx which does not have a matching opening square bracket. Therefore, that closing square bracket should be deleted from the equation. Equation (7-87) should be corrected to:

AlfClipC[ adaptation_parameter_set_id ][ j ] = Round( 2(BitDepthC * ( 4 − alf_chroma_clip_idx [ j ] ) / 4 ) - 1)	(7-87)

# Experimental results

The crosscheck was performed for All Intra (AI), Random Access (RA), Low Delay B (LDB), and Low Delay P (LDP) configurations with the VTM-5.0 software under the common test conditions, as described in JVET-N1010. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, hyperthreading off, turbo boost off) with Linux OS and a GCC 7.2.1 compiler. Only the Test 3 from JVET-O0188 was tested. Full experimental results can be found in Table 1.

Table 1. Crosscheck results for AI, RA, LDB, and LDP configurations

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.04% | 101% | 100% |
| Class A2 | 0.00% | -0.02% | 0.01% | 99% | 100% |
| Class B | 0.00% | -0.01% | 0.00% | 100% | 101% |
| Class C | 0.01% | -0.01% | -0.01% | 100% | 100% |
| Class E | 0.00% | -0.01% | 0.00% | 100% | 102% |
| Overall | 0.00% | -0.02% | -0.01% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | 0.01% | 0.00% | 0.05% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | 0.08% | 99% | 100% |
| Class A2 | 0.02% | 0.01% | 0.01% | 99% | 100% |
| Class B | 0.04% | 0.02% | -0.08% | 100% | 102% |
| Class C | -0.01% | -0.13% | -0.10% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.04% | -0.04% | 100% | 101% |
| Class D | 0.05% | 0.12% | 0.40% | 100% | 101% |
| Class F (optional) | 0.02% | 0.10% | 0.09% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.41% | 0.39% | 100% | 101% |
| Class C | 0.05% | -0.05% | 0.05% | 100% | 102% |
| Class E | 0.05% | 0.21% | 0.04% | 100% | 100% |
| Overall | 0.03% | 0.21% | 0.19% | 100% | 101% |
| Class D | 0.08% | 0.44% | 0.81% | 101% | 100% |
| Class F (optional) | 0.05% | -0.26% | 0.70% | 100% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.28% | 0.42% | 98% | 100% |
| Class C | 0.02% | -0.38% | 0.13% | 100% | 102% |
| Class E | -0.03% | 0.99% | -0.14% | 101% | 104% |
| Overall | 0.00% | 0.24% | 0.18% | 99% | 102% |
| Class D | -0.04% | 0.08% | -0.37% | 100% | 100% |
| Class F (optional) | -0.11% | 0.84% | -0.12% | 101% | 101% |

The cross checker could exactly reproduce the BD-rate results reported in JVET-O0188 (version 1) for the common test conditions, as described in JVET-N1010. Encoder and decoder run times are also very similar to the ones presented in JVET-O0188.