| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–17 July 2019 | Document: JVET-O1039-v2 |
| --- | --- |

| Title: | BoG report on CE4 inter prediction with merge modifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Haitao Yang | Email: | chunchic@qti.qualcomm.com haitao.yang@huawei.com |
| Source: | BoG |  |  |

_____________________________

# Summary

The sessions were held 2045~2200 on July 3, 0900~1330 and 1500~1740 on July 4.

## Recommendations

### Recommended adoptions to VTM

Normative changes (0)

Non-normative changes (0)

### Recommended CE4 tests from following contributions

## Open issues

### Revisit in track (0)

### Contribution not reviewed yet (0)

# Triangle prediction merge mode

JVET-O0128 Non-CE4: simplification of triangle mode MV context storage [J. Li, C.S. Lim (Panasonic)]

This contribution is a response to AHG16 comment on complexity of MV context storage of triangle mode. Two methods are proposed. The first method is code optimization that moves CU level MV index mapping to slice level by using mapping list table. The second method is further simplification of MV index mapping process. The first method has no impact on coding efficiency. The second method has coding impact of 0.01% luma BD-rate in RA and 0.00% luma BD-rate in LDB over VTM-5.0 anchor.

Method 1 [implementation]: Move CU-level MV mapping function to slice-level. It is reported no coding performance-wise change.

Method 2: In blending area, if both MVs are from the same reference picture list, the first MV is stored in the motion field of TPM CU. (RA: 0.01% loss; LDB: no impact.)

No action.

JVET-O0724 Crosscheck of JVET-O0128 (Non-CE4: Simplification of triangle mode MV context storage) [R.-L. Liao (Alibaba)]

JVET-O0217 Non-CE4: Simplification of motion vector storing process for triangle merge mode [Y. Ahn, D. Sim (Digital Insights)]

In current VVC WD, the additional motion vector storing process is used to store motion information of triangle merge mode. Especially, a reference picture mapping process is performed to store the motion information of the blending area. This contribution proposes a simplification of motion vector storing process for triangle merge mode to remove the reference picture mapping process. The proposed method applies the adaptive selection to determine which motion information to be stored in the blending area according to the reference index. The proposed method shows 0.00% and 0.16% luma BD-rate in RA and LB configuration, respectively.

When the two uni-prediction MVs of triangle partitions refer to the same ref list, then the MV with smaller refidx is selected to replace the bi-prediction MVs at the blending area. In case that first triangle partition MV and the second one happens to have identical refidx, the MV of the upper triangle partition’s MV is selected.

RA: 0.00%; LB: 0.16% loss (Class E 0.27% loss, 0.42% loss in Johnny).

Revisit in BoG. Missing crosscheck report.

JVET-O0222 Non-CE4: Modification of triangle merge mode enabling criterion [D. Liu, R. Yu (Ericsson)]

In VTM-5.0, the triangular merge mode divides a coding unit into two triangular partitions. Uni-directional motion compensation is applied for each triangular partition and the diagonal edge between the two triangular partitions is blended by weighting matrices. The triangular merge mode is allowed for CU with either of its dimension greater than 64. To accommodate VPDU pipeline design but still realize the correct blending, different weighting matrices need to be applied for each VPDU since the blending area for each VPDU is different. The contribution proposes to disable the triangular merge mode when width equals 128 or height equals 128, such that the handling for each VPDU is unified and storage of weighting matrices for different VPDU is saved. The proposed method was tested under the VTM-5.0 random access (RA) and low delay (LD) under common test configuration. The BD-rate impact is -0.02% for RA and 0.03% for LD.

Since TPM supports 128xN/Nx128 CUs which exceed the size of 64x64 VPDU, the proposal disables these CU sizes in the application of TPM. (RA: 0.02% gain; LDB: 0.03% loss.)

It is stated that removing 128xN/Nx128 CUs helps align the processing unit to that of VPDU and also stated that this proposed method can reduce the storage size of weighting values (if they are not computed on the fly).

Same proposals: JVET-O0362 and JVET-O0249 Test 3.

The proponents of JVET-O0249 comments that disabling SBT in TPM is similar in spirit to remove 128xN/Nx128 CUs.

It is stated from a participant that this same topic is discussed in Marrakesh meeting in JVET-M0207/JVET-M0438. The related note is quoted from the CE10 BoG report (JVET-M0873), as below:

- It is remarked that 128x128 TPM might also violate VPDU condition. It is answered that this not the case, as it can be done on a 64x64 block basis.

As commented from several participants, it is noted that performing TPM on large block has complexity implications but no performance-wise benefit.

Recommendation: Adopt. (JVET-O0222/ JVET-O0249 Test 3/ JVET-O0362)

JVET-O0265 Non-CE4: Simplified motion field storage for TPM [N. Zhang, H. Liu, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

In this contribution, three simplified motion field storage methods for TPM are proposed. In method #1, the mapping process is removed for the storage of motion information in the weighted area. In method #2, bi-prediction motion is always stored. In method #3, the changes of method #1 and method #2 are combined under the low delay check. Simulation results reportedly show that there is no overall coding performance change for method #3 compared to VTM5.0.

Method 1: If both MVs are from same ref list, select bottom triangle’s MV. (RA:0.00%; LB:0.07%)

Identical proposals: JVET-O0629 method 2 / JVET-O0418 method 1 / JVET-O0329 method 1A / JVET-O0378 method 2 / JVET-O0279 method 1B / JVET-O0411 test B

Method 2: If both MVs are from same ref list, change bottom triangle partitoin’s ref picture list idx to the other (RA:0.06%; LB:0.00%). However, refidx availability check in the other reference picture list is required but is not considered in this proposal, and because CTC happens to have two identical reference picture list in Low-Delay case, thus Method 2 happens to avoid this bug.

Method 3: For non low-delay case, Method 1 is taken; For low-delay case, Method 2 is taken (RA:0.00%; LB:0.00%)

The proponent mentioned that the number of lines in sepc regarding TPM can be greatly reduced.

Revisit in BoG.

JVET-O0676 Crosscheck of JVET-O0265: Non-CE4: Simplified motion field storage for TPM [J. Zhao (LGE)]

JVET-O0279 CE4-related: Simplification of motion vector storing process for triangle prediction mode [Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes two methods to simplify the motion vector (MV) storing process at partition boundary for triangle prediction mode. Compared with VTM5.0, in method 1, the sequential reference picture search procedure is removed, and motion vector selection either depending on partition direction or not is applied when the two MVs of the two partitions point to the same reference picture list. In method 2, the motion vector selection by partition direction is always applied. Results are summarized as follows.

Proposed method 1
VTM5.0-RA: {Y-BD-rate=0.00%, EncT=100%, DecT=100%}
VTM5.0-LB: {Y-BD-rate=0.06%, EncT=100%, DecT=100%}

Proposed method 1 variant a
VTM5.0-RA: {Y-BD-rate=0.00%, EncT=100%, DecT=98%}
VTM5.0-LB: {Y-BD-rate=0.08%, EncT=100%, DecT=100%}

Proposed method 1 variant b
VTM5.0-RA: {Y-BD-rate=0.00%, EncT=100%, DecT=98%}
VTM5.0-LB: {Y-BD-rate=0.07%, EncT=100%, DecT=100%}

Proposed method 2
VTM5.0-RA: {Y-BD-rate=0.04%, EncT=100%, DecT=102%}
VTM5.0-LB: {Y-BD-rate=0.17%, EncT=100%, DecT=101%}

Method 1: select MVA or MVB depending on partitioning direction, when both ref picture list are identical.

Identical proposal: JVET-O0378 method 1 / JVET-O0329 method 1 / JVET-O0411 Test C / JVET-O0660 method 3

Method 1a: select MVA, when both ref picture list are identical.

Identical proposals: JVET-O0418 method 2 / JVET-O0329 method 1-B / JVET-O0314 / JVET-O0411 Test A

Method 1b (identical to JVET-O0265 method 1): select MVB, when both ref picture list are identical.

Method 2: select MVA or MVB depending on partitioning direction and it does not depend on whether ref picture list are the same or not to each motion vector. The larger coding loss in LDB case is that because the motion field of the entire TPM

Identical proposal: JVET-O0660 method 1 / JVET-O0329 method 2.

Revisit in BoG.

JVET-O0835 Crosscheck of JVET-O0279 (CE4-related: Simplification of motion vector storing process for triangle prediction mode) [H. Lee, S.-C. Lim, J. Lee, J. Kang (ETRI)]

JVET-O0314 Non-CE4: Modification of MV storage in triangle merge mode [P.-H. Lin (Foxconn)]

(Identical to JVET-O0279 method 1a)

JVET-O0759 Crosscheck of JVET-O0314 (Non-CE4: Modification of MV storage in triangle merge mode) [H.-J. Jhu (Kwai Inc.)]

JVET-O0329 Non-CE4: Simplification of motion vector storage process for triangle merge mode [H. Lee, S.-C. Lim, J. Lee, J. Kang, H. Y. Kim (ETRI)]

This contribution presents simplified methods on motion vector storage process for triangle merge mode. In method 1, when a 4x4 subblock is type-2 and two triangular partitions have motion information in the same reference list, the reference picture mapping process is removed. Instead, one of two triangular partition motion information is assigned based on triangular split direction information. In method 1-variant A, MVB and RefIdxB is always assigned to 4x4 subblock. In method 1-variant B, MVA and RefIdxA is always assigned to 4x4 subblock. In method 2, type-2 is removed so that a 4x4 subblock can be either type-0 or type-1. Compared with VTM-5.0, the method 1 incurs an average 0.00% and 0.06% luma BD-rate changes for RA and LDB, respectively. The method 1-variant A incurs an average 0.XX% and 0.XX% luma BD-rate changes for RA and LDB, respectively. The method 1-variant B incurs an average 0.00% and 0.08% luma BD-rate changes for RA and LDB, respectively. The method 2 incurs an average 0.04% and 0.17% luma BD-rate changes for RA and LDB, respectively.

Method 1 is identical to JVET-0279 method 1 (partition-direction based selection, when both MVs share the same ref pic list).

Method 1-A is identical to JVET-O0265 method 1.

Method 1-B is identical to JVET-O0279 method 1a .

Method 2 is identical to JVET-O0279 method 2.

JVET-O0881 Crosscheck of JVET-O0329 (Non-CE4: Simplification of motion vector storage process for triangle merge mode) [R. Yu, D. Liu (Ericsson)]

JVET-O0362 Non-CE4: Modified enabling condition for triangle mode [F. Chen, L. Wang (Hikvision)]

(Identical to JVET-O0222).

JVET-O0725 Crosscheck of JVET-O0362 (Non-CE4: Modified Enabling Condition for Triangle Mode) [R.-L. Liao (Alibaba)]

JVET-O0378 Non-CE4: Simplification of MV context storage for triangle mode [F. Chen, L. Wang (Hikvision)]

Currently, the MV context storage of triangle merge mode seems to be unnecessarily complicated. In this contribution, three methods are proposed to simplify the MV context storage. The first method selects one of the uni-prediction motion vector based on the split mode for the weighted areas. The second one just selects the bottom MV. The third one selects one of the uni-prediction motion vector based on the split mode or the average MVs for the weighted areas. The forth one keeps current method unchanged if the two reference lists are totally the same. It is reported that the first method brings 0.00% and 0.06% loss for RA and LB configurations respectively compared to VTM-5.0. The second one only brings -0.01% and 0.07% loss for RA and LB respectively compared to VTM-5.0. The third one only brings 0.02% and 0.04% loss for RA and LB respectively compared to VTM-5.0. And the forth one only brings -0.01% and 0.00% loss for RA and LB respectively compared to VTM-5.0.

Method 1 is identical to JVET-0279 method 1 (direction-based selection)

Method 2 is identical to JVET-O0265 method 1

Method 3: Average MVA and MVB if they both come from the same reference picture list and same reference picture; otherwise, method 1 is taken. (RA:0.02%; LB:0.04%)

Method 4: Slice-level to check whether two ref lists are identical to each other. If yes, swap ref picture list from one to the other (RA:-0.01%; LB:0.00%); otherwise, method 1 is taken.

Revisit in BoG.

JVET-O0880 Crosscheck of JVET-O0378 (Non-CE4: Simplification of MV context storage for triangle mode) [D. Liu, R. Yu (Ericsson)]

JVET-O0379 Non-CE4: Modified mode decision process for triangle mode [F. Chen, L. Wang (Hikvision)]

In VTM-5.0, the triangle prediction mode brings additional 8% and 12% encoding time for RA and LB configurations, respectively. Especially for LB configuration, the complexity is a heavy burden. In this contribution, a fast mode decision method of the triangle prediction mode is reported for encoder. This method forces the triangle prediction mode in the encoder to conduct rough RDO process to save the candidate number of true RDO modes. It is reported that the method saves 5% and 7% encoding time for RA and LB configurations respectively. And it brings 0.01% gains and 0.02% loss for RA and LB configurations respectively compared to VTM-5.0.

Encoder-only fast algorithm to speed up TPM mode.

In VTM-5.0, there are 40 RDO checks at least for TPM mode when best mode is skip, this proposal reduces the number of RDO check from 40 to 3. These 3 are selected depending on SATD and bit count without CABAC estimation for the 40 TPM candidates. The same encoder-only algorithm also applies when the condition that the best mode is not skip is satisfied.

As reported, this encoder-only fast algorithm saves encoding time by 5% in RA and 7% in LDB with around 0.02% of BD-rate reduction in LBD case.

(RA:-0.01%/95%/101%;  LB:0.02%/93%/100%)

The cross-checker confirms that there is only one line of change in the current VTM software.

It is commended from a participant that JVET-O0280 proposes a SIMD-based implementation to speed up the blending process as for TPM. (~2% speed up to encoder)

Recommendation (SW): Adopt.

JVET-O0726 Crosscheck of JVET-O0379 (Non-CE4: Modified Mode Decision Process for Triangle Mode) [R.-L. Liao (Alibaba)]

JVET-O0388 Non-CE4: On the context of triangle merge index [F. Chen, L. Wang (Hikvision)]

Currently, both of the two indices of the triangle prediction mode reuse the context of the regular merge index. Although the candidates of the triangle prediction mode is the same as that of regular merge mode, the two indices of the triangle prediction mode have different distribution feature since they must be different from each other. For this purpose, two more contexts are added for the triangle index coding. It is reported that the modification brings 0.00% gains and 0.06% gains for RA and LB configurations respectively compared to VTM-5.0.

In VTM-5, TPM mode reuses the context of merge index of regular merge mode. This proposal introduces two new contexts respectively for the 2 merge idx of TPM mode.

The results show coding performance-wise benefit only for LDB case, with 0.06% gain on average, where the peak gain is from Class E with 0.19% gain and peak loss from SlideEditing with 0.21% loss.

A participant mentioned that this proposal does not increase the number of context-coded bins of TPM mode. The only thing required is to switch the context that is to be used for regular merge and TPM mode.

Further study.

General recommendation to Track B:

It is commented that at this stage it is suggested to not to make changes to the number of CABAC context models for minor coding performance-wise impact.

It is also mentioned what would be the right timing to start working on making changes to CABAC context models.

JVET-O0727 Crosscheck of JVET-O0388 (Non-CE4: On the Context of Triangle Merge Index) [R.-L. Liao (Alibaba)]

JVET-O0411 Non-CE4: Simplification of bi-prediction MV generation for triangle partition mode storage [K. Reuzé, C.-C. Chen, W.-J. Chien, H. Huang, M. Karczewicz (Qualcomm)]

(Chaired by Haitao)

Triangle Partition Mode (TPM) requires the generation of a bi-prediction motion vector (biMv) for motion field storage in the blending area. It was mentioned in AHG16 that the process for generating this biMv is unnecessarily complex. It is thus proposed to simplify the generation of this biMv. Three methods are proposed to simplify this process. A first method provides X.XX% in RA / X.XX% in LDB. A second method provides X.XX% in RA / X.XX% in LDB. A third method provides no BD-rate loss in RA / X.XX% in LDB. A fourth method provides X.XX% in RA / X.XX% in LDB.

Test A is identical to JVET-O0279 method 1a

Test B is identical to JVET-O0265 method 1

Test C is identical to JVET-O0279 method 1

Test D: non-square handling

JVET-O0418 CE4-related: Simplification of motion vector storage operation for triangle merge mode [X. W. Meng (PKU), X. Zheng (DJI), S. S. Wang, S. W. Ma (PKU)]

(Method 1 is identical to JVET-O0265 method 1)

(Method 2 is identical to JVET-O0279 method 1a)

Crosscheck number: JVET-O0995.

JVET-O0489 Non-CE4: Geometrical partitioning for inter blocks [S. Esenlik, H. Gao, A. Filippov, V. Rufitskiy, A. M. Kotra, B. Wang, E. Alshina (Huawei), M. Bläser, J. Sauer (RWTH Aachen)]

Flexible non-rectangular partitioning for motion compensation blocks is proposed in this contribution. Triangle inter block is subset of proposed solution. On top of VTM-5.0 proposed method solution provides 0.24% and 0.55% luma BD-rate gain for RA and LD-B with ~5% encoding run-time increment and no decoding run-time change. Beside BD-rate gain proposed solution provides clear visual quality improvement.

This proposal introduces new geometric partitions for merge mode. This mode supports 32 angles & 5 distance values to indicate a variety of geometric partitioning patterns. In total there are only 140 new modes (but not 160=32*5) because some of the redundant partitioning patterns as for TPM can be removed.

A lookup-table based weighting function is introduces for blending samples on partitioning boundary inside a CU. The lookup table stores pre-calculated values of sine and cosine functions.

For motion storage, it sums up the four weight values at the corner of each 4x4 unit, and depending on the sum to differentiate whether uni-pred mv of triangle partitions or bi-pred mv should be stored.

For transform coding, it is required to remap a parallelogram shape of prediction residuals along the partitioning edge to square block for transform. For other samples, they are assumed to be zero.

(RA:-0.24%/105%/100%;)

If proposed transform is not applied, RA:-0.22%/106%/101%; LB:-0.44%/108%/102%.

The proponent claims that GEO is a straightforward extension to current TPM in VTM.

Visual quality comparisons are provided in the proposal, showing comparable viewing experience to that of the VTM anchor.

Non-proponents support further studying this tool due to the improved visual quality as compared to VTM-5.0 anchor.

Crosscheck number: JVET-O1010.

Recommendation: Further study in a CE.

JVET-O0513 Non-CE4: Adaptive blending filtering for TPM [S. Esenlik, H. Gao, B. Wang, A. Kotra, Z. Zhao, E. Alshina (Huawei), M. Bläser, J. Sauer (RWTH Aachen)]

(To discuss in CE8 BoG)

JVET-O0921 Crosscheck of JVET-O0513: Non-CE4: Adaptive blending filtering for TPM [X. Xiu (Kwai Inc.)]

JVET-O0552 Non-CE4: Extension of triangle partition mode to P-slices [K. Reuzé, C.-C Chen, W.-J. Chien, H. Huang, M. Karczewicz (Qualcomm)]

(Chaired by Haitao)

It is proposed to extend the use of TPM to P-slices, by removing the blending operation performed on the diagonal, and by only storing uni motion vectors, using the different proposed simplifications from JVET-O0411. Using the first method provides X.XX% coding gains in LDP configuration. The second method provides X.XX% coding gains in LDP configuration. The third method provides 0.30% coding gains in LDP configuration. The fourth method provides 0.39% coding gains in LDP configuration.

It is commented that the worst-case memory access is increased by the proposed method for LDP configuration. 4x4 uni-prediction vs. 4x8 uni-prediction.

An increase of encoder run-time over 20% is observed.

It is commented that there is a redundancy in syntax design of merge mode in current VTM-5.0 for LDP. Simply removing this redundancy could give about 0.22% coding gain for LDP.

Further study is encouraged.

JVET-O0601 CE4-related: On motion vector population under triangle prediction mode [X. Wang, Y.-W. Chen, X. Xiu, T.-C. Ma (Kwai Inc.)]

In the current VVC under triangle prediction mode, 4x4 blocks in each triangle partition are populated with the respective uni-prediction motion vector (MV) for that partition. For those blocks sitting on the triangle partition border, a potentially bi-prediction MV is derived from the two uni-prediction MVs and assigned to them. At decoder side, certain logic is needed to classify each individual 4x4 block into one of the three cases and assign a MV to the block accordingly. In this document, a method is proposed to simplify such motion vector population process, with each quarter area of a triangle mode CU filled with a same motion vector. Simulation results show that the proposed simplification method leads to {Y, U, V} BD-rate variations of {0.00%, -0.07%, 0.02%} and {0.02%, 0.23%, 0.05%} for RA and LD configurations, respectively.

The proposed change is to store bi-pred MVs not only on the diagonal 4x4 units but also to the whole quarter area of the CU sitting along the partitioning boundary. Therefore, all the 4x4 units inside a quarter shares the same motion information.

RA: 0.00%; LB: 0.02% (higher chroma loss in Class E with 0.56% loss, and peak chroma loss of 1.11% in FourPeople).

A participant commented that the proposed quarter-by-quarter motion storage would introduce mismatch between the motion field that is used for MC and that for deblocking.

It is also commented that the mismatch could potentially impact on the visual quality at CU boundaries.

No action.

JVET-O0629 Non-CE4: Simplification of motion field for triangle partition [R.-L. Liao, Y. Ye, J. Chen (Alibaba)]

In this contribution, the motion field storage for triangle partition mode is simplified. Firstly, 8x8 subblock is used to store motion instead of 4x4 subblock in VVC draft 5. Secondly, the bi-prediction motion generation process for subblocks along the diagonal/anti-diagonal is modified: when both of the uni-prediction motion vectors of the two triangle partitions are from the same reference picture list, the process of converting one of them to another reference picture list is removed. Instead, the motion vector of the bottom triangle partition is stored. It is reported that, under the CTC, the overall performance impact for {Y, U, V} is {0.03%, 0.04%, 0.00%} for RA and {0.04%, 0.17%, -0.05%} for LDB when 8x8 subblock is used. {0.00%, -0.05%, -0.02%} for RA and {0.09%, 0.02%, -0.38%} for LDB when the motion vector of bottom triangle partition is used. {0.02%, -0.02%, 0.01%} for RA and {0.09%, 0.08%, 0.06%} for LDB when both simplifications are applied.

Method 1: Enlarge the unit of motion storage from 4x4 to 8x8 (RA:0.03%/99%/99%;  LB:0.04%/100%/99%)

A non-proponent commented that for larger CUs, this design creates motion field that fits blending process more closely.

With 8x8 unit storage, the number of comparisons to check whether each motion storage unit belongs to partition 1, partition 2 or blending area is reduced from 496 to 120.

It is also commented if a CE is formed to study TPM motion storage related techniques, complexity characteristics (i.e., number of comparisons to determine motion storage) should be reported.

A non-proponent supports adopting method 1 and also commented that the loss of 0.03% is relatively minor against VTM anchor.

Method 2: Identical to JVET-O0265 method 1 (i.e., if both MVs are from the same ref picture list, bottom triangle’s MV is taken) (RA:0.00%/99%/100%; LB:0.07%/99%/99%)

Method 3: Test 1+2 (RA:0.02%/99%/99%; LB:0.09%/99%/99%)

Recommendation: Adopt method 1.

JVET-O0792 Crosscheck of JVET-O0629 (Non-CE4: Simplification of motion field for triangle partition) [Y.-L. Hsiao (MediaTek)]

JVET-O0645 Non-CE4/8: Blending-off Switch for TPM Mode [C.-C. Chen, Y. Zhang, K. Reuze, Y.-J. Chang, W.-J. Chien, M. Karczewicz (Qualcomm)]

(To discuss in CE8 BoG)

JVET-O0912 Crosscheck of JVET-O0645: Non-CE4/8: Blending-off Switch for TPM Mode [W. Zhu (Bytedance)]

JVET-O0646 Non-CE4/8: Combination of Blending-off Switch (JVET-O0645) and Simplified Motion Storage (JVET-O0411) for TPM Mode [C.-C. Chen, K. Reuze, Y. Zhang, W.-J. Chien, M. Karczewicz (Qualcomm)]

(To discuss in CE8 BoG)

JVET-O0763 Crosscheck of JVET-O0646 (Non-CE4/8: Combination of Blending-off Switch (JVET-O0645) and Simplified Motion Storage (JVET-O0411) for TPM Mode) [H. Yang (InterDigital)]

JVET-O0653 Non-CE4: Motion field storage simplification for triangular prediction mode [T. Solovyev, S. Ikonin, E. Alshina (Huawei)]

This contribution describes modification of motion field storage for triangular prediction mode. Alternative motion buffer filling pattern is proposed. The only motion information is used for the whole block except the area adjacent to the block borders. For the area adjacent to the block borders motion buffer filling method is the same as it is in VTM 5.0. The simulation results reportedly show no performance change for RA and 0.07% luma BD-rate loss for LDB configurations.

Non-boundary 4x4 units and the 2 corner 4x4 units along the partitioning boundary are all with bi-pred MVs. The rest are assigned with either one of the triangle partitions’ uni-pred MV.

(RA: 0.00%; LDB: 0.07%)

A non-proponent commented the proposed method still needs to figure out whether uni-pred MV is to be store at the bottom or upper line or to bi-pred MVs are to be store at the corners along the partition boundary.

The proponent claimed that in one implementation is to fill the whole CU with bi-pred MVs and then fill the remaining units close to CU boundary with either the partition 1’s MV or partition 2’s MV, and the benefit is that the number of comparisons as compared to that of TPM can be reduced.

A non-proponent commented that the proposed method may have to access the memory buffer that is used to store motion field twice, firstly for bi-pred MVs to fill all the 4x4 units per CU and secondly to update some of the boundary 4x4 units with either one of the uni-pred motion from partition 1 and 2.

No action.

JVET-O0660 Simplification of MV storage for blending block in triangle mode [K. Panusopone, L. Wang (Nokia)] [late]

This contribution proposes two approaches to simplify MV storage for blending blocks in triangle mode. The first approach eliminates blending blocks and only one MV is stored for all blocks in triangle mode. The second approach always stores one MV when MVs of first and second triangle partition are from the same reference list. The results show that methods 1 and 2 from the first approach have luma BD-rate of approximately 0.04% and 0.03% for RA configuration, respectively. The results also show that methods 3 and 4 from the second approach have luma BD-rate of approximately 0.00% and -0.01% for RA configuration, respectively.

Method 1 is identical to JVET-O0279 method 2. (RA: 0.04% ; LB: 0.17%)

Method 2 refers to bottom triangle’s MV if CU width > CU height and diagonal split. Otherise, it refers to right triangle’s MV.  (RA: 0.03% ; LB: 0.12%)

Method 3 is identical to JVET-O0279 method 1. (RA: 0.00% ; LB: 0.06%)

Method 4: if both MVs share identical ref picture list, apply Method 2. (RA: -0.01% ; LB: 0.07%)

Missing crosscheck.

Further study.

JVET-O0666 Non-CE4: Fix for missing MV in 4xN and Nx4 triangle partition mode [K. Reuzé, C.-C Chen, W.-J. Chien, H. Huang, M. Karczewicz (Qualcomm)]

(Chaired by Haitao)

It is proposed to extend the use of TPM to P-slices, by removing the blending operation performed on the diagonal, and by only storing uni motion vectors, using the different proposed simplifications from JVET-O0411. Using the first method provides X.XX% coding gains in LDP configuration. The second method provides X.XX% coding gains in LDP configuration. The third method provides 0.30% coding gains in LDP configuration. The fourth method provides 0.39% coding gains in LDP configuration.

It is claimed that this proposal can fix a problem in existing TPM where an update of the motion buffer of current coding unit is required. In the current design, bi-directional motion is stored for all 4x4 subblocks inside a 4xN or Nx4 CU using TPM. Without this fix, 1 bit needs to be stored for each 4x4 block to distinguish which motion vector from the bi-directional motion is used for which partition, in one possible implementation. And 128 bytes buffer is required for the storage of this indication flag of the whole CTU. It is claimed this additional 128 bytes could be saved.

It is commented that in another implementation where the motion compensation is performed for each 16x16 block considering pipeline design, the buffer required is 2 bytes.

It is commented that the update of the motion buffer of the current coding unit already exists in the current VTM-5.0. And there is no practical problem if the update is performed for TPM as well.

Revisit.

JVET-O0280 CE4-related: SIMD implementation for weighted sample prediction process of triangle prediction mode [Y.-L. Hsiao, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes single instruction multiple data (SIMD) implementation for the weighted sample prediction process of triangle prediction mode. Compared with VTM5.0, the encoding time is reduced by 2% for RA and 3% for LB without BD-rate changes.

Encoder-only modification to TPM for SIMD implementation on TPM blending operation.

Recommendation (SW): Revisit in BoG. Adopt this, if there is no further issue spotted after code review by Xiang Li and (cc. to Frank Bossen).

JVET-O0859 Crosscheck of JVET-O0280 (CE4-related: SIMD implementation for weighted sample prediction process of triangle prediction mode) [T. Hashimoto (Sharp)]

JVET-01016 Non-CE4: simplified triangle signaling [A. Robert, F. Le Léannec, T. Poirier, F. Galpin (InterDigital)] [late]

Withdrawn.

(Discussion for TPM storage at 1600, July 4)

(A) If both MVs share same ref pic list, then use right triangle's MV.

RA:0.00%; LB:0.06%

JVET-O0279 method 1 /   JVET-O0378 method 1 /  JVET-O0329 method 1 /  JVET-O0411 Test C /   JVET-O0660 method 3

It is commented that the mapping function from one reference pic list to the other may be just of implementation issue and can be done in slice level, that is, to use mapping tables for TPM CU. How this mapping table is only an implementation-wise issue.

A participant commented that if a certain solution is selected from these solutions, the text in the spec as for TPM can be greatly reduced.

A participant commented that such simplification design (i.e., removing reference list mapping) can save 2 comparisons (i.e., one for mapping from L0 to L1 and the other for mapping from L1 to L0).

A non-proponent suggests to take no action.

(B) If both MVs share same ref pic list, then use bottom triangle partition's MV.

RA:0.00%; LB:0.07%

JVET-O0265 method 1 / JVET-O0629 method 2 / JVET-O0418 method 1 / JVET-O0329 method 1A /   JVET-O0378 method 2 / JVET-O0411 test B / JVET-O0279 method 1B

The consensus is based on the assumption that this simplification is desirable to select Solution B from Solutions A-C.

It is commented this Solution can simplify the text of the spec significantly.

(C) If both MVs share same ref pic list, then use upper triangle's MV.

RA:0.00%; LB:0.08%

JVET-O0279 method 1A / JVET-O0418 method 2 / JVET-O0329 method 1B / JVET-O0314 / JVET-O0411 Test A

One competing proponent commented that this configuration looks less promising as the upper one is located further away from succeeding CUs and would impact on the performance of spatial mv prediction, which explains why this configuration incurs slightly higher coding loss than others’.

(D) Always use right triangle's MV.

RA:0.04%; LB:0.17%

JVET-O0279 method 2 / JVET-O0660 method 1 / JVET-O0329 method 2

This solution saves 1 additional comparison as compared to Solution A-C; however, according to the loss in LDB, this configuration does not seem to reach an acceptable performance-complexity trade-offs. It is also commented that saving this comparison cannot justify 0.1% loss in LDB case.

(E) If both MVs share same ref pic list, then use the MV with smaller refIdx and if two MVs happen to have same refIdx, then use upper triangle's MV.

JVET-O0217 | RA: 0.00%; LB: 0.16% loss (Class E 0.27% loss, 0.42% loss in Johnny).

This solution actually imposes extra condition check compared with Solution B and the coding loss is the highest among all the solutions.

No action.

(F) If both MVs share same ref pic list, then map bottom triangle’s ref pic list idx to the other (incomplete design)

JVET-O0265 method 2 | RA:0.06%; LB:0.00%

(G) If both MVs share same ref pic list, then:

If non low-delay case, then use bottom triangle partition's MV

Otherwise, map bottom triangle’s ref pic list idx to the other (incomplete design)

JVET-O0265 method 3 | RA:0.06%; LB:0.00%

(H) If same ref pic list and same ref pic POC, then average MVA and MVB.

Otherwise, If same ref list, then use right triangle's MV

JVET-O0378 method 3 | RA:0.02%; LB:0.04%

Compared with Solution B, this solution introduces one more condition check and it also requires to one additional 2 ADD and 2 SHIFT operations to average x and y components.

The increase of complexity does not provide extra coding performance improvement and thus is not justifiable to be consider for adoption.

No action.

(I) If both MVs share same ref pic list and two ref pic lists are identical, then map ref pic list idx to the other

Otherwise, if same ref list, then use right triangle's MV

It is commented from a participant that the reference picture list condition is somewhat overfitting to the Low-Delay configuration in CTC, and thus there is no observable performance-wise difference in LDB case.

JVET-O0378 method 4 | RA:-0.01%; LB:0.00%

Since there is no non-proponent supports this solution, no further action is required.

Although Solution B is generally agreed, there is still one proponent who proposed opposing solution and raised opinion to take no action.

Revisit in Track B.

# Merge mode with MVD

JVET-O0415 Non-CE4: Fix of POC distance condition for MMVD [N. Park, J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

JVET-O0868 Crosscheck of JVET-O0415 (Non-CE4: Fix of POC distance condition for MMVD) [N. Zhang (Bytedance)]

JVET-O0420 CE4-related: Reduce scaling operation number for MMVD [S. H. Wang (PKU), X. Zheng (DJI), S. S. Wang, S. W. Ma (PKU)]

JVET-O0761 Crosscheck of JVET-O0420 (CE4-related: Reduce scaling operation number for MMVD) [H.-J. Jhu (Kwai Inc.)]

JVET-O0633 Non-CE4: On MMVD candidate flag coding [X. Chen, H. Yang (Huawei)]

JVET-O0927 Crosscheck of JVET-O0633 (Non-CE4: On MMVD candidate flag coding) [T. Hashimoto, T. Ikai (Sharp)]

# Combined intra and inter prediction

JVET-O0384 CE4-related: CIIP Simplification [J. Y. Lee (Sejong Univ.), W. Lim, G. Bang, H. Y. Kim (ETRI)] [miss] [late]

JVET-O0421 Non-CE4: On CIIP Merge mode with no residual [G. Laroche, C. Gisquet, J. Taquet, P. Onno (Canon)]

JVET-O0522 Non-CE4: CIIP using triangular partitions [S. Blasi, G. Kulupana, A. Seixas Dias (BBC)]

JVET-O0537 Non-CE: Weighted intra and inter prediction mode [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-O0778 Crosscheck of JVET-O0537 (Non-CE: Weighted intra and inter prediction mode) [Y.-W. Chen (Kwai Inc.)]

JVET-O0661 Non-CE4/8: Simplification of CIIP on IBC mode [Y. Zhang, C.-C. Chen, Y.-J. Chang, W.-J. Chien, M. Karczewicz (Qualcomm)]

# Merge list construction

JVET-O0285 CE4-related: On merging candidate list construction [Y.-C. Lin, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-O0879 Crosscheck of JVET-O0285 (CE4-related: On merging candidate list construction) [H. Gao (Huawei)]

JVET-O0501 Non-CE4: Temporal merge modes simplification [F. Galpin, F. Le Léannec, A. Robert, T. Poirier (InterDigital)]

JVET-O0904 Crosscheck of JVET-O0501 (non-CE4: temporal merge modes simplification) [H. Huang (Qualcomm)]

JVET-O0589 Non-CE4: Simplification of HMVP in merge list construction [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-O0698 Crosscheck of JVET-O0589 (Non-CE4: Simplification of HMVP in merge list construction) [Y.-H. Chao (Qualcomm)]

# Merge mode syntax

JVET-O0220 Non-CE4: On maximum number of subblock merge candidates [R. Yu, D. Liu (Ericsson)]

JVET-O0221 Non-CE4: On merge data syntax [R. Yu, D. Liu (Ericsson)]

JVET-O0249 Non-CE4: Merge Modes Signaling [H. Huang, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-O0728 Crosscheck of JVET-O0249 (Non-CE4: Merge Modes Signaling) [R.-L. Liao (Alibaba)]

JVET-O0263 Non-CE4: Cleanups on syntax design for sub-block coding tools [K. Zhang, L. Zhang, H. Liu, N. Zhang, Z. Deng, Y. Wang (Bytedance)]

JVET-O0278 CE4-related: On signalling of merge flags [Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-O0828 Crosscheck of JVET-O0278 (CE4-related: On signalling of merge flags) [H. Chen (Huawei)]

JVET-O0305 Non-CE4: Undefined condition of merge mode syntax [K. Abe, T. Toma, J. Li, V. Drugeon (Panasonic)]

JVET-O0309 Non-CE4: Merge mode signalling overhead reduction [G. Ko, D. Kim, J. Jung, J. Son, J. Kwak (WILUS)]

JVET-O0829 Crosscheck of JVET-O0309 (Non-CE4: Merge mode signalling overhead reduction) [H. Chen (Huawei)]

JVET-O0330 Non-CE4: Syntax element ‘regular_merge_flag’ signalling [H. Lee, S.-C. Lim, J. Lee, J. Kang, H. Y. Kim (ETRI)]

JVET-O0770 Crosscheck of JVET-O0330 (Non-CE4: Syntax element ‘regular_merge_flag’ signaling) [Y.-W. Chen (Kwai Inc.)]

JVET-O0361 Non-CE4: Remove the redundancy among the flag coding for merge modes [F. Chen, L. Wang (Hikvision)]

JVET-O0422 Non-CE4: On mode flags signalling for Merge [G. Laroche, C. Gisquet, J. Taquet, P. Onno (Canon)]

JVET-O0431 Non-CE4: On Merge data signaling [H. Chen, X. Chen, H. Yang (Huawei)]

JVET-O0534 Non-CE4: Redundancy removal for merge mode signaling [J. Ye, X. Li, S. Liu (Tencent)]

JVET-O0560 Merge Mode Syntax [L. Wang, K. Panusopone (Nokia)]

JVET-O0585 Non-CE4: Merge syntax for removing redundant syntax signaling or fall back merge mode [H. Jang, N. Park, J. Nam, S. Kim, J. Lim (LGE)]

JVET-O0586 Non-CE4: Redundancy removal in merge modes signaling [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, S. Ye, X. Wang (Kwai Inc.)]

JVET-O0697 Crosscheck of JVET-O0586 (Non-CE4: Redundancy removal in merge modes signaling) [Y.-H. Chao (Qualcomm)]

JVET-O0639 Non-CE4: Context reduction for merge mode [J. Ye, X. Li, C. Auyeung, S. Liu (Tencent)]

JVET-O0911 Crosscheck of JVET-O0639: Non-CE4: Context reduction for merge mode [W. Zhu (Bytedance)]

JVET-O0795 CE4-related: On the context variables and syntax elements of merge index [?? (Mediatek)] [miss] [late]

JVET-O0849 Crosscheck of JVET-O0795 (CE4-related: On the context variables and syntax elements of merge index) [R.-L. Liao (Alibaba)]

# Subblock Temporal motion vector prediction

JVET-O0163 Non-CE4: Simplification on SbTMVP [H. Chen, H. Yang (Huawei)]

JVET-O0926 Crosscheck of JVET-O0163 (Non-CE4: Simplification on SbTMVP) [Y.-C. Lin (MediaTek)]

JVET-O0167 Non-CE4: On SbTMVP motion data derivation [J. Zhao, S. Kim (LGE)]

JVET-O0867 Crosscheck of JVET-O0167 (Non-CE4: On SbTMVP base motion data derivation) [N. Zhang (Bytedance)]

JVET-O0280 CE4-related: SIMD implementation for weighted sample prediction process of triangle prediction mode [Y.-L. Hsiao, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-O0859 Crosscheck of JVET-O0280 (CE4-related: SIMD implementation for weighted sample prediction process of triangle prediction mode) [T. Hashimoto (Sharp)]

JVET-O0283 CE4-related: Simplification for SbTMVP [Y.-C. Lin, C.-C. Chen, C.-W. Hsu, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-O0328 Non-CE4: On Sub-block Merge Candidates List [H. Lee, S.-C. Lim, J. Lee, J. Kang, H. Y. Kim (ETRI)]

JVET-O0423 Non-CE4: On SbTMVP derivation using HMVP [C. Gisquet, G. Laroche, P. Onno, J. Taquet (Canon)]

JVET-O0887 Crosscheck of JVET-O0423 (Non-CE4: On SbTMVP derivation using HMVP) [F. Henry (Orange), G. Clare (bcom)]

JVET-O0588 CE4-related: On SbTMVP motion shift derivation [Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-O0780 Crosscheck of JVET-O0588: CE4-related: On SbTMVP motion shift derivation [Y.-C. Sun (Alibaba)]

JVET-O0604 CE4-related: Simplification on block location validation in SbTMVP [Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)]

JVET-O0785 Crosscheck of JVET-O0604: CE4-related: Simplification on block location validation in SbTMVP [Y.-C. Sun (Alibaba)]