| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1075 |
| --- | --- |

| Title: | AHG12: On tile and brick signalling (combination of JVET-O0390 and JVET-O0551) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland Naël Ouedraogo Eric Nassor Franck Denoual Canon Research Centre France | Email: | firstname.lastname@nokia.com nael.ouedraogo@crf.canon.fr eric.nassor@crf.canon.fr franck.denoual@crf.canon.fr |
| Source: | Nokia, Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution is a merge of JVET-O0390 and JVET-0551.

The contribution proposes modified signalling of tile and brick partitioning, which is asserted to reduce:

- the number of syntax elements from 13 to 7,

- the number of syntax lines from 26 to 20,

- the length of the semantics,

- the length of the scanning process in clause 6.5.1, and

- the bit count spent for tile and brick partitioning of the effective 6K schemes of OMAF by more than 50%.

The contribution contains the following items:

- Skipping tile row related syntax and directly indicating bricks per tile column

- Having a single signalling mode instead of separate uniform and explicit tile/brick spacing modes

- Two options to reuse the same brick split pattern for several tile columns

# Introduction

Partitioning to tiles and bricks is indicated in two phases:

- A tile grid (i.e., tile column widths and tile row heights) is provided 
(indicated with light grey shading in the syntax table below).

- Second, the indicated tiles are further partitioned to bricks 
(indicated with blue shading in the syntax table below).

There are two modes of indicating the tile grid and splitting of tiles to bricks:

- Uniform spacing (indicated with the yellow highlighting in the syntax table below) where:

  - Tiles have equal width with the possible exception of the rightmost tile column and equal height with the possible exception of the bottom-most tile row.

  - Bricks of a tile have equal height with the possible exception of the bottom-most brick of the tile.

- Explicit spacing (indicated with the green highlighting in the syntax table below) where:

  - The widths and heights of the tile columns and rows (respectively) are indicated (in CTUs) except for the rightmost column and bottom-most row (respectively).

  - The brick heights are indicated (in CTUs) except for the bottom-most brick of a tile.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| … |  |

This contribution proposes:

- Skipping tile row related syntax and directly indicating bricks per tile column

- A single signalling mode instead of separate uniform and explicit spacing modes

- Two options to reuse the same brick split pattern for several tile columns

# Proposal

## Overview of the proposed syntax

The proposed syntax can be summarized as follows, with reference to the syntax tables presented in 2.2.

Tile columns are indicated as follows (see light grey shading in the syntax table below):

- The number of tile columns is indicated (num_tile_columns_minus1).

- The following is indicated in a loop that traverses tile columns from right to left until all tile columns have been traversed or until the remaining tile columns are indicated to have an equal width:

  - If the remaining width (in units of CTBs) is divisible by the number of tile columns yet to be specified, it is indicated if the remaining tile columns have an equal width (rem_tile_col_equal_flag[ i ]).

  - If the remaining tile columns do not have an equal width, the tile column width is indicated (tile_column_width_minus1[ i ]).

In a first option to reuse the same brick split pattern for several tile columns, the following is indicated in a loop that traverses tile columns from left to right for rectangular slices and only includes one loop entry that specifies the tile row heights for raster-scan slices (see blue shading in the syntax table below):

- A flag if the brick partitioning of the tile column is identical to that of the previous tile column. The flag is not present for the left-most tile column (copy_previous_col_flag[ i ]).

- When the tile brick partitioning of the tile column is not identical to that of the previous tile column, bricks of the tile column are indicated as follows:

  - The number of bricks in the tile column is indicated (num_bricks_minus1[ i ]).

  - The following is indicated in a loop that traverses bricks from bottom to top until all bricks of the tile column have been traversed or until the remaining bricks of the tile column are indicated to have an equal height:

  - If the remaining height (in units of CTBs) is divisible by the number of bricks yet to be specified, it is indicated if the remaining bricks have an equal height (rem_brick_height_equal_flag[ i ][ j ]).

  - If the remaining bricks do not have an equal height, the brick height is indicated (brick_height_minus1[ i ][ j ]).

In a second option to reuse the same brick split pattern for several tile columns,  the PPS specifies a number of brick split patterns (num_brick_patterns_minus1) followed by a set of brick split patterns on the height of the picture. The brick split patterns are defined prior to the tile specification. The syntax elements that describe each patterns is similar as option 1 and is as follows for the i-th brick split pattern:

- The number of bricks in the tile column is indicated (num_bricks_minus1[ i ]).

- The following is indicated in a loop that traverses bricks from bottom to top until all bricks of the tile column have been traversed or until the remaining bricks of the tile column are indicated to have an equal height:

- If the remaining height (in units of CTBs) is divisible by the number of bricks yet to be specified, it is indicated if the remaining bricks have an equal height (rem_brick_height_equal_flag[ i ][ j ]).

- If the remaining bricks do not have an equal height, the brick height is indicated (brick_height_minus1[ i ][ j ]).

The loop that traverses tile columns from right to left further specifies for each tile column the index of the brick split pattern that applies to the tile column. When the number of brick split patterns equals to 1, the brick index of each tile column is inferred equal to 0.

For raster scan slice mode, the number of brick split patterns equals to 1 (num_brick_patterns_minus1 equals 0). Each tile column contains the same brick patterns and the horizontal brick boundaries are inferred as tile rows.

## Proposed syntax

### Option 1

The red strikethrough text indicates the syntax lines that are proposed to be removed, whereas the colour-shaded syntax lines indicate the proposed additions to the syntax.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| num_tile_columns_minus1 | ue(v) |
| for( i = num_tile_columns_minus1, remWidthInCtbsY = PicWidthInCtbsY; i > 0 && !rem_tile_col_equal_flag[ i ]; remWidthInCtbsY −= tile_column_width_minus1[ i ] + 1, i− − ) { |  |
| if( remWidthInCtbsY % ( i + 1 ) = = 0 ) |  |
| rem_tile_col_equal_flag[ i ] | u(1) |
| if( !rem_tile_col_equal_flag[ i ] ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| } |  |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| for( i = 0; i <= num_tile_columns_minus1 * rect_slice_flag; i++ ) { |  |
| if( i > 0 ) |  |
| copy_previous_col_flag[ i ] | u(1) |
| if( i = = 0 ¦ ¦ !copy_previous_col_flag[ i ] ) { |  |
| num_bricks_minus1[ i ] | ue(v) |
| for( j = num_bricks_minus1[ i ], remHeightInCtbsY = PicHeightInCtbsY; j > 0 && !rem_brick_height_equal[ i ][ j ]; remHeightInCtbsY −= brick_height_minus1[ i ][ j ] + 1, j− − ) { |  |
| if( remHeightInCtbsY % ( j + 1 ) = = 0 ) |  |
| rem_brick_height_equal_flag[ i ][ j ] | u(1) |
| if( !rem_brick_height_equal_flag[ i ][ j ] ) |  |
| brick_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |

### Option 2

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !single_tile_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) { |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| num_brick_patterns_minus1 | ue(v) |
| for( i = 0; i <= num_brick_patterns_minus1; i++ ) { |  |
| num_bricks_minus1[ i ] | ue(v) |
| for( j = num_bricks_minus1[ i ], remHeightInCtbsY = PicHeightInCtbsY; j > 0 && !rem_brick_height_equal[ i ][ j ]; remHeightInCtbsY −= brick_height_minus1[ i ][ j ] + 1, j− − ) { |  |
| if( remHeightInCtbsY % ( j + 1 ) = = 0 ) |  |
| rem_brick_height_equal_flag[ i ][ j ] | u(1) |
| if( !rem_brick_height_equal_flag[ i ][ j ] ) |  |
| brick_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| num_tile_columns_minus1 | ue(v) |
| for( i = num_tile_columns_minus1, remWidthInCtbsY = PicWidthInCtbsY; i > 0 && !rem_tile_col_equal_flag[ i ]; remWidthInCtbsY −= tile_column_width_minus1[ i ] + 1, i− − ) { |  |
| if( remWidthInCtbsY % ( i + 1 ) = = 0 ) |  |
| rem_tile_col_equal_flag[ i ] | u(1) |
| if( !rem_tile_col_equal_flag[ i ] ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| } |  |
| if( num_brick_patterns_minus1 > 0 ) |  |
| for( i = 0; i <= num_tile_columns_minus1; i++ ) |  |
| brick_layout[ i ] | u(v) |
| single_brick_per_slice_flag | u(1) |
| if( !single_brick_per_slice_flag ) |  |
| rect_slice_flag | u(1) |
| … |  |

num_brick_patterns_minus1 plus 1 is equal to the number of brick splitting patterns described in the PPS. The value of num_brick_patterns_minus1 should be in the range of 0 to num_tile_columns_minus1.

brick_layout[ i ] is the index of the brick pattern that specifies the brick partitioning of the i-th tile. When not present, brick_layout[ i ] is inferred equal to 0. The length of brick_layout[ i ] is equal to Ceil( Log2( num_brick_patterns_minus1 + 1 ) ) bits.

# Asserted benefits

## For Option 1

The following properties were derived from VVC Draft 5 and this contribution:

- The number of syntax elements for indicating tile and brick partitioning

- The number of syntax lines for indicating tile and brick partitioning

- The number of bits required for indicating the tile and brick partitioning for the schemes presented in Figure 1.

- The bit count saving in per cent provided by this proposal when compared to VVC Draft 5 for the schemes presented in Figure 1.

The properties derived with 128×128 luma CTB size are presented in Table 1 and more details can be observed from the accompanying Excel file. It is also remarked that with the proposal clause 6.5.1 become some lines shorter and the semantics became almost half a page shorter.

Table 1. Comparison of tile and brick signalling in VVC Draft 5 and in this contribution.

|  |  |  | 6K effective ERP |  | 6K effective CMP 3×6 tile grid |  | Effective 6K CMP 6×3 tile grid |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | # syntax elements | # syntax lines | # bits | #bits saving | # bits | #bits saving | # bits | #bits saving |
| VVC Draft 5 | 13 | 26 | 74 |  | 54 |  | 84 |  |
| Proposal | 7 | 20 | 33 | 55 % | 24 | 56 % | 28 | 67 % |

Figure 1a and Figure 1b present the tile and brick partitionings that achieve the 6K effective ERP and CMP schemes (respectively) that are described in OMAF clauses D.6.3 and D.6.4 (respectively) [1]. These schemes have been recommended in the VR Industry Forum Guidelines [2]. The scheme presented in Figure 1c is otherwise equivalent to that in Figure 1b but uses a different picture aspect ratio.

| a) 6K effective ERP scheme of OMAF clause D.6.3. 6×2 tile grid with tile column widths of 768, 768, 768, 768, 384, and 384 luma samples and tile row heights of 2048 and 256 luma samples | b) 6K effective CMP scheme of OMAF clause D.6.4. 3×6 tile grid with tile column widths of 768, 768, and 384 luma samples and all tile row heights equal to 768 luma samples. | c) 6K effective CMP scheme. 6×3 tile grid with tile column widths of 768, 768, 768, 768, 384, and 384 luma samples and all tile row heights equal to 768 luma samples. |
| --- | --- | --- |

Figure 1. VVC tile and brick partitioning to achieve 6K effective resolutions for 360° video.

## For Option 2

For examples of figures 1, similar results as for option 1 are excepted with a possible increase of a few bits to indicate the number of brick split patterns. The Option 2 would provide better results when same brick split patterns are used for non-adjacent tile columns.

# References

- "Revised text of ISO/IEC FDIS 23090-2 Omnidirectional Media Format," MPEG N17563, Apr. 2018.

- "VR Industry Forum Guidelines," version 1.1, June 2018. Available: https://www.vr-if.org/guidelines/.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).