| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1093-v2 |
| --- | --- |

| Title: | Report of BoG on intra prediction and mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera | Email: | geertv@qti.qualcomm.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met during following hours:

Thu 4 July: 14:30-19:45

Fri 5 July: 14:30-19:45

# Notes

## MIP-related contributions

Overview of common aspects of MIP-related contributions:

- 8-bit MIP: proposed in JVET-O0084, JVET-O0481, JVET-O0925

- MIP MPM:

JVET-O0169, JVET-O0312, JVET-O0171, JVET-O0401: replace MIP MPM construction with mode index coded TB

JVET-O0170, JVET-O0255: simplify MIP MPM list to 3 default lists, one for each size index

JVET-O0469, JVET-O0255, JVET-O0598, JVET-O0171, JVET-O0202, JVET-O0396, JVET-O0170, JVET-O0793 (*), JVET-O0312, JVET-O0187(**): simplifies MIP by removing the mapping table from regular intra mode to MIP mode (default mode is used for each size index) for MIP MPM construction and also the mapping table from MIP mode to regular mode for non-MIP MPM list construction (use planar mode), and also DM chroma mode mapping is removed in case of MIP luma (use planar mode)

(*) this method has slightly different approach for MIP MPM construction

(**) suggests removing mapping table from MIP to regular mode, and DM mode

JVET-O0202, JVET-O0464: suggests aligning L and A of MIP MPM construction with regular MPM construction

JVET-O0755: signalling matrix parameters

JVET-O0160, JVET-O0276, JVET-0407, JVET-O0408, JVET-O0168: suggest to move clipping to subsampled prediction

JVET-O0187, JVET-O0084, JVET-O0481, JVET-O0485, JVET-O0276: suggest to apply VVC reference sample construction process to MIP

JVET-O0261, JVET-O0187, JVET-O0084, JVET-O0481, JVET-O0485, JVET-O0161, JVET-O0345, JVET-O0276: suggest using unfiltered reference sample for upsampling stage

JVET-O0571: suggests to use fixed order for upsampling

JVET-O0161: suggests downsampling filter change for alignment

JVET-O0848: suggests 2D upsampling

Approximate MIP matrices, or reduce number of matrices

JVET-O0523, JVET-O0139, JVET-O0397, JVET-O0187, JVET-O0793, JVET-O0621, JVET-O0547, JVET-O0203, JVET-O0345

JVET-O0084 8-bit implementation and simplification of MIP [J. Pfaff, T. Hinz, P. Helle, P. Merkle, B. Stallenberger, M. Schäfer, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI)]

At the 14th JVET meeting, held in Geneva from March 19th to March 27th 2019, matrix-based intra prediction (MIP) was adopted. Here, the coefficients of the involved matrices and offset vectors were stored in 10-bit precision. In this document, as requested at the Geneva Meeting, a version of MIP where the coefficients are stored in 8-bit precision is presented. In the course of this asserted simplification, the number of coefficients needed for MIP is reported to be reduced: The addition of the offset vector is removed and for the largest matrices, one column can be removed. As a result, the overall memory requirement of MIP is now 5.188 Kilobyte, both in hardware and software, whereas the MIP version in VTM-5.0 had a memory requirement of 7.92 Kilobyte.  	
Furthermore, two modifications of MIP are proposed in the present document; these are asserted to simplify the MIP design. First, it is proposed to remove the averaging step in the derivation of the boundary samples that are used for the linear interpolation of the reduced prediction signal and to remove here the extra handling of negative values in the rounding. Second, it is proposed to derive unavailable reference samples using the same padding method as the conventional intra prediction and, in the derivation of the most probable modes for MIP, it is proposed to treat a neighbouring CIIP-block exactly as neighbouring CIIP-blocks are treated in the derivation of the most probable modes of the conventional intra prediction. 	
The proposed changes of MIP were integrated into the VTM-5.0 software. No changes concerning the encoder control were made in comparison to VTM-5.0. The reported results for the presented 8-bit implementation of MIP are -0.05% luma BD-rate gain for the AI and -0.03% luma BD-rate gain for the RA configuration, compared to the VTM-5.0 anchor. The encoding times are reported to be 100%, the decoding times are reported to be 100% . The tool-off test results for the presented 8-bit version of MIP are reported to be 0.41% for AI and 0.30% for RA. The encoding times are reported to be 88% and 95 %, the decoding times are reported to be 101% and 102% .

JVET-O0139 CE3 Related: Low Memory and Computational Complexity Matrix Based Intra Prediction (MIP) [M. Salehifar, S. Kim (LGE)]

This contribution proposes several modification to the current MIP design in VTM5. Currently three different MIP index defined based on the block size (index 0: for W=H=4 index 1: for maxW,H=8, and index 2: formaxW,H>8). Also there are 19, 10, and 6 kernels which represent 35, 19, and 11 modes for each index 0,1, and 2 respectively. These MIP modes are divideded into MPM and non-MPM for signaling, where 3 MPM modes are coded as TU and non-MPM coded as fixed length code. This proposal suggests following modifications:

- Removing MIP for block size index 0 and 1.

- Reducing number of the kernels for MIP index 2 from 6 to 3.

- Encoder modification

- Removing the MPM derivation, and coding MIP mode with truncated binary code.

| Test # | Modification |
| --- | --- |
| Test 1 | a |
| Test 2 | a, c |
| Test 3 | a, b, c, d |

The reported result for test 1 is 0.08 %, 0.07 % and 0.04 % luma BD-rates differences with 94 %, 98 %, and 100 % encoding time for the AI, RA, and LB settings respectively, under common test conditions. Meanwhile it uses ~ 4 KB memory (50% less memory).

The reported result for test 2 is 0.00 %, 0.02 % and 0.03 % luma BD-rates differences with 101 %, 101 %, and 101 % encoding time for the AI, RA, and LB settings respectively, under common test conditions. Meanwhile it uses ~ 4 KB memory (50% less memory).

The reported result for test 3 is 0.03 %, 0.03 % and 0.05 % luma BD-rates differences with 98 %, 99 %, and 99 % encoding time for the AI, RA, and LB settings respectively, under common test conditions. Meanwhile it uses ~2 KB memory (75% less memory).

The proponent requests to study the aspects a and b in a core experiment. It was commented that the group needs to agree whether reducing the kernels is needed and that this may be due to the characteristics of the training set. If other content is tested the BD-rate loss may be different. It was further commented that hardware implementations are ok with the current memory use of MIP. If an experiment is defined then the reduced set of kernels would need to be retrained (not on CTC set). It was further commented that there does not seem to be a complexity issue with 4x4 blocks, for example.

BoG recommends further study outside CE. It is suggested to bring evidence on an independent test set to prove that the kernel reduction is general enough. It is also suggested in addition to CTC to study low-Qp range where 4x4 blocks will be used more.

JVET-O0721 Crosscheck of JVET-O0139 (CE3 Related: Low Memory and Computational Complexity Matrix Based Intra Prediction (MIP)) [T. Biatek (Qualcomm)]

JVET-O0957 Crosscheck of JVET-O0139 (CE3 Related: Low Memory and Computational Complexity Matrix Based Intra Prediction (MIP)) [J. Pfaff, B. Stallenberger (HHI)]

JVET-O0160 Non-CE3: Sample value clipping on MIP reduced prediction [Z. Zhang, K. Andersson, R. Sjöberg, J. Ström, P. Wennersten, R. Yu (Ericsson)]

See comments JVET-O0925

JVET-O0961 Crosscheck of JVET-O0160 (Non-CE3: Sample value clipping on MIP reduced prediction) [J. Pfaff, M. Schäfer (HHI)]

JVET-O0161 Non-CE3: Simplification on MIP boundary sample downsampling process [Z. Zhang, K. Andersson, R. Sjöberg, J. Ström, P. Wennersten, R. Yu (Ericsson)]

This contribution presents a method to simplify the matrix based intra prediction (MIP) boundary sample downsampling process when the current block is predicted by MIP. The proposed method 1) enable parallel derivation of the reduced boundary for matrix multiplication and the reduced boundary for interpolation; 2) reduced the number of samples to be averaged to derive one reduced boundary sample. For the first aspect, the proposed method derives the reduced boundary for interpolation without averaging. For the second aspect, the proposed method simplifies the derivation of reduced boundary for matrix multiplication by averaging two original boundary samples to derive one reduced boundary sample. The proposed method has a luma BDR impact for AI / RA / LDB of -0.01% / -0.01% / 0.01% compared to VTM5.0 and decoding time impact for AI / RA / LDB of 100% / 100% /100% compared to VTM5.0.

It was commented that from the experiment it seems that the averaging of for example eight or sixteen samples does not differ much from the suggested two samples, but that averaging more samples may be more stable. In this case, retraining may be necessary. It suggested that another filter may be more appropriate.

In case a core experiment is established for studying MIP related methods, the BoG recommends to include this method (aspect 2). A variation of the downsampling filtering could be studied as well.

JVET-O0960 Crosscheck of JVET-O0161 (Non-CE3: Simplification on MIP boundary sample downsampling process) [J. Pfaff, M. Schäfer (HHI)]

JVET-O0168 Non-CE3: MIP low resolution clipping [T. Biatek, G. Van der Auwera, M. Karczewicz (Qualcomm)]

See comments JVET-O0925

JVET-O0882 Crosscheck of JVET-O0168 (Non-CE3: MIP low resolution clipping) [Z. Zhang (Ericsson)]

JVET-O0169 CE3 Related: No MPM Derivation for Matrix Based Intra Prediction (MIP) [M. Salehifar, S. Kim (LGE)]

See comments JVET-O0925

JVET-O0722 Crosscheck of JVET-O0169, (CE3 Related: No MPM Derivation for Matrix Based Intra Prediction (MIP)) [T. Biatek (Qualcomm)]

JVET-O0958 Crosscheck of JEVT-O169 (CE3 Related: No MPM Derivation for Matrix Based Intra Prediction (MIP)) [J. Pfaff, B. Stallenberger (HHI)]

JVET-O0170 Non-CE3: Simplification on Matrix-based intra prediction [B. Wang, A. M. Kotra, S. Esenlik, H. Gao, E. Alshina (Huawei)]

See comments JVET-O0925

JVET-O0883 Crosscheck of JVET-O0170 (Non-CE3: Simplifications of Intra Mode Coding for Matrix-based Intra Prediction) [Z. Zhang (Ericsson)]

JVET-O0171 Non-CE3: Simplifications for MIP [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

See comments JVET-O0925

JVET-O0850 Crosscheck of JVET-O0171 (Non-CE3: Simplifications for MIP) [Jie Yao (Fujitsu)]

JVET-O0943 Crosscheck of JVET-O0171 (Non-CE3: Simplifications for MIP) [T.-C. Ma (Kwai Inc.)]

JVET-O0187 Non-CE3: MIP simplifications [A. K. Ramasubramonian, G. Van der Auwera, T. Biatek, L. Pham van, M. Karczewicz (Qualcomm)]

See comments JVET-O0925

JVET-O0202 Non-CE3: MIP Modifications [Y.-U. Yoon, D.-H. Park, J.-G. Kim (KAU), J. Lee, J. Kang (ETRI)]

This contribution proposes two modifications of matrix-weighted intra prediction (MIP). The first modification is changing the positions of the neighboring blocks to be referred deriving MIP MPM. The second modification is removing inverse mapping table which maps MIP matrix modes to regular intra modes. This contribution reports the performances on the two tests below:

- Test 1: modified positions for deriving MIP MPM candidates

- Test 2: remove inverse mapping table that assigns MIP modes to regular intra prediction modes

The experiment results for test 1 reportedly show 0.00% and 0.XX% luma BD-rates performance with 101% and, XX% encoding time for the AI and RA, respectively.

The experiment results for test 2 reportedly show 0.01% and 0.XX% luma BD-rates performance with 99% and, XX% encoding time for the AI and RA, respectively.

Aspect 1 is not considered because it is understood that the MIP MPM construction can be removed and replaced with mode index coding using TB. For aspect 2 see comments under JVET-O0925.

JVET-O1024 Crosscheck of JVET-O0202 (Non-CE3: MIP Modifications) [Z.-Y. Lin (MediaTek)]

JVET-O0203 CE3-related: Simplification of Matrix-Based Intra Prediction (MIP) [A. Filippov, V. Rufitskiy, B. Wang, S. Esenlik, E. Alshina (Huawei)]

In this document, we propose a set of mechanisms for reducing computational complexity and memory consumption of Matrix-based Intra Prediction (MIP). In Test 1, we align the width of MIP multipliers with the 16-bit multipliers used for interpolation filtering in the conventional intra prediction mechanism. In addition to reducing the multiplier width, multiplication by the MIP coefficients used in Test 1 can be implemented using add-and-shift technique (with no more than either 1 addition or 1 subtraction operation). Test 1 reportedly shows the following overall results over VTM-5.0: 0.03% (Y), -0.02% (U), 0.03% (V) with encoding time of 100% and decoding time of 100% for AI configuration, 0.02% (Y), -0.01% (U), 0.00% (V) with encoding time of 99% and decoding time of 100% for RA configuration. Besides, we test MIP with 6-bit multipliers on top of JVET-O0084. This test (Test 2) reportedly shows the following overall results over VTM-5.0: -0.05% (Y), -0.10% (U), -0.09% (V) with encoding time of 100% and decoding time of 99% for AI configuration, -0.03% (Y), -0.17% (U), -0.07% (V) with encoding time of 100% and decoding time of 99% for RA configuration.

The method is tested on VTM5, JVET-O0084, JVET-O0481.

It is commented that the matrix coefficients are quantized by the method.

The proponent mentions that there are two implementations, either storing approximated matrices or computing approximation on the fly. In addition, there is a multiplication free method proposed which is tested with special tuning on VTM5. The other results are obtained with multiplications and without tuning.

A question is raised that the input samples could be downshifted to obtain 16-bit multiplications.

It is commented that the benefit will be larger if the method is applied on 10-bit coefficients as opposed to 8 bit. It may also be better to test such a method after stability of the MIP method is achieved.

BoG recommends to further study the method outside of CE. Additional feedback from hardware experts may be requested to get feedback on the importance of the method.

JVET-O0811 Crosscheck of JVET-O0203 (CE3-related: Simplification of Matrix-based Intra Prediction (MIP)) [T.-H. Li, J.-Y. Deng (Foxconn)]

JVET-O0255 Non-CE3: Fixed MPMs for MIP [K. Zhang, Z. Deng, L. Zhang, H. Liu, N. Zhang (Bytedance)]

It is understood that the MIP MPM may be replaced by mode index coded with TB (see comments under JVET-O0925). Therefore, the BOG recommends no action on the fixed MPM part.

For other aspects (removal of mapping tables) see comments under JVET-O0925.

JVET-O1005 Crosscheck of JVET-O0255 (Non-CE3: Fixed MPMs for MIP) [J. Choi (LGE)]

JVET-O0261 Non-CE3: MIP with one-stage boundary sample downsampling [Z. Deng, H. Liu, K. Zhang, L. Zhang, Y. Wang (Bytedance)]

See comments under JVET-O0925

JVET-O0826 Crosscheck of JVET-O0261 (Non-CE3: MIP with one-stage boundary sample downsampling) [H. Chen (Huawei)]

JVET-O0276 CE3-related: Simplifications for MIP [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

See comments under JVET-O0925

JVET-O0827 Crosscheck of JVET-O0276 (CE3-related: Simplifications for MIP) [H. Chen (Huawei)]

JVET-O0312 Non-CE3: Simplifications of MIP [P.-H. Lin (Foxconn)]

See comments under JVET-O0925

JVET-O1001 Crosscheck of JVET-O0312 (Non-CE3: Simplifications of MIP) [C.-C. Kuo, C.-H. YauLin, C.-L. Lin (ITRI)]

JVET-O0321 Non-CE3: Removal of MIP mapping table [J.-Y. Huo, Y.-Z. Ma, X.-W. Li, H.-X. Wang, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu, M. Li (OPPO)]

See comments under JVET-O0925

JVET-O1007 Crosscheck of JVET-O0321 (Non-CE3: Removal of MIP mapping table) [J. Choi (LGE)]

JVET-O0323 Non-CE3: Unification of shifting for MIP mode [J.-Y. Huo (Xidian Univ.), S. Wan (NPU), Y.-Z. Ma (Xidian Univ.), J.-K. Guo (NPU), H.-X. Wang, F.-Z. Yang (Xidian Univ.), Y.-F. Yu, Y. Liu, M. Li (OPPO)]

This document proposes modification of matrix-based intra prediction (MIP). In current VVC, the weight shift may be equal to 8 or 9. This contribution unifies weight shift into 8 in MIP prediction calculation.

Simulation results reportedly show BD rate on Y, Cb and Cr components and encoding/decoding time comparison over VTM5.0:

For AI configuration: 0.00%, -0.05%, and -0.02%, with 100% EncT, 100% DecT;

For RA configuration: 0.00%, -0.06%, and -0.09%, with 100% EncT, 100% DecT;

Benefit is that lookup table for sW is not needed.

It is commented that in the 8-bit MIP method the downshift is 6.

In case a core experiment is established, the BoG recommends studying this method with the 8-bit MIP algorithm.

It is commented that JVET-O0408 proposes the identical method.

JVET-O0717 Crosscheck of JVET-O0323 (CE3-related: Unification of shifting for MIP mode) [L. Xu, F. Chen, L. Wang (Hikvision)]

JVET-O0324 Non-CE3: MIP Simplification [J.-Y. Huo (Xidian Univ.), S. Wan (NPU), Y.-Z. Ma (Xidian Univ.), J.-K. Guo (NPU), F.-Z. Yang (Xidian Univ.), Y.-F. Yu, Y. Liu, M. Li (OPPO)]

See comments JVET-O0139.

JVET-O0718 Crosscheck of JVET-O0324 (Non-CE3: MIP Simplification) [L. Xu, F. Chen, L. Wang (Hikvision)]

JVET-O0345 Non-CE3: MIP simplification [C.-H. Yau, C.-C. Lin, C.-L. Lin (ITRI)]

Method 1: removal of 4x4 kernel (see comments under JVET-O0139)

Method 2: use unfiltered samples in upsampling stage (see comments JVET-O0925)

JVET-O0971 Crosscheck of JVET-O0345 (Non-CE3: MIP simplification) [M.-S. Chiang (MediaTek)]

JVET-O0396 Non-CE3: Simplified mode mapping in MIP [J. Choi, J. Heo, J. Lim, S. Kim (LGE)]

See comments JVET-O0925.

JVET-O0750 Crosscheck of JVET-O0396 (Non-CE3: Simplified mode mapping in MIP) [?? (??)]

JVET-O0397 Non-CE3: MIP mode reduction [J. Choi, J. Heo, J. Lim, S. Kim (LGE)]

In matrix intra prediction (MIP), the block sizes are classified into three categories and each category supports 35/19/11 modes, and it requires 18/10/6 matrices, respectively.This contribution proposes to reduce the number of modes in MIP. The proposed method reduces the number of MIP modes and matrices as follows:

- Method 1: each category uses 11/11/11 modes, and it requires 6/6/6 matrices.

- Method 2: each category uses 6/7/8 modes, and it requires 3/4/4 matrices

With the method 1, the memory of MIP matrix is reduced to 75.7% with negligible BD-rate change. In the method 2, the memory of MIP matrix is reduced to 49.2%, compared to the VTM5.0.

Experimental results for Method 1 show 0.01%, -0.03%, and 0.00% BD-rates on Y, Cb, and Cr components, respectively in All Intra configuration, and Method 2 shows 0.04%, -0.03%, and -0.03% BD-rates on Y, Cb, and Cr components, respectively in All Intra configuration.

BoG recommends further study outside CE. It is suggested to bring evidence on an independent test set to prove that the kernel reduction is general enough. It is also suggested in addition to CTC to study low-Qp range where 4x4 blocks will be used more.

JVET-O0918 Crosscheck of JVET-O0397 (Non-CE3: MIP mode reduction) [K. Zhang (Bytedance)]

JVET-O0401 Non-CE3: MIP mode simplifications [C. Rosewarne, J. Gan (Canon)]

4x4 removal: see comments JVET-O0139

MIP MPM table removal: see comments JVET-O0925

JVET-O0407 Non-CE3: On adaptive rounding offset of MIP [K. Kondo, M. Ikeda, T. Suzuki (Sony)]

See comments JVET-O0925

JVET-O1006 Crosscheck of JVET-O0407 (Non-CE3: On adaptive rounding offset of MIP) [J. Pfaff, M. Schäfer (HHI)]

JVET-O0408 Non-CE3: On rounding shift of MIP [K. Kondo, M. Ikeda, T. Suzuki (Sony)]

See comments JVET-O0323

JVET-O1011 Crosscheck of JVET-O0408 (Non-CE3: On rounding shift of MIP) [J. Pfaff, M. Schäfer (HHI)]

JVET-O0464 Non-CE3: Unification of Neighbouring Location for MPM List Construction [D. Kim, J. Jung, G. Ko, J. Son, J. Kwak (WILUS)]

No need to present, because it is related to MIP MPM construction, which was suggested to be removed (BoG).

JVET-O0469 Non-CE3: Simplification of MIP MPM Derivation [D. Kim, J. Jung, G. Ko, J. Son, J. Kwak (WILUS)]

Method 1: use planar mode in case MIP is neigbouring block in regular MPM (see JVET-O0925)

Method 2: remove mapping table regular intra mode to MIP for MIP MPM construction (see JVET-O0925)

JVET-O0481 Variations of the 8-bit implementation of MIP [P. Helle, J. Pfaff, T. Hinz, P. Merkle, B. Stallenberger, M. Schäfer, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

At the 14th JVET meeting, held in Geneva from March 19th to March 27th 2019, matrix-based intra prediction (MIP) was adopted. Here, the coefficients of the involved matrices and offset vectors were stored in 10-bit precision. At the Geneva Meeting, it was requested to present a version of MIP where the coefficients are stored in 8-bit precision. In JVET-O0084, such an 8-bit version is proposed. Responding to feedback that was given to the proponents of JVET-O0084, in the present document, two modifications of the 8-bit implementation of JVET-O0084 are proposed. The first proposed modification is the 7-bit version of JVET-O0084. If one implements this proposed method as an 8-bit method, the matrix offsets fW, introduced in JVET-O0084, can be removed. The second proposed modification is proposed on top of the first proposed modification, namely replacing the mean value of the reduced boundary in the algorithm of JVET-O0084 by the first entry of the reduced boundary. Therefore, in comparison to JVET-O0084, the computation of the mean value of the reduced boundary vector can be avoided. Note, that both proposed methods as well as the method of JVET-O0084 are different 8-bit (resp. 7-bit) integer realizations of the same floating point MIP-models. Moreover, both proposed methods are combined with the MIP design simplifications proposed in section 2 of JVET-O0084. The reported results of the first proposed method over VTM-5.0 are -0.04% in the AI configuration and -0.03% in the RA configuration for the luma-component. The tool-off test for MIP of the first proposed method is 0.4% in the AI and 0.3% in the RA configuration. The reported results of the second proposed method over VTM-5.0 are -0.04% in the AI configuration and -0.03% in the RA configuration. The tool-off test for MIP of the second proposed method is 0.4% in the AI and 0.3% in the RA configuration.

It was commented by the cross-checker that the cleanups proposed do not have any BD-rate impact, therefore, the change in integer representation has minor gain -0.04% (AI), -0.03% (RA).

Blocks 16x16 and larger, matrix size 8x64 changed to 7x64; from SW memory access point of view there is little benefit of this change.

Regarding DC value computation in JVET-O0084, using one value in JVET-O0481 is preferable.

JVET-O0485 Non-CE3: Harmonization of 8-Bit MIP with Unified-MPM and LFNST [J. Pfaff, T. Hinz, P. Helle, P. Merkle, B. Stallenberger, M. Schäfer, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

No need to present. Included in JVET-O0925.

JVET-O0885 Crosscheck of JVET-O0485 (Non-CE3: Harmonization of 8-Bit MIP with Unified-MPM and LFNST) [Z. Zhang (Ericsson)]

JVET-O0486 Non-CE3: Harmonization of 8-Bit MIP with Unified-MPM and LFNST using a single type of MIP prediction modes [J. Pfaff, T. Hinz, P. Helle, P. Merkle, B. Stallenberger, M. Schäfer, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

See comments JVET-O0139

JVET-O0886 Crosscheck of JVET-O0486 (Non-CE3: Harmonization of 8-Bit MIP with Unified-MPM and LFNST using a single type of MIP prediction modes) [Z. Zhang (Ericsson)]

JVET-O0514 Non-CE3: Analysis of Matrix-based Intra Prediction (Informative) [S. Keating, K. Sharman (Sony)]

Informational. No need to present (proponent).

JVET-O0523 CE3-related: simplification of Matrix Intra Prediction [T. Dumas, F. Galpin, G. Rath, F. Racapé (InterDigital)]

This contribution reports the result of adding one Matrix-based intra prediction mode per block size, called Simplified Matrix Intra Prediction (SMIP) mode, in addition to the classical intra modes. Compared to the Matrix-based Intra Prediction (MIP) implemented in VTM-5.0, the memory footprint is reduced from~8 kilobytes to 1 kilobyte. In addition, the decoding process does not require any extra MPM list derivation anymore, nor any mapping table between conventional modes and MIP modes. A loss of 0.14% in AI, and a loss of 0.10% in RA is reported for the luminance, when the proposed mode is not combined with MTS.

Proponent has retrained matrices for the proposed reduction of kernels. Aspects related MIP MPM (table removal) see comments JVET-O0925. Other simplification aspects: see comments JVET-O0139.

JVET-O0547 Non-CE: Modified MIP Matrices for Fast Implementation [K. Naser, L. Kerofsky, F. Le Léannec, T. Poirier (InterDigital)]

Recently, MIP was adopted to VTM5.0 as an additional prediction mode that is based on partial multiplication of the reference samples by a prediction matrix. Unlike the ordinary intra prediction mode, MIP requires matrix-vector multiplication that can be time consuming.

This contribution proposes a slight modification to the MIP matrices so that faster implementation can be achieved. The modification allows dual implementation support, in which an MIP transform can be either implemented by matrix multiplications or fast algorithm to yield exactly the same results.

Experimental results show some negligible RD loss, while the number of multiplications is significantly reduced to 18% and 7% of the original number.

Implemented on JVET-O0084.

It was commented that the formula for approximation changes row by row of the matrix, which would need to be implemented without reuse. Proponent asserts that this is similar to a proposal by the proponent for DCT-8 implementation. However, in the DCT-8 case the data is more regular.

It is commented that it is more appropriate to have such a method after MIP kernels have stabilised.

BoG recommends to further study the method outside of CE.

JVET-O0962 Crosscheck of JVET-O0547 (Non-CE: Modified MIP Matrices for Fast Implementation) [J. Pfaff, M. Schäfer (HHI)]

JVET-O0571 Non-CE3: Up-sampling with a fixed order in MIP [H. Liu, L. Zhang, K. Zhang, Z. Deng, J. Xu (Bytedance)]

When the reduced prediction signal in MIP needs to be up-sampled in both directions, the up-sampling is always firstly performed along the shorter side of a CU. Afterwards, the up-sampling along the longer side is performed. Therefore, the up-sampling order in MIP varies with different CUs. This contribution proposes to perform the up-sampling in a fixed up-sampling order for the prediction signal up-sampling process in MIP. Simulation results reportedly show that the proposed method has negligible impact on the coding performance.

It is commented that both upsampling processes are implemented and, therefore, the benefit of having the same upsampling order is rather a cleanup of the algorithm.

In case a core experiment is established, the BoG recommends studying the method on the 8-bit MIP algorithm.

JVET-O0843 Crosscheck of JVET-O0571: Non-CE3: Up-sampling with a fixed order in MIP [J. Chen (Alibaba)]

JVET-O0598 CE3-related: Simplification on MIP Look-up Table [T.-C. Ma, X. Xiu, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)]

See comments JVET-O0925 (removal of mapping table)

JVET-O0693 Crosscheck of JVET-O0598 (CE3-related: Simplification on MIP Look-up Table) [L. Zhao (Tencent)]

JVET-O0599 CE3-related: Simplification on MIP flag context model [T.-C. Ma, X. Xiu, Y.-W. Chen, H.-J. Jhu, X. Wang (Kwai Inc.)]

This contribution proposes to reduce context model from 3 to 1 and remove selection logic when signaling 1-bit MIP flag. In this proposal, two tests and results are provided as following:

- Test 1: Proposed method versus VTM5

  - The BD-rate results are -0.04%, 0.00%, and 0.00% for AI, RA and LD, respectively.

- Test 2: Proposed method versus JVET-O0084

  - The BD-rate results are -0.04%, 0.00%, and 0.00% for AI, RA and LD, respectively.

It is commented that context reduction is better performed at a later stage of the VVC standardization when MIP algorithm stability has been achieved. BoG recommends no action at this stage.

JVET-O0815 Crosscheck of JVET-O0599 (CE3-related: Simplification on MIP flag context model) [J. Heo, L. Li (LGE)]

JVET-O0602 Non-CE3/Non-CE8: Remove MIP/IBC mode storage from line buffer [H.-J. Jhu, Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

This contribution proposes to reduces the line buffer of Matrix Intra Prediction (MIP)/Intra Block Copy (IBC) mode flag storage. For MIP mode flag storage, the context selection for the CABAC coding of MIP flag is not depending on the MIP flag of above CU when the above CU is not located in the same CTU as current CU. The proposed method is compared with both VTM5 JVET-O0084, with simulation results as follows:

- Test 1: Proposed method versus VTM5

  - The BD-rate results are 0.00%, 0.02%, and 0.02% for AI, RA and LD, respectively, without increasing encoding and decoding time.

- Test 2: Proposed method versus JVET-O0084

  - The BD-rate results are -0.05%, -0.03%, and -0.01% for AI, RA and LD, respectively, without increasing encoding and decoding time.

For IBC mode flag storage, the context selection for the CABAC coding of IBC flag is not depending on the IBC flag of above CU when the above CU is not located in the same CTU as current CU. Furthermore, the above neighbor candidate for IBC AMVP/merge/skip mode prediction is not used when the above CU is not in the same CTU as current CU. Compared to CE8 anchors, for Class SCC sequences, The BD-rate results are 0.11%, 0.1%, and 0.05% for AI, RA and LD, respectively, without increasing encoding and decoding time.

It is commented that there are many other syntax elements that access line buffer memory for flag retrieval and that it is appropriate to study the proposed cases together with other cases in a comprehensive proposal at a later stage of standardization of VVC (BoG recommendation).

JVET-O0813 Crosscheck of JVET-O0602 (Non-CE3/Non-CE8: Remove MIP/IBC mode storage from line buffer) [T.-H. Li, J.-Y. Deng (Foxconn)]

JVET-O0621 Non-CE3: MIP simplification [Y. Yasugi, T. Ikai (Sharp)] [miss] [late]

See comments JVET-O0139

JVET-O0855 Crosscheck of JVET-O0621 (Non-CE3: MIP simplification) [Z. Deng (Bytedance)]

JVET-O0755 Non-CE3: On signalling of MIP parameters [A. K. Ramasubramonian, G. Van der Auwera, T. Hsieh, T. Biatek, L. Pham Van, M. Karczewicz (Qualcomm)] [late]

To be presented.

JVET-O0793 CE3-related: MIP with simplified mode index coding [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)] [late]

In this proposal, three methods are proposed to simplify the VTM5.0 matrix-based intra prediction (MIP). Method 1 removes mapping between non-MIP and MIP modes and uses power-of-2 modes in each MIP group. To remove the non-MIP-to-MIP mapping table, the MPMs in MIP mode are removed, and MIP mode indices are coded using fixed length binarization without using any context variables. To remove MIP-to-non-MIP mapping table, during the non-MIP MPMs derivation and the direct mode (DM) derivation of a chroma intra block, MIP mode index is mapped to planar mode index. To efficiently utilize the fixed length binarization codewords, only power-of-2 modes are used in each MIP group. The number of MIP modes in the three MIP groups are reduced from (35, 19, 11) to (32, 16, 8). Based on method 1, method 2 further simplifies the matrix index derivation by changing the composition of the MIP mode index from “matrixIndex + (numberOfMode/2)*isTranspose” to “2*matrixIndex + isTranspose.” Therefore, the matrix index can be easily derived by right shifting the MIP mode index by one bit, and whether the MIP mode is transposed mode is derived by checking the parity bit of the MIP mode index. Method 3 further simplifies method 2 by reducing the numbers of MIP modes in the three MIP mode groups from (32, 16, 8) to (8, 8, 8).

Test results of method 2 are reported as follows.
VTM5.0-AI: {BD-rates = 0.03% (Y), -0.03% (Cb), 0.00% (Cr); EncT = 98%; DecT = 100%}
VTM5.0-RA: {BD-rates = 0.02% (Y), -0.08% (Cb), 0.09% (Cr); EncT = 99%; DecT = 99%}
VTM5.0-LB: {BD-rates = -0.01% (Y), 0.32% (Cb), 0.15% (Cr); EncT = 99%; DecT = 99%}

Test results of method 3 are reported as follows.
VTM5.0-AI: {BD-rates = 0.05% (Y), -0.01% (Cb), 0.02% (Cr); EncT = 97%; DecT = 100%}
VTM5.0-RA: {BD-rates = 0.04% (Y), 0.02% (Cb), 0.03% (Cr); EncT = 99%; DecT = 97%}
VTM5.0-LB: {BD-rates = 0.06% (Y), 0.28% (Cb), 0.10% (Cr); EncT = 99%; DecT = 99%}

Regarding MIP MPM construction removal (table mappings): see comments JVET-O0925.

Regarding reduction of kernels: see comments JVET-O0139.

Regarding MIP mode index coding with fixed length coding, the BoG recommends to study the method in a core experiment (if established).

JVET-O0848 Non CE3: Modification of MIP interpolation [B.-J. Fuh, C.-H. Yau, C.-C. Lin, C.-C. Kuo, C.-L. Lin (ITRI)] [late]

This contribution proposed the modified interpolation of MIP. The 1D linear interpolation is replaced by a 2D linear interpolation by an interpolation unit to improve the flexibility for the hardware and reduce the interpolation process latency. The results of the proposed method show 0.00%(Y), -0.01%(U) and -0.01%(V) coding gain.

It is commented that the number of operations is increased. It is further commented that with current separable interpolation a similar benefit can be achieved if smaller areas are interpolated consecutively. It is further commented that a similar argument holds for other separable processing.

The BoG recommends to further study the method outside of CE and it is recommended to request feedback on the latency versus complexity trade-off from, for example, hardware experts.

JVET-O0925 Non-CE3: Simplifications of MIP [J. Pfaff, P. Merkle, P. Helle, H. Schwarz, D. Marpe, T. Wiegand (HHI), A. K. Ramasubramonian, T. Biatek, G. Van der Auwera, L. Pham Van, M. Karczewicz (Qualcomm), J. Choi, J. Heo, J. Lim, M. Salehifar, S. Kim (LGE), K. Kondo, M. Ikeda, T. Suzuki (Sony), Z. Zhang, K. Andersson, R. Sjöberg, J. Ström, P. Wennersten (Ericsson)] [late]

It proposes to adopt the following changes of MIP:

- MIP is implemented in 8 bit, as proposed in JVET-O0481, Method 2.

- The reference sample derivation for MIP is performed exactly as for the conventional intra prediction modes. The identical method is proposed in JVET-O0187 (proposal 3), JVET-O0084, JVET-O0481, JVET-O0485, JVET-O0276.

- For the upsampling step used in the MIP-prediction, original reference samples are used instead of downsampled ones. The identical method is proposed in JVET-O0161 (aspect 1), JVET-O0261, JVET-O0187, JVET-O0084, JVET-O0481, JVET-O0485, JVET-O0345, JVET-O0276.

- The extra handling of negative values is removed from the upsampling. The identical method is proposed  in JVET-O0160,  JVET-O0407, JVET-O0084, JVET-O0481, JVET-O0485.

- Clipping is performed before upsampling and not after upsampling. The identical method is proposed in JVET-O0160, JVET-O0168, JVET-O0276,  .

- The mapping tables from MIP modes to conventional intra prediction modes are removed. Instead, MIP modes are always mapped to the planar mode. Method is also proposed in JVET-O0187 ( Proposal 1), JVET-O0396 (Method 1a), JVET-O0469, JVET-O0255, JVET-O0598, JVET-O0171, JVET-O0202, JVET-O0396, JVET-O0170, JVET-O0793 (*), JVET-O0312, JVET-O0187(**), JVET-O0321:

  - (*) this method has slightly different approach for MIP MPM construction

  - (**) suggests removing mapping table from MIP to regular mode, and DM mode

- For the coding of the MIP-modes, the MIP-MPMs are no longer used and the mapping from conventional intra prediction modes to MIP modes is removed. Instead, the MIP modes are coded using truncated binary code. The same method is proposed in JVET-O0169, JVET-O0312, JVET-O0171, JVET-O0401.

- MIP is harmonized with unified MPM-coding: In the list of the five non-planar MPMs, the DC mode is replaced with an angular mode if one of the neighboring modes is angular, as proposed in JVET-O0485, JVET-O0484.

- LFNST is enabled for MIP on large blocks. Here, the LFNST transforms corresponding to the planar mode are used, as proposed in JVET-O0485.

It was commented that 8-bit MIP change has 0.04% gain, UMPM change has some gain also, but overall there appears to be an element that has some loss. This is reportedly due to the encoder speedup (7% reduction for AI).

It was commented that an adopted LFNST proposal may reduce the gain from applying the LFNST to MIP large blocks.

A question was raised about the sequence set for training the new matrices. The proponent shared the training code, but did not share the sequences. Proponent of JVET-O0523 is willing to also share a training set. For future contributions, it would be useful to have such a set available that is different from the CTC sequences.

It was commented that the cross-check is pending as well as some results in the proposal are missing.

The proponent asserts that the draft spec text is finalized and can be studied.

The proponent of JVET-O0203 suggests that 16-bit implementation of multiplications is desirable and agreed by the proponent and a non-proponent. This means that the result of the multiplication is in the 16-bit range after clipping and before downshifting.

It was commented that ‘MIP harmonization with unified MPM-coding’ affects regular intra mode MPM construction and is not necessarily related to MIP.

BoG recommends adoption of this contribution, subject to the finalization of the cross-check, completion of results, studying of software and working draft text, review of other proposals on this topic.

## Intra mode coding

JVET-O0111 Non-CE3: Modified CABAC Coding for Intra Prediction Mode [B. Wang, A. M. Kotra, S. Esenlik, H. Gao, E. Alshina (Huawei)]

A planar flag is introduced in VTM5.0 due to the high occurrence of planar mode in intra prediction. The planar flag uses CABAC regular coding with two contexts based on whether current block is applied with Intra Sub-Partition (ISP) coding tool. In this contribution, four contexts are proposed based on the prediction mode of left and above blocks of the current block. Furthermore, the first bin of the MPM index is changed from bypass coding to regular CABAC coding, and the same context derivation logic to the planar flag is used. Compared to VTM5.0, the proposed method achieves a coding performance of -0.07% and -0.02% for all intra and random access configurations, respectively, with negligible runtime difference.

Results shown for 4 context used for planar flag and 1st bin of MPM index and additional results for 4 context for planar flag only (minor 0.04% coding gain for AI, 0.01% for RA).

BoG recommends further study.

JVET-O1003 Crosscheck of JVET-O0111 (Non-CE3: Modified CABAC Coding for Intra Prediction Mode) [L. Li (LGE)]

JVET-O0192 Non-CE3: MPM flag first signaling for intra luma mode coding [Jie Yao, Jianqing Zhu, Wenting Cai, Kimihiko Kazui (Fujitsu)]

In the current VVC draft [1], for intra luma mode coding, MRL (multiple reference line intra prediction) index and ISP (intra sub-partitions) flag are first signaled even if the block mode is remainder mode (non MPM mode). Since MRL and ISP can only be enabled on MPM mode, this document propose to clean-up the draft syntax by first signal MPM flag. The results reportedly show an average luma BD rate performance of (0.01%, −0.04%, 0.00%) for (Y, Cb, Cr) AI, (0.00%, −0.13%, −0.03%) for (Y, Cb, Cr) RA without increase of encoding running time over VTM-5.0.

The proponent comments that the context initialization was not retrained and that perhaps coding gain can be improved. The proponent claims that the syntax is also simplified. The coding gain is not the goal of the proposal. One inconsistency in the current design is that ISP flag and MRL index are signalled for regular remaining modes (non-MPM modes), while they are not used. It was commented for regular MPM modes, the ISP flag and MRL index are also not used and this is not addressed by this proposal. It was commented that currently ISP and MRL modes only allow MPM modes and that allowing remaining modes, for example ISP, does not seem compatible with the proposed syntax design.

BoG recommends further study outside of CE, with clear analysis of the benefits.

JVET-O0768 Crosscheck of JVET-O0192: Non-CE3: MPM flag first signaling for intra luma mode coding [L. Pham Van, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-O0197 Non-CE3: cleanup for ISP flag and MRL index signalling [H. Liu, L. Zhang, K. Zhang, N. Zhang, J. Xu, Y. Wang (Bytedance)]

In current VVC, Intra Subblock Partitioning and Multiple Reference Line are disallowed for the non-Most Probable Modes. However, the on/off control flag of Intra Subblock Partitioning and the reference line index are still signalled for such intra prediction methods, which is unnecessary. To tackle this problem, this contribution proposes to signal the on/off control flag of Intra Subblock Partitioning and the reference line index only when Most Probable Mode is used. It is reported that the proposed method can obtain 0.01% luma BD-rate gain for the AI and RA configurations on top of VTM-5.0 without encoding and decoding run-time changes.

This proposal is identical to JVET-O0192 regarding the proposed syntax change. It does include an encoder change that has a small coding gain (0.01% AI gain).

BoG recommends further study outside of CE, with clear analysis of the benefits.

JVET-O0851 Crosscheck of JVET-O0197 (Non-CE3: Cleanup for ISP flag and MRL index signalling) [Jie Yao (Fujitsu)]

JVET-O0224 CE3-related: Further simplifications of the unified MPM list for intra mode coding [G. Rath, F. Racapé, F. Urban (InterDigital)]

This contribution reports the result of using a simplified unified MPM list in the intra mode coding. The list construction follows the same logic as in the unified MPM list adopted in the JVET meeting in Geneva in March 2019, but consists of only 5 MPMs. The complex logic for the adjacency check of the intra modes of the left and top blocks is replaced with a simpler logic. Like the unified list, the simplified list can be used with all coding tools such as the intra prediction with multiple reference lines (MRL), or intra prediction with sub-partitions (ISP), or the regular intra prediction. Compared to the unified list in VTM5.0, 0.01% coding gain in AI, with 98% encoding and 98% decoding time is observed.

The proposal aims at reducing the MPM list construction complexity. The proponent asserts that the worst-case number of conditions is similar to the current list construction.

It was commented that it is 4 MPMs in addition to planar mode. It was also commented that there is an additional condition used in the worst-case. This was acknowledged by the proponent.

BoG recommends no action.

JVET-O0968 Crosscheck of JVET-O0224 (Non-CE3: Further simplifications of the unified MPM list for intra mode coding) [S. De-Luxán-Hernández (HHI)]

JVET-O0275 CE3-related: Chroma DM derivation for IBC and PCM [M.-S. Chiang, O. Chubach, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution reports the result of using a simplified unified MPM list in the intra mode coding. The list construction follows the same logic as in the unified MPM list adopted in the JVET meeting in Geneva in March 2019, but consists of only 5 MPMs. The complex logic for the adjacency check of the intra modes of the left and top blocks is replaced with a simpler logic. Like the unified list, the simplified list can be used with all coding tools such as the intra prediction with multiple reference lines (MRL), or intra prediction with sub-partitions (ISP), or the regular intra prediction. Compared to the unified list in VTM5.0,  0.01% coding gain in AI, with 98% encoding and 98% decoding time is observed.

There is a related contribution JVET-O0651 that proposes to use the horizontal mode for IBC case. It was commented that IBC is used for screen content coding and in this case the planar mode is rarely used. It was commented that in this case (JVET-O0651) an additional check may be required. This was not agreed by the proponent. The proponent of JVET-O0651 reports a minor gain of 0.02% on class F (AI), 0.01% in class TGM (AI). It was commented that there is insignificant difference between gains of both proposals.

It was commented that the simpler mode is preferable for screen content coding, i.e., the horizontal mode in the case of IBC and PCM luma block modes for chroma DM.

The BoG recommends adopting JVET-O0651.

JVET-O0823 Crosscheck of JVET-O0275 (CE3-related: Chroma DM derivation for IBC and PCM) [S. Yoo, J. Lim (LGE)]

JVET-O0351 Non-CE3: Clean-up intra type signaling [J. Heo, J. Nam, J. Choi, J. Lim, S. Kim (LGE)]

This contribution proposes two clean-up methods on intra type signaling. The first method changes the intra type signaling order. In this method, after MPM flag, MRL index and ISP flag are signaled. The proposed intra type signaling order change provides the coding performance 0.01% and 0.00% BD-rate in AI and RA configuration, respectively. The second method signals a unified intra type by aligning the MPM flag, reference index for MRL, and split flag for ISP. The proposed unified intra type signaling method provides the coding performance 0.00% and 0.02% BD-rate in AI and RA configuration, respectively without increasing the complexity of encoding and decoding.

It is claimed that the current luma signalling is broken. The non-MPM regular mode signalling is used a lot in particular for lower Qp range while for CTC Qp range the MPM regular modes are used a lot. Therefore, it is claimed by the proponent that it makes sense to move up the mpm_flag as well as the remaining modes. The proponent asserts that the proposed method can be aligned with the adoption that allows all 67 intra modes to be used for ISP, but it requires a redesign of the proposed method.

Regarding method 2, the syntax is simplified because conditions are removed due to using one combined syntax element that is used to signal the various intra mode types, however, the conditions are moved to the parsing of this syntax element. It is asked how the method would support the signalling of non-MPM ISP modes. It was also commented that the semantics of the combined syntax element may be more confusing than having separate syntax elements. It was commented that there is some minor loss observed for RA. It was further commented that the MIP mode is not included in the combined syntax element.

BoG recommends further study outside of CE, with clear analysis of the benefits.

JVET-O0363 CE3-related: Context reduction for intra MPM related syntax element [L. Xu, X. Cao, F. Chen, L. Wang (Hikvision)]

This contribution proposed to simplify the context modelling and reduces the number of contexts for CABAC for syntax intra_luma_not_planar_flag to simplify the coding process. With the context reduction, the BD-rate increase is reported as 0.00% and 0.00% for AI and RA, respectively.

The BoG recommends no action at this time and bring context reduction proposals at a later stage of the VVC standardization when algorithms have stabilized.

JVET-O0816 Crosscheck of JVET-O0363 (CE3-related: Context reduction for intra MPM related syntax element) [T. Ikai (Sharp)]

JVET-O0369 CE3-related: Context reduction for ISP related syntax element [L. Xu, X. Cao, F. Chen, L. Wang (Hikvision)]

This contribution proposed to simplify the context modelling and reduces the number of contexts for CABAC by using bypass coding to replace the CABAC coding for the coding for syntax intra_subpartitions_split_flag to simplify the coding process. With the context reduction, the BD-rate increase is reported as 0.02% and 0.01% for AI and RA, respectively.

The BoG recommends no action at this time and bring context reduction proposals at a later stage of the VVC standardization when algorithms have stabilized.

JVET-O0817 Crosscheck of JVET-O0369 (CE3-related: Context reduction for ISP related syntax element) [T. Ikai (Sharp)]

JVET-O0377 CE3-related: Context reduction for CCLM syntax element [L. Xu, X. Cao, F. Chen (Hikvision)]

This contribution proposed to simplify the context modelling and reduces the number of contexts for CABAC by using bypass coding to replace the CABAC coding for the coding for syntax intra_chroma_pred_mode to simplify the coding process. With the context reduction, the BD-rate increase is reported as 0.01% and 0.02% for AI and RA, respectively.

The BoG recommends no action at this time and bring context reduction proposals at a later stage of the VVC standardization when algorithms have stabilized.

JVET-O0818 Crosscheck of JVET-O0377 (CE3-related: Context reduction for CCLM syntax element) [T. Ikai (Sharp)]

JVET-O0416 Non-CE3: Removal of CCLM context derivation dependency [J. Q. Zhang (PKU), X. Zheng (DJI), S. S. Wang, S. W. Ma (PKU)]

At current VVC working draft, the CCLM mode (INTRA_LT_CCLM) and the CCLM modes using top or left templates (INTRA_L_CCLM, INTRA_T_CCLM) were adopted. When using CCLM prediction, the parsing process of chroma prediction mode exist dependency which causes latency for hardware design. In order to remove such dependency of parsing process of chroma CCLM prediction, this contribution removes the parsing dependency by using the following method:

- Use bypass coding mode when parsing the fourth bin of intra_chroma_pred_mode.

- Use bypass coding mode when parsing the third and fourth bin of intra_chroma_pred_mode.

Simulation results show negligible coding performance impact compared to VTM-5.0 anchor with minor encoding and decoding time change.

It was commented that this proposal is not removing contexts but removes parsing dependency. There seems to be some loss in LB test condition for method 1 and somewhat less loss when more bins are bypass coded, however, overall the reported loss is minor.

It was commented that the first method is identical to JVET-O0512. It seems to be reasonable to have this fix.

The BoG recommends adoption of method 1 of the current proposal as well as JVET-O0512.

JVET-O0841 Crosscheck of JVET-O0416: Non-CE3: Removal of CCLM context derivation dependency [J. Chen (Alibaba)]

JVET-O0467 Non-CE3: simplified MRL and ISP mode coding [F. Le Léannec, Y. Chen, T. Poirier (InterDigital)]

This contribution proposes to simplify the coding of the multiple reference line (MRL) index and the Intra Sub-Partition (ISP) mode.

In VTM-5.0, the MRL index is coded by the syntax element intra_luma_ref_idx, made of two context-based coded bins. This contribution’s method 1 proposes to code the second bin in bypass mode. It reportedly brings +0.01% Luma BD-rate loss in RA and +0.01% Luma BD-rate loss in AI over VTM-5.0.

Moreover, the ISP mode is signaled through two context-based coded flags, intra_subpartitions_mode_flag and intra_subpartitions_split_flag. The proposed method 2 codes the second flag intra_subpartitions_split_flag in bypass mode. When the method 2 is used on top of the method 1, it leads to 0.01% Luma BD-rate loss in RA and 0.02% loss in AI over VTM-5.0.

The BoG recommends no action at this time and bring context reduction proposals at a later stage of the VVC standardization when algorithms have stabilized.

JVET-O0856 Crosscheck of JVET-O0467 (Non-CE3: simplified MRL and ISP mode coding) [H. Liu (Bytedance)]

JVET-O0477 Non-CE3: Simplified entropy coding for intra prediction mode [D. Park, Y.-U. Yoon, J. Do, J.-G. Kim (KAU), J. Lee, J. Kang (ETRI)]

This proposal is identical to JVET-O0363 test 1. The BoG recommends no action at this time and bring context reduction proposals at a later stage of the VVC standardization when algorithms have stabilized.

JVET-O1009 Crosscheck of JVET-O0477 (Non-CE3: Simplified entropy coding for intra prediction mode) [J. Choi (LGE)]

JVET-O0484 CE3-related: Improved intra mode coding based on unified MPM list generation [J. Pfaff, T. Hinz, P. Merkle, P. Helle, B. Stallenberger, M. Schäfer, H. Schwarz, D. Marpe, T. Wiegand (HHI), B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

In this contribution, an alignment of the newly adopted Unified-MPM (UMPM) generation of VTM 5.0 is proposed. The proposal has two aspects. First, it is proposed to keep all the design features of the adopted UMPM and to only align the generation of the list of the five non-planar MPMs. Second, the planar flag in VTM 5.0 uses two contexts based on whether a current block is applied with Intra Sub-Partition (ISP) coding tool. In this proposal, four contexts are used by also considering the prediction mode of left and above blocks. It is reported that a combined method of these two aspects yields 0.12% coding gain in AI over VTM-5.0, with 99% encoding and 100% decoding time and that it yields … coding gain in RA over VTM-5.0, with 100% encoding and 100% decoding time. Both modifications are jointly proposed by the proponents of UMPM (JVET N0185) and MIP (JVET N0217).

It is asserted that there is a change in mode usage statistics due to the adoption of MIP and that the non-MIP MPM construction needs to be modified to reflect this change.

The non-MIP MPM construction method proposed is also included in the proposal JVET-O0925 on 8-bit MIP. The BoG recommended adoption of the JVET-O0925 proposal and, therefore, the proposed unified MPM list modification is also recommended for adoption by the BoG.

The aspect of context coding improvement (4 contexts for planar flag), which is also proposed in JVET-O0111 can be revisited after reviewing proposals with related methods. Further study for this aspect.

JVET-O0938 Crosscheck of JVET-O0484 (CE3-related: Improved intra mode coding based on unified MPM list generation) [M. Salehifar (LGE)]

JVET-O0502 Non-CE3: Proposed ISP cleanup [S. De-Luxán-Hernández, V. George, G. Venugopal, J. Brandenburg, B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

This contribution removes the MPM only and PDPC restrictions and applies always the cubic filter on CUs using ISP. Results show a gain of 0.10% and 0.06% for AI and RA respectively. Encoding and decoding run-times are unaffected. The removal of the mentioned restrictions simplifies the design and harmonizes it with other intra coding tools.

This contribution proposes the following changes to unify the intra mode coding and prediction stages between ISP and non-ISP blocks:

- Remove the MPM restriction so that ISP can code all modes. The signalling of the new available intra modes is exactly the same as with non-ISP blocks. This change requires modifying the encoder search, given that now ISP can code 67 intra modes instead of only 6. The removal of the MPM restriction was proposed during the last meeting in JVET-N0437: Non-CE3: On allowing non-MPM modes for ISP and non-zero reference line. A further study of the topic was recommended.

- Apply PDPC on the sub-partitions if their corresponding width and height is at least 4 samples long (and if all the other regular conditions of PDPC apply).

- Use always the cubic filter in the prediction stage of the sub-partitions (like MRL and BDPCM).

It was commented that it would be good to know what the contributions to the reported gain are for each aspect. For PDC and filtering there is a minor gain of approximately 0.02% (AI) on top of VTM5, however, the majority of the gain is coming from allowing all 67 intra modes together with PDPC.

BoG recommends adoption of the proposed methods, given that there is some coding gain benefit for ISP while the encoder runtime is not significantly increased. The proposal includes an encoder search optimization for ISP.

JVET-O0944 Crosscheck of JVET-O0502 (Non-CE3: Proposed ISP cleanup) [T.-C. Ma (Kwai Inc.)]

JVET-O0509 Non-CE3: MPM flag signaling first [D. Y. Lee, W. Gwun, W. Lee, T. H. Kim, G. H. Park (KHU), J. Lee, J. Kang (ETRI)]

Method 1 is identical to JVET-O0192, … (see comments)

Method 2 is variation of method 1.

BoG recommends further study outside of CE, with clear analysis of the benefits.

JVET-O0512 Non-CE3: CCLM Coding bug fix [S. Keating, K. Sharman (Sony)]

See comments JVET-O0416.

JVET-O0716 Crosscheck of JVET-O0512 (Non-CE3: CCLM Coding bug fix) [L. Xu, F. Chen, L. Wang (Hikvision)]

JVET-O0515 Non CE3: Intra mode coding of non-MPM modes [S. Blasi, G. Kulupana, A. Seixas Dias (BBC)]

In the VVC WD, the process of decoding a mode that is not in the MPM list depends on the computation of the MPM list, and sorting of the MPM modes. This contribution proposes to remove this dependency, allowing the process of decoding modes that are not in the MPM list to be performed with no need to evaluate the MPM list. An average BD-rate of XXX is reported for AI configuration with an encoder complexity of XX%. The reported decoder complexity is XX% on average.

It was commented that there is an MPM list adjustment from the recommended adoption of JVET-O0925 with one change that the list would not have DC in some cases and whether this would affect the method. It is suggested to provide results with the adjusted MPM list. It is commented that there is redundancy between the MPM list and remaining modes in the method. It is claimed that complexity is significantly reduced, but a detailed analysis is not included in the proposal.

BoG recommends further study together with the adjusted MPM list construction (JVET-O0925), including an analysis of the complexity reduction benefits.

JVET-O0606 Non-CE3: Context Reduction for MPM Signaling [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

The BoG recommends no action at this time and bring context reduction proposals at a later stage of the VVC standardization when algorithms have stabilized.

JVET-O1020 Crosscheck of JVET-O0606 (Non-CE3: Context Reduction for MPM Signaling) [?? (??)]

JVET-O0647 Non-CE3: On signalling of intra coded blocks [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, H. Huang, M. Karczewicz (Qualcomm)]

[Notes by L. Li]

Test 1 is identical to method included in JVET-O0192, JVET-O0197, JVET-O0509, JVET-O0351

Test 2 code MPM flag before MIP

Test 3 code regular intra flag first

Proponents suggests to adopt test 3

It was commented test 2 is not compatible with recent adoption of ISP enabling more intra modes. Test 3 was suggest from non-proponent having cleaner syntax. It was noted semantics for newly proposed syntax is missing. The peak loss is 0.13% in RA, and this commented from non-proponent that it is due to MIP is coded after regular intra flag in test 3. It was also commented hardware benefit is not clear for test 3.

CE is claimed that not required because the goal is not clear. Proponent states that the goal is to have a “logical design”, and since encoder is jointly designed, encoder optimization could also be done to work with logical design.

It was also commented that combined syntax is not required in the case when encoder choose not to code some intra tools, then it is better to have separate flags for each intra tool. It is safer for CABAC updates separate flags.

BoG recommends further study.

JVET-O0651 Non-CE8: Intra prediction mode derivation for DM chroma block with corresponding IBC/PCM luma [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)]

See notes JVET-O0275

JVET-O0694 Crosscheck of JVET-O0651 (Non-CE8: Intra prediction mode derivation for DM chroma block with corresponding IBC/PCM luma) [L. Zhao (Tencent)]

JVET-O0655 Non-CE3: On chroma DM derivation table for 4:2:2 chroma format [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

This contribution reports a defect on chroma intra angle mapping table for 4:2:2 chroma format implemented in the current VTM and proposes a modification of the table values. In HEVC, due to the limitation of the prediction angle, luma intra prediction modes ranging from 2 to 5 are mapped to 2 for chroma intra mode derivation in 4:2:2 chroma format. VTM adopts the similar mapping table ported from HM and extended from 35 to 67 entries, by which luma intra prediction modes ranging from 2 to 8 are also mapped to 2 even though wide angle intra prediction (WAIP) was adopted in VVC. The proposal replaces some values of the entries of the mapping table to convert prediction angle more precisely for chroma blocks. The experimental results show slight but consistent coding gain for all intra configuration without encoding and decoding time change.

It was commented that the number of wide-angle modes depends on the aspect ratio of the block. To support this there would need to be a table for each aspect ratio. This is not the case in the proposal, but would be unnecessary.

It is asserted that in program exchange, high-bit rate transmission, the 4:2:2 format is used.

BoG recommends adoption, because it fixes a problem with wide-angle mode mapping for 4:2:2 in the current VVC working draft.

JVET-O0687 Crosscheck of JVET-O0655 (Non-CE3: On chroma DM derivation table for 4:2:2 chroma format) [T. Chujoh (Sharp)]

JVET-O0671 Non-CE3: Intra chroma mode coding simplification [B. Ray, G. Van der Auwera, M. Karczewicz (Qualcomm)]

[Notes by L. Li]

It is proposed to use same chroma mode for DM so that extra table is removed from the spec. Method 2 proposes a unified binarization table for chroma mode. It is asserted that bin is reduced in average. Three contexts are used which is the same with existing. It shows slight improvement in chroma in all classes.

It was commented the proposed change is a reasonable design.

BoG recommend adoption

JVET-O0748 Non-CE3: On signalling for intra coding mode in the MPM list [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)] [late]

[Notes by L. Li]

Test 1 reduces 1 context of planar_flag

Test 2 proposes to code planar_flag with 3 context

Test 3 proposes to code first bin of mpm index with 3 contexts on top of test 2.

It is commented that adding contexts to planar_flag is desirable since it shows slight improvement. Adding context coded bin for mpm index is not recommended from non-proponent.

It was also commented that this is a late contribution (5 days) and the first version of document does not include spec text. From the third version the spec text was included.

BoG recommend further study.

## Intra prediction

JVET-O0229 CE3-related: Simplifications of PDPC for diagonal and adjacent diagonal directions [G. Rath, F. Racapé, F. Urban (InterDigital)]

JVET-O0533 Crosscheck of JVET-O0229 (CE3-related: Simplifications of PDPC for diagonal and adjacent diagonal directions)) [L. Zhao (Tencent)] [miss] [late]

JVET-O0230 CE3-related: Interpolation filtering for intra prediction in non-diagonal directions [G. Rath, F. Racapé, F. Urban, F. Le Léannec (InterDigital)]

JVET-O1030 Crosscheck of JVET-O0230 (Non-CE3: Interpolation filtering for intra prediction in non-diagonal directions) [J. Lee, S.-C. Lim, H. Lee, J. Kang (ETRI)]

JVET-O0260 Non-CE3: Rounding offset removal for intra prediction [Z. Deng, H. Liu, L. Zhang, K. Zhang, Y. Wang (Bytedance)]

JVET-O0862 Cross-check of JVET-O0260: "Non-CE3: Rounding offset removal for intra prediction" [F. Le Léannec (Interdigital)]

JVET-O0277 CE3-related: Simplification and unification for intra reference sample filtering [C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-O0825 Crosscheck of JVET-O0277 (CE3-related: Simplification and unification for intra reference sample filtering) [J. Shingala (Ittiam)]

JVET-O0311 CE3-related: Intra sub-partition improvement for low decoding latency and hardware implementation efficiency [H. Yang, Y. He (InterDigital)]

JVET-O0974 Crosscheck of JVET-O0311 (CE3-related: Intra sub-partition improvement for low decoding latency and hardware implementation efficiency) [L. Zhao (Tencent)]

JVET-O0316 Non-CE3：Unification of ISP and Non-ISP [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-O0317 CE3-related: MPM List Modification for Zero Reference Line [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-O0318 Non-CE3: Intra Combined predict [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-O0320 Non-CE3: CCLM simplification [J.-Y. Huo, Y.-Z. Ma, X.-W. Li, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu, M. Li (OPPO)]

JVET-O0978 Crosscheck of JVET-O0320 (Non-CE3: CCLM simplification) [X. Ma (Huawei)]

JVET-O0322 Non-CE3: Simplification of PDPC for chroma component [J.-Y. Huo (Xidian Univ.), S. Wan (NPU), Y.-Z. Ma (Xidian Univ.), J.-K. Guo (NPU), F.-Z. Yang (Xidian Univ.), Y.-F. Yu, Y. Liu, M. Li (OPPO)]

JVET-O0980 Crosscheck of JVET-O0322 (Non-CE3: Simplification of PDPC for chroma component) [X. Ma (Huawei)]

JVET-O0341 Non-CE3: Unification of intra interpolation filter selection [J. Lee, J. Kang, H. Lee, S.-C. Lim, H. Y. Kim (ETRI)]

JVET-O0765 Crosscheck of JVET-O0341: Non-CE3: Unification of intra interpolation filter selection [F. Racapé, G. Rath (Interdigital)]

JVET-O0343 CE3-related: Simplification on CCLM process [D.-Y. Kim, J.-O. Lee (Chips&Media), S.-C. Lim, J. Lee, J. Kang (ETRI)]

JVET-O0845 Crosscheck of JVET-O0343 (CE3-related: Simplification on CCLM process) [M. Ikeda (Sony)]

JVET-O0353 Non-CE3: Simplification on transform selection for multiple reference line (MRL) [J. Heo, M. Koo, J. Choi, J. Lim, S. Kim (LGE)]

Moved to CE6.

JVET-O0707 Crosscheck of JVET-O0353 (Non-CE3: Simplification on transform selection for multiple reference line (MRL)) [L. Zhao (Tencent)]

JVET-O0355 Non-CE3: MRL restriction [J. Heo, J. Choi, H. Jang, J. Lim, S. Kim (LGE)]

JVET-O0942 Crosscheck of JVET-O0355 (Non-CE3: MRL restriction) [T.-C. Ma (Kwai Inc.)]

JVET-O0356 Non-CE3: Restriction on WAIP for MRL and ISP intra prediction [J. Heo, J. Choi, L. Li, J. Lim, S. Kim (LGE)]

JVET-O0836 Crosscheck of JVET-O0356 (Non-CE3: Restriction on WAIP for MRL and ISP intra prediction) [A. Tamse (Samsung)]

JVET-O0364 On general intra sample prediction [F. Bossen (Sharp)]

JVET-O0365 CE3-related: Simplification of Multi-reference line prediction scheme [L. Xu, X. Cao, F. Chen, L. Wang (Hikvision)]

JVET-O0749 Crosscheck of JVET-O0365 (CE3-related: Simplification of Multi-reference line prediction scheme) [?? (??)]

JVET-O0371 CE3-related: On 1xN and 2xN sub-partition in ISP [Y. Sun, L. Xu, X. Cao (Hikvision)]

JVET-O0771 Crosscheck of JVET-O0371 (CE3-related: On 1xN and 2xN sub-partition in ISP) [Y.-W. Chen (Kwai Inc.)]

JVET-O0399 Non-CE3: CCLM block size restriction [J. Choi, J. Heo, J. Lim, S. Kim (LGE)]

JVET-O1014 Crosscheck of JVET-O0399 (Non-CE3: CCLM block size restriction) [F. Chen (Hikvision)]

JVET-O0404 Non-CE3: Linear Model-based Intra Prediction [R. Ghaznavi-Youvalari (Nokia)]

JVET-O0412 CE3-related: Sampling point extension for CCLM [M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)] [miss] [late]

JVET-O0833 Crosscheck of JVET-O0412 (CE3-related: Sampling point extension for CCLM) [S.-C. Lim, J. Lee, H. Lee, J. Kang (ETRI)]

JVET-O0426 Non-CE3: DC intra prediction mode alignment [A. K. Ramasubramonian, G. Van der Auwera, L. Pham Van, M. Karczewicz (Qualcomm)]

JVET-O0762 Crosscheck of JVET-O0426 (Non-CE3: DC intra prediction mode alignment) [H.-J. Jhu (Kwai Inc.)]

JVET-O0449 Non-CE3: Decoder-side Intra Mode Derivation (DIMD) with prediction fusion using Planar [M. Abdoli, T. Guionnet, E. Mora, M. Raulet (ATEME), S. Blasi, A. Seixas Dias, G. Kulupana (BBC)]

JVET-O0720 Crosscheck of JVET-O0449 Decoder-side Intra Mode Derivation with Prediction Fusion Using Planar [V. Drugeon (Panasonic)]

JVET-O0507 Non-CE3: Intra-sub-partition coding mode only performing on non-square blocks [W. Gwun, W. Lee, D. Y. Lee, T. H. Kim, G. H. Park (KHU), G. Bang, W. Lim, H.Y. Kim (ETRI)]

JVET-O0988 Cross-check of Non-CE3: Intra-sub-partition coding mode only performing on non-square blocks (JVET-O0507) [M. W. Park (Samsung)]

JVET-O0510 CE3-related: Extensions to CCLM reference sample filtering in CE3-3.2.1 [S. Keating, K. Sharman (Sony)]

JVET-O0535 Non-CE3: Unification of PDPC for diagonal intra prediction modes [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-O0766 Crosscheck of JVET-O0535: Non-CE3: Unification of PDPC for diagonal intra prediction modes [F. Racapé, G. Rath (Interdigital)]

JVET-O0536 CE3-related: Redundant bit saving for CE3-2.2 [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-O0779 Crosscheck of JVET-O0536: CE3-related: Redundant bit saving for CE3-2.2 [L. Pham Van, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-O0632 Non-CE3: WAIP restriction for square sub-CU blocks partitioned by ISP [A. Kumar, S. Shrestha, B. Lee (Chosun Univ), Y. Lee, J. Park (Humax)]

JVET-O0640 CE3-related: On 2x2/2x4/4x2 chroma blocks at the corner of pictures [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-O0649 Non-CE3: Simplification of reference sample smoothing in intra prediction [A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz (Qualcomm)]

JVET-O0672 Non-CE3: MRL intra prediction for chroma DM [C.-C. Kuo, C.-C. Lin, C.-L. Lin (ITRI)]

JVET-O0673 Harmonized MIP/CCLM boundary downsampling [T. Biatek, G. Van Der Auwera, M. Karczewicz (Qualcomm)]

JVET-O0674 Non-CE3: Multiple reference set for CCLM [P.-H. Lin, J.-Y. Deng (Foxconn)] [late]

JVET-O0760 Crosscheck of JVET-O0674 (Non-CE3: Multiple reference set for CCLM) [H.-J. Jhu (Kwai Inc.)]